3GPP TSG-SA WG3-LI Meeting #45 	SA3LI12_043r2

# Bratislava, Slovakia, 24-26. April 2012

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR |  | rev | - | Current version: | 11.2.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handover indication at HI2 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia Siemens Networks) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI11 |  |  |  |  |  |  | Date: |  | 2012-04-25 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | The Inter RAT handover indication in the GTPv2 protocol is implemented as a flag. Currently this implemented as NULL type in the ASN.1 definition of the EPS HI2. There is ASN.1 encoding method for which this NULL type encoding is not suitable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new ASN.1 parameter is introduced to represent the GTPv2 handover indication as an OCTET STRING. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The proper indication of handover from GTPv2 protocol is not possible in certain configurations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5, B.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

## 10.5	IRI for evolved packet domain

The IRI will in principle be available in the following phases of a data transmission:

1.	At connection attempt when the target identity becomes active, at which time packet transmission may or may not occur (set up of a bearer/tunnel, target may be the originating or terminating party);

2.	At the end of a connection, when the target identity becomes inactive (removal of a bearer/tunnel);

3.	At certain times when relevant information are available.

In addition, information on non-transmission related actions of a target constitute IRI and is sent via HI2.

The IRI may be subdivided into the following categories:

1.	Control information for HI2 (e.g. correlation information);

2.	Basic data context information, for standard data transmission between two parties.

# The events defined in TS 33.107 [19] are used to generate records for the delivery via HI2.

There are several different event types received at DF2 level. According to each event, a Record is sent to the LEMF if this is required. The following table gives the mapping between event type received at DF2 level and record type sent to the LEMF. The applicability of the events to specific access (E-UTRAN, trusted non-3GPP access, untrusted non-3GPP access) and network protocols (GTP/PMIP S5/S8 interface) is specified in [19]. Additional events and mapping with IRI Record type are applicable to EPS in case of interworking between SGSN and PDN-GW over Gn/Gp interface, as specified in this document for PS interception.

# Table 10.5.1: Mapping between EPS Events and HI2 records type

| Event | IRI Record Type |
| --- | --- |
| E-UTRAN attach | REPORT |
| E-UTRAN detach | REPORT |
| Bearer activation (successful) | BEGIN |
| Bearer modification | CONTINUE |
| UE Requested bearer resource modification | REPORT |
| Bearer activation (unsuccessful) | REPORT |
| Start of interception with active bearer | BEGIN or optionally CONTINUE |
| Bearer deactivation | END |
| UE requested PDN connectivity | REPORT |
| UE requested PDN disconnection | REPORT |
| Tracking Area update | REPORT |
| Serving Evolved Packet System | REPORT |
| PMIP attach/tunnel activation (successful) | BEGIN |
| PMIP attach/tunnel activation (unsuccessful) | REPORT |
| PMIP session modification | CONTINUE |
| PMIP detach/tunnel deactivation | END |
| Start of interception with active PMIP tunnel | BEGIN (or optionally CONTINUE) |
| PMIP PDN-GW initiated PDN disconnection | END |
| MIP registration/tunnel activation (successful) | BEGIN |
| MIP registration/tunnel activation (unsuccessful) | REPORT |
| MIP deregistration/tunnel deactivation | END |
| Start of interception with active MIP tunnel | BEGIN |
| DSMIP registration/tunnel activation (successful) | BEGIN |
| DSMIP registration/tunnel activation (unsuccessful) | REPORT |
| DSMIP session modification | CONTINUE |
| DSMIP deregistration/tunnel deactivation | END |
| Start of interception with active DSMIP tunnel | BEGIN |
| DSMIP HA Switch | REPORT |
| PMIP Resource Allocation Deactivation | END |
| MIP Resource Allocation Deactivation | END |
| Start of interception with E-UTRAN attached UE | REPORT |

A set of information is used to generate the records. The records used transmit the information from mediation function to LEMF. This set of information can be extended in the network nodes or DF2 MF, if this is necessary in a specific country. The following table gives the mapping between information received per event and information sent in records.

# Table 10.5.2: Mapping between Events information and IRI information

| parameter | description | HI2 ASN.1 parameter |
| --- | --- | --- |
| observed MSISDN | Target Identifier with the MSISDN of the target subscriber (monitored subscriber). | partyInformation (party-identiity) |
| observed IMSI | Target Identifier with the IMSI of the target subscriber (monitored subscriber). | partyInformation (party-identity) |
| observed ME Id | Target Identifier with the ME Id of the target subscriber (monitored subscriber) | partyInformation (party-identity) |
| observed MN NAI | Target Identifier with the NAI of the target subscriber | partyInformation (party-identity) |
| event type | Description which type of event is delivered | ePSevent |
| event date | Date of the event generation in the node | Timestamp |
| event time | Time of the event generation in the node |  |
| access point name | When provided by the MME, the parameter carries the Access Point Name provided by the UE. When provided by the S-GW/PDN-GW, it is the APN used for the PDN connection | aPN |
| APN-AMBR | Contains the Aggregate Maximum Bit Rate for the APN | aPN-AMBR |
| PDN type | Indicated the used IP version (IPv4, IPv6, IPv4/IPv6) | pDNAddressAllocation |
| PDN address allocation | Provides the IP version (IPv4, IPv6, IPv4/IPv6) and the IP address(es) allocated for the UE. | pDNAddressAllocation |
| Protocol Configuration Options | Are used to transfer parameters between the UE and the PDN-GW (e.g. address allocation preference by DHCP) | protConfigOptions |
| Attach type | Indicates the type of attach and may carry indication of handover in case of mobility with non-3GPP access. | attachType |
| RAT type | Radio Access Type | rATType |
| initiator | This field indicates whether the procedure is UE or network initiated. | initiator |
| Handover indication | Provides information that the procedure is triggered as part of a handover | handoverIndication extendedHandoverIndication |
| Procedure Transaction Identifier | Identifies a set of messages belonging to the same procedure; the parameter is dynamically allocated by the UE | procedureTransactionId |
| EPS bearer identity | Identifies an EPS bearer for one UE accessing via E-UTRAN. It is allocated by the MME. | ePSBearerIdentity |
| Bearer activation/ deactivation type | Indicates the type of bearer being activated/deactivated, i.e. default or dedicated. | bearerActivationType, bearerDeactivationType |
| Linked EPS bearer identity | Indicates, in case of dedicated bearer, the EPS bearer identity of the default bearer. | linkedEPSBearerId |
| Switch off indicator | Indicates whether a detach procedure is due to a switch off situation or not. | detachType |
| Detach type | Parameter sent by the network to the UE to indicate the type of detach. | detachType |
| Traffic Flow Template (TFT) | Collection of all packet filters associated with the EPS bearer. | tFT |
| Traffic Aggregate Description (TAD) | Consists of the description of the packet filter(s) for the traffic flow aggregate. | trafficAggregateDescription |
| correlation number | Unique number for each target connection delivered to the LEMF, to help the LEA, to have a correlation between each target connection and the IRI. | ePSCorrelationNumber |
| lawful interception identifier | Unique number for each lawful authorization. | lawfulInterceptionIdentifier |
| location information | When authorized, this field provides the location information of the target that is present at the node at the time of event record production. | ePSlocationOfTheTarget |
| Old location information | Location information of the target subscriber before Tracking Area Update. | ePSlocationOfTheTarget |
| Failure reason | The reason for the failure or rejection of the Tracking Area Update | failedTAUReason |
| failed bearer activation reason | This field gives information about the reason for a failed bearer activation of the target subscriber. | failedBearerActivationReason |
| failed attach reason | This field gives information about the reason for a failed attach attempt of the target subscriber. | failedEUTRANAttachreason, status, code (depending on the protocol) |
| Session modification failure reason | This field gives information about the reason for a failed session modification attempt of the target subscriber | status |
| EPS bearer QOS | This field indicates the Quality of Service associated with the EPS bearer procedure. | ePSBearerqOS |
| bearer deactivation reason | This field gives information about the reason for bearer deactivation of the target subscriber. | bearerDeactivationCause |
| network identifier | Operator ID plus node address. | networkIdentifier |
| Failed Bearer Modification reason | The reason for failure of Bearer Modification | failedBearerModReason |
| Lifetime | Lifetime of the tunnel; it is set to a nonzero value in case of registration or lifetime extension; is set to zero in case of deregistration. | lifetime |
| Access technology type | Indicates the Radio Access Type | accessTechnologyType |
| UE address info | Includes one or more IP addresses allocated to the UE. | iPv6HomeNetworkPrefix, iPv4HomeAddress, iPv6careOfAddress, iPv4careOf Address |
| Additional parameters | Additional information provided by the UE, such as protocol configuration options | protConfigurationOption |
| serving MME address | Diameter Origin-Host and Origin-Realm of the serving MME. | servingMME-Address |
| Revocation trigger | Contains the reason which triggered a PDN-GW initiated PDN-disconnection (revocation) procedure. | revocationTrigger |
| Home Address | Contains the UE Home IP address | homeAddress |
| Home Agent Address | Contains the IP address of the Home Agent | homeAgentAddress |
| Requested IPv6 Home Prefix | The IPv6 Home Prefix requested by the UE. | requestedIPv6HomePrefix |
| Care of Address | The local IP address assigned to the UE by the Access Network. | careOfAddress |
| HSS/AAA address | The address of the HSS/AAA triggering a pDN-GW reallocation. | hSS-AAA-address |
| Target PDN-GW address | The address of the PDN-GW which the UE will be reallocated to. | targetPDN-GW-Address |
| Foreign domain address | The relevant IP address in the foreign domain. | foreignDomainAddress |
| Visited network identifier | An identifier that allows the home network to identify the visited network [53] | visitedNetworkId |
| DHCP v4 Address Allocation Indication | Indicates that DHCPv4 is to be used to allocate the IPv4 address to the UE | dHCPv4AddressAllocationInd |
| Serving Network | Identifies, for E-UTRAN access, the serving network the UE is attached to | servingNetwork |
| Request type | Provides the type of UE requested PDN connectivity | requestType |
| Failed reason | Provides the failure cause for UE requested PDN connectivity | uEReqPDNConnFailReason |

# NOTE:	LIID parameter must be present in each record sent to the LEMF.

* * * Next Change * * *

# B.9	Intercept related information (HI2 SAE/EPS and IMS)

Declaration of ROSE operation umts-sending-of-IRI is ROSE delivery mechanism specific. When using FTP delivery mechanism, data UmtsIRIsContent must be considered.

# ASN1 description of IRI (HI2 interface)

EpsHI2Operations {itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulintercept(2) threeGPP(4) hi2eps(8) r11(11) version-2(2)}

# DEFINITIONS IMPLICIT TAGS ::=

# BEGIN

# IMPORTS

OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t(2) remote-operations(4) informationObjects(5) version1(0)}

LawfulInterceptionIdentifier,

TimeStamp,

Network-Identifier,

National-Parameters,

National-HI2-ASN1parameters,

DataNodeAddress,

IPAddress,

IP-value,

X25Address

# FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulIntercept(2) hi2(1) version10(10)}; -- Imported from TS 101 671v3.3.1

-- Object Identifier Definitions

-- Security DomainId

lawfulInterceptDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0)

securityDomain(2) lawfulIntercept(2)}

-- Security Subdomains

threeGPPSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP(4)}

hi2epsDomainId OBJECT IDENTIFIER	::= {threeGPPSUBDomainId hi2eps(8) r11(11) version-2(2)}

eps-sending-of-IRI  OPERATION ::=

{

ARGUMENT	EpsIRIsContent

ERRORS		{ OperationErrors }

CODE		global:{threeGPPSUBDomainId hi2eps(8) opcode(1)}

}

-- Class 2 operation . The timer shall be set to a value between 3 s and 240 s.

-- The timer.default value is 60s.

-- NOTE:	The same note as for HI management operation applies.

EpsIRIsContent		::= CHOICE

{

epsiRIContent			EpsIRIContent,

epsIRISequence			EpsIRISequence

}

EpsIRISequence		::= SEQUENCE OF EpsIRIContent

-- Aggregation of EpsIRIContent is an optional feature.

-- It may be applied in cases when at a given point in time

-- several IRI records are available for delivery to the same LEA destination.

-- As a general rule, records created at any event shall be sent

-- immediately and not withheld in the DF or MF in order to

-- apply aggragation.

-- When aggregation is not to be applied,

-- UmtsIRIContent needs to be chosen.

EpsIRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,	-- include at least one optional parameter

iRI-End-record			[2] IRI-Parameters,

iRI-Continue-record		[3] IRI-Parameters,	-- include at least one optional parameter

iRI-Report-record		[4] IRI-Parameters	-- include at least one optional parameter

}

unknown-version 		ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter-value	ERROR ::= { CODE local:2}

unknown-parameter		ERROR ::= { CODE local:3}

# OperationErrors ERROR ::=

{

unknown-version |

missing-parameter |

unknown-parameter-value |

unknown-parameter

}

-- This values may be sent by the LEMF, when an operation or a parameter is misunderstood.

-- Parameters having the same tag numbers must be identical in Rel-5 and onwards modules.

IRI-Parameters		::= SEQUENCE

{

hi2epsDomainId				[0]	OBJECT IDENTIFIER,  -- 3GPP HI2 EPS domain

lawfulInterceptionIdentifier	[1] LawfulInterceptionIdentifier,

-- This identifier is associated to the target.

timeStamp				[3] TimeStamp,

-- date and time of the event triggering the report.)

initiator 				[4] ENUMERATED

{

not-Available		(0),

originating-Target	(1),

-- in case of GPRS, this indicates that the PDP context activation, modification

-- or deactivation is MS requested

-- in case of EPS, this indicated that the EPS detach, bearer activation, modification

-- or deactivation is UE requested

terminating-Target	(2),

-- in case of GPRS, this indicates that the PDP context activation, modification or

-- deactivation is network initiated

-- in case of EPS, this indicated that the EPS detach, bearer activation, modification

-- or deactivation is network initiated

...

} OPTIONAL,

locationOfTheTarget		[8] Location OPTIONAL,

-- location of the target subscriber

partyInformation 		[9] SET SIZE (1..10) OF PartyInformation OPTIONAL,

-- This parameter provides the concerned party, the identiy(ies) of the party

--)and all the information provided by the party.

serviceCenterAddress	[13] PartyInformation OPTIONAL,

-- e.g. in case of SMS message this parameter provides the address of  the relevant

-- server within the calling (if server is originating) or called (if server is

-- terminating) party address parameters

sMS						[14] SMS-report OPTIONAL,

-- this parameter provides the SMS content and associated information

national-Parameters		[16] National-Parameters OPTIONAL,

ePSCorrelationNumber	[18] EPSCorrelationNumber OPTIONAL,

ePSevent 				[20] EPSEvent OPTIONAL,

-- This information is used to provide particular action of the target

-- such as attach/detach

sgsnAddress 			[21] DataNodeAddress OPTIONAL,

gPRSOperationErrorCode 	[22] GPRSOperationErrorCode OPTIONAL,

ggsnAddress 			[24] DataNodeAddress OPTIONAL,

qOS						[25] UmtsQos OPTIONAL,

networkIdentifier		[26] Network-Identifier OPTIONAL,

sMSOriginatingAddress 	[27] DataNodeAddress OPTIONAL,

sMSTerminatingAddress 	[28] DataNodeAddress OPTIONAL,

iMSevent				[29] IMSevent OPTIONAL,

sIPMessage				[30] OCTET STRING  OPTIONAL,

servingSGSN-number		[31] OCTET STRING (SIZE (1..20))	OPTIONAL,

servingSGSN-address		[32] OCTET STRING (SIZE (5..17)) 	OPTIONAL,

-- Octets are coded according to 3GPP TS 23.003 [25]

...,

-- Tag			[33] was taken into use by ETSI module in TS 101 671v2.13.1

ldiEvent	 			[34] LDIevent OPTIONAL,

correlation 			[35] CorrelationValues OPTIONAL,

ePS-GTPV2-specificParameters   [36] EPS-GTPV2-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of GTPV2 based intercepted messages

ePS-PMIP-specificParameters    [37] EPS-PMIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of PMIP based intercepted messages

ePS-DSMIP-SpecificParameters   [38] EPS-DSMIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of DSMIP based intercepted messages

ePS-MIP-SpecificParameters     [39] EPS-MIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of MIP based intercepted messages

servingNodeAddress             [40] OCTET STRING               OPTIONAL,

-- this parameter is kept for backward compatibility only and should not be used

-- as it has been superseeded by parameter visitedNetworkId

visitedNetworkId               [41] UTF8String                 OPTIONAL,

-- contains the visited network identifier inside the EPS Serving System Update for

-- non 3GPP access, coded according to [53]

mediaDecryption-info	       [42] MediaDecryption-info OPTIONAL,

servingS4-SGSN-address	       [43] OCTET STRING OPTIONAL,

-- Diameter Origin-Host and Origin-Realm of the S4-SGSN based on the TS 29.272 [59].

-- Only the data fields from the Diameter AVPs are provided concatenated

-- with a semicolon to populate this field.

national-HI2-ASN1parameters	[255]	National-HI2-ASN1parameters	OPTIONAL

}

-- Parameters having the same tag numbers must be identical in Rel-5 and onwards modules

-- PARAMETERS FORMATS

PartyInformation 			::= SEQUENCE

{

party-Qualifier 	[0]  ENUMERATED

{

gPRSorEPS-Target(3),

...

},

partyIdentity 		[1] SEQUENCE

{

imei					[1] OCTET STRING (SIZE (8)) OPTIONAL,

-- See MAP format [4]

imsi					[3] OCTET STRING (SIZE (3..8)) OPTIONAL,

-- See MAP format [4] International Mobile

-- Station Identity E.212 number beginning with Mobile Country Code

msISDN					[6] OCTET STRING (SIZE (1..9)) OPTIONAL,

-- MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document TS 29.002 [4]

e164-Format				[7] OCTET STRING    (SIZE (1 .. 25)) OPTIONAL,

-- E164 address of the node in international format. Coded in the same format as

-- the calling party number  parameter of the ISUP (parameter part:[5])

sip-uri					[8] OCTET STRING 	OPTIONAL,

-- See [26]

...,

tel-url					[9] OCTET STRING 	OPTIONAL,

-- See [36]

nai                    [10] OCTET STRING    OPTIONAL

-- NAI of the target, encoded in the same format as defined by [EPS stage 3 specs]

},

services-Data-Information 	[4] Services-Data-Information OPTIONAL,

-- This parameter is used to transmit all the information concerning the

-- complementary information associated to the basic data call

...

}

Location	::= SEQUENCE

{

e164-Number         [1] OCTET STRING (SIZE (1..25)) OPTIONAL,
        -- Coded in the same format as the ISUP location number (parameter
        -- field) of the ISUP (see EN 300 356 [30]).    
	globalCellID		[2] GlobalCellID	OPTIONAL,

--see MAP format (see [4])

rAI					[4] Rai		OPTIONAL,

-- the Routeing Area Identifier in the current  SGSN is coded in accordance with the

-- § 10.5.5.15 of document [9] without the Routing Area Identification IEI

-- (only the last 6 octets are used)

gsmLocation			[5] GSMLocation OPTIONAL,

umtsLocation		[6] UMTSLocation OPTIONAL,

sAI					[7] Sai	OPTIONAL,

-- format:	PLMN-ID	3 octets (no. 1 – 3)

--			LAC		2 octets (no. 4 – 5)

--			SAC		2 octets (no. 6 – 7)

--			(according to 3GPP TS 25.413 [62])

...,

oldRAI				[8] Rai		OPTIONAL

-- the Routeing Area Identifier in the old SGSN is coded in accordance with the

-- § 10.5.5.15 of document [9] without the Routing Area Identification IEI

-- (only the last 6 octets are used).

}

GlobalCellID	::= OCTET STRING  (SIZE (5..7))

Rai				::= OCTET STRING (SIZE (6))

Sai				::=	OCTET STRING (SIZE (7))

GSMLocation 	::= CHOICE

{

geoCoordinates 	[1] SEQUENCE

{

latitude		[1]	PrintableString (SIZE(7..10)),

-- format : 	XDDMMSS.SS

longitude		[2]	PrintableString (SIZE(8..11)),

-- format : 	XDDDMMSS.SS

mapDatum		[3]	MapDatum DEFAULT wGS84,

...,

azimuth			[4] INTEGER (0..359) OPTIONAL

-- The azimuth is the bearing, relative to true north.

},

-- format : 	XDDDMMSS.SS

-- 				X 			: N(orth), S(outh), E(ast), W(est)

--				DD or DDD 	: degrees (numeric characters)

--				MM			: minutes (numeric characters)

--				SS.SS 		: seconds, the second part (.SS) is optionnal

-- Example :

--			latitude short form		N502312

--			longitude long form		E1122312.18

utmCoordinates	[2] SEQUENCE

{

utm-East 		[1] PrintableString (SIZE(10)),

utm-North 		[2] PrintableString (SIZE(7)),

-- example	utm-East	32U0439955

--			utm-North	5540736

mapDatum		[3]	MapDatum DEFAULT wGS84,

...,

azimuth			[4] INTEGER (0..359) OPTIONAL

-- The azimuth is the bearing, relative to true north.

},

utmRefCoordinates  	[3] SEQUENCE

{

utmref-string		PrintableString (SIZE(13)),

mapDatum			MapDatum DEFAULT wGS84,

...

},

-- example	32UPU91294045

wGS84Coordinates  	[4] OCTET STRING

-- format is as defined in [37].

}

# MapDatum ::= ENUMERATED

{

wGS84,

wGS72,

eD50, 	-- European Datum 50

...

}

# UMTSLocation ::= CHOICE {

point					[1]	GA-Point,

pointWithUnCertainty	[2]	GA-PointWithUnCertainty,

polygon					[3]	GA-Polygon

}

# GeographicalCoordinates ::= SEQUENCE {

latitudeSign			ENUMERATED { north, south },

latitude				INTEGER (0..8388607),

longitude				INTEGER (-8388608..8388607),

...

}

# GA-Point ::= SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

# GA-PointWithUnCertainty ::=SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

uncertaintyCode				INTEGER (0..127)

}

maxNrOfPoints						INTEGER ::= 15

# GA-Polygon ::= SEQUENCE (SIZE (1..maxNrOfPoints)) OF

# SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

SMS-report		::= SEQUENCE

{

sMS-Contents	[3] SEQUENCE

{

sms-initiator		[1] ENUMERATED 	-- party which sent the  SMS

{

target			(0),

server			(1),

undefined-party	(2),

...

},

transfer-status		[2] ENUMERATED

{

succeed-transfer	(0), 		-- the transfer of the SMS message succeeds

not-succeed-transfer(1),

undefined			(2),

...

} OPTIONAL,

other-message		[3] ENUMERATED 	-- in case of terminating call, indicates if

-- the server will send other SMS

{

yes			(0),

no			(1),

undefined	(2),

...

} OPTIONAL,

content				[4] OCTET STRING (SIZE (1 .. 270)) OPTIONAL,

-- Encoded in the format defined for the SMS mobile

...

}

}

# EPSCorrelationNumber ::= OCTET STRING

-- In case of PS interception, the size will be in the range (8..20)

CorrelationValues ::= CHOICE {

iri-to-CC 	[0]		IRI-to-CC-Correlation, -- correlates IRI to Content(s)

iri-to-iri	[1]		IRI-to-IRI-Correlation, -- correlates IRI to IRI

both-IRI-CC	[2]		SEQUENCE { -- correlates IRI to IRI and IRI to Content(s)

iri-CC	[0]	IRI-to-CC-Correlation,

iri-IRI	[1]	IRI-to-IRI-Correlation}

}

# IRI-to-CC-Correlation ::= SEQUENCE { -- correlates IRI to Content

cc 		[0] SET OF 	OCTET STRING,-- correlates IRI to multiple CCs

iri 	[1] 		OCTET STRING OPTIONAL

-- correlates IRI to CC with signaling

}

# IRI-to-IRI-Correlation ::= OCTET STRING -- correlates IRI to IRI

# EPSEvent ::= ENUMERATED

{

pDPContextActivation 					(1),

startOfInterceptionWithPDPContextActive	(2),

pDPContextDeactivation					(4),

gPRSAttach 								(5),

gPRSDetach 								(6),

locationInfoUpdate 						(10),

sMS 									(11),

pDPContextModification					(13),

servingSystem							(14),

... ,

startOfInterceptionWithMSAttached		(15),

e-UTRANAttach							(16),

e-UTRANDetach                           (17),

bearerActivation                        (18),

startOfInterceptionWithActiveBearer     (19),

bearerModification                      (20),

bearerDeactivation                      (21),

uERequestedBearerResourceModification   (22),

uERequestedPDNConnectivity              (23),

uERequestedPDNDisconnection             (24),

trackingAreaUpdate                      (25),

servingEvolvedPacketSystem              (26),

pMIPAttachTunnelActivation              (27),

pMIPAttachTunnelDeactivation            (28),

startOfInterceptWithActivePMIPTunnel    (29),

pMIPPdnGwInitiatedPdnDisconnection      (30),

mIPRegistrationTunnelActivation         (31),

mIPDeregistrationTunnelDeactivation     (32),

startOfInterceptWithActiveMIPTunnel     (33),

dSMIPRegistrationTunnelActivation       (34),

dSMIPDeregistrationTunnelDeactivation   (35),

startOfInterceptWithActiveDsmipTunnel   (36),

dSMipHaSwitch                           (37),

pMIPResourceAllocationDeactivation      (38),

mIPResourceAllocationDeactivation       (39),

pMIPsessionModification                 (40),

startOfInterceptWithEUTRANAttachedUE    (41),

dSMIPSessionModification                (42)

}

-- see [19]

# IMSevent ::= ENUMERATED

{

unfilteredSIPmessage (1),

-- This value indicates to LEMF that the whole SIP message is sent.

...,

sIPheaderOnly (2),

-- If warrant requires only IRI then specific content in a 'sIPMessage'

-- (e.g. 'Message', etc.) has been deleted before sending it to LEMF.

decryptionKeysAvailable (3)

-- This value indicates to LEMF that the IRI carries CC decryption keys for the session

-- under interception.

}

# Services-Data-Information ::= SEQUENCE

{

gPRS-parameters [1] GPRS-parameters OPTIONAL,

...

}

# GPRS-parameters ::= SEQUENCE

{

pDP-address-allocated-to-the-target 	[1] DataNodeAddress OPTIONAL,

aPN 					[2] OCTET STRING (SIZE(1..100)) OPTIONAL,

pDP-type 				[3] OCTET STRING (SIZE(2)) OPTIONAL,

-- when PDP-type is IPv4 or IPv6, the IP address is carried by parameter

-- pDP-address-allocated-to-the-target

-- when PDP-type is IPv4v6, the additional IP address is carried by parameter

-- additionalIPaddress

...,

nSAPI					[4] OCTET STRING (SIZE (1)) OPTIONAL,

additionalIPaddress		[5] DataNodeAddress OPTIONAL

}

# GPRSOperationErrorCode ::= OCTET STRING

-- The parameter shall carry the GMM cause value or the SM cause value, as defined in the

-- standard [9], without the IEI.

# LDIevent ::= ENUMERATED

{

targetEntersIA 			(1),

targetLeavesIA			(2),

...

}

# UmtsQos ::= CHOICE

{

qosMobileRadio [1] OCTET STRING,

-- The qosMobileRadio parameter shall be coded in accordance with the § 10.5.6.5 of

-- document [9] without the Quality of service IEI and Length of

-- quality of service IE (. That is, first

-- two octets carrying 'Quality of service IEI' and 'Length of quality of service

-- IE' shall be excluded).

qosGn [2] OCTET STRING

-- qosGn parameter shall be coded in accordance with § 7.7.34 of document [17]

}

# EPS-GTPV2-SpecificParameters ::= SEQUENCE

{

pDNAddressAllocation                 [1]   OCTET STRING                       OPTIONAL,

aPN                                  [2]   OCTET STRING (SIZE (1..100))       OPTIONAL,

protConfigOptions                    [3]   ProtConfigOptions                  OPTIONAL,

attachType                           [4]   OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

ePSBearerIdentity                    [5]   OCTET STRING                       OPTIONAL,

detachType                           [6]   OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47], includes switch off indicator

rATType                              [7]   OCTET STRING (SIZE (1))            OPTIONAL,

failedBearerActivationReason         [8]   OCTET STRING (SIZE (1))            OPTIONAL,

ePSBearerQoS                         [9]   OCTET STRING                       OPTIONAL,

bearerActivationType                 [10]  TypeOfBearer                       OPTIONAL,

aPN-AMBR                             [11]  OCTET STRING                       OPTIONAL,

procedureTransactionId               [12]  OCTET STRING                       OPTIONAL,

linkedEPSBearerId                    [13]  OCTET STRING                       OPTIONAL,

tFT                                  [14]  OCTET STRING                       OPTIONAL,

handoverIndication                   [15]  NULL                               OPTIONAL,

failedBearerModReason                [16]  OCTET STRING (SIZE (1))            OPTIONAL,

trafficAggregateDescription          [17]  OCTET STRING                       OPTIONAL,

failedTAUReason                      [18]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

failedEUTRANAttachReason             [19]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

servingMMEaddress                    [20]  OCTET STRING                       OPTIONAL,

-- Contains the data fields from the Diameter Origin-Host and Origin-Realm AVPs

-- as received in the HSS from the MME according to the TS 29.272 [59].

-- Only the data fields from the Diameter AVPs are provided concatenated

-- with a semicolon to populate this field.

bearerDeactivationType               [21]  TypeOfBearer                       OPTIONAL,

bearerDeactivationCause              [22]  OCTET STRING (SIZE (1))            OPTIONAL,
    ePSlocationOfTheTarget               [23]  EPSLocation                        OPTIONAL,
      -- the use of ePSLocationOfTheTarget is mutually exclusive with the use of locationOfTheTarget

-- ePSlocationOfTheTarget allows using the coding of the paramater according to SAE stage 3.

...,

pDNType                              [24]   OCTET STRING (SIZE (1))           OPTIONAL,

-- coded according to TS 24.301 [47]

requestType                          [25]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

uEReqPDNConnFailReason               [26]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

extendedHandoverIndication			 [27]  OCTET STRING( SIZE(1)) 			  OPTIONAL

-- This parameter with value 1 indicates handover based on the flags in the TS 29.274 [46].

-- Otherwise set to the value 0.

-- The use of extendHandoverIndication and handoverIndication parameters is

-- mutually exclusive and depends on the actual ASN.1 encoding method.

-- All the parameters are coded as the corresponding IEs without the octets containing type and

-- length. Unless differently stated, they are coded according to 3GPP TS 29.274 [46]; in this

-- case the octet containing the instance shall also be not included.
}

# TypeOfBearer ::= ENUMERATED

{

defaultBearer          (1),
    dedicatedBearer        (2),

...

}

EPSLocation	::= SEQUENCE

{

userLocationInfo    [1] OCTET STRING (SIZE (1..34)) OPTIONAL,
        -- coded according to 3GPP TS 29.274 [46]; the type IE is not included

gsmLocation			[2] GSMLocation OPTIONAL,

umtsLocation		[3] UMTSLocation OPTIONAL,

olduserLocationInfo	[4] OCTET STRING (SIZE (1..34))	OPTIONAL,

-- coded in the same way as userLocationInfo

lastVisitedTAI	    [5] OCTET STRING (SIZE (1..5))	OPTIONAL,

-- the Tracking Area Identity is coded in accordance with the TAI field in 3GPP TS 29.274
        -- [46].

tAIlist			    [6] OCTET STRING (SIZE (7..97))	OPTIONAL,

-- the TAI List is coded acording to 3GPP TS 24.301 [47], without the TAI list IEI

...,

3gpp2Bsid			[7] OCTET STRING (SIZE (1..12)) OPTIONAL

-- contains only the payload from the 3GPP2-BSID AVP described in the 3GPP TS 29.212 [56].

}

ProtConfigOptions ::= SEQUENCE

{

ueToNetwork                           [1]  OCTET STRING (SIZE(1..251))        OPTIONAL,

networkToUe                           [2]  OCTET STRING (SIZE(1..251))        OPTIONAL,

...

}

# EPS-PMIP-SpecificParameters ::= SEQUENCE

{

lifetime                              [1]  INTEGER (0..65535)                 OPTIONAL,

accessTechnologyType                  [2]  OCTET STRING (SIZE (4))            OPTIONAL,

aPN                                   [3]  OCTET STRING (SIZE (1..100))       OPTIONAL,

iPv6HomeNetworkPrefix                 [4]  OCTET STRING (SIZE (20))           OPTIONAL,

protConfigurationOption               [5]  OCTET STRING                       OPTIONAL,

handoverIndication                    [6]  OCTET STRING (SIZE (4))            OPTIONAL,

status                                [7]  INTEGER (0..255)                   OPTIONAL,

revocationTrigger                     [8]  INTEGER (0..255)                   OPTIONAL,

iPv4HomeAddress                       [9]  OCTET STRING (SIZE (4))            OPTIONAL,

iPv6careOfAddress                     [10] OCTET STRING                       OPTIONAL,

iPv4careOfAddress                     [11] OCTET STRING                       OPTIONAL,

...,

servingNetwork                        [12] OCTET STRING (SIZE (3))            OPTIONAL,

dHCPv4AddressAllocationInd            [13] OCTET STRING (SIZE (1))            OPTIONAL,

ePSlocationOfTheTarget                [14] EPSlocation                        OPTIONAL

-- parameters coded according to 3GPP TS 29.275 [48] and RFCs specifically

-- referenced in it.

}

# EPS-DSMIP-SpecificParameters ::= SEQUENCE

{

lifetime                              [1]   INTEGER (0..65535)                OPTIONAL,

requestedIPv6HomePrefix               [2]   OCTET STRING (SIZE (25))          OPTIONAL,

-- coded according to RFC 5026

homeAddress                           [3]   OCTET STRING (SIZE (8))           OPTIONAL,

iPv4careOfAddress                     [4]   OCTET STRING (SIZE (8))           OPTIONAL,

iPv6careOfAddress                     [5]   OCTET STRING (SIZE(16))           OPTIONAL,

aPN                                   [6]   OCTET STRING (SIZE (1..100))      OPTIONAL,

status                                [7]   INTEGER (0..255)                  OPTIONAL,

hSS-AAA-address                       [8]   OCTET STRING                      OPTIONAL,

targetPDN-GW-Address                  [9]   OCTET STRING                      OPTIONAL,

...

-- parameters coded according to 3GPP TS 24.303 [49] and RFCs specifically

-- referenced in it.

}

# EPS-MIP-SpecificParameters ::= SEQUENCE

{

lifetime                              [1]   INTEGER (0.. 65535)                  OPTIONAL,

homeAddress                           [2]   OCTET STRING (SIZE (4))           OPTIONAL,

careOfAddress                         [3]   OCTET STRING (SIZE (4))           OPTIONAL,

homeAgentAddress                      [4]   OCTET STRING (SIZE (4))           OPTIONAL,

code                                  [5]   INTEGER (0..255)                  OPTIONAL,

foreignDomainAddress                  [7]   OCTET STRING (SIZE (4))           OPTIONAL,

...

-- parameters coded according to 3GPP TS 29.279 [63] and RFCs specifically

-- referenced in it.

}

# MediaDecryption-info ::= SEQUENCE OF CCKeyInfo

-- One or more key can be available for decryption, one for each media streams of the

-- intercepted session.

# CCKeyInfo ::= SEQUENCE

{

cCCSID	 [1]	OCTET STRING,

-- the parameter uniquely mapping the key to the encrypted stream.

cCDecKey [2]	OCTET STRING,

cCSalt   [3]    OCTET STRING OPTIONAL,

-- The field reports the value from the CS_ID field in the ticket exchange headers as

-- defined in IETF RFC 6043 [61] “draft-mattsson-mikey-ticket”.

...

}

# END -- OF EpsHI2Operations

* * * End of Changes * * *