Source:			Research In Motion UK Ltd.

Title:				Proposed Changes for MIKEY-IBAKE Living Document v0.4.0

Document for:		Discussion and Approval

Agenda Item:

# Work Item / Release:	LI Rel-11

# Contact Information:	N. Alfano, Research In Motion, nalfano@rim.com

# Date:				28 January 2013

# Introduction

Based on points raised in contribution SA3LI13_018 the following changes are proposed for MIKEY-IBAKE Living Document v0.4.0

# Change 1

## 5.4	Evaluation

In this clause the proposed solutions are evaluated against the LI requirements for encrypted services as specified in clause 5.7 of TS 33.106 [4].

# Change 2

### 5.4.2	Second LI Requirement

In all four solutions described in clauses 5.2 and 5.3, call setup is not affected by an ongoing interception, thus there is no difference in latency during call setup or during communications compared to a non-intercepted communications.  Due to the necessity to re-key if SIP messages are lost during call interception the interception may be detectable. Therefore the second LI requirement is not satisfied.

# Change 3

### 5.4.4	Fourth LI Requirement

All four LI solutions for MIKEY-IBAKE described above rely on information contained in SIP messages executed during the call setup with the target of interception.  Therefore, commencement of interception and decryption of an existing communication requires SIP messages executed during the call setup with the Target of interception as well as other session related information to be available.  If these SIP messages are no longer available, the forced or periodic rekeying could be executed.

Therefore, all four solutions described in clauses 5.2 and 5.3 do not satisfy the fourth LI requirement for encrypted services.

# Change 4

### 5.4.6	Summary

The table below summarizes evaluation of the proposed LI approaches against the LI requirements for encrypted services specified in clause 5.7 of TS 33.106 [4].

| Requirements | LI Approaches |  |  |  |
| --- | --- | --- | --- | --- |
|  | Consolidated PRG | Split PRG | Consolidated Nonces | Split Nonces |
| 1. When an encryption service is provided by the PLMN, lawful interception shall take place as for an unencrypted communications (subclause 5.4.1). |  |  |  |  |
| 1. a. In addition, encrypted communications shall be decrypted, or the decryption keys and any required associated information (e.g. roll over counters) shall be provided to the LEMF (subclause 5.4.1). |  |  |  |  |
| 1. b. For the specific case where a key server based solution is used, it is a national option for the operator to make keys and any associated information (e.g. roll over counters) directly available to the LEMF for the decryption of communications (subclause 5.4.1). |  |  |  |  |
| 2. Interception shall be performed in such a manner as to avoid detection by the Target or others (subclause 5.4.2). | X | X | X | X |
| 2. a. There shall be no significant difference in latency during call setup or during communications compared to a non-intercepted communications (subclause 5.4.2). |  |  |  |  |
| 2. b. Interception of a Target shall not prevent the use of key exchange applications which provide a user key confirmation mechanism (subclause 5.4.2). |  |  |  |  |
| 2. c. Should interception fail during a call (or during call setup), the call shall be unaffected (subclause 5.4.2). |  |  |  |  |
| 3. Where the CSP provides decryption of the communication, it is the operator’s choice where in the network this decryption is performed. However, following decryption, all IRI and CC shall be provided to the LEMF using handover mechanisms as per an unencrypted communication (subclause 5.4.3). |  |  |  |  |
| 4. An encryption solution shall not prohibit commencement of Interception and decryption of an existing communication (subclause 5.4.4). | X | X | X | X |
| 5. If key material and any associated information are available, it shall be possible to retrospectively decrypt encrypted communications (subclause 5.4.5). |  |  |  |  |