3GPP TSG-SA3 LI Meeting #51	SA3LI13_094

# Burlington, VT, USA; October 2013

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | CRNum | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition a new clause to describe LI details for VoIP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia Solutions and Networks) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI12 |  |  |  |  |  |  | Date: |  | 2013-10-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | To add a new clause into 3GPP TS 33.107 to provide the details pertaining to the invocation of VoIP lawful interception. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause is added to 3GPP TS 33.107 under the title “Invocation of Lawful Interception for VoIP”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This is part of defining the LI architecture for CC interception for VoIP. Without this, the details of the lawful intrception for VoIP remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 33.108 |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** START OF FIRST MODIFICATION ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI TS 101 331: "Telecommunications security; Lawful Interception (LI); Requirements of Law Enforcement Agencies".

[2]	ETSI ES 201 158: "Lawful Interception; Requirements for network functions".

[3]	ETSI ES 201 671: "Handover Interface for the lawful interception of telecommunications traffic".

[4]	GSM 01.33: "Lawful Interception requirements for GSM".

[5]	GSM 02.33: "Lawful Interception - stage 1".

[6]	GSM 03.33: "Lawful Interception - stage 2".

Editor's note: 
References 4, 5 & 6 are not cited in the body of the present TS, and anyway they do not exist in this Release.

[7]	3GPP TS 33.106: "3G Security; Lawful Interception Requirements".

[8]	ANSI J-STD-025-A: "Lawfully Authorised Electronic Surveillance".

[9]	IETF RFC 2806: "URLs for Telephone Calls ".

[10]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description".

[11]	3GPP TS 33.108: "3G Security; Handover interface for Lawful Interception".

[12]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[13]	3GPP TS 21.905: "Vocabulary for 3GPP Specifications".

[14]	3GPP TS 23.234: "3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[15]	3GPP TS 23.008: "Organization of subscriber data".

[16]	3GPP TS 29.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; Stage 3".

[17]	3GPP TS 24.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; User Equipment (UE) to network protocols; Stage 3".

[18]	IETF RFC 1122: "Requirements for Internet Hosts -- Communication Layers".

[19]	IETF RFC 1123: "Requirements for Internet Hosts -- Application and Support".

[20]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and Functional Description".

[21]	3GPP TS 24.147: "Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[22]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[23]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[25]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[26]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[27]	(void)

[28]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[29]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[30]	3GPP TS 23.272: “Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2”

[31]	3GPP TS 22.220:  “Service Requirements for Home NodeBs and Home eNodeBs”.

[32]	3GPP TS 36.300:  Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network  (E-UTRAN); Overall description; Stage 2”.

[33]	3GPP TS 25.467:  “UTRAN architecture for 3G Home Node B (HNB); Stage 2”

[34]	3GPP TS 33.320:  “Security of Home Node B (HNB) / Home evolved Node B (HeNB)”.

[35]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[y1]	3GPP TS 29.334: “IMS Application Level Gateway (IMS-ALG) – IMS Access Gateway (IMS-AGW); Iq Interface (Stage 3)”

[y2]	3GPP TS 23.228: “IP Multimedia Subsystem; Stage 2”

[y3]	3GPP TS 23.334: “IP Multimedia Subsystem (IMS) Application Level Gateway (IMS-ALG) – IMS Access Gateway (IMS-AGW) interface: Procedures descriptions”

[y4]	ATIS-0700005.2007, ATIS-0700005.a: “LAES for 3GPP IMS-based VoIP and other Multimedia Services”

[y5]	3GPP TS 23.203: “Policy Charging and Control Architecture”

[y6]	3GPP TS 23.292: “IP Multimedia Subsystem (IMS) Centralized Services; Stage 2”

[y7]	3GPP TS 23.237: “IP Multimedia Subsystem (IMS) Service Continuity; Stage 2”

[y8]	3GPP TS 29.162: “Interworking between IM CN subsystem and IP Networks”

[y9]	3GPP TS 29.163” “Interworking between IP Multimedia Core Network (CN) subsystem and Circuit Switched (CS) Networks”

*** END OF FIRST MODIFICATION ***

*** START OF SECOND MODIFICATION ***

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [13] and the following apply:

3GMS	3rd Generation Mobile Communications System

3G GGSN	3rd Generation Gateway GPRS Support Node

3G GSN		3rd Generation GPRS Support Node (GGSN/SGSN)

3G MSC		3rd Generation Mobile Switching Center

3G SGSN	3rd Generation Serving GPRS Support Node

3G UMSC	3rd Generation Unified Mobile Switching Centre

AAA	Authentication, Authorization, and Accounting

ADMF	Administration Function

AN	Access Network

AP	Access Provider

AS	Application Server

BM-SC	Broadcast-Multicast Service Centre

BSF	Bootstrapping Serving Function

B-TID	Bootstrapping Transaction Identifier

CC	Content of Communication

CS	Circuit Switched

CSCF	Call Session Control Function

CSG	Closed Subscriber Group

DF	Delivery Function

DSMIP		Dual Stack Mobile IP

ECT	Explicit Call Transfer

EPC	Evolved Packet Core

ePDG	Evolved PDG

EPS	Evolved Packet System

E-UTRAN	Evolved UTRAN

FTP	File Transfer Protocol

GBA	Generic Bootstrapping Architecture

GGSN	Gateway GPRS Support Node

GPRS	General Packet Radio Service

GSM	Global System for Mobile communications

GSN	GPRS Support Node (SGSN or GGSN)

HA	Home Agent

HeMS	HeNB Management System

HeNB	Home enhanced NodeB

HeNB GW	HeNB Gateway

H(e)NB	Home and Home enhanced NodeB

HI	Handover Interface

HMS	HNB Management System

HNB	Home NodeB

HNB GW	HNB Gateway

HRPD	High Rate Packet Data

HSS	Home Subscriber Server

IA	Interception Area

IBCF	Interconnecting Border Control Function

ICEs	Intercepting Control Elements (3G MSC Server, 3G GMSC Server, P-CSCF, S-CSCF, SGSN, GGSN, HLR, AAA Server, PDG, MME, S-GW, PDN-GW, HSS)

IETF	Internet Engineering Task Force

IM-MGW	IMS Media Gateway

IMEI	International Mobile station Equipment Identity

IMS	IP Multimedia Core Network Subsystem

IMS-AGW	IMS Access Gateway

IMSI	International Mobile Subscriber Identity

INEs	Intercepting Network Elements (3G MSC Server, 3G GMSC Server, P-CSCF, S-CSCF, SGSN, GGSN, MGW, HLR, AAA Server, PDG)

IP	Internet Protocol

IRI	Intercept Related Information

I-WLAN	Interworking WLAN (3GPP WLAN interworking subnetwork)

LAN	Local Area Network

LDI	Location Dependent Interception

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LIPA	Local IP Access

LTE	Long Term Evolution

MBMS	Multimedia Broadcast/Multicast Service

MGCF	Media Gateway Control Function

MGW	Media Gateway

ME	Mobile Entity

MIP	Mobile IP

MME	Mobility Management Entity

MN	Mobile Node

MSISDN	Mobile Subscriber ISDN Number

NAF	Network Application Function

NAI	Network Access Identifier

NO	Network Operator

PCRF	Policy and Charging Rules Function

P-CSCF	Proxy CSCF

PDG	Packet Data Gateway

PDN 	Packet Data Network

PDN-GW	PDN Gateway

PMIP	Proxy Mobile IP

PoC	Push to talk over Cellular

PS	Packet Switched

RA	Routing Area

RAI	Routing Area Identity

SAI	Service Area Identity

S-CSCF	Serving CSCF

SeGW	Security Gateway

SGSN	Serving GPRS Support Node

SIP	Session Initiation Protocol

SMS	Short Message Service

S-GW	Serving Gateway

TEL URL	"tel" URL, as defined in RFC 2806 [9]

TLS	Transport Layer Security

TrGW	Transit Gateway

UE	User Equipment

UMTS	Universal Mobile Telecommunication System

URI	Universal Resource Identifier

URL	Universal Resource Locator

VoIP	Voice over IP

VoLTE	Voice over LTE

WLAN	Wireless LAN

*** END OF SECOND MODIFICATION ***

*** THIRD MODIFICATION *** NEW CLAUSE ***

X.X Invocation of Lawful Interception for VoIP

The capabilities defined in this clause shall apply when the interception of content of communications at the packet data network through a separate activation and invocation method, as outlined in clause 7A.1, does not meet the national regulations for the lawful interception of IMS-based VoIP calls.

X.X.1 Background

One of the use-cases of IMS-based VoIP service is VoLTE.  The term “VoLTE” is used to refer to an IMS-based VoIP service when EPS (see 3GPP TS 23.401 [22]) happens to be the access network.  When EPS is not the access network, the lawful interception capabilities defined in this clause shall  apply for any IMS-based VoIP service with the presumption that in those cases the media on the target’s access goes through an IMS-AGW (see 3GPP TS 29.334 [y1]) or a PDN-GW (see 3GPP TS 23.401 [22] and 3GPP TS 402 [23]).

Note 1: Even with the EPS-based access network, the media may still go through the IMS-AGW. And, in this case, a VoLTE shall be treated very similar to any other VoIP service.

Furthermore, it is presumed that an inter-CSP call enters or leaves an IMS network via an IBCF/TrGW or an MGCF/IM-MGW depending on whether the inter-working CSP network is an IMS-based network or a CS-based network (see 3GPP TS 23.228 [y2]).

The figure X.1 illustrates the VoIP configuration considered for the lawful interception capabilities defined in this clause:

Figure X.1: VoIP Configuration

The network nodes, involved in providing the interception of a VoIP call, shall be determined based on the deployment configuration and the call scenario. The scenario, where other network nodes that are not defined in this clause,  but may be involved in providing the lawful interception functions for VoIP calls, are beyond the scope of this release. Also, the scenarios, where the access nodes and signalling nodes are governed by different CSPs are beyond the scope of this release.

Note 2: Lawful interception of VoIP as it applies voice call service continuity (see 23.237 [y7]) is for further study.

Note 3: Lawful interception of VoIP as it applies and IMS centralized services (see 3GPP TS 23.292 [y6]) is for further study.

The term “access node” is used to denote the network node present on media path. The term “signaling node” is used to denote the network node present on the signalling path.

The general concepts of VoIP LI is shown in figure X.2

Figure X.2: General Concepts of VoIP LI

The interception of IRI for a VoIP call shall be done according to X.X.3. The interception of VoIP CC shall be done according to X.X.2.

X.X.2 Provision of Content of Communications

The figure X3 is extracted from figures 1g and 1h to explain the CC interception and delivery for a VoIP call.

Figure X3: VoIP CC Interception and Delivery – an overview

The interception and delivery of CC for VoIP may be done at the following access nodes:

- PDN-GW

- IMS-AGW

- TrGW

- IM-MGW.

The trigger for the CC interception may be generated at the following signalling nodes:

- P-CSCF, for PDN-GW and IMS-AGW

- IBCF for TrGW

- MGCF for IM-AGW.

At any given time and for any given call, only one access node is required to provide the CC interception. The access node that provides the CC interception can be different, primarily, based on the call scenario.

X.X.2.1 General Principles of CC Interception

X.X.2.1.1 Intercept Trigger

This is referring to the intercept trigger that a P-CSCF sends through the PCRF to the PDN-GW over the Rx and Gx reference points, the P-CSCF sends to the IMS-AGW over the Iq reference point, the IBCF sends to the TrGW over the Ix reference point and the MGCF sends to the IM-MGW over the Mn reference point.

The intercept trigger, at the minimum, shall consist the following:

- Correlation Identifier

- Media Identifier

The Correlation Identifier is used correlate the CC with the corresponding IRI data and is delivered from the intercepting access node in the intercepted media packet (i.e., CC) over the X3 interface to the Delivery Function 3.

The Media Identifier is used to identify the call of which the media packets have to be intercepted. The technique used in defining the Media Identifier is an implementation specific.

The information passed on this intercept trigger shall adhere to the security requirements outlined in clause 8.

X.X.2.1.2 X3-Interface

This is referring to the delivery of intercepted media packets (i.e., CC) from the intercepting access node to the Delivery Function 3. In addition to the actual intercepted packets, the following information shall be passed from the intercepting access node to the Delivery Function 3:

- target identity;

- correlation number;

- time stamp (optional);

- direction (indicates media is from or to the target) – optional;

The intercepted packets shall include the IP layer and above (e.g., IP/UDP/RTP). The Delivery Function 3 delivers the information to the LEMF over the HI3 interface based on the national regulations.

X.X.2.2 VoIP CC Interception at IMS-AGW

The interception of CC at an IMS-AGW is done for the following call scenarios:

- When a target originates a call or receives an incoming call – in both cases, target’s media passes through the IMS-AGW.

- When an incoming call to the target is forwarded within the same CSP’s network – in this case, the target is not physically on the call. The media of the forwarded call passes through the IMS-AGW.

The figure X4 illustrates the CC interception at the IMS-AGW for a basic call and figure X5 illustrates the CC interception at the IMS-AGW for an intra-CSP forwarded call.

Figure X4: VoIP CC Interception at the IMS-AGW for basic calls

In figure X4, the Intercepted Subscriber is the target and the Other Party is the called party when the target originates a call; and the Other Party is the calling party when the target receives an incoming call. In both cases, the media passes through the IMS-AGW present on the side of target’s access network and that IMS-AGW provides the CC interception.

In figure X5 (below), there is no Intercepted Subscriber (i.e., target) shown because this is the scenario where an incoming call to a target gets forwarded in the same CSP’s network. The figure X5 shows the calling party who originated call and the forwarded-to-party who receives the forwarded call. The media passes through the IMS-AGW associated with the forwarded-to-party and that IMS-AGW provides the CC interception.

Figure X5: VoIP CC Interception at the IMS-AGW for intra-CSP forwarded calls

The P-CSCF sends the CC intercept trigger to the IMS-AGW over the Iq reference point according X.X.2.1.1.   The IMS-AGW intercepts media packets for the call (identified based on the Media Identifier information received over the intercept trigger) and delivers the media packets as according to X.X.2.1.2.

Note 4: Iq reference point is defined in 3GPP TS 23.334 [y3].

X.X.2.3 VoIP CC Interception at PDN-GW

The capability defined in this clause is an optional capability, when the IMS-AGW deployed on the target’s access network provides the CC interception (see X.X.2.2).

The interception of CC at a PDN-GW shall apply to the following call scenarios:

- When a target originates a call or receives an incoming call – in both cases, target’s media passes through the PDN-GW.

- When an incoming call to the target is forwarded within the same CSP’s network – in this case, the target is not physically on the call. The media of the forwarded call passes through the PDN-GW.

The figure X6 illustrates the CC interception at the PDN-GW for a basic call and figure X7 illustrates the CC interception at the PDN-GW for an intra-CSP forwarded call.

Figure X6: VoIP CC Interception at the PDN-GW for basic calls

In figure X5, the Intercepted Subscriber is the target and the Other Party is the called party when the target originates a call and the Other Party is the calling party when the target receives an incoming call. In both cases, the media passes through the PDN-GW present on the target’s access network and that PDN-GW provides the CC interception.

In figure X7 (below), there is no Intercepted Subscriber (i.e., target) shown because this is the scenario where an incoming call to a target gets forwarded in the same CSP’s network. The figure X7 shows the calling party who originated call and the forwarded-to-party who receives the forwarded call. The media passes through the PDN-GW associated with the forwarded-to-party and that PDN-GW provides the CC interception.

Figure X7: VoIP CC Interception at the PDN-GW for intra-CSP forwarded calls

The P-CSCF sends the CC intercept trigger to the PDN-GW through the PCRF over the Rx and Gx reference points according X.X.2.1.1.     The PDN-GW intercepts media packets for the call (identified based on the Media Identifier information received over the intercept trigger) and delivers the media packets as according to 12.2.2 and X.X.2.1.2.

Note 5: PCRF, Rx and Gx reference points are defined in 3GPP TS 23.203 [y5].

X.X.2.4 VoIP CC Interception at TrGW

The interception of CC at a TrGW is done for the following call scenario:

- When an incoming call to the target is forwarded to another IMS subscriber in another CSP’s network.   The media of the forwarded call passes through the TrGW.

The figure X8 illustrates the CC interception at the TrGW for an inter-CSP (IMS) forwarded call.    In figure X8 (below), there is no Intercepted Subscriber (i.e., target) shown because this is the scenario where an incoming call to a target gets forwarded to an IMS subscriber in another CSP’s network. The figure X8 shows the calling party who originated call and the forwarded-to-party who receives the forwarded call. The media passes through the TrGW before entering the other CSP’s IMS network and that TrGW provides the CC interception.

Figure X8: VoIP CC Interception at the TrGW for forwarded calls

IBCF sends the CC intercept trigger to the TrGW over the Ix reference point according X.X.2.1.1. TrGW intercepts media packets for the call (identified based on the Media Identifier information received over the intercept trigger) and delivers the media packets as according to X.X.2.1.2.

Note 6: Ix reference point is defined in 3GPP TS 29.162[y8].

X.X.2.5 VoIP CC Interception at IM-MGW

The interception of CC at an IM-MGW is done for the following scenarios:

- When an incoming call to the target is forwarded to a subscriber in a CS domain of another CSP’s network.   The media of the forwarded call passes through the IM-MGW.

The figure X9 illustrates the CC interception at the IM-MGW for an inter-CSP (IMS) forwarded call.

In figure X9 (below), there is no Intercepted Subscriber (i.e., target) shown because this is the scenario where an incoming call to a target gets forwarded to a subscriber in the CS domain of another CSP’s network. The figure X9 shows the calling party who originated call and the forwarded-to-party who receives the forwarded call. The media passes through the IM-MGW before entering the other CSP’s network and that IM-MGW provides the CC interception.

Figure X9: VoIP CC Interception at the IM-MGW for forwarded calls

MGCF sends the CC intercept trigger to the IM-MGW over the Mn reference point according X.X.2.1.1. IM-MGW intercepts media packets for the call (identified based on the Media Identifier information received over the intercept trigger) and delivers the media packets as according to X.X.2.1.2.

Note 7: Mn reference point is defined in 3GPP TS 29.163[y9].

X.X.3 Provision of Intercept Related Information for VoIP

The figure X10 is extracted from figures 1d, 1g and 1h to explain the IRI interception and delivery for a VoIP call.

Figure X10: VoIP IRI Interception and Delivery – an overview

The interception and delivery of IRI for VoIP is done in the same way as defined for IMS in clause 7.A. The clause 7.A defines that the interception of IRI for IMS is done at the S-CSCF or at the P-CSCF.

For VoIP, the interception of IRI can also be done at the AS that provides the voice-specific services (e.g., supplementary voice services).  The delivery method used for the IRI intercepted at the AS is same as the delivery method used for the IRI intercepted at the S-CSCF/P-CSCF.

Any additional capabilities that may be needed for the AS-based interception and delivery of IRI are for further study.

Note 8: While the clause 7A defines the IRI interception and delivery for all SIP messages (voice and other multimedia services), the Interception of IRI at a voice-specific AS may allow the VoIP CSP to have the ability to limit the interception and delivery of IRI for the voice services.

Note 9: The interception at the voice specific AS may also allow the CSP to have the ability to provide the interception for voice-related supplementary services. The details of these are for further study.

The handling of IMS media plane security as it applies to a VoIP call shall be done according to clause 7A.

*** END OF THIRD MODIFICATION *** NEW CLAUSE ***