3GPP TSG-SA3-LI Meeting #51 	SA3LI13_116r2

# Burlington, VT., USA, 22-24, Oct 2013

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | CRNum | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 107 UMTS IRI Packet Header Information Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (FBI, Ministère du Redressement Productif - France) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI12 |  |  |  |  |  |  | Date: |  | 2013-10-22 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | To provide UMTS IRI Packet Header Information reporting to support national requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | References added, new events defined to report per packet and summary reports. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory implications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.3, 7.4.4, New 7.4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	Void

[2]	ETSI ES 201 158 (V1.2.1 April 2002): "Lawful Interception; Requirements for network functions".

[3]	ETSI ES 201 671 (V3.1.1 May 2007): "Handover Interface for the lawful interception of telecommunications traffic".

[4]	Void

[5]	Void

[6]	Void

[7]	3GPP TS 33.106: "3G Security; Lawful Interception Requirements".

[8]	ANSI J-STD-025-A (April 2003): "Lawfully Authorised Electronic Surveillance".

[9]	 VOID

[10]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description".

[11]	3GPP TS 33.108: "3G Security; Handover interface for Lawful Interception".

[12]	 Void

[13]	3GPP TS 21.905: "Vocabulary for 3GPP Specifications".

[14]	3GPP TS 23.234: "3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[15]	3GPP TS 23.008: "Organization of subscriber data".

[16]	3GPP TS 29.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; Stage 3".

[17]	3GPP TS 24.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; User Equipment (UE) to network protocols; Stage 3".

[18]	IETF RFC 1122 (October 1989): "Requirements for Internet Hosts -- Communication Layers".

[19]	IETF RFC 1123 (October 1989): "Requirements for Internet Hosts -- Application and Support".

[20]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and Functional Description".

[21]	3GPP TS 24.147: "Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[22]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[23]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[25]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[26]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[27]	Void

[28]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[29]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[30]	3GPP TS 23.272: " Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2"

[31]	3GPP TS 22.220: " Service Requirements for Home NodeBs and Home eNodeBs".

[32]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network  (E-UTRAN); Overall description; Stage 2".

[33]	3GPP TS 25.467: "UTRAN architecture for 3G Home Node B (HNB); Stage 2"

[34]	3GPP TS 33.320: "Security of Home Node B (HNB) / Home evolved Node B (HeNB) ".

[35]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[36]	IETF RFC 3966 (December 2004): "The Tel URILs for Telephone Numbers ".

[a]	IETF RFC 791:  “Internet Protocol”.

[b]	IETF RFC 2460:  “Internet Protocol, Version 6 (IPv6) Specification”.

[c]	IEFT RFC 3697:  “IPv6 Flow Label Specification”.

*** NEXT MODIFICATION ***

## 7.3	Provision of Intercept Related Information

Intercept Related Information (Events) are necessary at the Mobile Station Attach, Mobile Station Detach, PDP Context Activation, Start of intercept with PDP context active, PDP Context Deactivation, RA update, Serving System, Packet Header Information Reporting, and SMS events.

Serving System event reporting is a national option.

Packet Header Information Reporting is a national option.

Figure 21 shows the transfer of intercept related information to the DF2. If an event for / from a mobile subscriber occurs, the 3G GSN or the Home Location Register (HLR) sends the relevant data to the DF2. For Packet Header Information Reporting, a 3G GSN either isolates the relevant data and sends it to the DF2 or sends the packet stream to another entity in the network (e.g., DF3) for isolation which then provides the relevant data to the DF2.

See clause 7A for multi-media Intercept Related Information produced at the CSCF.

intercepted

subscriber

HLR

other

party

Delivery

Function 2

LEMF

3G GSN

Figure 21: Provision of Intercept Related Information

### 7.3.1	X2-interface

The following information needs to be transferred from the 3G GSN or the HLR to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity (MSISDN, IMSI, IMEI);

-	events and associated parameters as defined in clauses 7.3.2 and 7.4 may be provided;

-	the target location (if available) or the IAs in case of location dependent interception;

-	Correlation number;

-	Quality of Service (QoS) identifier;

-	Encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

The IRI should be sent to DF2 using a reliable transport mechanism.

The 3G GSN detects packets containing packet header information in the communications path but the information needed for Packet Header Information Reporting may need to be transferred from the 3G GSN either directly to the DF2 or via another entity in order to allow the DF2 to perform its functionality.

### 7.3.2	Structure of the events

There are several different events in which the information is sent to the DF2 if this is required. Details are described in the following clause. The events for interception are configurable (if they are sent to DF2) in the 3G GSN or the HLR and can be suppressed in the DF2.

The following events are applicable to 3G SGSN:

-	Mobile Station Attach;

-	Mobile Station Detach;

-	PDP context activation;

-	Start of interception with mobile station attached (national option);

-	Start of intercept with PDP context active;

-	PDP context modification;

-	PDP context deactivation;

-	RA update;

-	SMS;

-	Packet Header Information Reporting.

NOTE:	3G GGSN interception is a national option. Location information may not be available in this case.  If interception is performed at the 3G GGSN, then Packet Header Information Reporting shall also be performed at the 3G GGSN and not at the 3G SGSN.

If 3G direct tunnel functionality with the GGSN, as defined in TS 23.060 [10], is used in the network, then both the SGSN and the GGSN shall perform the interception of intercept related information.

When the SGSN connects an UE to a S-GW through the S4 interface ([10]), the SGSN is not required to report events PDP context activation (successful), Start of intercept with PDP context active, PDP context modification, PDP context deactivation; the SGSN shall report unsuccessful PDP context activation event.

The following events are applicable to the 3G GGSN:

-	PDP context activation;

-	PDP context modification;

-	PDP context deactivation;

-	Start of interception with PDP context active,

-	Packet Header Information Reporting.

The following events are applicable to the HLR:

-	Serving System.

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from 3G GSN or HLR to DF2, perhaps via a MF in the case of Packet Header Information Reporting. Available IEs from this set of elements as shown below can be extended in the 3G GSN or HLR, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option e.g. a unique number for each surveillance warrant.

Table 2: Information Events for Packet Data Event Records

Observed MSISDN MSISDN of the target subscriber (monitored subscriber).

Observed IMSI IMSI of the target subscriber (monitored subscriber).

Observed IMEI IMEI of the target subscriber (monitored subscriber),it shall be checked for each activation over the radio interface.

Event type Description which type of event is delivered: MS attach, MS detach, PDP context activation, Start of intercept with PDP context active, PDP context deactivation, SMS, Serving System, Packet Data Header Information, Cell and/or RA update.

Event date Date of the event generation in the 3G GSN or the HLR.

Event time Time of the event generation in the 3G GSN or the HLR. Timestamp shall be generated relative to GSN or HLR internal clock.

PDP address The PDP address of the target subscriber. Note that this address might be dynamic. In case the PDP type is IPv4v6, the parameter may carry two IP addresses.

Access Point Name The APN of the access point. (Typically the GGSN of the other party).

Location Information Location Information is the Service Area Identity (SAI), RAI and/or location area identity that is present at the GSN at the time of event record production.

Old Location Information Location Information of the subscriber before Routing Area Update

PDP Type The used PDP type.

Correlation Number The correlation number is used to correlate CC and IRI.

SMS The SMS content with header which is sent with the SMS-service. The header also includes the SMS-Centre address.

Network Element Identifier Unique identifier for the element reporting the ICE.

Failed attach reason Reason for failed attach of the target subscriber.

Failed context activation reason Reason for failed context activation of the target subscriber.

IAs The observed Interception Areas.

Initiator The initiator of the PDP context activation, deactivation or modification request either the network or the 3G MS.

SMS Initiator SMS indicator whether the SMS is MO or MT.

Deactivation / termination cause The termination cause of the PDP context.

QoS This field indicates the Quality of Service associated with the PDP Context procedure.

Serving System Address Information about the serving system (e.g. serving SGSN number or serving SGSN address).

NSAPI Network layer Service Access Point Identifier The NSAPI information element contains an NSAPI identifying a PDP Context in a mobility management context specified by the Tunnel Endpoint Identifier Control Plane. This is an optional parameter to help DF/MF and LEA's to distinguish between the sending mobile access networks when the GGSN is used as element of the PDG according TS 23.234 [14], Annex F.

Destination IP Address The IP address, including type IPv4 or IPv6, of the destination of the IP packet.

Destination Port Number The port number of the destination of the IP packet.

Flow Label (IPv6 only) The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [c]).

Packet Count The number of packets detected and reported (for a particular summary period).

Packet Data Summary Reason The reason for a Packet Data Summary message being sent to the LEMF (e.g., timed out, counter expiration, end of session)

Packet Size The size of the packet. (i.e., Total Length Field in IPv4 or Payload Length field in IPv6)

Source IP Address The IP address, including type IPv4 or IPv6, of the source of the IP packet.

Source Port Number The port number of the source of the IP packet.

Sum of Packet Sizes (for a particular summary period) The sum of values contained in the Total Length fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets.

Summary Period Includes the dates and times of the first and last packets in a particular packet data interval.

Transport Protocol (e.g., TCP) The identification of the transport protocol of the packet or packet flow being reported.

*** NEXT MODIFICATION ***

### 7.4.4	Start of interception with PDP context active

This event will be generated if interception for a target is started and if the target has at least one PDP context active. If more than one PDP context is open, for each of them an event record is generated. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

PDP address of observed party

Event Type

Event Time

Event Date

Correlation number

Access Point Name

PDP Type

Network Element Identifier

Location Information

Old Location Information (optional)

IAs (if applicable)

QoS (optional)

Initiator (optional)

NSAPI (optional)

Presence of the optional Old Location Information field indicates that PDP context was already active, and being intercepted. However, the absence of this information does not imply that interception has not started in the old location SGSN for an active PDP context.

Start of interception with PDP context active shall be sent regardless of whether a Start of interception with mobile station attached has already been sent.

*** NEXT MODIFICATION ***

### 7.4.X	Packet Header Information Reporting

Packet Header Information Reporting can be done either on a per-packet (i.e., non-summarized) basis or in a summary report.

### 7.4.X.1	Packet Data Header Report

This event is used to provide packet header reports on a per packet basis (non-summarized reporting) and is triggered by each packet sent or received by the target.  These elements will be delivered either directly to DF2 or via another network entity if available:

Observed MSISDN

Observed IMSI

Observed IMEI

PDP address of observed party

Event Type

Event Time

Event Date

Correlation Number

Access Point Name

PDP Type

Network Element Identifier

Source IP Address

Source Port Number

Destination IP Address

Destination Port Number

Transport Protocol (e.g., TCP)

Packet Size

Flow Label (IPv6 only)

### 7.4.X.2	Packet Data Summary Report

This event is used to report:

- the source and destination information derived from the packet headers, including:

- source and destination IP Addresses,

- IP next-layer protocol,

- Layer-4 ports, and

- Flow label, if the packet is IPv6

- summary information for the number of packets and bytes transmitted or received by the subject for each unique packet flow within a PDP context,  and

- the date and the time of the first and last packets associated with that packet flow.  A packet flow is defined as the 6-tuple of source/destination IP address/port number and the layer 4 protocol, and PDP Context.

IP addresses and the IP next-layer protocol are always reported, the flow label is reported if the packet is IPv6, and the layer-4 ports are reported.

The event provides packet summary reports for each unique packet data session (PDP context) and packet flow, and is triggered by one of the following:

- start of a packet flow associated with a PDP Context

- an interim report for a packet flow associated with a PDP Context is to be reported

- end of a packet flow associated with a PDP Context (including end of the PDP Context itself).

An interim report can be triggered by

- The expiration of a configurable timer per intercept (called a Summary Timer). The Summary Timer is configurable in units of seconds;

- A per-intercept configurable count threshold is reached;

These elements will be delivered either directly to DF2 or via an MF for each packet flow if available:

Observed MSISDN

Observed IMSI

Observed IMEI

PDP address of observed party

Event Type

Event Time

Event Date

Correlation Number

Access Point Name

PDP Type

Network Element Identifier

Source IP Address

Source Port Number

Destination IP Address

Destination Port Number

Transport Protocol (e.g., TCP)

Flow Label (IPv6 only)

Summary Period

Packet Count (for this summary period)

Sum of Packet Sizes (for this summary period)

If the packets are IPv4, the sum of all observed packet sizes is the sum of the values contained in the Total Length field of each packet as specified in IETF RFC 791[a].

If the packet is IPv6, the sum of all observed packet sizes is the sum of the values contained in the Payload Length field for each packet as specified in IETF RFC 2460 [b].

If no packets were detected for the duration of the Summary Timer, then the Packet Data Summary Report shall not be sent.

*** END OF MODIFICATIONS ***