3GPP TSG-SA3-LI Meeting #51 	SA3LI13_120

# Burlington, VT., 22-24 October 2013

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | CRNum | rev | - | Current version: | 12.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 108 WLAN IRI Packet Header Information Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (FBI, Ministère du Redressement Productif - France) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI12 |  |  |  |  |  |  | Date: |  | 2013-10-22 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | To provide WLAN IRI Packet Header Information reporting to support national requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | References added, new events defined to report per packet and summary reports. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory implications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.5, 8.5.1.4, 8.5.1.x, Annex B.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | TS 33.108 LTE companion CR is SA3LI13_121. TS 33.108 UMTS companion contribution is SA3LI13_119. TS 33.107 companion CR is SA3LI13_117. |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI TS  101 331: "Lawful Interception (LI); Requirements of Law Enforcement Agencies".

[2]	ETSI ES 201 158: "Telecommunications security; Lawful Interception (LI); Requirements for network functions".

[3]	ETSI ETR 330: "Security Techniques Advisory Group (STAG); A guide to legislative and regulatory environment".

[4]	3GPP TS 29.002: "3rd Generation Partnership Project; Technical Specification Group Core Network; Mobile Application Part (MAP) specification".

[5A]	ITU-T Recommendation X.680: "Abstract Syntax Notation One (ASN.1): Specification of Basic Notation".

[5B]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Information Object Specification".

[5C]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Constraint Specification".

[5D]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 Specifications".

[6]	ITU-T Recommendation X.690: "ASN.1 encoding rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER)".

NOTE 1:	It is recommended that for [5A], [5B], [5C], [5D] and [6] the 2002 specific versions should be used.

[7]	ITU-T Recommendation X.880: "Information technology - Remote Operations: Concepts, model and notation".

[8]	ITU-T Recommendation X.882: "Information technology - Remote Operations: OSI realizations - Remote Operations Service Element (ROSE) protocol specification".

NOTE 2:	It is recommended that for [8] the 1994 specific versions should be used.

[9]	3GPP TS 24.008: "3GPP Technical Specification Group Core Network; Mobile radio interface Layer 3 specification, Core network protocol; Stage 3".

[10] - [12]	Void.

[13]	IETF STD 9 (RFC 0959): "File Transfer Protocol (FTP)".

[14]	3GPP TS 32.215: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication Management; Charging Management; Charging data description for the Packet Switched (PS) domain)".

[15]	IETF STD0005 (RFC 0791: "Internet Protocol".

[16]	IETF STD0007 (RFC 0793): "Transmission Control Protocol".

[17]	3GPP TS 29.060: "3rd Generation Partnership Project; Technical Specification Group Core Network; General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[18]	3GPP TS 33.106: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful Interception Requirements".

[19]	3GPP TS 33.107: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful interception architecture and functions".

[20]	3GPP TS 23.107: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Quality of Service QoS concepts and architecture".

[21] – [22]	Void.

[23]	ANSI/J-STD-025-A: "Lawfully Authorized Electronic Surveillance".

[24]	ETSI TS 101 671: "Handover Interface for the lawful interception of telecommunications traffic".

[25]	3GPP TS 23.003: "3rd Generation Partnership Project; Technical Specification Group Core Network; Numbering, addressing, and identification".

[26]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[27]	IETF RFC 1006: "ISO Transport Service on top of the TCP".

[28]	IETF RFC 2126: "ISO Transport Service on top of TCP (ITOT)".

[29]	ITU-T Recommendation Q.763: "Signalling System No. 7 - ISDN User Part formats and codes".

[30]	ETSI EN 300 356 (all parts): "Integrated Services Digital Network (ISDN); Signalling System No.7; ISDN User Part (ISUP) version 3 for the international interface".

[31]	ETSI EN 300 403-1 (V1.3.2): "Integrated Services Digital Network (ISDN); Digital Subscriber Signalling System No. one (DSS1) protocol; Signalling network layer for circuit-mode basic call control; Part 1: Protocol specification [ITU-T Recommendation Q.931 (1993), modified]".

NOTE 3:	Reference [31] is specific, because ASN.1 parameter "release-Reason-Of-Intercepted-Call" has the following comment: "Release cause coded in [31] format". In case later version than the given one indicated for ISDN specification ETSI EN 300 403-1 has modified format of the "release cause", keeping the reference version specific allows to take proper actions in later versions of this specification.

[32] - [33]	Void

[34]	ITU-T Recommendation Q.931: "ISDN user-network interface layer 3 specification for basic call control".

[35]	Void.

[36]	Void.

[37]	3GPP TS 23.032: "3rd Generation Partnership Project; Technical Specification Group Core Network; Universal Geographical Area Description (GAD)".

[38]	3GPP TR 21.905: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

[39]	ISO 3166-1: "Codes for the representation of names of countries and their subdivisions - Part 1: Country codes".

[40]	3GPP TS 23.228: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Stage 2".

[41]	3GPP TS 29.234: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals: 3GPP System to Wireless Local Area Network (WLAN) interworking; Stage 3".

[42]	3GPP TS 23.060: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS); Service description".

[43]	3GPP TS 23.234: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[44]		3GPP TS 23.401: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”.

[45]	3GPP TS 23.402:  "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Architecture enhancements for non-3GPP accesses".

[46]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Access (GPRS) Tunneling Protocol for Control Plane (GTPv2-C); Stage 3".

[47]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[48]	3GPP TS 29.275: "Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunneling protocols; Stage 3".

[49]	3GPP TS 24.303: "Mobility management based on Dual-Stack Mobile IPv6; Stage 3".

[50]	(void)

[51]	(void)

[52]	3GPP TS 24.147: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[53]	3GPP TS 29.273: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[54]	3GPP TS 33.328: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS) media plane security".

[55]	ATIS-0700005 "Lawfully Authorized Electronic Surveillance (LAES) for 3GPP IMS-based VoIP and other Multimedia Services".

[56]	3GPP TS 29.212: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Policy and Charging Control(PCC); Reference points".

[57]	Void.

[58]	IETF RFC 4217: "Securing FTP with TLS".

[59]	3GPP TS 29.272: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Evolved Packet System (EPS); Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[60]	3GPP TS 33.310: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Network Domain Security (NDS); Authentication Framework (AF)".

[61]	IETF RFC 6043: "MIKEY-TICKET: Ticket-Based Modes of Key Distribution in Multimedia Internet KEYing (MIKEY)", available at www.ietf.org

[62]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[63]	3GPP TS 29.279: "Mobile IPv4 (MIPv4) based mobility protocols; Stage 3".

[64]	3GPP TS 29.118: "Mobility Management Entity (MME) –Visitor Location Register (VLR) SGs interface specification"

[65]	ANSI/J-STD-025-B: "Lawfully Authorized Electronic Surveillance”, July 17, 2006.

[66]	3GPP TS 24.007: "Mobile Radio Interface Signalling Layer 3; General Aspects".

[67]	IETF RFC 3966: "The Tel URIs for Telephone Numbers", December, 2004.

[a]	IETF RFC 791:  “Internet Protocol”.

[b]	IETF RFC 2460:  “Internet Protocol, Version 6 (IPv6) Specification”.

[c]	IEFT RFC 3697:  “IPv6 Flow Label Specification”.

*** NEXT MODIFICATION ***

## 8.5	IRI for I-WLAN

The IRI will in principle be available in the following phases of a data transmission:

1.	At I-WLAN access initiation attempt, when the target identity becomes active, at which time packet transmission may or may not occur (at the set up of a I-WLAN tunnel, the target may be the originating or terminating party);

2.	At the end of a connection, when the target identity becomes inactive (removal of a I-WLAN tunnel);

3.	At certain times when relevant information are available.

In addition, information on non-transmission related actions of a target constitute IRI and is sent via HI2, e.g. information on subscriber controlled input.

The IRI may be subdivided into the following categories:

1.	Control information for HI2 (e.g. correlation information);

2.	Basic data communication information, for standard data transmission between two parties.

# The events defined in TS 33.107 [19] are used to generate records for the delivery via HI2.

There are multiple different event types received at DF2 level. According to each event, a Record is sent to the LEMF if this is required. The following table gives the mapping between event type received at DF2 level and record type sent to the LEMF.

Table 8.1: Mapping between I-WLAN Events and HI2 records type

| Event | IRI Record Type |
| --- | --- |
| I-WLAN Access Initiation | REPORT |
| I-WLAN Access Termination | REPORT |
| I-WLAN Tunnel Establishment (successful) | BEGIN |
| I-WLAN Tunnel Establishment (unsuccessful) | REPORT |
| I-WLAN Tunnel Disconnect | END |
| Start of intercept with I-WLAN Communication Active | BEGIN or REPORT |
| Packet Data Header Information | CONTINUE or optionally END |

A set of information is used to generate the records. The records used transmit the information from mediation function to LEMF. This set of information can be extended in the ICE or DF2 MF, if this is necessary in a specific country. The following table gives the mapping between information received per event and information sent in records.

For the event “Start of intercept with I-WLAN Communication Active” reported from a AAA server, this event is reported using a:

- REPORT record to provide an indication that I-WLAN Access Initiation event has already occurred, but there are no tunnels established yet.

- BEGIN record to provide an indication that one or more I-WLAN Tunnels are already established.

Table 8.2: Mapping between Events information and IRI information

| parameter | description | HI2 ASN.1 parameter |
| --- | --- | --- |
| observed MSISDN | Target Identifier with the MSISDN of the target subscriber (monitored subscriber). | partyInformation (partyIdentiity) |
| observed IMSI | Target Identifier with the IMSI of the target subscriber (monitored subscriber). | partyInformation (partyIdentity) |
| observed NAI | Target Identifier with the NAI of the target subscriber (monitored subscriber). | partyInformation (partyIdentity) |
| event type | Description which type of event is delivered: I-WLAN Access Initiation, I-WLAN Access Termination, I-WLAN Tunnel Establishment, I-WLAN Tunnel Disconnect, Start of Intercept with I-WLAN Communication Active, Packet Data Header Information, etc. | i-WLANevent |
| event date | Date of the event generation in the PDG or AAA server. | timestamp |
| event time | Time of the event generation in the PDG or AAA server. |  |
| WLAN access point name | The WLAN Access Point Name contains a logical name of the access point (see 3GPP TS 23.060 [---TBD---]) | partyInformation (services-Data-Information) |
| initiator | This field indicates whether the event being reported is the result of an MS directed action or network initiated action when either one can initiate the action. | initiator |
| correlation number | Unique number for each I-WLAN tunnel delivered to the LEMF, to help the LEA, to have a correlation between each I-WLAN tunnel and the IRI. | correlationNumber |
| lawful interception identifier | Unique number for each lawful authorization. | lawfulInterceptionIdentifier |
| WLAN UE Local IP address | The Local IP address used by the target in a WLAN AN. | partyInformation (services-data-information) |
| WLAN UE MAC address | MAC Address of WLAN UE on the WLAN | i-WLANInformation (wLANMACAddress) |
| WLAN Remote IP address | It is the IP address of the WLAN UE in the network being accessed by the WLAN UE and is used in the data packet encapsulated by the WLAN UE-initiated tunnel. In addition, it is the source address used by applications in the WLAN UE. | partyInformation (services-data-information) |
| network identifier | Operator ID plus PDG or AAA server address. | networkIdentifier |
| WLAN Operator name | This field identifies the WLAN Operator serving the intercept subject. | i-WLANInformation (wLANOperatorName) |
| WLAN Location Name | This field identifies the name of the location of the WLAN serving the subject. | i-WLANInformation (wLANLocationName) |
| WLAN Location Information | This field provides detailed location information about the WLAN serving the intercept subject. | i-WLANInformation (wLANLocationInformation) |
| NAS IP/IPv6 address | An IP address of the serving Network Access Server. | i-WLANInformation (nasIPIPv6Address) |
| visited PLMN ID | This field identifies the visited PLMN that will either terminate or tunnel the intercept subject's communications to the Home PLMN. | visitedPLMNID |
| session alive timer | Thi field identifies the expected maximum duraton of the I-WLAN access being initiated. | i-WLANInformation (sessionAliveTimer) |
| failed access reason | This field gives information about the reason for a failed access initiation attempt of the target subscriber. | i-WLANOperationErrorCode |
| session termination reason | This field identifies the reason for the termination of the I-WLAN access. | i-WLANOperationErrorCode |
| failed tunnel establishment reason | This field gives information ("Authentication failed" or Authorization failed") about the reason for a failed tunnel establishment of the target subscriber. | i-WLANOperationErrorCode |
| tunnel disconnect reason | This field gives information about the reason for tunnel disconnect of the target subscriber. (For Further Study). | i-WLANOperationErrorCode |
| NSAPI | Network layer Service Access Point Identifier. Information element contains an NSAPI identifying a PDP Context in a mobility management context specified by the Tunnel Endpoint Identifier Control Plane. This is an optional parameter to help DF/MF and LEA's to distinguish between the sending mobile access networks when the GGSN is used as element of the PDG according TS 23.234 [43]. | nSAPI |
| destination IP address | Identifies the destination IP address of a packet. | destinationIPAddress |
| destination port number | Identifies the destination port number of a packet | destinationPortNumber |
| source IP address | Identifies the source IP address of a packet. | sourceIPAddress |
| source port number | Identifies the source port number of a packet. | sourcePortNumber |
| transport protocol | Identifies the transport protocol (i.e., Protocol Field in IPv4 or Next Header Field in IPv6. | transportProtocol |
| flow label | The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [c]) | flowLabel |
| packet count | The number of packets detected and reported in a particular packet data summary report. | packetCount |
| packet size | The size of a packet (i.e., Total Length Field in IPv4 [a] or Payload Length Field in IPv6 [b]) | packetsize |
| packet direction | Identifies the direction of the intercepted packet (from subject or to subject) | packetDirection |
| packet header copy | Provides a copy of the packet headers including IP layer and next layer, and extensions, but excluding content. | packetHeaderCopy |
| summary period | Provides the period of time during which the packets of the summary report were sent or received by the subject. | summaryPeriod |
| sum of packet sizes | Sum of values in Total Length Fields in IPv4 packets or Payload Length Field in IPv6 packets. | sumOfPacketSizes |
| packet data summary reason | Provides the reason for a summary report. | packetDataSummaryReason |
| packet data summary | For each particular packet flow, identifies pertinent reporting information (e.g., source IP address, destination IP address, source port, destination port, transport protocol, packet count, time interval, sum of packet sizes) associated with the particular packet flow. | packetDataSummary |

NOTE:	LIID parameter must be present in each record sent to the LEMF.

*** NEXT MODIFICATION ***

#### 8.5.1.4	END record information

The END record is used to convey the last event of packet-data communication.

The END record shall be triggered when:

-	I-WLAN tunnel disconnect occurs (triggered by the AAA server or the PDG).

Table 8.12: I-WLAN Tunnel Disconnect END Record - PDG

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed NAI |  |  |
| event type | C | Provide I-WLAN Tunnel Disconnect event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| WLAN access point name | C | Provide to identify the packet data network to which the intercept subject is connected. |
| initiator | C | Provide to indicate whether the I-WLAN tunnel disconnection is network-initiated, intercept-subject-initiated, or not available. |
| network identifier | M | Shall be provided. |
| WLAN local IP address | M | Provide to identify the IP address associated with the intercept subject in the WLAN. |
| WLAN remote IP address | M | Provide to identify the IP address associated with the intercept subject in the network being accessed by the intercept subject for the I-WLAN tunnel. |
| correlation number | C | Provide to allow correlation of CC and IRI and the correlation of IRI records. |
| lawful intercept identifier | M | Shall be provided. |
| packet data header information | C | Provide to report the final packet header summary report(s) for one or more packet flow summaries associated with I-WLAN interworking communications. The content of a single packet data summary parameter is defined in Table 8.c. May be excluded if the information is reported via the Packet Data Header Information Continue record. |
| NSAPI | O | Provided for additional information. |

Table 8.13: I-WLAN Tunnel Disconnect END Record – AAA Server

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed NAI |  |  |
| event type | C | Provide I-WLAN Tunnel Disconnect event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| WLAN access point name | C | Provide to identify the packet data network to which the intercept subject is connected. |
| initiator | C | Provide to indicate whether the I-WLAN tunnel disconnection is network-initiated, intercept-subject-initiated, or not available. |
| network identifier | M | Shall be provided. |
| correlation number | C | Provide to allow correlation of IRI records. |
| lawful intercept identifier | M | Shall be provided. |

*** NEXT MODIFICATION ***

#### 8.5.1.x	CONTINUE record information

The CONTINUE record is used to convey events related to I-WLAN interworking communications.

The CONTINUE record shall be triggered when:

-	packet data header reporting is performed on an individual intercepted packet basis and a packet is detected as it is sent or received by the target for I-WLAN interworking communications;

-	when packet data summary reporting is performed on an summary basis for I-WLAN interworking communications.associated with particular packet flow (defined as the combination of source IP address, destination IP address, source port, destination port, and protocol and for IPv6 also include the flow label) and:

-	the packet flow starts,

-	an interim packet summary report is to be provided, or

-	packet flow ends including the case where the I-WLAN interworking tunnel is deactivated.

An interim packet summary report is triggered if:

-	the expiration of a configurable Summary Timer per intercept occurs.  The Summary Timer is configurable in units of seconds.

-	a per-intercept configurable count threshold is reached, or

-	a change in information being counted (e.g., change of IP Address associated with packet flow being counted).

Packet Header Information Reporting is reported either on a per-packet (i.e., non-summarised) basis or in a summary report.  These reports provide IRI associated with the packets detected and are used when the reporting of content is not authorised. The packet header information related CONTINUE record is used to convey packet header information during active I-WLAN interworking communications.

In order to enable the LEMF to correlate the information on HI3, a new correlation number shall not be generated within

Table 8.a: Packet Data Header CONTINUE Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| event type |  | Provide the Continue interception with Packet Data Header event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| WLAN access point name | M | Provide to identify the packet data network to which the intercept subject is connected. |
| WLAN local IP address | M | Provide to identify the IP address associated with the intercept subject in the WLAN. |
| WLAN remote IP address | M | Provide to identify the IP address associated with the intercept subject in the network being accessed by the intercept subject for the I-WLAN tunnel. |
| network identifier | M | Shall be provided. |
| correlation number | M | Provide to uniquely identify the I-WLAN interworking communications.delivered to the LEMF used to correlate IRI records with CC. |
| lawful intercept identifier | M | Shall be provided. |
| packet data header information | M | Shall be provided to identify the packet header information to be reported on a per-packet basis as defined in Table 8.b or on a summary basis. For summary reporting includes one or more packet flow summaries where each packet flow summary is associated with a particular packet flow as defined in Table 8.c. |
| NSAPI | O | Provided for additional information. |

Table 8.b: Contents of per-packet, packet data header information parameter

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| source IP address | C | Provide when mapping packet header information to identify the source IP address for a particular packet flow. |
| source port number | C | Provide when mapping packet header information to report the source port number for a particular packet flow when the transport protocol supports port numbers. |
| destination IP address | C | Provide when mapping packet header information to Identify the destination IP address for a particular packet flow. |
| destination port number | C | Provide when mapping packet header information to report the destination port number for a particular packet flow when the transport protocol supports port numbers. |
| transport protocol | C | Provide when mapping packet header information to identify the transport protocol (e.g., TCP) for a particular packet flow. |
| flow label | C | Provide when mapping packet header information for IPv6 only for a particular packet flow. |
| direction | M | Shall be provided. Identifies the direction of the packet (from subject or to subject). |
| packet size | C | Provide when mapping packet header information to convey the value contained in Total Length Fields of the IPv4 packets or the value contained in the Payload Length fields of the IPv6 packets. |
| packet data header copy | C | Provide when reporting a copy of the entire packet header information rather than mapping individual information. |

Table 8.c: Contents of a single summary flow packet data header information parameter

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| source IP address | M | Shall be provided. Identifies the source IP address for a particular packet flow. |
| source port number | C | Provide to report the source port number for a particular packet flow when the transport protocol supports port numbers. |
| destination IP address | M | Shall be provided. Identifies the destination IP address for a particular packet flow. |
| destination port number | C | Provide to report the destination port number for a particular packet flow when the transport protocol supports port numbers. |
| transport protocol | M | Identifies the transport protocol (e.g., TCP) for a particular packet flow. |
| flow label | C | Provide for IPv6 only for a particular packet flow. |
| summary period | M | Provides the period of time during which the packets of a particular packet flow of the summary report were sent or received by the subject and defined by specifying the time when the first packet and the last packet of the reporting period were detected. |
| packet count | M | Provides the number of packets detected for a particular packet flow. |
| sum of packet sizes | M | Provides the sum of values contained in Total Length Fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets. |
| packet data summary reason | M | Provides the reason for the report being delivered to the LEMF (i.e., timeout, count limit, end of session). |

*** NEXT MODIFICATION ***

# B.7	Intercept related information (and I-WLAN)

Declaration of ROSE operation iwlan-umts-sending-of-IRI is ROSE delivery mechanism specific. When using FTP delivery mechanism, data IWLANUmtsIRIsContent must be considered.

# ASN1 description of IRI (HI2 interface)

IWLANUmtsHI2Operations {itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulintercept(2) threeGPP(4) hi2wlan(6) r12(12) version-1 (1)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t(2) remote-operations(4) informationObjects(5) version1(0)}

LawfulInterceptionIdentifier,

TimeStamp,

Network-Identifier,

National-Parameters,

National-HI2-ASN1parameters,

DataNodeAddress,

IPAddress

FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulIntercept(2) hi2(1) version10 (10)}; -- Imported from TS 101 671

-- Object Identifier Definitions

-- Security DomainId

lawfulInterceptDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0)

securityDomain(2) lawfulIntercept(2)}

-- Security Subdomains

threeGPPSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP(4)}

hi2wlanDomainId OBJECT IDENTIFIER	::= {threeGPPSUBDomainId hi2wlan(6) r12(12) version-1(1)}

iwlan-umts-sending-of-IRI  OPERATION ::=

{

ARGUMENT	IWLANUmtsIRIsContent

ERRORS		{ OperationErrors }

CODE		global:{threeGPPSUBDomainId hi2wlan(6) opcode(1)}

}

-- Class 2 operation . The timer shall be set to a value between 3 s and 240 s.

-- The timer.default value is 60s.

-- NOTE:	The same note as for HI management operation applies.

IWLANUmtsIRIsContent		::= CHOICE

{

iWLANumtsiRIContent			IWLANUmtsIRIContent,

iWLANumtsIRISequence		IWLANUmtsIRISequence

}

IWLANUmtsIRISequence		::= SEQUENCE OF IWLANUmtsIRIContent

-- Aggregation of IWLANUmtsIRIContent is an optional feature.

-- It may be applied in cases when at a given point in time

-- several IRI records are available for delivery to the same LEA destination.

-- As a general rule, records created at any event shall be sent

-- immediately and not withheld in the DF or MF in order to

-- apply aggragation.

-- When aggregation is not to be applied,

-- IWLANUmtsIRIContent needs to be chosen.

IWLANUmtsIRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,

iRI-End-record			[2] IRI-Parameters,

iRI-Report-record		[3] IRI-Parameters,

…

}

unknown-version 		ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter-value	ERROR ::= { CODE local:2}

unknown-parameter		ERROR ::= { CODE local:3}

OperationErrors ERROR ::=

{

unknown-version |

missing-parameter |

unknown-parameter-value |

unknown-parameter

}

-- These values may be sent by the LEMF, when an operation or a parameter is misunderstood.

IRI-Parameters		::= SEQUENCE

{

hi2iwlanDomainId				[0]	OBJECT IDENTIFIER,  -- 3GPP HI2 WLAN domain

lawfulInterceptionIdentifier	[2] LawfulInterceptionIdentifier,

-- This identifier is associated to the target.

timeStamp				[3] TimeStamp,

-- date and time of the event triggering the report.

initiator 				[4] ENUMERATED

{

not-Available		(0),

originating-Target	(1),

-- in case of I-WLAN, this indicates that the I-WLAN tunnel disconnect is WLAN UE

-- requested.

terminating-Target	(2),

-- in case of I-WLAN, this indicates that the I-WLAN tunnel disconnect is network

-- initiated.

...

} OPTIONAL,

partyInformation 		[5] SET SIZE (1..10) OF PartyInformation OPTIONAL,

-- This parameter provides the concerned party, the identiy(ies) of the party

-- and all the information provided by the party.

national-Parameters		[6] National-Parameters OPTIONAL,

networkIdentifier		[7] Network-Identifier OPTIONAL,

i-WLANevent				[8] I-WLANEvent	OPTIONAL,

correlationNumber		[9] CorrelationNumber OPTIONAL,

i-WLANOperationErrorCode[10] I-WLANOperationErrorCode 	OPTIONAL,

i-wLANinformation		[11] I-WLANinformation OPTIONAL,

visitedPLMNID			[12] VisitedPLMNID 	OPTIONAL,

national-HI2-ASN1parameters	[255]	National-HI2-ASN1parameters	OPTIONAL,

...,

nSAPI					[13] OCTET STRING (SIZE (1)) OPTIONAL,

-- Include either Octet 2 of the NSAPI IE of 3GPP TS 24.008 [9]

-- or Octet 2 of the NSAPI IE of 3GPP TS 29.060 [17].

packetDataHeaderInformation		[xx] PacketDataHeaderInformation	OPTIONAL

}

-- PARAMETERS FORMATS

PartyInformation 			::= SEQUENCE

{

party-Qualifier 	[0]  ENUMERATED

{

iWLAN-Target(1),

...

},

partyIdentity 		[1] SEQUENCE

{

imsi					[2] OCTET STRING (SIZE (3..8)) OPTIONAL,

-- See MAP format [4] International Mobile

-- Station Identity E.212 number beginning with Mobile Country Code

msISDN					[3] OCTET STRING (SIZE (1..9)) OPTIONAL,

-- MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document TS 29.002 [4]

nai						[7]  OCTET STRING  OPTIONAL,

-- NAI of the target, encoded in the same format as

-- defined in 3GPP TS 29.234 [41].

...

},

services-Data-Information 	[2] Services-Data-Information OPTIONAL,

-- This parameter is used to transmit all the information concerning the

-- complementary information associated to the basic data call

...

}

CorrelationNumber ::= OCTET STRING (SIZE(8..20))

I-WLANEvent ::= ENUMERATED

{

i-WLANAccessInitiation 					(1),

i-WLANAccessTermination					(2),

i-WLANTunnelEstablishment				(3),

i-WLANTunnelDisconnect					(4),

startOfInterceptionCommunicationActive	(5),

...

packetDataHeaderInformation				(xx)

}

-- see [19]

Services-Data-Information ::= SEQUENCE

{

i-WLAN-parameters [1] I-WLAN-parameters OPTIONAL,

...

}

I-WLAN-parameters ::= SEQUENCE

{

wlan-local-IP-address-of-the-target 	[1] DataNodeAddress OPTIONAL,

w-APN 									[2] OCTET STRING (SIZE(1..100))	OPTIONAL,

-- The Access Point Name (APN) is coded in accordance with

-- 3GPP TS 24.008 [9] without the APN IEI (only the last 100 octets are used).

-- Octets are coded according to 3GPP TS 23.003 [25].

wlan-remote-IP-address-of-the-target 	[3] DataNodeAddress 	OPTIONAL,

...

}

I-WLANOperationErrorCode ::= OCTET STRING

-- The parameter shall carry the I-WLAN failed tunnel establishment reason, the I-WLAN Failed Access -- Initiation reason or the I-WLAN session termination reason.

I-WLANinformation ::= SEQUENCE

{

wLANOperatorName 					[1] OCTET STRING 		OPTIONAL,

wLANLocationName 					[2] OCTET STRING		OPTIONAL,

wLANLocationInformation 			[3] OCTET STRING		OPTIONAL,

nASIPIPv6Address					[4]	IPAddress			OPTIONAL,

wLANMACAddress						[5]	OCTET STRING		OPTIONAL,

sessionAliveTimer					[6]	SessionAliveTime	OPTIONAL,

...

--These parameters are defined in 3GPP TS 29.234.

}

VisitedPLMNID ::= OCTET STRING

-- The parameter shall carry the VisitedPLMNID as defined in 3GPP TS 29.234.

SessionAliveTime ::= OCTET STRING

--The parameter shall carry the SessionAliveTime as defined in 3GPP TS 29.234.

PacketDataHeaderInformation ::= CHOICE

{

packetDataHeader		[1]	PacketDataHeader,

packetDataHeaderSummary	[2]	PacketDataHeaderSummary,

...

}

PacketDataHeader ::= CHOICE

{

packetDataHeaderMapped	[1]	PacketDataHeaderMapped,

packetDataHeaderCopy	[2]	PacketDataHeaderCopy,

...

}

PacketDataHeaderMapped ::= SEQUENCE

{

sourceIPAddress			[1] IPAddress	OPTIONAL,

sourcePortNumber		[2] INTEGER (0..65535) OPTIONAL,

destinationIPAddress	[3] IPAddress	OPTIONAL,

destinationPortNumber	[4] INTEGER (0..65535) OPTIONAL,

transportProtocol		[5] INTEGER  OPTIONAL,

-- For IPv4, report the “Protocol” field and for IPv6 report “Next Header” field.

-- Assigned Internet Protocol Numbers can be found at

-- http://www.iana.org/assignments/protocol-numbers/protocol-numbers.xml

packetsize				[6] INTEGER OPTIONAL,

flowLabel				[7] INTEGER OPTIONAL,

packetCount				[8] INTEGER OPTIONAL,

direction				[9] TPDU-direction,

...

}

TPDU-direction ::= ENUMERATED

{

from-target 	(1),

to-target 		(2),

unknown 		(3)

}

PacketDataHeaderCopy ::= SEQUENCE

{

direction				[1] TPDU-direction,

headerCopy				[2] OCTET STRING,	-- includes a copy of the packet header at the IP

-- network layer and above including extension headers, but excluding contents.

...

}

PacketDataSummary ::= SEQUENCE OF PacketFlowSummary

PacketFlowSummary ::= SEQUENCE

{

sourceIPAddress			[1] IPAddress,

sourcePortNumber		[2] INTEGER (0..65535) OPTIONAL,

destinationIPAddress	[3] IPAddress,

destinationPortNumber	[4] INTEGER (0..65535) OPTIONAL,

transportProtocol		[5] INTEGER,

-- For IPv4, report the “Protocol” field and for IPv6 report “Next Header” field.

-- Assigned Internet Protocol Numbers can be found at

-- http://www.iana.org/assignments/protocol-numbers/protocol-numbers.xml

flowLabel				[6] INTEGER OPTIONAL,

summaryPeriod			[7] ReportInteval,

packetCount				[8] INTEGER,

sumOfPacketSizes		[9] INTEGER,

packetDataSummaryReason	[10] ReportReason,

...

}

ReportReason ::= ENUMERATED

{

timerExpired			[0],

countThresholdHit		[1],

pDPComtextDeactivated   [2],

pDPContextModification	[3],

other or unknown		[4],

...

}

ReportInterval ::= SEQUENCE

{

firstPacketTimeStamp	[0] TimeStamp,

lastPacketTimeStamp		[1] TimeStamp,

...

}

END -- OF IWLANUmtsHI2Operations

*** END MODIFICATIONS ***