3GPP TSG-SA3LI Meeting #51 	SA3LI13_135r1

Burlington, Vermont, USA, 22-24 October 2013

| CR-Form-v11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 33.107 |  | CR |  |  | CRNum |  | rev |  |  |  |  | Current version: |  |  |  |  |  |  |  | 12.4.0 |  |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  | UICC apps |  |  |  | ME |  |  |  |  | Radio Access Network |  |  |  |  |  | Core Network |  | X |  |  |  |  |  |
| Title: |  | Editorial Fix of implementation of SA3LI13_073r3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: |  | SA3-LI (Nokia Solutions and Networks) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  | S3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  | LI12 |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  | 2013-10-23 |  |  |  |  |  |  |  |  |
| Category: |  | D |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  | Rel-12 |  |  |  |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |  |  |  |  |  |  |  |  |
| Reason for change: |  |  | SA3LI13_073r3 was approved at SA3LI#50 and the text was inserted into 3GPP TS 33.107 at a wrong place, after the informative Annexes. The text should be moved upfront in thr 33.107 before the Annexes. This is a CR implementation error. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  | Move the text from Annex E to the main section of the document, before the Annexes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  | If LI capabilities of GBA are placed along with the informative Annexes, it is quite possible that consumers (i.e., the readers) may miss reading the requirements or categorize them in the same way as the other text in the informative annex. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  | E, Y (new clause) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Y | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

*** START of FIRST CHANGE ****

***** END of FIRST CHANGE *****

***** START of SECOND CHANGE **** NEW CLAUSE *** Before the Annexes *****

# Y	Interception of Generic Bootstrapping Architecture (GBA) Secured Communications

## Y.1	Introduction

The Generic Bootstrapping Architecure (GBA) is defined in the 3GPP TS 33.220 [35]. This section details the stage 2 Lawful Interception architecture and functions that are needed to provide the GBA based application specific encryption keys from the GBA architecture towards the DF2 for a subscriber that is target of interception.

Figure Y.1 shows the LI architecture for the GBA where the BSF provides the events and associated information towards the DF2 over the X2 interface.

Figure Y.1: GBA Intercept Configuration

## Y.2	Provision of Content of Communications

The GBA interception provides the application specific cryptographic keys (aka GBA application specific keys) which are used to decrypt the intercepted communication secured using those GBA application specific keys. Interception of the content of communications for GBA secured services is not part of this section and can be achieved via other methods outlined in this specification. The Ua protocol Id and the NAF Id along with the GBA application specific keys will allow the LEMF to decrypt the received intercepted packets.

Note 1: The details of LI capabilities for GBA in a roaming scenario is for further study.

Note 2: The delivery by the CSP of intercepted packets in a decrypted form is for further study.

## Y.3	Provision of Intercept Related Information

### Y.3.1	Provision of Intercept Related Information Data Flow

Figure Y.2 shows the transfer of intercept related information to the DF2. If an event related to a target subscriber occurs, the BSF shall send the relevant data to the DF2.

Figure Y.2: Provision of Intercept Related Information

### Y.3.2	X2-interface

The following information needs to be transferred from the BSF to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity;

-	events and associated parameters as defined in clauses Y.3.3 may be provided;

The IRI should be sent to DF2 using a reliable transport mechanism.

### Y.3.3	GBA LI Events and Event Information

Intercept Related Information (Events) are necessary for the following;

- Bootstrapping

- Query from NAF

- Start of interception with GBA key

A set of possible elements as shown in Table Y.3.1 are used to generate the events.

Table Y.3.1: Information Events for GBA Event Records

Element

Observed IMSI IMSI of the target subscriber (monitored subscriber).

Observed Other Identity Other Identity of the target subscriber (monitored subscriber).

Event type Description which type of event is delivered: Bootstrapping, Query from NAF, Start of interception with GBA key

Event date Date of the event generation in the BSF

Event time Time of the event generation in the BSF.

Network Element Identifier Unique identifier for the element reporting the BSF.

B-TID Bootstrapping transaction identifier, TS 33.220 [xy].

Key lifetime The lifetime of the key material is set according to the local policy of the BSF, TS 33.220 [xy].

Bootstrapping time The timestamp of the bootstrapping event.

Ks_int_NAF GBA application specific key (internal), if GBA_U has been used, TS 33.220 [xy].

Ks_ext_NAF GBA application specific key (external), if GBA_U has been used, TS 33.220 [xy].

Ks_NAF GBA application specific key, if GBA_ME has been used, TS 33.220 [xy].

Ua protocol id Ua interface security protocol id defined in clause Annex H in TS 33.220 [xy].

NAF_Id The FQDN of the NAF, concatenated with the Ua security protocol identifier, TS 33.220 [xy].

## Y.4	Structure of GBA Events

### Y.4.1	Bootstrapping

This event will be generated when the UE triggers a bootstrapping procedure towards the BSF when the UE wants to interact with a NAF. The actual bootstrapping procedure is defined in the TS 33.220 [35], in sections 4.5.2 and in 5.3.2. The information elements shown in Table Y.4.1 table, if available, will be delivered to the DF2, by the BSF.

Table Y.4.1: Bootstrapping

Observed IMSI

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

B-TID

Key lifetime

Bootstrapping time

### Y.4.2	Query from NAF

The Query from NAF event is generated when the BSF receives an application specific key query from a NAF in order to retrieve GBA based application specific keys and related information. A new event is generated for each individual query events. The information elements shown in Table Y.4.2 will be delivered to the DF2, if available, by the BSF.

Table Y.4.2: Query from NAF

Observed IMSI

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

Ks_ext_NAF

NAF_Id

Ks_int_NAF

Ks_NAF

Key lifetime

Bootstrapping time

Ua protocol id

### Y.4.3	Start of Interception with GBA key

For start of interception where GBA application specific key is already in use a Start of Interception with GBA key event is generated. The elements, shown in Table Y.4.3 will be delivered to the DF2, if available, by the BSF.

Table Y.4.3: Start of Interception with GBA key

Observed IMSI

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

B-TID [Note]

NAF_Id [Note]

Ks_ext_NAF [Note]

Ks_int_NAF [Note]

Ks_NAF [Note]

Key lifetime [Note]

Bootstrapping time [Note]

Ua protocol id [Note]

Note: These are repeated for each GBA application specific key associated with the target subscriber.

***** END of SECOND CHANGE ****