3GPP TSG-SA3-LI Meeting #54	SA3LI14_108

# Amsterdam, Netherlands, 22-24 July 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | CRNum | rev | - | Current version: | 12.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interception of Group Communications at the BM-SC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI12 |  |  |  |  |  |  | Date: |  | 2014-07-22 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Clarify 16.3 to describe how the BM-SC can perform interception of group communications when a target is received downstream multicast communications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify Clause 16.3 to add in procedures for interception at the BM-SC to intercept based on TMGI. The mechanism for identifying the association of the LE provided target identity to the TMGI to be intercepted is not specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory implications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

## 16.3	GCSE AS outside Operator Network

## 16.3.1	Overview

When the GCSE AS is outside the operator’s network, the interception will need to occur at the PDN-GW and the BM-SC used for downlink communications in multicast mode. The uplink communications will go through the PDN-GW towards the GCSE AS that is external to the network.  The downlink communications may be sent either over a unicast bearer to the UE via the PDN-GW or over a multicast bearer via the BM-SC.  Thus, the PDN-GW and the BM-SC will need to be able to identify a target’s communication and perform the interception.

Interception at the PDN-GW shall follow the requirements in Clause 12. Additional requirements pertain to interception at the BM-SC and are specified herein. Any requirements pertaining to decryption aspects are for further study.

The general model for interception at the BM-SC is specified in Clause 10.  However, the applicability of the concept of subscription joining, leaving and modification is not clear in the GCSE context.  Thus, new events are defined in this clause for interception of GCSE communications at an BM-SC.

Figure 16.3 shows the extract from the reference configuration which is relevant for the invocation of Lawful Interception of GCSE communications at the BM-SC.

Delivery

Function 2

BM-SC

X2

X3

Delivery

Function 3

HI2

HI3

LEAs

Figure 16.3: Functional model for invocation of Lawful Interception for GCSE at an BM-SC

## 16.3.2	Provision of Content of Communications

### 16.3.2.1	CC Interception at the BM-SC

Figure 16.4 depicts the interception of GCSE content of communications at a BM-SC.  Interception at a BM-SC is based on a TMGI that is associated with the target of interception who is a member of a multicast group identified by a TMGI.  How the BM-SC may be provisioned with the TMGI(s) to intercept on or may be provided with the TMGI(s) dynamically.

intercepted

subscriber

Delivery

Function 3

LEMF

BM-SC

Figure 16.4: Provision of Intercept Product from an MBMS Server for GCSE

### 16.3.2.2	X3-interface

In addition to the intercepted content of communications, the following information may need to be transferred from the BM-SC to the DF3 in order to allow the DF3 to perform its functionality:

-	identity used for interception include TMGI and others if known (e.g., IMSI, IMEI, MSISDN);

-	correlation number;

-	the identity of source (i.e., group communications party identity) of a media stream, if known;

-	time stamp;

## 16.3.3	Provision of Intercept Related Information

### 16.3.3.1	IRI Interception at the BM-SC

Figure 16.5 shows the transfer of intercept related information to the DF2. If an event for / from a GCSE user occurs, the BM-SC shall send the relevant data to the DF2.

intercepted

subscriber

Delivery

Function 2

LEMF

BM-SC

Figure 16.5: Provision of Intercept Related Information

### 16.3.3.2	X2-interface

The following information needs to be transferred from the BM-SC Server to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity include the TMGI and others (e.g., IMSI, IMEI, MSIDN) if known;

-	events and associated parameters as defined in clauses 16.3.3.2 may be provided;

-	For Further Study:- Encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

The IRI should be sent to DF2 using a reliable transport mechanism.

### 16.3.3.3	GCSE MBMS LI Events and Event Information

#### 16.3.3.3.1	Event Information overview

Intercept Related Information events to be reported for GCSE include:

- Start of Interception with an Active Multicast Group identified by a TMGI

- When a communication associated with a target TMGI is detected

- When a TMGI associated with a multicast group involving a target of interception is released.

Events shall include changes resulting from direct communication between the GCSE AS and the BM-SC, between the target UE and the BM-SC and BM-SC and off-line subscription changes (e.g., changes made by the operator on behalf of the GCSE AS).

A set of possible elements as shown in Table 16.12 are used to generate the events.

Table 16.12: Information Events for GCSE MBMS Event Records

Element

Observed IMSI IMSI of the target.

Observed IMEI IMEI of the target

Observed TMGI TMGI of the target

Observed Other Identity Other Identity of the target

Event type Description which type of event is delivered: Start of Intercept at BM-SC with Active GCSE Multicast Group, GCSE Multicast Communication Detected at BM-SC, GCSE Multicast Communication Detected at BM-SC .

Event date Date of the event generation in the BM-SC.

Event time Time of the event generation in the BM-SC. Timestamp shall be generated relative to the BM-SC internal clock.

Correlation Number The correlation number is used to correlate CC and IRI. The correlation number is also used to allow the correlation of IRI records.

Network Element Identifier Unique identifier for the element reporting the ICE.

Identity of GCSE AS Identity of the GCSE AS which required resources from the MBMS Server for GCSE.

Length of TMGI Reservation Identifies the length of time for which a TMGI is reserved by the MBMS Server at the request of a GCSE AS.

NOTE: Generation of Correlation Number is FFS.

#### 16.3.3.3.2	Start of Intercept with an Active Multicast Group at BM-SC for GCSE

When an intercept is started with an active GCSE multicast group as identified by a TMGI, a Start of Intercept for Multicast Group event is generated in the following cases:

-	When interception is activated for a target of interception who is already a member of an active GCSE multicast group.

The fields, shown in Table 16.13, will be delivered to the DF2, if available, by the BM-SC.

Table 16.13 Start of Intercept with Active MBMS Multicast Group

Observed IMSI

Observed IMEI

Observed TMGI

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

Observed Multicast Group ID

Length of TMGI reservation time (if known)

Identity of GCSE AS

#### 16.3.3.3.3	GCSE Multicast Communication Detected at BM-SC

When a GCSE multicast communication is received by the BM-SC from a GCS AS for a target TMGI, a GCSE Multicast Communication Detected event is generated in the following cases:

-	When a multicast communication request is received by the BM-SC from the GCS AS for a target TMGI.

The fields, shown in Table 16.14, will be delivered to the DF2, if available, by the BM-SC.

Table 16.14 GCSE Multicast Communication Detected at BM-SC

Observed IMSI

Observed IMEI

Observed TMGI

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

TMGI

Length of TMGI reservation time (if known)

Identity of GCSE AS

#### 16.3.3.3.4	Release of GCSE Multicast Group at BM-SC

When a multicast group is released at the BM-SC for GCSE, a Release of GCSE Multicast Group event is generated in the following cases:

-	When the BM-SC successfully releases a multicast group for GCSE in which a member is a target of interception.

The fields, shown in Table 16.15, will be delivered to the DF2, if available, by the BM-SC.

Table 16.15 Release of GCSE Multicast Group

Observed IMSI

Observed IMEI

Observed TMGI

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

TMGI

Identity of GCSE AS

*** END OF MODIFICATIONS ***