3GPP TSG-SA3-LI Meeting #56	SA3LI15_024

# Sophia Antipolis, France, 20-22 January 2015

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | CRNum | rev | - | Current version: | 12.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | WebRTC Interworking LI Stage 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI12 |  |  |  |  |  |  | Date: |  | 2015-01-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | The current specification for 33.107 does not address the LI Stage 2 X2 & X3 changes needed to support WebRTC Interworking. This needs to be addressed to meet Regulatory requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution incorporates changes to the LI Stage 2 X2 & X3 to support WebRTC Interworking. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory implications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7A.2.1, 15.2, E.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

### 7A.2.1	Provision of IRI with SIP messaging

SIP messaging is reported as Intercept Related Information for the interception of multi-media service. As shown in figure 22 below, all SIP messages executed on behalf of a target are subject to intercept at the S-CSCF and Optionally P-CSCF. Based upon network configuration, the ADMF shall provision P-CSCFs, or S-CSCFs, or both P-CSCFs and S-CSCFs with SIP URI, TEL URI, or IMEI target identifiers. These resulting intercepted SIP messages shall be sent to DF2 for mediation prior to transmittal across the HI2 interface.  For the support of WebRTC Interworking as specified in [43], the P-CSCF shall be interpreted as the eP-CSCF when handling WebRTC Interworking.  That is, all LI requirements pertaining to the P-CSCF shall apply to the eP-CSCF and wherever a P-CSCF appears in this specification, it shall be considered to be the eP-CSCF for the WebRTC Interworking cases. Any additional requirements that only apply to the eP-CSCF are described distinctly.

For roaming scenarios, interception at the P-CSCF shall be Mandatory, in order to provide IRI Interception in the visited network, where the P-CSCF is located in the Visited Network. Where the P-CSCF is located in the Home Network, interception at the P-CSCF shall be Optional, subject to national regulation.

ADMF

P

-

CSCF

S-CSCF

DF2

LEMF

Intercepted

Subscriber

Other
party

Figure 22: Provision of Intercept Related Information for multi-media

*** NEXT MODIFICATION ***

## 15.2	Provision of Content of Communications

As the interception of CC needs to be done at a network node that has access to the voice media, and that interception of CC is required for all targeted calls, including forwarded calls and transferred calls, the CSP needs to support the capability to dynamically trigger CC interception for a call at a network node that has access to the voice media. Depending on the CSP’s network configuration and the call scenario, different network elements will intercept the CC.

The interception and delivery of CC for VoIP may be done at the following functional element:

1)	PDN-GW/GGSN;

2)	IMS-AGW, eIMS-AGW in case of WebRTC Interworking;

3)	TrGW;

4)	IM-MGW;

5)	MRF.

Note 2:  Other functional elements may also be applicable in specific deployment scenarios.

Note 3:  The redirection of target communications to a specific network element purely for LI purposes is undesirable.

The functional elements that provide the signalling to generate the trigger for the CC interception may be any of the following functional elements:

-	P-CSCF (eP-CSCF in case of WebRTC Interworking), for PDN-GW/GGSN and IMS-AGW (eIMS-AGW in case of WebRTC Interworking);

-	IBCF for TrGW;

-	MGCF for IM-MGW

-	S-CSCF or AS for MRF.

At any given time, for a specific target and for any given call, only one functional element is required to provide the CC interception. The functional element that provides the CC interception may vary, primarily, based on the call scenario.

Annex E shows scenarios where the use of the functional elements listed above are applicable.

*** NEXT MODIFICATION ***

# E.2	Background

One of the use-cases of IMS-based VoIP service is VoLTE.  The term "VoLTE" is used to refer to an IMS-based VoIP service when EPS (see 3GPP TS 23.401 [22]) happens to be the access network. When EPS is not the access network, the lawful interception capabilities defined in this informative annex applies for any IMS-based VoIP service with the presumption that in those cases the media on the target’s access goes through an IMS-AGW (see 3GPP TS 29.334 [42]) or a PDN-GW (see 3GPP TS 23.401 [22] and 3GPP TS 402 [23]) or a GGSN (UMTS network).  For the support of WebRTC Interworking as specified in [43], the P-CSCF and IMS-AGW shall be interpreted as the eP-CSCF and the eIMS-AWG, respectively, when handling WebRTC Interworking.  That is, all LI requirements pertaining to the P-CSCF and IMS-AGW shall apply to the eP-CSCF and eIMS-AGW, respectively, and wherever a P-CSCF or IMS-AGW appears in this specification, it shall be considered to be the eP-CSCF for the WebRTC Interworking cases. Any additional requirements that only apply to the eP-CSCF or eIMS-AGW are described distinctly.

Note 1: Even with the EPS-based access network, the media may still go through the IMS-AGW. And, in this case, a VoLTE shall be treated very similar to any other VoIP service.

Furthermore, it is presumed that an inter-CSP call enters or leaves an IMS network via an IBCF/TrGW or an MGCF/IM-MGW depending on whether the inter-working CSP network is an IMS-based network or a CS-based network (see 3GPP TS 23.228 [43]). Also, for an IMS roaming scenario, it is presumed that the signalling and media enters or leaves the home CSP through the IBCF/TrGW.

The figure E.1 illustrates the VoIP configuration considered for the lawful interception capabilities defined in this clause:

Figure E.1: IMS-based VoIP Configuration

Note 2: The configuration scenario where the media does not go through the GGSN, or PDN-GW or the IMS-AGW is outside the scope of this document.

Note 3: In the remaining part of this informative annex, the PDN-GW/GGSN are shown in one box and should read as either the GGSN (UMTS) or the PDN-GW (EPS).

In figure E.1, the term "media node" is used to denote the network node present on media path and the term "signalling node" is used to denote the network node present on the signalling path.

The general concepts of VoIP LI is shown in figure E.2

Figure E.2: General Principles of VoIP Interception

In clause 15, the SIP signalling nodes provide signalling information to the CC Interception Triggering Function.  The CC Interception Triggering Function triggers the CC interception at a media node that implements the CC Intercept Function.

The following functional elements provide the CC Intercept Function in the example call scenarios:

-	PDN-GW/GGSN;

-	IMS-AGW, eIMS-AGW in case of WebRTC Interworking;

-	TrGW;

-	IM-MGW

-	MRF.

The following functional elements provide the signalling to the CC Intercept Triggering Function:

-	P-CSCF (eP-CSCF in case of WebRTC Interworking), for PDN-GW/GGSN and IMS-AGW (eIMS-AGW in case of WebRTC Interworking);

-	IBCF for TrGW;

-	MGCF for IM-MGW

-	S-CSCF for MRF.

At any given time, for a specific target and for any given call, only one functional element is required to provide the CC interception. The functional element that provides the CC interception may vary, primarily, based on the CSP network implementation and the call scenario.

*** END OF MODIFICATIONS ***