3GPP TSG-SA3-LI Meeting #56	S3i150037

# Sophia Antipolis, France, 20-22 January 2015		(Update of S3i150025)

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | CRNum | rev | - | Current version: | 12.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | HI2_HI3 Updates for WebRTC Interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI12 |  |  |  |  |  |  | Date: |  | 2015-01-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | LI for WebRTC Interworking has not been addressed and should be to meet regulatory requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper propses updates to include the eP-CSCF and the eIMS-AGW as reporting elements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory implications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.1, 12.1.4, 12.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

### 12.1.1	Overview

Specific identifiers are necessary to identify a target for interception uniquely and to correlate between the communication information, which is conveyed over the different handover interfaces (HI2 and HI3). The identifiers are defined in the subsections below. For the support of WebRTC Interworking as specified in [40], the P-CSCF shall be interpreted as the eP-CSCF when handling WebRTC Interworking.  That is, all LI requirements pertaining to the P-CSCF shall apply to the eP-CSCF and wherever a P-CSCF appears in this specification, it shall be considered to be the eP-CSCF for the WebRTC Interworking cases. Any additional requirements that only apply to the eP-CSCF are described distinctly.

As indicated in [40], target of WebRTC interworking system may be :

-    individual Public User Identity using IMS authentication, individual Public User Identity based on web authentication

-    individual Public User Identity from a pool of Public User Identities based on web authentication could be used if the interception is activated on NAI used during the authentication.

-    individual Public User Identity from a pool of Public User Identities if the service by the third party is a target itself.

Note:

As U.2.1.4 of TS 23 228 [40] indicates that WWSF may be located in a third party network and have a business arrangement with the IMS operator, this third party network will have its on LI functions according to national regulation. This point and the definition of a target or parties in the annex B9 is FFS.

Some national regulations may prohibit the WWSF or WAF from using the option of not authenticating the user, especially as unauthenticated users are anonymous to the third party but may still be authorized for IMS service.

*** NEXT MODIFICATION ***

### 12.1.4	Correlation Number

For a given target, the Correlation Number is unique per VoIP session and used for the following purposes:

-	Correlate CC with IRI,

-	Correlate different IRI records and different CC data within one VoIP session.

For IMS-based VoIP, the S-CSCF and optionally, the P-CSCF (eP-CSCF in case of WebRTC Interworking) provide the IRI events. For IMS-based VoIP, the functional element that provides the CC interception depends on the call scenario and network configuration.

As described in TS 33.107 [19], CC interception is done by one of the following functional elements (referred to as CC Intercept Function):

-	PDN-GW/GGSN

-	IMS-AGW (eIMS-AGW in case of WebRTC Interworking)

-	TrGW

-	IM-MGW

-	MRF.

And, the trigger to perform the CC interception at the above functional elements may be provided by the following functional elements (referred to as CC Interception Triggering Function):

-	P-CSCF (eP-CSCF in case of WebRTC Interworking) for PDN-GW/GGSN

-	P-CSCF for IMS-AGW (eIMS-AGW in case of WebRTC Interworking)

-	IBCF for TrGW

-	MGCF for IM-MGW

-	S-CSCF or AS for MRF.

For the delivery of CC, the CC Intercept Triggering Function provides the Correlation Number to the CC Intercept Function.   This Correlation Number is delivered to the LEMF on the handover interface HI3 and is also delivered to the LEMF on the handover interface HI2.

The IMS-VoIP-Correlation delivered to the LEMF on the HI2, contains the Correlation Number (s) used for the IRI messages as ims-iri (IRI-to-IRI-Correlation) and Correlation Number (s) used for the CC data as ims-cc (IRI-to-CC-Correlation). The LEMF shall interpret that the IRI messages and the CC data containing those Correlation Number values belong to the one single IMS VoIP session.

*** NEXT MODIFICATION ***

## 12.6	CC for IMS-based VoIP

Annex B.12 provides the definitions of the data structures to be used for the delivery of CC for IMS-based VoIP (see Annex K for the detailed description). The Correlation Number received from the CC Intercept Triggering Function shall be used in the CC Data sent over the HI3.

For PDN-GW based interception of CC for IMS-based VoIP, optionally, the data structures defned in B.10 can be used if the combined delivery option is not required. In the same way, for GGSN based interception of CC for IMS-based VoIP, optionally, the data structures defined in B.10 or B.4 can be used if the combined delivery option is not required.

The Correlation Number received from the P-CSCF (eP-CSCF in case of WebRTC Interworking) shall be used in the CC data sent over the handover interface (HI3).

*** END OF MODIFICATIONS ***