3GPP TSG-SA3-LI Meeting #56	S3i150051

# Sophia Antipolis, France, 20-22 January 2015

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | CRNum | rev | - | Current version: | 12.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | HI2_HI3 Updates for WebRTC Interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI12 |  |  |  |  |  |  | Date: |  | 2015-01-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | LI for WebRTC Interworking has not been addressed and should be to meet regulatory requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper propses updates to include the eP-CSCF and the eIMS-AGW as reporting elements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no LI solution for WebRTC Interworking resulting in Regulatory implications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.0, 12.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Revision of S3i150037 |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	IETF RFC 7118: "The WebSocket Protocol as a Transport for the Session Initiation Protocol (SIP)".

[3]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[4]	3GPP TS 23.228: " IP Multimedia Subsystem (IMS); Stage 2".

[5]	IETF RFC 5763: "Framework for Establishing a Secure Real-time Transport Protocol (SRTP) Security Context Using Datagram Transport Layer Security (DTLS)".

[6]	IETF RFC 5764: "Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP)".

[7]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[8]	3GPP TS 24.173: "IMS multimedia telephony communication service and supplementary services; Stage 3".

[9]	3GPP TS 33.203: "Access security for IP based services".

[10]	RFC 6750 (October 2012): "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[11]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) Centralized Services; Stage 2".

[12]	RFC 5009 (September 2007): "Private Header (P-Header) Extension to the Session Initiation Protocol (SIP) for Authorization of Early Media".

[13]	3GPP TS 23.334: "IMS Application Level Gateway (IMS-ALG) – IMS Access Gateway (IMS-AGW) interface".

[14]	RFC 4145 (September 2005): "TCP-Based Media Transport in the Session Description Protocol (SDP)".

[15]	RFC 4572 (July 2006): "Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP)".

[16]	draft-ietf-rtcweb-data-channel-12 (September 2014): "WebRTC Data Channels".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[17]	draft-ietf-rtcweb-data-protocol-08 (September 2014): "WebRTC Data Channel Establishment Protocol".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[18]	draft-ietf-mmusic-sctp-sdp-07 (July 2014): "Stream Control Transmission Protocol (SCTP)-Based Media Transport in the Session Description Protocol (SDP)".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[19]	RFC 3261 (June 2002): "SIP: Session Initiation Protocol".

[20]	RFC 3264 (June 2002): "An Offer/Answer Model with the Session Description Protocol (SDP)".

[21]	draft-ietf-rtcweb-stun-consent-freshness-08(October 2014): "STUN Usage for Consent Freshness".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[22]	RFC 5245 (April 2010): "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols".

[23]	draft-ietf-tsvwg-sctp-dtls-encaps-05 (July 2014): "DTLS Encapsulation of SCTP Packets".

Editor's note (WI: IMS_WebRTC): The above document cannot be formally referenced until it is published as an RFC.

[24]	RFC 6455 (December 2011): "The WebSocket Protocol".

[25]	draft-ietf-mmusic-sdp-bundle-negotiation-12 (October 2014): "Negotiating Media Multiplexing Using the Session Description Protocol (SDP)".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[26]	RFC 3581 (August 2003): "An Extension to the Session Initiation Protocol (SIP) for Symmetric Response Routing".

[27]	draft-yusef-sipcore-sip-oauth-01 (October 2014): " The Session Initiation Protocol (SIP) OAuth".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[28]	RFC 6544 (March 2012): "TCP Candidates with Interactive Connectivity Establishment (ICE)".

[29]	draft-nandakumar-mmusic-proto-iana-registration-00 (September 2014) " IANA registration of SDP 'proto' attribute for transporting RTP Media over TCP under various RTP profiles".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[30]	draft-ietf-rtcweb-overview-12 (October 2014): "Overview: Real Time Protocols for Brower-based Applications".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[31]	draft-alvestrand-rtcweb-gateways-01 (October 2014): "WebRTC Gateways".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[32]	RFC 3310 (September 2002): "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA)".

[33]	RFC 4169 (November 2005): "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA) Version-2".

[xx]	3GPP TS 24.371: " Web Real-Time Communications (WebRTC) access to the
 IP Multimedia (IM) Core Network (CN) subsystem (IMS); Stage 3".

*** NEXT MODIFICATION ***

## 7.0	Introduction

Clause 7 deals with IRI reporting in the IMS. IRI reporting in the multi-media domain specified in this clause does not depend on the IP-Connectivity Access Network (IP-CAN), defined in TS 23.228 [40], used to transport the CC. When the IP-CAN is the UMTS PS domain, annexes C and G apply for CC interception at the SGSN/GGSN.  However, such CC interception may intercept more than just the CC associated with an IMS based voice service.  Hence, for separated VoIP CC intercept and reporting, refer to clause 12.

According to TS 33.107 [19], interception shall be supported in the S-CSCF and optionally in the P-CSCF where the S-CSCF and the P-CSCF are in the same network. For roaming scenarios where the P-CSCF is in the Visited Network, interception at the P-CSCF is mandatory. The target identities for the intercept of traffic at the CSCFs are only the SIP-URI, TEL-URI and IMEI (described in 3GPP TS 23.003 [25], obtained from the Instance IDs, described also in TS 23.003 [25] as requested in clause7A.8 of TS 33.107 [19]. In the intercepting nodes (CSCF's) the relevant SIP-Messages are duplicated and forwarded to the MF HI2.

For the support of WebRTC Interworking as specified in [40], the eP-CSCF and eIMS-AGW shall adhere to all the LI requirements pertaining to a P-CSCF and IMS-AGW, respectively. Any additional requirements that only apply to the eP-CSCF or eIMS-AGW are described distinctly.

In case of target manipulation of IMS supplementary service setting, the interception shall be made by XCAP servers maintaining XCAP resources related to the supplementary service settings defined in TS 22 173[78] made on the interface Ut as described in TS 24 623 [77]. Any other points related to attempts to access to Target’s XCAP servers or, XCAP change/transaction in services setting related to the target, are for further studies.

Ut based XCAP manipulation messages for the IMS services for the target is reported. Any copy "en clair" of the XCAP exchanges (aggregated or not), between the UE and the AS, will be transmitted to the LEMF in the HI2 interface through the DF 2, that will encapsulate the XCAP Ut transactions in ASN.1. Such XCAP transactions on the Ut interface have to include any exchange of data, which are contained in the XCAP payload (e.g. the get, put, and delete operations on the XCAP resources).

NOTE: 	Interception of the target’s supplementary service setting management or modifications that are made outside the Ut interface is for further studies.

For clarification, see Figure 7.1. If the P-CSCF and S-CSCF are in the same network and LI is provided at both P-CSCF and S-CSCF, the events are sent twice to the LEMF.

Figure 7.1: IRI Interception at a CSCF

*** NEXT MODIFICATION ***

### 12.1.1	Overview

Specific identifiers are necessary to identify a target for interception uniquely and to correlate between the communication information, which is conveyed over the different handover interfaces (HI2 and HI3). The identifiers are defined in the subsections below. For the support of WebRTC Interworking as specified in [40], the eP-CSCF and eIMS-AGW shall adhere to all the LI requirements pertaining to a P-CSCF and IMS-AGW, respectively. Any additional requirements that only apply to the eP-CSCF or eIMS-AGW are described distinctly.

Based on the WebRTC Interworking as described in [40], an individual Public User Identity is used as the target of interception in a WebRTC interworking system.  Some additional considerations are included below.

1)	When a Public User Identity may be temporarily assigned to a WIC from a pool of Public User Identities, an underlying identity for the WIC used during authentication (called a web identity in [xx], e.g., NAI) will need to be correlated to the temporary Public User Identity assigned to the WIC.  This is needed to ensure that target identified in the lawful authorization is associated with the Public User Identity assigned to the user.

2)	When a lawful authorization is targeting an entire pool of Public User Identites, the target should still be each individual Public User Identity associated with the pool of PUIs.

NOTE:  	As U.2.1.4 of TS 23 228 [40] indicates that WWSF may be located in a third party network and have a business arrangement with the IMS operator, this third party network will have its on LI functions according to national regulation. This point and the definition of a target or parties in the annex B9 is FFS.  Also, some national regulations may prohibit the WWSF or WAF from using the option of not authenticating the user, especially as unauthenticated users are anonymous to the third party but may still be authorized for IMS service.

*** END OF MODIFICATIONS ***