3GPP TSG-SA3-LI Meeting #58	S3i150183

# Singapore, Singapore, 14-16 July 2015

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | 0180 | rev | - | Current version: | 12.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Separate Delivery of Messaging Services – ICE-DF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI13 |  |  |  |  |  |  | Date: |  | 2015-07-07 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | There are no capabilities defined in 33.107 for the support of separate delivery of Messaging Services. Such capabilities are needed to meet legal requirements based on national regulations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper propses the addition of functionality to 33.107 to describe the information that is passed from an Intercept Control Element (ICE) to a Delivery Function (DF) regardless of the specific Messaging Service being used or the protocol used to convey this to the target. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSPs may not be able to meet national regulations that require the separate delivery of Messaging Services from other services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New Clause X. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |

# X	Invocation of Lawful Interception for Messaging Services

## X.1	Overview of Messaging Services Interception

The capabilities defined in this clause apply when the interception of Messaging services is to be separated from the interception of all other services.

The network nodes, involved in providing the interception of a Messaging service, shall be determined based on the deployment configuration and the messaging scenario.

When lawfully authorized, Law Enforcement requires access to CC and IRI for the events pertaining to the target’s authorization, access to, and use of the Message Service, regardless of the service architecture used. This includes where the communications between the target and associates are sent and received over separate channels, or may be accessed at different ICEs at different geographical locations in the service provider’s network. Accessible communications includes communications originated by, terminating to, and redirected by the target, including both two-way and multi-way communications.

When available to the Message Service Provider, access control events may also be reported.

The interception of IRI and CC for a Message service shall be done according to X.2.

## X.2	Message Service Events

### X.2.1	X2-interface

The following information needs to be transferred from the ICE to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity (MSISDN, IMSI, IMEI);

-	events and associated parameters as defined in clauses X.2.2 through X.2.4 may be provided;

-	the target location (if available) or the IAs in case of location dependent interception;

-	Correlation number;

-	Encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

The IRI should be sent to DF2 using a reliable transport mechanism.

The interception and delivery of IRI/CC for Messaging services may be done at the following functional element:

1)	SGSN;

2)	PDN-GW/GGSN;

3)	CSCF;

4)	Message Centre/Server.

### X.2.2	Structure of the Events

At any given time, for a specific target and for any given Message service message, only one functional element is required to provide the CC interception. The functional element that provides the CC interception may vary, primarily, based on the Message service scenario.

Table X: Information Events for Message Service Event Records

Observed MSISDN MSISDN of the target.

Observed IMSI IMSI of the target.

Observed IMEI IMEI of the target. It shall be checked for each activation over the radio interface.

Event type Description which type of event is delivered: Message Service Authorisation, Message Service Deauthorisation, and Message Service Message.

Event date Date of the event generation in the ICE.

Event time Time of the event generation in the ICE. Timestamp shall be generated relative to ICE internal clock.

Network Element Identifier Unique identifier for the element reporting the ICE.

Message Service Identity Unique identifier for the message service being intercepted.

Message Identity Unique identifier for the message service message being intercepted.

Target Identity Unique identifier for the target whose Message Services messages are being intercepted.

Message Direction Identifies whether the target originated the message or is the intended recipient of the message.

Associate Identity Unique identifier for the other party(ies) either receiving the message or sending the message.

Location Information Location Information is the Service Area Identity (SAI), RAI and/or location area identity that is present at the ICE at the time of event record production.

Message Centre/Server Address Unique address for the Message Centre/Server that is handling the target’s message service.

Message Service Specific Information Consists of one or more sets of information that is integral to the message service (e.g., Presence information, platform identifier).

Message Content The actual content of the Message Service message or a pointer to where the contents reside.

Correlation Number The correlation number is used to correlate CC and IRI as well as related IRI.

Reason Indicates the reason (target, network, or other initiated, network failure).

System Identity Identifies the serving system when the intercept subject is authorised for service.

Requesting Identity Identifies the entity requesting authorisation, when different from the target identity.

Result Indicates whether the authorisation was successful or unsuccessful.

Failure Reason Indicates the reason for unsuccessful authorisation.

Authorisation Period Identifies the authorisation period.

IAs The observed Interception Areas.

### X.2.3	Message Service Authorisation

The Message Service Authorisation event occurs when a target attempts or gains authorisation for Message Services.  In cases where the Message Service Provider is also providing the access to the target, this event could be reported by a registration or authorisation event.  These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Message Service Identity

Target Identity

Result

Failure Reason

Location Information

System Identity

Requesting Identity

Authorisation Period

IAs (if applicable)

### X.2.4	Message Service Deauthorisation

The Message Service Termination event occurs when a target is no longer authorised, either temporarily or permanently, for Message Services. In the cases where the Message Service Provider is also providing the access to the target, this event could be reported by a deregistration or an access termination event.

These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Message Service Identity

Target Identity

Reason

Location Information

System Identity

Requesting Identity

IAs (if applicable)

### X.2.5	Message Service Message

For a message originated by the target, a Messaging Service ICE. Dependent on national requirements, event generation shall occur either when the ICE receives the message from the target MS or when the ICE receives notification that the Message Centre/Server successfully receives the message from ICE (when the ICE is not the Message Centre/Server); for a message intended for the target the event is generated in the Messaging Service ICE. Dependent on national requirements, event generation shall occur either when the Messaging Service ICE receives the message from the Message Centre/Server or when the Messaging Service ICE receives notification that the target MS successfully received the message. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Message Service Identity

Message Identity

Target Identity

Message Direction

Associate Identity

Location Information

Message Centre/Server Address

Message Service Specific Information

Message Content

Correlation Number

IAs (if applicable)