SA WG3LI Meeting #58	S3i150195

14 – 16 July, Singapore

Source:	SPRINT

Title:	P-CR proposing Introduction text for LIV8 study

Document for:	Approval

Agenda Item:	XXX

Work Item / Release:	Rel- 14

Abstract of the contribution: Proposes text for the introduction clause in the TR.

**************************************************start of first change*******************************

# Introduction

GSMA has been investigating a new VoLTE roaming architecture option, "S8 Home Routed (S8HR)".

S8HR is an architecture for IMS voice roaming whereby the PGW, PCRF, P-CSCF, ATCF and ATGW are located in the HPLMN also when the UE is roaming in a VPLMN. One of the main implications is that S8HR roaming architecture is not using Local Breakout anymore and therefore the IMS APN is resolved to point to a PGW in the HPLMN (rather than in the VPLMN). An IMS NNI between VPLMN and HPLMN may not be needed.

GSMA Product and Services Management Committee (PSMC) has informed 3GPP that they endorsed S8HR architecture as a candidate for VoLTE roaming (S2-150704), subject to the related issues (inclusive of Lawful Interception, Emergency Call, SRVCC, and others) being resolved by 3GPP.

The S8HR architecture limits existing Lawful Interception solutions  based on 3GPP standards in the serving PLMN in detecting, retrieving and reporting to Law Enforcement Agencies targeted roaming IMS communication  when obligated to do so by a intercept request (e.g., lawful authorization or warrant).

************************************end of last change*****************************************