3GPP TSG-SA3-LI Meeting #58	S3i150213

# Singapore, Singapore, 14-16 July 2015

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | 0176 | rev | 1 | Current version: | 12.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Multiple Delivery Addresses for Resilience and Failover Mechanism |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI13 |  |  |  |  |  |  | Date: |  | 2015-07-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | To support resiliency and failover of the HI2 and HI3 interfaces to the LE, it is necessary to be able to support delivery to more than one delivery address at the LE side. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper propses the addition of an optional capability to support multiple delivery addresses that can be used for LI data distribution and for failover purposes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Delivery of IRI and CC may be temporarily stopped and such intercept data may be lost. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2 and 5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Revision of S3i150174 |  |  |  |  |  |  |  |  |

### 5.1.2	X1_2-interface (IRI)

For the activation of IRI the message sent from the ADMF to the DF contains:

-	the target identity;

-	the address for delivery of IRI (= LEMF address);

- optionally multiple addresses for distributed delivery of IRI to a single LEMF

- optionally a primary and failover address(es) for delivery of IRI to either a single LEMF or two different LEMFs for the same LIID.

-	which subset of information shall be delivered;

-	a DF2 activation identity, which uniquely identifies the activation for DF2 and is used for further interrogation or deactivation, respectively;

-	the IA in case of location dependent interception;

-	the warrant reference number if required by national option.

If a target is intercepted for several LEAs and/or several identities simultaneously, a single activation of delivery is necessary for each combination of LEA and identity.

Figure 4: Information flow on X1_2-interface for Lawful Interception activation

### 5.1.3	X1_3-interface (CC)

For the activation of intercepted Content of Communications the message sent from the ADMF to the Delivery Function contains:

-	the target identity;

-	the address of delivery for CC (= LEMF address);

- optionally multiple addresses for delivery of CC to a single LEMF

- optionally a primary and failover address(es) for delivery of CC to either a single LEMF or two different LEMFs for the same LIID.

-	a DF3 activation identity, which uniquely identifies the activation for DF3 and is used for further interrogation or deactivation, respectively;

-	the IA in case of location dependent interception;

-	the warrant reference number if required by national option.

If a target is intercepted by several LEAs and/or several identities simultaneously, a single activation of delivery is necessary for each combination of LEA and identity.

Figure 5: Information flow on X1_3-interface for Lawful Interception activation