3GPP TSG-SA3-LI Meeting #59	S3i150265

# Austin, TX, USA, 5-7 October 2015

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | 0283 | rev | - | Current version: | 12.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correlation Number parameter required for LTE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI13 |  |  |  |  |  |  | Date: |  | 2015-10-05 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | The Correlation Number parameter is identified as being Conditional in LTE events reported to LE. However, if this parameter is not reported, LE may not be able to correlate related LTE event messages. Thus, this CR proposes changing the inclusion condition of this parameter to mandatory. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the M/C/O condition to M for Correlation Number and the description to indicate that it is mandatory. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Correlation Number may not be reported and if so, will not allow the CSP to meet their regulatory obligations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5.1.2, 10.5.1.3, 10.5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |

#### 10.5.1.2	BEGIN record information

The BEGIN record is used to convey the first event of EPS communication interception.

The BEGIN record shall be triggered in the following cases:

-	successful EPS bearer activation or tunnel establishment;

-	the interception of a target's communications is started and at least one EPS bearer or tunnel is active. In this case, some of the parameters, available at EPS bearer or tunnel activation may be not available any longer at the node. It is not required to store these parameters at the node to be used just in case of LI activation at later stage. If more than one EPS bearer or tunnel is active, a BEGIN record shall be generated for each EPS bearer or tunnel that is active;

-	during the S-GW relocation, when there is a change in the PLMN or when the information about the change in the PLMN is not available at the DF/MF;

-	the target entered an interception area and has at least one EPS bearer/tunnel active (FFS).

Table 10.5.1.2.1: Bearer Activation (successful) and Start of Interception with active bearer BEGIN Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide, depending on the reported event, Bearer activation or Start of interception with active bearer event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| access point name | C | Provide to identify the packet data network to which the connection is made (valid only for default bearer activation). |
| PDN address allocation | C | Provides the PDN type and PDN address(es) used by the network. |
| initiator | C | Provide to indicate whether the EPS bearer activation is network-initiated, target-initiated, or not available. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's UE. |
| EPS bearer QOS | C | Provide to identify the QOS parameters. It carries the EPS bearer QOS associated to the established bearer. |
| Bearer activation type | C | Provides information on default or dedicated bearer activation. |
| APN-AMBR | C | The Aggregate Maximum Bit Rate foreseen for the APN. The parameter carries the APN-AMBR used for the established bearer |
| Protocol configuration options | C | Provide information about the protocol configuration options requested by the UE. |
| Procedure transaction identifier | C | Used to associate the EPS bearer activation to other messages triggering the procedure. |
| EPS bearer id | C | Provides the EPS bearer id allocated by the network. |
| Linked EPS bearer identity | C | Provides, in case of dedicated bearer activation, the EPS bearer id of the associated default bearer; not applicable in case of default bearer activation. |
| Traffic Flow Template(s) TFT | C | The TFT associated to the dedicated bearer activation; not applicable in case of default bearer activation. |
| Handover indication | C | Provide information that the procedure is triggered as part of a handover. |
| RAT type | C | The Radio Access Type used by the target subscriber (only applicable to default bearer activation). |
| Correlation number | M | Shall be provided to uniquely identify the EPS bearer delivered to the LEMF and to correlate IRI records with CC. |

Table 10.5.1.2.2: PMIP Attach/tunnel activation (successful) and Start of Interception with active PMIP tunnel BEGIN Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed MSISDN |  |  |
| Observed ME Id |  |  |
| observed IMSI |  |  |
| event type | C | Provide, depending on the reported event, PMIP Attach/tunnel activation or Start of interception with active PMIP tunnel event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| Lifetime | C | The lifetime for the tunnel |
| Access technology type | C | Provide the radio access type |
| Handover indicator | C | Provide information that the procedure is triggered as part of the handover |
| APN | C | Provides the Access Point Name |
| UE address info | C | Includes one or more addresses allocated to the UE |
| Correlation number | M | Shall be provided to uniquely identify tunnel delivered to the LEMF and to correlate IRI records with CC. |
| Serving Network | C | Provide to identify the serving network the UE is attached to in case of E-UTRAN access and PMIP based S5/S8 interfaces. |
| DHCPv5 Address Allocation Indication | C | Indicates that DHCPv5 is to be used to allocate the IPv4 address to the UE in case of E-UTRAN access and PMIP based S5/S8 interfaces. |
| Location information | C | Provide, when authorized, to identify location information for the target’s UE. |

Table 10.5.1.2.3: MIP registration/tunnel activation (successful) and Start of Interception with active MIP tunnel BEGIN Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed IMSI |  |  |
| event type | C | Provide, depending on the reported event, MIP registration/tunnel activation or Start of interception with active MIP tunnel event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| Lifetime | C | The lifetime for the tunnel. |
| Home Address | C | Provide the UE Home IP Address. |
| Care of address | C | The IP address provided by the access network. |
| Home Agent Address | C | Provide the Home Agent address |
| Correlation number | M | Shall be provided to uniquely identify tunnel delivered to the LEMF and to correlate IRI records with CC. |
| APN | C | Provides the Access Point Name |

Table 10.5.1.2.4: DSMIP registration/tunnel activation (successful) and Start of Interception with active DSMIP tunnel BEGIN Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed IMSI |  |  |
| event type | C | Provide, depending on the reported event, DSMIP registration/tunnel activation or Start of interception with active DSMIP tunnel event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| lifetime | C | The lifetime for the tunnel |
| Requested IPv6 Home Prefix | C | Provide the UE IPv6 Home Prefix |
| Home address | C | Provide the assigned home address |
| APN | C | Provides the Access Point Name |
| Care of address | C | The IP address provided by the access network |
| Correlation number | M | Shall be provided to uniquely identify tunnel delivered to the LEMF and to correlate IRI records with CC. |

#### 10.5.1.3	CONTINUE record information

The CONTINUE record is used to convey events during an active EPS bearer/tunnel.

The CONTINUE record shall be triggered in the following cases:

-	An active EPS bearer/session is modified;

-	During the S-GW relocation, when target has got at least one EPS bearer/tunnel active, the PLMN does not change and the triggering event information is available at the DF/MF.
NOTE: This scenario does not apply to DSMIP and MIP protocol cases.

-	In case of handover between different accesses when GTP based messages are intercepted. In this case, the RAT type indicates the new access after the handover.

In order to enable the LEMF to correlate the information on HI3, a new correlation number shall not be generated within a CONTINUE record.

Table 10.5.1.3.1: Bearer Modification CONTINUE Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide Bearer modification event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| initiator | C | Provide to indicate whether the EPS bearer modification is network-initiated, target-initiated, or not available. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's UE. |
| EPS bearer QOS | C | Provide to identify the QOS parameters. |
| APN-AMBR | C | The Aggregate Maximum Bit Rate for the APN. |
| Procedure transaction identifier | C | Used to associate the EPS bearer modification to other messages triggering the procedure. |
| EPS bearer id | C | Provides the EPS bearer id allocated by the network. |
| Traffic Flow Template(s) TFT | C | The TFT associated to the EPS bearer modification; |
| RAT type | C | The Radio Access Type used by the target. |
| APN-AMBR | C | The Aggregate Maximum Bit Rate foreseen for the APN. |
| Handover indication | C | Provide information that the procedure is triggered as part of a handover. |
| Correlation number | M | Shall be provided to uniquely identify the EPS bearer delivered to the LEMF and to correlate IRI records with CC. |
| Failed bearer modification reason | C | Provide information about the reason for failed bearer modification |

Table 10.5.1.3.2: Start of Interception with active bearer CONTINUE Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide Start of interception with active bearer event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| access point name | C | Provide to identify the packet data network to which the connection is made (valid only for default bearer). |
| PDN address allocation | C | Provides the PDN type and PDN address(es) used by the network. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's UE. |
| EPS bearer QOS | C | Provide to identify the QOS parameters. |
| Bearer activation type | C | Provides information on default or dedicated bearer. |
| APN-AMBR | C | The Aggregate Maximum Bit Rate foreseen for the APN. |
| Protocol configuration options | C | Provide, if available, information about the protocol configuration options requested by the UE (valid only for default bearer). |
| Procedure transaction identifier | C | Used, if available, to associate the EPS bearer to other messages triggering the previous bearer activation. |
| EPS bearer id | C | Provides the EPS bearer id allocated by the network. |
| Linked EPS bearer identity | C | Provides, in case of dedicated bearer, the EPS bearer id of the associated default bearer; not applicable in case of default bearer. |
| Traffic Flow Template(s) TFT | C | The TFT associated to the dedicated bearer; not applicable in case of default bearer. |
| Handover indication | C | Provide information that the procedure is triggered as part of a handover. |
| RAT type | C | The Radio Access Type used by the target (only applicable to default bearer). |
| Correlation number | M | Shall be provided to uniquely identify the EPS bearer delivered to the LEMF and to correlate IRI records with CC. |

Table 10.5.1.3.3: Start of Interception with active PMIP tunnel CONTINUE Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed MSISDN |  |  |
| observed ME Id |  |  |
| observed IMSI |  |  |
| event type | C | Provide Start of interception with active PMIP tunnel event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| Lifetime | C | The lifetime for the tunnel |
| Access technology type | C | Provide the radio access type |
| Handover indicator | C | Provide information that the procedure is triggered as part of the handover |
| APN | C | Provides the Access Point Name |
| UE address info | C | Includes one or more addresses allocated to the UE |
| Additional parameters | C | Provide additional parameters sent by the UE. |
| Correlation number | M | Shall be provided to uniquely identify tunnel delivered to the LEMF and to correlate IRI records with CC. |
| Serving Network | C | Provide to identify the serving network the UE is attached to in case of E-UTRAN access and PMIP based S5/S8 interfaces. |
| Location information | C | Provide, when authorized, to identify location information for the target’s UE. |

Table 10.5.1.3.4: PMIP session modification CONTINUE Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed MSISDN |  |  |
| observed ME Id |  |  |
| observed IMSI |  |  |
| event type | C | Provide PMIP session modification. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| Lifetime | C | The lifetime for the tunnel |
| Access technology type | C | Provide the radio access type |
| Handover indicator | C | Provide information that the procedure is triggered as part of the handover |
| APN | C | Provides the Access Point Name |
| UE address info | C | Includes one or more addresses allocated to the UE |
| Additional parameters | C | Provide additional parameters sent by the UE. |
| Correlation number | M | Shall be provided to uniquely identify tunnel delivered to the LEMF and to correlate IRI records with CC. |
| Serving Network | C | Provide to identify the serving network the UE is attached to |
| DHCPv4 Address Allocation Indication | C | Indicates that DHCPv4 is to be used to allocate the IPv4 address to the UE |
| Location information | C | Provide, when authorized, to identify location information for the target’s UE. |

Table 10.5.1.3.5: DSMIP session modification CONTINUE Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed IMSI |  |  |
| event type | C | Provide DSMIP session modification. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| lifetime | C | The lifetime for the tunnel |
| Requested IPv6 Home Prefix | C | Provide the UE IPv6 Home Prefix |
| Home address | C | Provide the assigned home address |
| APN | C | Provides the Access Point Name |
| Care of address | C | The IP address provided by the access network |
| Correlation number | M | Shall be provided to uniquely identify tunnel delivered to the LEMF and to correlate IRI records with CC. |
| Session modification failure reason | C | Provides the reason for failure |

#### 10.5.1.4	END record information

The END record is used to convey the last event of EPS communication.

The END record shall be triggered in the following cases:

- EPS bearer deactivation;

- Tunnel deactivation;

- Resource allocation deactivation.

Table 10.5.1.4.1: Bearer Deactivation END Record

| Parameter | MOC | Description/Conditions |  |
| --- | --- | --- | --- |
| observed MSISDN |  |  |  |
| observed IMSI | C | Provide at least one and others when available. |  |
| observed ME Id |  |  |  |
| event type | C | Provide Bearer Deactivation event type. |  |
| event date | M | Provide the date and time the event is detected. |  |
| event time |  |  |  |
| initiator | C | Provide to indicate whether the EPS deactivation is network-initiated, target-initiated, or not available. |  |
| network identifier | M | Shall be provided. |  |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |  |
| correlation number | M | Shall be provided to uniquely identify the PDP context delivered to the LEM and to correlate IRI records with CC. |  |
| lawful intercept identifier | M | Shall be provided. |  |
| location information | C | Provide, when authorized, to identify location information for the target's MS. |  |
| Bearer deactivation type | C | Provides information on default or dedicated bearer deactivation. |  |
| Bearer deactivation cause | C | Provide to indicate reason for deactivation. |  |
| EPS bearer id | O | Provides the identity of the deactivated bearer. |  |
| Procedure Transaction Identifier | C | Used to associate the EPS bearer deactivation to other messages triggering the procedure. |  |
| ULI Timestamp | O | Indicates the time when the User Location Information was acquired. |

Table 10.5.1.4.2: PMIP Detach/tunnel deactivation END Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed MSISDN |  |  |
| observed ME Id |  |  |
| observed IMSI |  |  |
| event type | C | Provide PMIP Detach/tunnel deactivation event type |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| APN | C | The access point name |
| Initiator | C | Provide to indicate whether the tunnel deactivation is network-initiated, target-initiated |
| Correlation number | M | Shall be provided to uniquely identify tunnel delivered to the LEMF and to correlate IRI records with CC. |
| Location information | C | Provide, when authorized, to identify location information for the target’s UE. |

Table 10.5.1.4.3: MIP deregistration/tunnel deactivation END Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed IMSI |  |  |
| event type | C | Provide MIP deregistration/tunnel deactivation. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| Home Agent address | C | Provide the Home Agent address |
| Home Address | C | Provide the UE Home IP Address |
| Care of address | C | The local IP address provided by the access network. |
| Initiator | C | Provide to indicate whether the tunnel deactivation is network-initiated, target-initiated |
| Correlation number | M | Shall be provided to uniquely identify tunnel delivered to the LEMF and to correlate IRI records with CC. |

Table 10.5.1.4.4: DSMIP deregistration/tunnel deactivation END Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed IMSI |  |  |
| event type | C | Provide DSMIP deregistration/tunnel deactivation. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| Home address | C | Provide the IPv6 home address |
| Care of Address | C | The IP address provided by the access network |
| Initiator | C | Provide to indicate whether the tunnel deactivation is network-initiated, target-initiated |
| Correlation number | M | Shall be provided to uniquely identify tunnel delivered to the LEMF and to correlate IRI records with CC. |

Table 10.5.1.4.5: PMIP Resource Allocation Deactivation END Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed MSISDN |  |  |
| observed ME Id |  |  |
| observed IMSI |  |  |
| event type | C | Provide PMIP Resource Allocation Deactivation event type |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| Revocation trigger | C | Provide the cause for the revocation procedure |
| UE Address Info | C | Includes one or more addresses allocated to the UE (i.e. UE PMIP tunnel information) |
| Correlation number | M | Shall be provided to uniquely identify tunnel delivered to the LEMF and to correlate IRI records with CC. |
| Location information | C | Provide, when authorized, to identify location information for the target’s UE. |

Table 10.5.1.4.6: PMIP PDN-GW initiated PDN disconnection END Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed MSISDN |  |  |
| observed ME Id |  |  |
| observed IMSI |  |  |
| event type | C | Provide PMIP PDN-GW initiated PDN disconnection event type |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| Revocation trigger | C | Provide the cause for the revocation procedure |
| PDN address(es) | C | Provide the PDN address(es) for which the disconnection is done |
| Correlation number | M | Shall be provided to uniquely identify tunnel delivered to the LEMF and to correlate IRI records with CC. |
| Location information | C | Provide, when authorized, to identify location information for the target’s UE. |

Table 10.5.1.4.7: MIP Resource Allocation Deactivation END Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed IMSI |  |  |
| event type | C | Provide MIP deregistration/tunnel deactivation. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| Revocation trigger | C | Provide the cause for the revocation procedure |
| Home Address | C | Provide the UE Home IP Address |
| Foreign domain address | C | The relevant IP address in the foreign domain. |
| Correlation number | M | Shall be provided to uniquely identify tunnel delivered to the LEMF and to correlate IRI records with CC. |