SA WG3LI Meeting #58	S3i150288

5 – 7 October 2015, Austin, Texas, USA	Revised from

Source:	NTT DOCOMO

Title:	P-CR on implicit registrations for LIV-8 study

Document for:	Approval

Agenda Item:	XXX

Work Item / Release:	Rel- 14

Abstract of the contribution: Proposes a description and a solution for implicit registrations by LMISF extracting the implicitly registered identities form the registration OK messages.

**************************************************start of first change*******************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Y]	3GPP TS 24.229: "IP Multimedia Call Control based on SIP and SDP; Stage 3".

**************************************************start of next change*******************************

## 5.8	Key Issue #8 – Detecting implicit IMS registrations

### 5.8.1	Description

Several identities may be registered with one registration event by implicit registrations.

### 5.8.2	Architectural Requirements

### All public identities registered for a UE shall be available at the LMIRSF.

### 5.8.3	Stage 3 Requirements

### According to 24.229 [Y], subclause 5.4.1.2.2F, the registered distinct public user identity and its associated set of implicitly registered public user identities is sent to UE from S-CSCF in the "200 (OK)" message. Thus, this information is available in the LMISF

### 5.8.4	Solution Approaches

### The LMISF shall racking both IMS registration requests as well as the registration responses, in order to be able to make sure all implicitly registered IMS user identities are tracked.

## 5.9	Key Issue #9 – Retained data

### 5.9.1	Description

Editor's Note:	Describe the key Issue (i.e. problem statement), including technical constraints and interpretations.

### 5.9.2	Architectural Requirements

Editor's Note:	Capture agreements on architectural requirements for solving the key issue. This clause may be omitted if deemed unnecessary.

### 5.9.3	Stage 3 Requirements

Editor's Note:	Capture agreements on Stage 3 requirements for solving the key issue. This clause may be omitted if deemed unnecessary.

### 5.9.4	Solution Approaches

Editor's Note:	Describe approaches possible to solve the key issue described.

************************************end of last change*****************************************