3GPP TSG-SA3-LI Meeting #59	S3i150290

# Austin, TX, USA, 5-7 October 2015

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | 289 | rev | - | Current version: | 12.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Delivery of Messaging Services to LE separate from other services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI13 |  |  |  |  |  |  | Date: |  | 2015-10-05 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | National requirements may require that Messaging Services be delivered separate from other services. This CR provides modifications to 33.108 to meet this requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause B.X and clause X is proposed to contain the ASN.1 and the requirements for separate delivery of Messaging Services, respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSP will not be able to meet regulatory requiements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause B.X and new clause X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

# B.X	Intercept related information & CC (HI2 Messaging Services)

Declaration of ROSE operation mess-sending-of-IRI  is ROSE delivery mechanism specific. When using FTP delivery mechanism, data MessIRIsContent must be considered.

# ASN1 description of IRI & CC (HI2 interface)

MessagingServicesHI2Operations {itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulintercept(2) threeGPP(4) hi2mess(x) r13(13) version-1 (1)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t(2) remote-operations(4) informationObjects(5) version1(0)}

LawfulInterceptionIdentifier,

TimeStamp,

Network-Identifier,

National-Parameters,

National-HI2-ASN1parameters,

DataNodeAddress,

IPAddress,

IP-value,

X25Address

FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulIntercept(2) hi2(1) version18(18)} -- Imported from TS 101 671v3.12.1

CivicAddress

FROM UmtsHI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulintercept(2) threeGPP(4) hi2(1) r12(12) version-8 (8)};

-- Imported from 3GPP TS 33.108, UMTS PS HI2

-- Object Identifier Definitions

-- Security DomainId

lawfulInterceptDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0)

securityDomain(2) lawfulIntercept(2)}

-- Security Subdomains

threeGPPSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP(4)}

hi2epsDomainId OBJECT IDENTIFIER	::= {threeGPPSUBDomainId hi2mess(x) r13(13) version-1 (1)}

mess-sending-of-IRI  OPERATION ::=

{

ARGUMENT	MessIRIsContent

ERRORS		{ OperationErrors }

CODE		global:{threeGPPSUBDomainId hi2mess(x) opcode(1)}

}

-- Class 2 operation . The timer shall be set to a value between 3 s and 240 s.

-- The timer.default value is 60s.

-- NOTE:	The same note as for HI management operation applies.

MessIRIsContent		::= CHOICE

{

messiRIContent			MessIRIContent,

messIRISequence			MessIRISequence

}

MessIRISequence		::= SEQUENCE OF MessIRIContent

-- Aggregation of MessIRIContent is an optional feature.

-- It may be applied in cases when at a given point in time

-- several IRI records are available for delivery to the same LEA destination.

-- As a general rule, records created at any event shall be sent

-- immediately and not withheld in the DF or MF in order to

-- apply aggragation.

-- When aggregation is not to be applied,

-- MessIRIContent needs to be chosen.

-- MessIRIContent includes events that correspond to Messaging Services (e.g., SMS).

MessIRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,	-- include at least one optional parameter

iRI-End-record			[2] IRI-Parameters,

iRI-Continue-record		[3] IRI-Parameters,	-- include at least one optional parameter

iRI-Report-record		[4] IRI-Parameters	-- include at least one optional parameter

}

-- the MessIRIContent may provide events that correspond to UMTS/GPRS or IMS as well.

unknown-version 		ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter-value	ERROR ::= { CODE local:2}

unknown-parameter		ERROR ::= { CODE local:3}

OperationErrors ERROR ::=

{

unknown-version |

missing-parameter |

unknown-parameter-value |

unknown-parameter

}

-- These values may be sent by the LEMF, when an operation or a parameter is misunderstood.

-- Parameters having the same tag numbers must be identical in Rel-5 and onwards modules.

IRI-Parameters		::= SEQUENCE

{

hi2messDomainId				[0]	OBJECT IDENTIFIER,  -- 3GPP HI2 MESS domain

lawfulInterceptionIdentifier	[1] LawfulInterceptionIdentifier,

-- This identifier is associated to the target.

timeStamp				[2] TimeStamp,

-- date and time of the event triggering the report.)

initiator 				[3] ENUMERATED

{

not-Available		(0),

originating-Target	(1),

-- indicates the target MS originated the communications, e.g., SMS-MO.

terminating-Target	(2),

-- indicates the target MS is the intended recipient of the communications.

-- for example, SMS-MT.

...

},

locationOfTheTarget		[4] EPSLocation OPTIONAL,

-- location of the target

partyInformation 		[5] SET SIZE (1..10) OF PartyInformation OPTIONAL,

-- This parameter provides the concerned party, the identiy(ies) of the party

--)and all the information provided by the party.

serviceCenterAddress	[6] PartyInformation OPTIONAL,

-- e.g. in case of SMS message this parameter provides the address of  the relevant

-- server within the calling (if server is originating) or called (if server is

-- terminating) party address parameters

messageContent						[14] MessageContent OPTIONAL,

-- this parameter provides the Messaging Services content and associated information

national-Parameters		[7] National-Parameters OPTIONAL,

messCorrelationNumber	[8] EPSCorrelationNumber OPTIONAL,

-- this parameter provides GPRS Correlation number when the event corresponds to UMTS/GPRS.

messevent 				[9] MessEvent OPTIONAL,

-- This information is used to provide particular action of the target

-- such as attach/detach

networkIdentifier		[10] Network-Identifier OPTIONAL,

associateAddress 		[11] PartyInformation OPTIONAL,

messageIdentity         [12] MessageIdentity  OPTIONAL,

-- contains the visited network identifier inside the EPS Serving System Update for

-- non 3GPP access, coded according to [53]

uLITimestamp            [13] OCTET STRING (SIZE (8)) OPTIONAL,

-- Coded according to 3GPP TS 29.060 [17]; Only the ULI Timestamp value is reported.

pANI-Header-Info		[14] SEQUENCE OF PANI-Header-Info		OPTIONAL,

-- information extracted from P-Access-Network-Info headers of SIP message;

national-HI2-ASN1parameters	[256]	National-HI2-ASN1parameters	OPTIONAL

}

-- Parameters having the same tag numbers must be identical in Rel-5 and onwards modules

-- PARAMETERS FORMATS

PANI-Header-Info	::= SEQUENCE

{

access-Type 		[1] OCTET STRING 		OPTIONAL,

-- ASCII chain ‘3GPP-UTRAN-TDD’, ‘3GPP-E-UTRAN-TDD’,... : see TS 24.229 §7.2A.4 [76]

access-Class  		[2] OCTET STRING		OPTIONAL,

-- ASCII chain ‘3GPP-UTRAN’, ‘3GPP-E-UTRAN’,... : see TS 24.229 §7.2A.4 [76]

network-Provided	[3] NULL				OPTIONAL,

-- present if provided by the network

pANI-Location		[4] PANI-Location 		OPTIONAL,

...

}

PANI-Location  ::= SEQUENCE

{

raw-Location     [1] OCTET STRING 	OPTIONAL,

-- raw copy of the location string from the P-Access-Network-Info header

location          [2] Location      OPTIONAL,

ePSLocation       [3] EPSLocation   OPTIONAL,

...

}

PartyInformation 			::= SEQUENCE

{

party-Qualifier 	[0]  ENUMERATED

{

gPRSorEPS-Target(3),

...

},

partyIdentity 		[1] SEQUENCE

{

imei					[1] OCTET STRING (SIZE (8)) OPTIONAL,

-- See MAP format [4]

imsi					[3] OCTET STRING (SIZE (3..8)) OPTIONAL,

-- See MAP format [4] International Mobile

-- Station Identity E.212 number beginning with Mobile Country Code

msISDN					[6] OCTET STRING (SIZE (1..9)) OPTIONAL,

-- MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document TS 29.002 [4]

e164-Format				[7] OCTET STRING    (SIZE (1 .. 25)) OPTIONAL,

-- E164 address of the node in international format. Coded in the same format as

-- the calling party number  parameter of the ISUP (parameter part:[29])

sip-uri					[8] OCTET STRING 	OPTIONAL,

-- See [26]

...,

tel-uri					[9] OCTET STRING 	OPTIONAL,

-- See [67]

nai                    [10] OCTET STRING    OPTIONAL,

-- NAI of the target, encoded in the same format as defined by [EPS stage 3 specs]

x-3GPP-Asserted-Identity [11] OCTET STRING 	OPTIONAL,

-- X-3GPP-Intended-Identity header (3GPP TS 24 109 [79]) of the target, used in

-- some XCAP transactions as a complement information to SIP URI or Tel URI.

xUI						[12] OCTET STRING 	OPTIONAL

-- XCAP User Identifier (XUI)is a string, valid as a path element in an XCAP URI, that is

-- may be associated with each user served by a XCAP resource server. Defined in IETF RFC

-- 4825[80] as a complement information to SIP URI or Tel URI.

},

services-Data-Information 	[4] Services-Data-Information OPTIONAL,

-- This parameter is used to transmit all the information concerning the

-- complementary information associated to the basic data call

...

}

GlobalCellID	::= OCTET STRING  (SIZE (5..7))

Rai				::= OCTET STRING (SIZE (6))

Sai				::=	OCTET STRING (SIZE (7))

GSMLocation 	::= CHOICE

{

geoCoordinates 	[1] SEQUENCE

{

latitude		[1]	PrintableString (SIZE(7..10)),

-- format : 	XDDMMSS.SS

longitude		[2]	PrintableString (SIZE(8..11)),

-- format : 	XDDDMMSS.SS

mapDatum		[3]	MapDatum DEFAULT wGS84,

...,

azimuth			[4] INTEGER (0..359) OPTIONAL

-- The azimuth is the bearing, relative to true north.

},

-- format : 	XDDDMMSS.SS

-- 				X 			: N(orth), S(outh), E(ast), W(est)

--				DD or DDD 	: degrees (numeric characters)

--				MM			: minutes (numeric characters)

--				SS.SS 		: seconds, the second part (.SS) is optionnal

-- Example :

--			latitude short form		N502312

--			longitude long form		E1122312.18

utmCoordinates	[2] SEQUENCE

{

utm-East 		[1] PrintableString (SIZE(10)),

utm-North 		[2] PrintableString (SIZE(7)),

-- example	utm-East	32U0439955

--			utm-North	5540736

mapDatum		[3]	MapDatum DEFAULT wGS84,

...,

azimuth			[4] INTEGER (0..359) OPTIONAL

-- The azimuth is the bearing, relative to true north.

},

utmRefCoordinates  	[3] SEQUENCE

{

utmref-string		PrintableString (SIZE(13)),

mapDatum			MapDatum DEFAULT wGS84,

...

},

-- example	32UPU91294045

wGS84Coordinates  	[4] OCTET STRING

-- format is as defined in [37].

}

MapDatum ::= ENUMERATED

{

wGS84,

wGS72,

eD50, 	-- European Datum 50

...

}

UMTSLocation ::= CHOICE {

point					[1]	GA-Point,

pointWithUnCertainty	[2]	GA-PointWithUnCertainty,

polygon					[3]	GA-Polygon

}

GeographicalCoordinates ::= SEQUENCE {

latitudeSign			ENUMERATED { north, south },

latitude				INTEGER (0..8388607),

longitude				INTEGER (-8388608..8388607),

...

}

GA-Point ::= SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

GA-PointWithUnCertainty ::=SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

uncertaintyCode				INTEGER (0..127)

}

maxNrOfPoints						INTEGER ::= 15

GA-Polygon ::= SEQUENCE (SIZE (1..maxNrOfPoints)) OF

SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

MessageContent ::= CHOICE

{

sMS-Report	[1]	SMS-report,

mMS-Report	[2]	MMS-report, -- to be defined

...

}

MessageIdentity ::= SEQUENCE

{

iDofMessage			[1]	INTEGER,

messageSessionID	[2]	INTEGER, OPTIONAL

...

}

SMS-report		::= SEQUENCE

{

sMS-Contents	[3] SEQUENCE

{

sms-initiator		[1] ENUMERATED 	-- party which sent the  SMS

{

target			(0),

associate		(1),

server			(2),

undefined-party	(3),

...

},

transfer-status		[2] ENUMERATED

{

succeed-transfer	(0), 		-- the transfer of the SMS message succeeds

not-succeed-transfer(1),

undefined			(2),

...

} OPTIONAL,

other-message		[3] ENUMERATED 	-- in case of terminating call, indicates if

-- the server will send other SMS

{

yes			(0),

no			(1),

undefined	(2),

...

} OPTIONAL,

content				[4] OCTET STRING (SIZE (1 .. 270)) OPTIONAL,

-- Encoded in the format defined for the SMS mobile

...

}

}

MessEvent ::= ENUMERATED

{

sMS					(1),

mMS					(2),

... ,

}

-- see [19]

EPSLocation	::= SEQUENCE

{

userLocationInfo    [1] OCTET STRING (SIZE (1..39)) OPTIONAL,
        -- coded according to 3GPP TS 29.274 [46]; the type IE is not included

gsmLocation			[2] GSMLocation OPTIONAL,

umtsLocation		[3] UMTSLocation OPTIONAL,

olduserLocationInfo	[4] OCTET STRING (SIZE (1..39))	OPTIONAL,

-- coded in the same way as userLocationInfo

lastVisitedTAI	    [5] OCTET STRING (SIZE (1..5))	OPTIONAL,

-- the Tracking Area Identity is coded in accordance with the TAI field in 3GPP TS 29.274
        -- [46].

tAIlist			    [6] OCTET STRING (SIZE (7..97))	OPTIONAL,

-- the TAI List is coded acording to 3GPP TS 24.301 [47], without the TAI list IEI

...,

threeGPP2Bsid		[7] OCTET STRING (SIZE (1..12)) OPTIONAL,

-- contains only the payload from the 3GPP2-BSID AVP described in the 3GPP TS 29.212 [56].

civicAddress		[8] CivicAddress OPTIONAL

}

END -- OF MessHI2Operations

*** SECOND MODIFICATION ***

# X	Separate Delivery of Messaging Services

## X.1	Background

National regulations may require the delivery to the LEMF of the results of LI of Messaging Services separate from other services as described in TS 33.107 [19].

This clause is intended to define the capabilities to meet this requirement.

## X.2	Separate Delivery of SMS

SMS interception in the PS domain is described in Clause 7.4.7 of TS 33.107 [19] while 7.A.6 of that same specification addresses SMS over IMS.  For both cases of SMS, the results of interception shall be delivered separately from all other services.

To enable the separate delivery of the results of LI of Messaging Services, a common Messaging Services Report Record is defined as shown in Table X.1.  This common format is intended to allow a common report format for the LEMF to handle regardless of the method by which SMS is passed in the network.

The common Messaging Services REPORT record shall be triggered when:

-	the target's mobile station sends an SMS-Mobile Originated (MO) communication. Dependent on national requirements, the triggering event shall occur either when the ICE receives the SMS from the target MS or, when the ICE receives notification that the SMS-Centre successfully received the SMS;

national regulations may mandate that a REPORT record shall be triggered when the ICE receives an SMS-MO communication from the target's mobile station;

-	the target's mobile station receives a SMS Mobile-Terminated (MT) communication. Dependent on national requirements, the triggering event shall occur either when the ICE receives the SMS from the SMS-Centre or, when the ICE receives notification that the target MS successfully received the SMS;

national regulations may mandate that a REPORT record shall be triggered when the ICE receives an SMS-MT communication from the SMS-Centre destined for the target's mobile station;

Table X.1: Messaging Services (SMS) REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| observed IMPU |  |  |
| observed IMPI |  |  |
| event type | M | Provide type of Message Service (SMS event type). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| message identity | M | Shall be provided to uniquely identify the message within a system. Includes, if applicable, a message session identity. |
| associate identity | M | Shall be provided to identify the associate’s address(es)/identify(ies) (e.g., IMSI, IMEI, MSISDN, email) for the message. May be more than one, if the target’s message is sent to multiple addresses. |
| location information | C | Provide, when authorized, to identify location information for the target's MS. |
| message content | C | Provide, when authorized, to deliver SMS content, including header which is sent with the SMS-service. |
| service center address | C | Provide to identify the address of the relevant SMS-C server. If SMS content is provided, this parameter is optional. |
| message service specific information | C | Provide, when information is integral to the Message Service (e.g., Presence information, platform identity). May be more than one instance. |
| message initiator | M | Indicates whether the SMS is MO, MT, or Undefined. |

*** END OF MODIFICATIONS ***