SA WG3LI Meeting #59	S3i150341

6-8 October 2015, Austin, Texas, USA	Revised from S3i150285

Source:	SPRINT, NTT Docomo

Title:	P-CR proposing text for 4.2 Architecture

Document for:	Approval

Agenda Item:	XXX

Work Item / Release:	Rel- 14

Abstract of the contribution: Proposes text for the Architecture clause in the TR.

**************************************************start of first change*******************************

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BBIFF	Bearer Binding Intercept and Forwarding Function

LMISF	LI Mirror IMS State Function

LPCF	LI Policy Control Function

S8HR	S8 Home Routing

************************************end of first change/start of second change *******************************

## 4.2	Architecture and Architecture Requirements

### 4.2.X	Architecture

#### 4.2.X.1	Architecture reference model

Firgure Y: S8HR with BBIFF target GTP tunnel extraction

#### 4.2.X.2	Reference points

Xia:	Reference point between BBIFF or S-GW and LMISF for the delivery of IMS signalling (Gm extracted from S8 GTP tunnel) messages extracted from IMS signalling GTP tunnels  (S8) for roaming users where S8HR is utilized.

Xib:	Provides the transfer of GTP tunnel LI extraction rules from the LPCF to the BBIFF or S-GW. Provides indications from BBIFF or S-GW of GTP tunnel setup or disconnect meeting LI extraction rules.

Xic:	Reference point for informing the LPCF by the LMISF of activation and deactivation of LI for targets operating through S8HR.

#### 4.2.X.3	Network elements

##### 4.2.X.3.1	LMISF

LMISF functions include:

- Receive and process IMS signalling (Gm) over the Xia reference point (extracted from the S8 GTP tunnel)

- Tracking all IMS registrations, re-registrations and de-registrations of all roaming users operating through S8HR

- Establish and maintain mapping between LEA requested  target identity and  the GTP tunnel identities available in the BBIFF or S-GW.

- Provide IMS  IAP capabilities (IRI) equivalent to those defined in 33.107[Y]  & 33.108 [X]

- Inform the LPCF of activation and deactivation of LI on targets roaming through S8HR.

- Generation of Communications records for the purpose of Data Retention.

##### 4.2.X.3.2	LPCF

LPCF functions include:

- Directing the BBIFF or S-GW to start or stop extracting and delivering IMS media in identified GTP tunnels to the DF3.

- Receive from the LMISF indications of activation and deactivation of LI on targets roaming through S8HR.

##### 4,2.X.3.3	BBIFF

BBIFF functions include:

- Executing the policy rules established by the LPCF against S8HR traffic.

- Deliver IMS signalling (Gm) over the Xia reference point (extracted from the S8 GTP tunnel.

- Direct the S-GW to deliver IMS media (content) extracted from the S8 GTP tunnel over X3 to DF3.

************************************end of last change*****************************************