SA WG3LI Meeting #59	S3i150346

5 – 7 October 2015, Austin, Texas, USA	Revised from S3i150289

Source:	NTT DOCOMO

Title:	P-CR on intra-access handover for LIV-8 study

Document for:	Approval

Agenda Item:	XXX

Work Item / Release:	Rel- 14

Abstract of the contribution: Proposes architectural requirements for intra-access handovers.

**************************************************start of first change*******************************

## 5.5	Key Issue #5 – Intra-access handover or anchor change impact

### 5.5.1	Description

There are two types of intra-access handovers that may impact LI:

1)	SGW relocation: SGW relocation may happen any time the visited network deems necessary. It is necessary to ensure that the LI system is able to follow SGW relocations for targeted IMS sessions and for IMS registrations.

2) MME relocation: MME relocation may happen any time the visited network deems necessary. It is necessary to ensure that the LI system is able to follow MME relocations for targeted IMS sessions and for IMS registrations.

### 5.5.2	Architectural Requirements

1)	SGW relocation: LMISF will need to be made aware of the handover by MME or by the new BBIFF and then trigger the target SGW to continue LI.

2)	MME relocations have no bearing on IMS as such, LI for LTE already needs to be able to deal with MME relocations.

************************************end of last change*****************************************