3GPP TSG-SA3LI Meeting #64	S3i160521

# San Antonio, Texas, The USA, October 25-28, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | 0341 | rev |  | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ASN.1 tag number correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (PIDS) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI13 |  |  |  |  |  | Date: |  |  | 18/10/2016 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Incorrect tag number causes decoding errors |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changing tag number back to correct value |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Decoding errors can accure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

****** FIRST CHANGE ******

# B.9	Intercept related information (HI2 SAE/EPS and IMS)

Declaration of ROSE operation eps-sending-of-IRI is ROSE delivery mechanism specific. When using FTP delivery mechanism, data EpsIRIsContent must be considered.

# ASN1 description of IRI (HI2 interface)

EpsHI2Operations {itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulintercept(2) threeGPP(4) hi2eps(8) r13(13) version-3 (3)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t(2) remote-operations(4) informationObjects(5) version1(0)}

LawfulInterceptionIdentifier,

TimeStamp,

Network-Identifier,

National-Parameters,

National-HI2-ASN1parameters,

DataNodeAddress,

IPAddress,

IP-value,

X25Address

FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulIntercept(2) hi2(1) version18(18)} -- Imported from TS 101 671v3.12.1

CivicAddress

FROM UmtsHI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulintercept(2) threeGPP(4) hi2(1)  r13 (13) version-0 (0)};

-- Imported from 3GPP TS 33.108, UMTS PS HI2

-- Object Identifier Definitions

-- Security DomainId

lawfulInterceptDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0)

securityDomain(2) lawfulIntercept(2)}

-- Security Subdomains

threeGPPSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP(4)}

hi2epsDomainId OBJECT IDENTIFIER	::= {threeGPPSUBDomainId hi2eps(8) r13(13) version-3 (3)}

eps-sending-of-IRI  OPERATION ::=

{

ARGUMENT	EpsIRIsContent

ERRORS		{ OperationErrors }

CODE		global:{threeGPPSUBDomainId hi2eps(8) opcode(1)}

}

-- Class 2 operation . The timer shall be set to a value between 3 s and 240 s.

-- The timer.default value is 60s.

-- NOTE:	The same note as for HI management operation applies.

EpsIRIsContent		::= CHOICE

{

epsiRIContent			EpsIRIContent,

epsIRISequence			EpsIRISequence

}

EpsIRISequence		::= SEQUENCE OF EpsIRIContent

-- Aggregation of EpsIRIContent is an optional feature.

-- It may be applied in cases when at a given point in time

-- several IRI records are available for delivery to the same LEA destination.

-- As a general rule, records created at any event shall be sent

-- immediately and not withheld in the DF or MF in order to

-- apply aggragation.

-- When aggregation is not to be applied,

-- EpsIRIContent needs to be chosen.

-- EpsIRIContent includes events that correspond to EPS and UMTS/GPRS.

EpsIRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,	-- include at least one optional parameter

iRI-End-record			[2] IRI-Parameters,

iRI-Continue-record		[3] IRI-Parameters,	-- include at least one optional parameter

iRI-Report-record		[4] IRI-Parameters	-- include at least one optional parameter

}

-- the EpsIRIContent may provide events that correspond to UMTS/GPRS as well.

unknown-version 		ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter-value	ERROR ::= { CODE local:2}

unknown-parameter		ERROR ::= { CODE local:3}

OperationErrors ERROR ::=

{

unknown-version |

missing-parameter |

unknown-parameter-value |

unknown-parameter

}

-- These values may be sent by the LEMF, when an operation or a parameter is misunderstood.

-- Parameters having the same tag numbers must be identical in Rel-5 and onwards modules.

IRI-Parameters		::= SEQUENCE

{

hi2epsDomainId				[0]	OBJECT IDENTIFIER,  -- 3GPP HI2 EPS domain

lawfulInterceptionIdentifier	[1] LawfulInterceptionIdentifier,

-- This identifier is associated to the target.

timeStamp				[3] TimeStamp,

-- date and time of the event triggering the report.)

initiator 				[4] ENUMERATED

{

not-Available		(0),

originating-Target	(1),

-- in case of GPRS, this indicates that the PDP context activation, modification

-- or deactivation is MS requested

-- in case of EPS, this indicated that the EPS detach, bearer activation, modification

-- or deactivation is UE requested

terminating-Target	(2),

-- in case of GPRS, this indicates that the PDP context activation, modification or

-- deactivation is network initiated

-- in case of EPS, this indicated that the EPS detach, bearer activation, modification

-- or deactivation is network initiated

...

} OPTIONAL,

locationOfTheTarget		[8] Location OPTIONAL,

-- location of the target

partyInformation 		[9] SET SIZE (1..10) OF PartyInformation OPTIONAL,

-- This parameter provides the concerned party, the identiy(ies) of the party

--)and all the information provided by the party.

serviceCenterAddress	[13] PartyInformation OPTIONAL,

-- e.g. in case of SMS message this parameter provides the address of  the relevant

-- server

sMS						[14] SMS-report OPTIONAL,

-- this parameter provides the SMS content and associated information

national-Parameters		[16] National-Parameters OPTIONAL,

ePSCorrelationNumber	[18] EPSCorrelationNumber OPTIONAL,

-- this parameter provides GPRS Correlation number when the event corresponds to UMTS/GPRS.

ePSevent 				[20] EPSEvent OPTIONAL,

-- This information is used to provide particular action of the target

-- such as attach/detach

sgsnAddress 			[21] DataNodeAddress OPTIONAL,

gPRSOperationErrorCode 	[22] GPRSOperationErrorCode OPTIONAL,

ggsnAddress 			[24] DataNodeAddress OPTIONAL,

qOS						[25] UmtsQos OPTIONAL,

networkIdentifier		[26] Network-Identifier OPTIONAL,

sMSOriginatingAddress 	[27] DataNodeAddress OPTIONAL,

sMSTerminatingAddress 	[28] DataNodeAddress OPTIONAL,

iMSevent				[29] IMSevent OPTIONAL,

sIPMessage				[30] OCTET STRING  OPTIONAL,

servingSGSN-number		[31] OCTET STRING (SIZE (1..20))	OPTIONAL,

servingSGSN-address		[32] OCTET STRING (SIZE (5..17)) 	OPTIONAL,

-- Octets are coded according to 3GPP TS 23.003 [25]

...,

-- Tag			[33] was taken into use by ETSI module in TS 101 671v2.13.1

ldiEvent	 			[34] LDIevent OPTIONAL,

correlation 			[35] CorrelationValues OPTIONAL,

ePS-GTPV2-specificParameters   [36] EPS-GTPV2-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of GTPV2 based intercepted messages

ePS-PMIP-specificParameters    [37] EPS-PMIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of PMIP based intercepted messages

ePS-DSMIP-SpecificParameters   [38] EPS-DSMIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of DSMIP based intercepted messages

ePS-MIP-SpecificParameters     [39] EPS-MIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of MIP based intercepted messages

servingNodeAddress             [40] OCTET STRING               OPTIONAL,

-- this parameter is kept for backward compatibility only and should not be used

-- as it has been superseeded by parameter visitedNetworkId

visitedNetworkId               [41] UTF8String                 OPTIONAL,

-- contains the visited network identifier inside the EPS Serving System Update for

-- non 3GPP access, coded according to [53]

mediaDecryption-info	       [42] MediaDecryption-info OPTIONAL,

servingS4-SGSN-address	       [43] OCTET STRING OPTIONAL,

-- Diameter Origin-Host and Origin-Realm of the S4-SGSN based on the TS 29.272 [59].

-- Only the data fields from the Diameter AVPs are provided concatenated

-- with a semicolon to populate this field.

sipMessageHeaderOffer   [44] OCTET STRING OPTIONAL,

sipMessageHeaderAnswer  [45] OCTET STRING OPTIONAL,

sdpOffer                [46] OCTET STRING OPTIONAL,

sdpAnswer               [47] OCTET STRING OPTIONAL,

uLITimestamp            [48] OCTET STRING (SIZE (8)) OPTIONAL,

-- Coded according to 3GPP TS 29.060 [17]; Only the ULI Timestamp value is reported.

packetDataHeaderInformation		   [49] PacketDataHeaderInformation	OPTIONAL,

mediaSecFailureIndication          [50] MediaSecFailureIndication OPTIONAL,

csgIdentity			[51] OCTET STRING (SIZE (4)) OPTIONAL,  -- Octets are coded

-- according to 3GPP TS 23.003 [25].  The 27 bits specified in TS 23.003 shall be encoded as.

-- follows The most significant bit of the CSG Identity shall be encoded in the most

-- significant bit of the first octet of the octet string and the least significant bit coded

-- in bit 6 of octet 4.

heNBIdentity			[52] OCTET STRING OPTIONAL,

-- 4 or 6 octets are coded with the HNBUnique Identity

-- as specified in 3GPP TS 23.003            [25], Clause 4.10.

heNBiPAddress		[53] IPAddress  OPTIONAL,

heNBLocation			[54] HeNBLocation  OPTIONAL,

tunnelProtocol		[55] TunnelProtocol  OPTIONAL,

pANI-Header-Info		[56] SEQUENCE OF PANI-Header-Info		OPTIONAL,

-- information extracted from P-Access-Network-Info headers of SIP message;

-- described in TS 24.229 §7.2A.4 [76]

imsVoIP 				[57] IMS-VoIP-Correlation					OPTIONAL,

xCAPmessage			[58] OCTET STRING OPTIONAL,

-- The HTTP message (HTPP header and any XCAP body) of any of the target’s IMS supplementary

-- service setting management or manipulation XCAP messages occuring through the Ut interface

-- defined in the 3GPP TS 24 623 [77].

logicalFunctionInformation	[59] DataNodeIdentifier OPTIONAL,

ccUnavailableReason		[60] PrintableString	OPTIONAL,

carrierSpecificData					[61] OCTET STRING OPTIONAL,

-- Copy of raw data specified by the CSP or his vendor related to HSS.

current-previous-systems			[62] Current-Previous-Systems OPTIONAL,

change-Of-Target-Identity				[63] Change-Of-Target-Identity OPTIONAL,

requesting-Network-Identifier		[64] OCTET STRING OPTIONAL,

-- the requesting network identifier PLMN id (Mobile Country Code and Mobile Network Country,

-- defined in E212 [87]).

requesting-Node-Type					[65] Requesting-Node-Type OPTIONAL,

serving-System-Identifier				[66] OCTET STRING OPTIONAL,

-- the serving network identifier PLMN id (MNC, Mobile Country Code and MNC,Mobile Network

-- Country, defined in E212 [87]) and 3GPP TR 21.905 [38], that may be included in the Diameter

-- AVP to and from the HSS.

national-HI2-ASN1parameters	[255]	National-HI2-ASN1parameters	OPTIONAL

}

-- Parameters having the same tag numbers must be identical in Rel-5 and onwards modules

-- PARAMETERS FORMATS

DataNodeIdentifier ::= SEQUENCE

{

dataNodeAddress		[1] DataNodeAddress OPTIONAL,

logicalFunctionType	[2] LogicalFunctionType OPTIONAL,

dataNodeName		[3] PrintableString(SIZE(7..25)) OPTIONAL,

--Unique identifier of a Data Node within the CSP domain. Could be a name/number combination.

...

}

LogicalFunctionType ::= ENUMERATED

{

pDNGW		(0),

mME			(1),

sGW			(2),

ePDG		(3),

hSS			(4),

...

}

PANI-Header-Info	::= SEQUENCE

{

access-Type 		[1] OCTET STRING 		OPTIONAL,

-- ASCII chain ‘3GPP-UTRAN-TDD’, ‘3GPP-E-UTRAN-TDD’,... : see TS 24.229 §7.2A.4 [76]

access-Class  		[2] OCTET STRING		OPTIONAL,

-- ASCII chain ‘3GPP-UTRAN’, ‘3GPP-E-UTRAN’,... : see TS 24.229 §7.2A.4 [76]

network-Provided	[3] NULL				OPTIONAL,

-- present if provided by the network

pANI-Location		[4] PANI-Location 		OPTIONAL,

...

}

PANI-Location  ::= SEQUENCE

{

raw-Location     [1] OCTET STRING 	OPTIONAL,

-- raw copy of the location string from the P-Access-Network-Info header

location          [2] Location      OPTIONAL,

ePSLocation       [3] EPSLocation   OPTIONAL,

...

}

PartyInformation 			::= SEQUENCE

{

party-Qualifier 	[0]  ENUMERATED

{

gPRSorEPS-Target(3),

...

},

partyIdentity 		[1] SEQUENCE

{

imei					[1] OCTET STRING (SIZE (8)) OPTIONAL,

-- See MAP format [4]

imsi					[3] OCTET STRING (SIZE (3..8)) OPTIONAL,

-- See MAP format [4] International Mobile

-- Station Identity E.212 number beginning with Mobile Country Code

msISDN					[6] OCTET STRING (SIZE (1..9)) OPTIONAL,

-- MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document TS 29.002 [4]

e164-Format				[7] OCTET STRING    (SIZE (1 .. 25)) OPTIONAL,

-- E164 address of the node in international format. Coded in the same format as

-- the calling party number  parameter of the ISUP (parameter part:[29])

sip-uri					[8] OCTET STRING 	OPTIONAL,

-- See [26]

...,

tel-uri					[9] OCTET STRING 	OPTIONAL,

-- See [67]

nai                    [10] OCTET STRING    OPTIONAL,

-- NAI of the target, encoded in the same format as defined by [EPS stage 3 specs]

x-3GPP-Asserted-Identity [11] OCTET STRING 	OPTIONAL,

-- X-3GPP-Asserted-Identity header (3GPP TS 24 109 [79]) of the target, used in

-- some XCAP transactions as a complement information to SIP URI or Tel URI.

xUI						[12] OCTET STRING 	OPTIONAL

-- XCAP User Identifier (XUI)is a string, valid as a path element in an XCAP URI, that is

-- may be associated with each user served by a XCAP resource server. Defined in IETF RFC

-- 4825[80] as a complement information to SIP URI or Tel URI.

},

services-Data-Information 	[4] Services-Data-Information OPTIONAL,

-- This parameter is used to transmit all the information concerning the

-- complementary information associated to the basic data call

...

}

Location	::= SEQUENCE

{

e164-Number         [1] OCTET STRING (SIZE (1..25)) OPTIONAL,
        -- Coded in the same format as the ISUP location number (parameter
        -- field) of the ISUP (see EN 300 356 [30]).    
	globalCellID		[2] GlobalCellID	OPTIONAL,

--see MAP format (see [4])

rAI					[4] Rai		OPTIONAL,

-- the Routeing Area Identifier in the current  SGSN is coded in accordance with the

-- § 10.5.5.15 of document [9] without the Routing Area Identification IEI

-- (only the last 6 octets are used)

gsmLocation			[5] GSMLocation OPTIONAL,

umtsLocation		[6] UMTSLocation OPTIONAL,

sAI					[7] Sai	OPTIONAL,

-- format:	PLMN-ID	3 octets (no. 1 – 3)

--			LAC		2 octets (no. 4 – 5)

--			SAC		2 octets (no. 6 – 7)

--			(according to 3GPP TS 25.413 [62])

...,

oldRAI				[8] Rai		OPTIONAL,

-- the Routeing Area Identifier in the old SGSN is coded in accordance with the

-- § 10.5.5.15 of document [9] without the Routing Area Identification IEI

-- (only the last 6 octets are used).

civicAddress		[9] CivicAddress OPTIONAL

}

GlobalCellID	::= OCTET STRING  (SIZE (5..7))

Rai				::= OCTET STRING (SIZE (6))

Sai				::=	OCTET STRING (SIZE (7))

GSMLocation 	::= CHOICE

{

geoCoordinates 	[1] SEQUENCE

{

latitude		[1]	PrintableString (SIZE(7..10)),

-- format : 	XDDMMSS.SS

longitude		[2]	PrintableString (SIZE(8..11)),

-- format : 	XDDDMMSS.SS

mapDatum		[3]	MapDatum DEFAULT wGS84,

...,

azimuth			[4] INTEGER (0..359) OPTIONAL

-- The azimuth is the bearing, relative to true north.

},

-- format : 	XDDDMMSS.SS

-- 				X 			: N(orth), S(outh), E(ast), W(est)

--				DD or DDD 	: degrees (numeric characters)

--				MM			: minutes (numeric characters)

--				SS.SS 		: seconds, the second part (.SS) is optionnal

-- Example :

--			latitude short form		N502312

--			longitude long form		E1122312.18

utmCoordinates	[2] SEQUENCE

{

utm-East 		[1] PrintableString (SIZE(10)),

utm-North 		[2] PrintableString (SIZE(7)),

-- example	utm-East	32U0439955

--			utm-North	5540736

mapDatum		[3]	MapDatum DEFAULT wGS84,

...,

azimuth			[4] INTEGER (0..359) OPTIONAL

-- The azimuth is the bearing, relative to true north.

},

utmRefCoordinates  	[3] SEQUENCE

{

utmref-string		PrintableString (SIZE(13)),

mapDatum			MapDatum DEFAULT wGS84,

...

},

-- example	32UPU91294045

wGS84Coordinates  	[4] OCTET STRING

-- format is as defined in [37].

}

MapDatum ::= ENUMERATED

{

wGS84,

wGS72,

eD50, 	-- European Datum 50

...

}

UMTSLocation ::= CHOICE {

point					[1]	GA-Point,

pointWithUnCertainty	[2]	GA-PointWithUnCertainty,

polygon					[3]	GA-Polygon

}

GeographicalCoordinates ::= SEQUENCE {

latitudeSign			ENUMERATED { north, south },

latitude				INTEGER (0..8388607),

longitude				INTEGER (-8388608..8388607),

...

}

GA-Point ::= SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

GA-PointWithUnCertainty ::=SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

uncertaintyCode				INTEGER (0..127)

}

maxNrOfPoints						INTEGER ::= 15

GA-Polygon ::= SEQUENCE (SIZE (1..maxNrOfPoints)) OF

SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

SMS-report		::= SEQUENCE

{

sMS-Contents	[3] SEQUENCE

{

sms-initiator		[1] ENUMERATED 	-- party which sent the  SMS

{

target			(0),

server			(1),

undefined-party	(2),

...

},

transfer-status		[2] ENUMERATED

{

succeed-transfer	(0), 		-- the transfer of the SMS message succeeds

not-succeed-transfer(1),

undefined			(2),

...

} OPTIONAL,

other-message		[3] ENUMERATED 	-- in case of terminating call, indicates if

-- the server will send other SMS

{

yes			(0),

no			(1),

undefined	(2),

...

} OPTIONAL,

content				[4] OCTET STRING (SIZE (1 .. 270)) OPTIONAL,

-- Encoded in the format defined for the SMS mobile

...

}

}

EPSCorrelationNumber ::= OCTET STRING

-- In case of PS interception, the size will be in the range (8..20)

CorrelationValues ::= CHOICE {

iri-to-CC 	[0]		IRI-to-CC-Correlation, -- correlates IRI to Content(s)

iri-to-iri	[1]		IRI-to-IRI-Correlation, -- correlates IRI to IRI

both-IRI-CC	[2]		SEQUENCE { -- correlates IRI to IRI and IRI to Content(s)

iri-CC	[0]	IRI-to-CC-Correlation,

iri-IRI	[1]	IRI-to-IRI-Correlation}

}

IMS-VoIP-Correlation ::= SET OF SEQUENCE {

ims-iri		[0]		IRI-to-IRI-Correlation,

ims-cc		[1]		IRI-to-CC-Correlation 		OPTIONAL

}

IRI-to-CC-Correlation ::= SEQUENCE { -- correlates IRI to Content

cc 		[0] SET OF 	OCTET STRING,-- correlates IRI to multiple CCs

iri 	[1] 		OCTET STRING OPTIONAL

-- correlates IRI to CC with signaling

}

IRI-to-IRI-Correlation ::= OCTET STRING -- correlates IRI to IRI

EPSEvent ::= ENUMERATED

{

pDPContextActivation 					(1),

startOfInterceptionWithPDPContextActive	(2),

pDPContextDeactivation					(4),

gPRSAttach 								(5),

gPRSDetach 								(6),

locationInfoUpdate 						(10),

sMS 									(11),

pDPContextModification					(13),

servingSystem							(14),

... ,

startOfInterceptionWithMSAttached		(15),

e-UTRANAttach							(16),

e-UTRANDetach                           (17),

bearerActivation                        (18),

startOfInterceptionWithActiveBearer     (19),

bearerModification                      (20),

bearerDeactivation                      (21),

uERequestedBearerResourceModification   (22),

uERequestedPDNConnectivity              (23),

uERequestedPDNDisconnection             (24),

trackingAreaEpsLocationUpdate           (25),

servingEvolvedPacketSystem              (26),

pMIPAttachTunnelActivation              (27),

pMIPDetachTunnelDeactivation            (28),

startOfInterceptWithActivePMIPTunnel    (29),

pMIPPdnGwInitiatedPdnDisconnection      (30),

mIPRegistrationTunnelActivation         (31),

mIPDeregistrationTunnelDeactivation     (32),

startOfInterceptWithActiveMIPTunnel     (33),

dSMIPRegistrationTunnelActivation       (34),

dSMIPDeregistrationTunnelDeactivation   (35),

startOfInterceptWithActiveDsmipTunnel   (36),

dSMipHaSwitch                           (37),

pMIPResourceAllocationDeactivation      (38),

mIPResourceAllocationDeactivation       (39),

pMIPsessionModification                 (40),

startOfInterceptWithEUTRANAttachedUE    (41),

dSMIPSessionModification                (42),

packetDataHeaderInformation				(43),

hSS-Subscriber-Record-Change			(44),

registration-Termination				(45),

-- FFS

location-Up-Date						(46),

-- FFS

cancel-Location							(47),

register-Location						(48),

location-Information-Request			(49)

}

-- see [19]

IMSevent ::= ENUMERATED

{

unfilteredSIPmessage (1),

-- This value indicates to LEMF that the whole SIP message is sent , i.e. without filtering

-- CC; location information is removed by the DF2/MF if not required to be sent.

...,

sIPheaderOnly (2),

-- If warrant requires only IRI then specific content in a 'sIPMessage'

-- (e.g. 'Message', etc.) has been deleted before sending it to LEMF.

decryptionKeysAvailable (3),

-- This value indicates to LEMF that the IRI carries CC decryption keys for the session

-- under interception.

startOfInterceptionForIMSEstablishedSession (4),

-- This value indicates to LEMF that the IRI carries information related to

-- interception started on an already established IMS session.

xCAPRequest (5),

-- This value indicates to LEMF that the XCAP request is sent.

xCAPResponse (6) ,

-- This value indicates to LEMF that the XCAP response is sent.

ccUnavailable	(7)

-- This value indicates to LEMF that the media is not available for interception for intercept

-- orders that requires media interception.

}

Services-Data-Information ::= SEQUENCE

{

gPRS-parameters [1] GPRS-parameters OPTIONAL,

...

}

GPRS-parameters ::= SEQUENCE

{

pDP-address-allocated-to-the-target 	[1] DataNodeAddress OPTIONAL,

aPN 					[2] OCTET STRING (SIZE(1..100)) OPTIONAL,

-- The Access Point Name (APN) is coded in accordance with

-- 3GPP TS 24.008 [9] without the APN IEI (only the last 100 octets are used).

-- Octets are coded according to 3GPP TS 23.003 [25].

pDP-type 				[3] OCTET STRING (SIZE(2)) OPTIONAL,

-- Include either Octets 3 and 4 of the Packet Data Protocol Address information element

-- of 3GPP TS 24.008 [9] or Octets 4 and 5 of the End User Address IE of 3GPP TS 29.060 [17].

-- when PDP-type is IPv4 or IPv6, the IP address is carried by parameter

-- pDP-address-allocated-to-the-target

-- when PDP-type is IPv4v6, the additional IP address is carried by parameter

-- additionalIPaddress

...,

nSAPI					[4] OCTET STRING (SIZE (1)) OPTIONAL,

-- Include either Octet 2 of the NSAPI IE of 3GPP TS 24.008 [9]

-- or Octet 2 of the NSAPI IE of 3GPP TS 29.060 [17].

additionalIPaddress		[5] DataNodeAddress OPTIONAL

}

GPRSOperationErrorCode ::= OCTET STRING

-- The parameter shall carry the GMM cause value or the SM cause value, as defined in the

-- standard [9], without the IEI.

LDIevent ::= ENUMERATED

{

targetEntersIA 			(1),

targetLeavesIA			(2),

...

}

UmtsQos ::= CHOICE

{

qosMobileRadio [1] OCTET STRING,

-- The qosMobileRadio parameter shall be coded in accordance with the § 10.5.6.5 of

-- document [9] without the Quality of service IEI and Length of

-- quality of service IE (. That is, first

-- two octets carrying 'Quality of service IEI' and 'Length of quality of service

-- IE' shall be excluded).

qosGn [2] OCTET STRING

-- qosGn parameter shall be coded in accordance with § 7.7.34 of document [17]

}

EPS-GTPV2-SpecificParameters ::= SEQUENCE

{

pDNAddressAllocation                 [1]   OCTET STRING                       OPTIONAL,

aPN                                  [2]   OCTET STRING (SIZE (1..100))       OPTIONAL,

protConfigOptions                    [3]   ProtConfigOptions                  OPTIONAL,

attachType                           [4]   OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

ePSBearerIdentity                    [5]   OCTET STRING                       OPTIONAL,

detachType                           [6]   OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47], includes switch off indicator

rATType                              [7]   OCTET STRING (SIZE (1))            OPTIONAL,

failedBearerActivationReason         [8]   OCTET STRING (SIZE (1))            OPTIONAL,

ePSBearerQoS                         [9]   OCTET STRING                       OPTIONAL,

bearerActivationType                 [10]  TypeOfBearer                       OPTIONAL,

aPN-AMBR                             [11]  OCTET STRING                       OPTIONAL,

-- see 3GPP TS 29.274 [46] parameters coding rules defined for EPS-GTPV2-SpecificParameters.

procedureTransactionId               [12]  OCTET STRING                       OPTIONAL,

linkedEPSBearerId                    [13]  OCTET STRING                       OPTIONAL,

--The Linked EPS Bearer Identity shall be included and coded according to 3GPP TS 29.274 [46].

tFT                                  [14]  OCTET STRING                       OPTIONAL,

-- Only octets 3 onwards of TFT IE from 3GPP TS 24.008 [9] shall be included.

handoverIndication                   [15]  NULL                               OPTIONAL,

failedBearerModReason                [16]  OCTET STRING (SIZE (1))            OPTIONAL,

trafficAggregateDescription          [17]  OCTET STRING                       OPTIONAL,

failedTAUReason                      [18]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

failedEUTRANAttachReason             [19]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

servingMMEaddress                    [20]  OCTET STRING                       OPTIONAL,

-- Contains the data fields from the Diameter Origin-Host and Origin-Realm AVPs

-- as received in the HSS from the MME according to the TS 29.272 [59].

-- Only the data fields from the Diameter AVPs are provided concatenated

-- with a semicolon to populate this field.

bearerDeactivationType               [21]  TypeOfBearer                       OPTIONAL,

bearerDeactivationCause              [22]  OCTET STRING (SIZE (1))            OPTIONAL,
    ePSlocationOfTheTarget               [23]  EPSLocation                        OPTIONAL,
      -- the use of ePSLocationOfTheTarget is mutually exclusive with the use of locationOfTheTarget

-- ePSlocationOfTheTarget allows using the coding of the parameter according to SAE stage 3.

...,

pDNType                              [24]   OCTET STRING (SIZE (1))           OPTIONAL,

-- coded according to TS 24.301 [47]

requestType                          [25]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

uEReqPDNConnFailReason               [26]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

extendedHandoverIndication			 [27]  OCTET STRING (SIZE (1)) 			  OPTIONAL,

-- This parameter with value 1 indicates handover based on the flags in the TS 29.274 [46].

-- Otherwise set to the value 0.

-- The use of extendedHandoverIndication and handoverIndication parameters is

-- mutually exclusive and depends on the actual ASN.1 encoding method.

uLITimestamp                         [28]  OCTET STRING (SIZE (8))            OPTIONAL,

uELocalIPAddress                     [29]  OCTET STRING                       OPTIONAL,

uEUdpPort                            [30]  OCTET STRING (SIZE (2))            OPTIONAL,

tWANIdentifier                       [31]  OCTET STRING                       OPTIONAL,

tWANIdentifierTimestamp              [32]  OCTET STRING (SIZE (4))            OPTIONAL

}

-- All the parameters within EPS-GTPV2-SpecificParameters are coded as the corresponding IEs

-- without the octets containing type and length. Unless differently stated, they are coded

-- according to 3GPP TS 29.274 [46]; in this case the octet containing the instance

-- shall also be not included.

TypeOfBearer ::= ENUMERATED

{

defaultBearer          (1),
    dedicatedBearer        (2),

...

}

EPSLocation	::= SEQUENCE

{

userLocationInfo    [1] OCTET STRING (SIZE (1..39)) OPTIONAL,
        -- see 3GPP TS 29.274 [46] parameters coding rules defined for EPS-GTPV2-SpecificParameters.

gsmLocation			[2] GSMLocation OPTIONAL,

umtsLocation		[3] UMTSLocation OPTIONAL,

olduserLocationInfo	[4] OCTET STRING (SIZE (1..39))	OPTIONAL,

-- coded in the same way as userLocationInfo

lastVisitedTAI	    [5] OCTET STRING (SIZE (1..5))	OPTIONAL,

-- the Tracking Area Identity is coded in accordance with the TAI field in 3GPP TS 29.274
        -- [46].

tAIlist			    [6] OCTET STRING (SIZE (7..97))	OPTIONAL,

-- the TAI List is coded acording to 3GPP TS 24.301 [47], without the TAI list IEI

...,

threeGPP2Bsid		[7] OCTET STRING (SIZE (1..12)) OPTIONAL,

-- contains only the payload from the 3GPP2-BSID AVP described in the 3GPP TS 29.212 [56].

civicAddress		[8] CivicAddress OPTIONAL

}

ProtConfigOptions ::= SEQUENCE

{

ueToNetwork                           [1]  OCTET STRING (SIZE(1..251))        OPTIONAL,

-- This shall be coded with octet 3 onwards of the Protocol Configuration Options IE in

-- accordance with 3GPP TS 24.008 [9].

networkToUe                           [2]  OCTET STRING (SIZE(1..251))        OPTIONAL,

-- This shall be coded with octet 3 onwards of the Protocol Configuration Options IE in

-- accordance with 3GPP TS 24.008 [9].

...

}

EPS-PMIP-SpecificParameters ::= SEQUENCE

{

lifetime                              [1]  INTEGER (0..65535)                 OPTIONAL,

accessTechnologyType                  [2]  OCTET STRING (SIZE (4))            OPTIONAL,

aPN                                   [3]  OCTET STRING (SIZE (1..100))       OPTIONAL,

iPv6HomeNetworkPrefix                 [4]  OCTET STRING (SIZE (20))           OPTIONAL,

protConfigurationOption               [5]  OCTET STRING                       OPTIONAL,

handoverIndication                    [6]  OCTET STRING (SIZE (4))            OPTIONAL,

status                                [7]  INTEGER (0..255)                   OPTIONAL,

revocationTrigger                     [8]  INTEGER (0..255)                   OPTIONAL,

iPv4HomeAddress                       [9]  OCTET STRING (SIZE (4))            OPTIONAL,

iPv6careOfAddress                     [10] OCTET STRING                       OPTIONAL,

iPv4careOfAddress                     [11] OCTET STRING                       OPTIONAL,

...,

servingNetwork                        [12] OCTET STRING (SIZE (3))            OPTIONAL,

dHCPv4AddressAllocationInd            [13] OCTET STRING (SIZE (1))            OPTIONAL,

ePSlocationOfTheTarget                [14] EPSLocation                        OPTIONAL

-- parameters coded according to 3GPP TS 29.275 [48] and RFCs specifically

-- referenced in it.

}

EPS-DSMIP-SpecificParameters ::= SEQUENCE

{

lifetime                              [1]   INTEGER (0..65535)                OPTIONAL,

requestedIPv6HomePrefix               [2]   OCTET STRING (SIZE (25))          OPTIONAL,

-- coded according to RFC 5026

homeAddress                           [3]   OCTET STRING (SIZE (8))           OPTIONAL,

iPv4careOfAddress                     [4]   OCTET STRING (SIZE (8))           OPTIONAL,

iPv6careOfAddress                     [5]   OCTET STRING (SIZE(16))           OPTIONAL,

aPN                                   [6]   OCTET STRING (SIZE (1..100))      OPTIONAL,

status                                [7]   INTEGER (0..255)                  OPTIONAL,

hSS-AAA-address                       [8]   OCTET STRING                      OPTIONAL,

targetPDN-GW-Address                  [9]   OCTET STRING                      OPTIONAL,

...

-- parameters coded according to 3GPP TS 24.303 [49] and RFCs specifically

-- referenced in it.

}

EPS-MIP-SpecificParameters ::= SEQUENCE

{

lifetime                              [1]   INTEGER (0.. 65535)               OPTIONAL,

homeAddress                           [2]   OCTET STRING (SIZE (4))           OPTIONAL,

careOfAddress                         [3]   OCTET STRING (SIZE (4))           OPTIONAL,

homeAgentAddress                      [4]   OCTET STRING (SIZE (4))           OPTIONAL,

code                                  [5]   INTEGER (0..255)                  OPTIONAL,

foreignDomainAddress                  [7]   OCTET STRING (SIZE (4))           OPTIONAL,

...

-- parameters coded according to 3GPP TS 29.279 [63] and RFCs specifically

-- referenced in it.

}

MediaDecryption-info ::= SEQUENCE OF CCKeyInfo

-- One or more key can be available for decryption, one for each media streams of the

-- intercepted session.

CCKeyInfo ::= SEQUENCE

{

cCCSID	 [1]	OCTET STRING,

-- the parameter uniquely mapping the key to the encrypted stream.

cCDecKey [2]	OCTET STRING,

cCSalt   [3]    OCTET STRING OPTIONAL,

-- The field reports the value from the CS_ID field in the ticket exchange headers as

-- defined in IETF RFC 6043 [61].

...

}

MediaSecFailureIndication  ::= ENUMERATED

{

genericFailure  (0),

...

}

PacketDataHeaderInformation ::= CHOICE

{

packetDataHeader		[1]	PacketDataHeaderReport,

packetDataSummary	[2]	PacketDataSummaryReport,

...

}

PacketDataHeaderReport ::= CHOICE

{

packetDataHeaderMapped	[1]	PacketDataHeaderMapped,

packetDataHeaderCopy	[2]	PacketDataHeaderCopy,

...

}

PacketDataHeaderMapped ::= SEQUENCE

{

sourceIPAddress			[1] IPAddress,

sourcePortNumber		[2] INTEGER (0..65535) OPTIONAL,

destinationIPAddress	[3] IPAddress,

destinationPortNumber	[4] INTEGER (0..65535) OPTIONAL,

transportProtocol		[5] INTEGER,

-- For IPv4, report the “Protocol” field and for IPv6 report “Next Header” field.

-- Assigned Internet Protocol Numbers can be found at

-- http://www.iana.org/assignments/protocol-numbers/protocol-numbers.xml

packetsize				[6] INTEGER OPTIONAL,

flowLabel				[7] INTEGER OPTIONAL,

packetCount				[8] INTEGER OPTIONAL,

direction				[9] TPDU-direction,

...

}

TPDU-direction ::= ENUMERATED

{

from-target 	(1),

to-target 		(2),

unknown 		(3)

}

PacketDataHeaderCopy ::= SEQUENCE

{

direction				[1] TPDU-direction,

headerCopy				[2] OCTET STRING,	-- includes a copy of the packet header at the IP

-- network layer and above including extension headers, but excluding contents.

...

}

PacketDataSummaryReport ::= SEQUENCE OF PacketFlowSummary

PacketFlowSummary ::= SEQUENCE

{

sourceIPAddress			[1] IPAddress,

sourcePortNumber		[2] INTEGER (0..65535) OPTIONAL,

destinationIPAddress	[3] IPAddress,

destinationPortNumber	[4] INTEGER (0..65535) OPTIONAL,

transportProtocol		[5] INTEGER,

-- For IPv4, report the “Protocol” field and for IPv6 report “Next Header” field.

-- Assigned Internet Protocol Numbers can be found at

-- http://www.iana.org/assignments/protocol-numbers/protocol-numbers.xml

flowLabel				[6] INTEGER OPTIONAL,

summaryPeriod			[7] ReportInterval,

packetCount				[8] INTEGER,

sumOfPacketSizes		[9] INTEGER,

packetDataSummaryReason	[10] ReportReason,

...

}

ReportReason ::= ENUMERATED

{

timerExpired			(0),

countThresholdHit		(1),

pDPComtextDeactivated   (2),

pDPContextModification	(3),

otherOrUnknown			(4),

...

}

ReportInterval ::= SEQUENCE

{

firstPacketTimeStamp	[0] TimeStamp,

lastPacketTimeStamp		[1] TimeStamp,

...

}

TunnelProtocol ::= CHOICE

{

rfc2868ValueField [0] OCTET STRING, -- coded to indicate the type of tunnel established between

-- the HeNB and the SeGW as specified in TS 33.320.  The actual coding is provided in 3 octets

-- with the Value field of the Tunnel Type RADIUS attribute as specified in IETF RFC 2868.

-- This corresponds to the outer layer tunnel between the HeNB and the SeGW as viewed by the

-- SeGW

nativeIPSec       [1] NULL, -- if native IPSec is required by TS 33.320 between HeNB and SeGW

...

}

HeNBLocation ::= EPSLocation

Requesting-Node-Type ::= ENUMERATED

{

mSC				(1),

sMS-Centre		(2),

gMLC			(3),

mME				(4),

sGSN			(5),

...

}

Change-Of-Target-Identity ::= SEQUENCE

{

new-MSISDN								[1] PartyInformation OPTIONAL,

-- new MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document TS 29.002 [4]

new-AMSISDN				[2] PartyInformation OPTIONAL,

-- new A MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document TS 29.002 [4]

old-MSISDN				[3] PartyInformation OPTIONAL,

-- new MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document TS 29.002 [4]

old-AMSISDN				[4] PartyInformation OPTIONAL,

-- new A MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document TS 29.002 [4]

new-IMSI 								[5] PartyInformation OPTIONAL,

-- See MAP format [4] International Mobile

-- Station Identity E.212 number beginning with Mobile Country Code

old-IMSI 								[6] PartyInformation OPTIONAL,

-- See MAP format [4] International Mobile

-- Station Identity E.212 number beginning with Mobile Country Code

new-IMEI 								[7] PartyInformation OPTIONAL,

-- See MAP format [4] International Mobile

-- Equipement Identity defined in MAP format document TS 29.002 [4]

old-IMEI 								[8] PartyInformation OPTIONAL,

-- See MAP format [4] International Mobile

-- Equipement Identity defined in MAP format document TS 29.002 [4]

...

}

Current-Previous-Systems ::= SEQUENCE

{

serving-System-Identifier			[1] OCTET STRING OPTIONAL,

-- VPLMN id (Mobile Country Code and Mobile Network Country, E. 212 number [87]).

current-Serving-MME-Address			[2] DataNodeIdentifier OPTIONAL,

-- The IP address of the serving SGSN or the Diameter Origin-Host and Origin-Realm of the

-- serving node.

previous-Serving-System-Identifier		[3] OCTET STRING OPTIONAL,

-- VPLMN id (Mobile Country Code and Mobile Network Country, defined in E212 [87]).

previous-Serving-MME-Address			[4] DataNodeIdentifier OPTIONAL,

-- The IP address of the previous serving node or its Diameter Origin-Host and Origin-Realm.

...

}

END -- OF EpsHI2Operations

****** END OF FIRST CHANGE ******

****** START OF SECOND CHANGE ******

****** END OF SECOND CHANGE ******

****** END OF ALL CHANGES ******