3GPP TSG-SA3 Meeting #64	s3i170025

# Sophia-Antipolis, France, 24th Jan 2017 - 27th Jan 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | 0347 | rev | - | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Separate Delivery of MMS - Handover Details |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  |  |  |  |  | Date: |  |  | 2017-01-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI TS 101 331: "Lawful Interception (LI); Requirements of Law Enforcement Agencies".

[2]	ETSI ES 201 158: "Telecommunications security; Lawful Interception (LI); Requirements for network functions".

[3]	ETSI ETR 330: "Security Techniques Advisory Group (STAG); A guide to legislative and regulatory environment".

[4]	3GPP TS 29.002: "3rd Generation Partnership Project; Technical Specification Group Core Network; Mobile Application Part (MAP) specification".

[5A]	ITU-T Recommendation X.680: "Abstract Syntax Notation One (ASN.1): Specification of Basic Notation".

[5B]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Information Object Specification".

[5C]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Constraint Specification".

[5D]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 Specifications".

[6]	ITU-T Recommendation X.690: "ASN.1 encoding rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER)".

NOTE 1:	It is recommended that for [5A], [5B], [5C], [5D] and [6] the 2002 specific versions should be used.

[7]	ITU-T Recommendation X.880: "Information technology - Remote Operations: Concepts, model and notation".

[8]	ITU-T Recommendation X.882: "Information technology - Remote Operations: OSI realizations - Remote Operations Service Element (ROSE) protocol specification".

NOTE 2:	It is recommended that for [8] the 1994 specific versions should be used.

[9]	3GPP TS 24.008: "3GPP Technical Specification Group Core Network; Mobile radio interface Layer 3 specification, Core network protocol; Stage 3".

[10] - [12]	Void.

[13]	IETF STD 9 (RFC 0959): "File Transfer Protocol (FTP)".

[14]	3GPP TS 32.215: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication Management; Charging Management; Charging data description for the Packet Switched (PS) domain)".

[15]	IETF STD0005 (RFC 0791: "Internet Protocol".

[16]	IETF STD0007 (RFC 0793): "Transmission Control Protocol".

[17]	3GPP TS 29.060: "3rd Generation Partnership Project; Technical Specification Group Core Network; General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[18]	3GPP TS 33.106: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful Interception Requirements".

[19]	3GPP TS 33.107: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful interception architecture and functions".

[20]	3GPP TS 23.107: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Quality of Service QoS concepts and architecture".

[21] – [22]	Void.

[23]	ANSI/J-STD-025-A: "Lawfully Authorized Electronic Surveillance".

[24]	ETSI TS 101 671: "Handover Interface for the lawful interception of telecommunications traffic".

[25]	3GPP TS 23.003: "3rd Generation Partnership Project; Technical Specification Group Core Network; Numbering, addressing, and identification".

[26]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[27]	IETF RFC 1006: "ISO Transport Service on top of the TCP".

[28]	IETF RFC 2126: "ISO Transport Service on top of TCP (ITOT)".

[29]	ITU-T Recommendation Q.763: "Signalling System No. 7 - ISDN User Part formats and codes".

[30]	ETSI EN 300 356 (all parts): "Integrated Services Digital Network (ISDN); Signalling System No.7; ISDN User Part (ISUP) version 3 for the international interface".

[31]	ETSI EN 300 403-1 (V1.3.2): "Integrated Services Digital Network (ISDN); Digital Subscriber Signalling System No. one (DSS1) protocol; Signalling network layer for circuit-mode basic call control; Part 1: Protocol specification [ITU-T Recommendation Q.931 (1993), modified]".

NOTE 3:	Reference [31] is specific, because ASN.1 parameter "release-Reason-Of-Intercepted-Call" has the following comment: "Release cause coded in [31] format". In case later version than the given one indicated for ISDN specification ETSI EN 300 403-1 has modified format of the "release cause", keeping the reference version specific allows to take proper actions in later versions of this specification.

[32] - [33]	Void

[34]	ITU-T Recommendation Q.931: "ISDN user-network interface layer 3 specification for basic call control".

[35]	Void.

[36]	Void.

[37]	3GPP TS 23.032: "3rd Generation Partnership Project; Technical Specification Group Core Network; Universal Geographical Area Description (GAD)".

[38]	3GPP TR 21.905: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

[39]	ISO 3166-1: "Codes for the representation of names of countries and their subdivisions - Part 1: Country codes".

[40]	3GPP TS 23.228: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Stage 2".

[41]	3GPP TS 29.234: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals: 3GPP System to Wireless Local Area Network (WLAN) interworking; Stage 3".

[42]	3GPP TS 23.060: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS); Service description".

[43]	3GPP TS 23.234: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[44]	3GPP TS 23.401: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[45]	3GPP TS 23.402: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Architecture enhancements for non-3GPP accesses".

[46]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Access (GPRS) Tunneling Protocol for Control Plane (GTPv2-C); Stage 3".

[47]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[48]	3GPP TS 29.275: "Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunneling protocols; Stage 3".

[49]	3GPP TS 24.303: "Mobility management based on Dual-Stack Mobile IPv6; Stage 3".

[50]	(void)

[51]	(void)

[52]	3GPP TS 24.147: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[53]	3GPP TS 29.273: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[54]	3GPP TS 33.328: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS) media plane security".

[55]	ATIS-0700005 "Lawfully Authorized Electronic Surveillance (LAES) for 3GPP IMS-based VoIP and other Multimedia Services".

[56]	3GPP TS 29.212: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Policy and Charging Control(PCC); Reference points".

[57]	Void.

[58]	IETF RFC 4217: "Securing FTP with TLS".

[59]	3GPP TS 29.272: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Evolved Packet System (EPS); Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[60]	3GPP TS 33.310: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Network Domain Security (NDS); Authentication Framework (AF)".

[61]	IETF RFC 6043: "MIKEY-TICKET: Ticket-Based Modes of Key Distribution in Multimedia Internet KEYing (MIKEY)", available at www.ietf.org

[62]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[63]	3GPP TS 29.279: "Mobile IPv4 (MIPv4) based mobility protocols; Stage 3".

[64]	3GPP TS 29.118: "Mobility Management Entity (MME) –Visitor Location Register (VLR) SGs interface specification"

[65]	ANSI/J-STD-025-B: "Lawfully Authorized Electronic Surveillance", July 17, 2006.

[66]	3GPP TS 24.007: "Mobile Radio Interface Signalling Layer 3; General Aspects".

[67]	IETF RFC 3966: "The Tel URIs for Telephone Numbers", December, 2004.

[68]	IETF RFC 791: "Internet Protocol"

[69]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[70]	IEFT RFC 3697: "IPv6 Flow Label Specification".

[71]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[72]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[73]	ISO.3166-2: International Organization for Standardization, "Codes for the representation of names of countries and their subdivisions - Part 2: Country subdivision code".

[74]	UPS SB42-4: Universal Postal Union (UPU), "International Postal Address Components and Templates".

[75]	ISO 639-1:2002: "Codes for the representation of names of languages -- Part 1: Alpha-2 code".

[76]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[77]	3GPP TS 24.623: "Technical Specification Group Core Network and Terminals; Extensible Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services".

[78]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[79]	3GPP TS 24.109: "Universal Mobile Telecommunications System (UMTS); Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[80]	IETF RFC 4825:"The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[81]	IETF RFC 7254: "A Uniform Resource Name Namespace for the Global System for Mobile Communications Association (GSMA) and the International Mobile station Equipment Identity (IMEI)"

[82]	IETF RFC 7255: "Using the International Mobile station Equipment Identity (IMEI) Uniform Resource Name (URN) as an Instance ID".

[83]	3GPP TS 22.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[84]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[85]	3GPP TS 25.321: "Medium Access Control (MAC) protocol specification".

[86]	3GPP TS 24.371: " Web Real-Time Communications (WebRTC) access to the IP Multimedia (IM) Core Network (CN) subsystem (IMS); Stage 3".

[87]	ITU-T Recommendation E.212: "The international identification plan for public networks and subscriptions".

[AA]	MMS Architecture OMA-AD-MMS-V1_3-20110913-A.

[BB]	Multimedia Messaging Service Encapsulation Protocol OMA-TS-MMS_ENC-V1_3-20110913-A.

[CC]	3GPP TS 22.140: "Multimedia Messaging Service (MMS); Stage 1".

*** END FIRST MODIFICATION ***

*** SECOND MODIFICATION ***

## 15.X	MMS

### 15.X.1	Introduction

LI for Separated Delivery of MMS (see Clause 15.3 of [Y]), the events of Clause 15.3 need to be reported to the LEMF from DF2 and for CC from DF3, respectively.  A Report record shall be utilized to report the IRI events while a CC may be reported using a ULIC header as described in this clause.

### 15.X.2	Identifiers

#### 15.X.2.1	Overview

Specific identifiers are necessary to identify a target for interception uniquely and to correlate between the data, which is conveyed over the different handover interfaces (HI2 and HI3). The identifiers are defined in the subsections below.

For the delivery of CC, the MMS Proxy-Relay provides correlation numbers and target identities to the HI3. The MMS Proxy-Relay reports the IRI associated with the MMS services.

For the delivery of CC and IRI, the MMS Proxy-Relay provides correlation numbers and target identities to the HI2 and HI3. For a given target the correlation number for a given MMS is unique across all MMS involving the target.

NOTE:	If two or more target identities are involved in the same MMS the same Correlation Number may be assigned by the relevant network element to the MMS of the different target identities.

#### 15.X.2.2	Lawful Interception Identifier

For each target identity related to an interception measure, the authorized operator (NO/AN/SP) shall assign a special Lawful Interception Identifier (LIID), which has been agreed between the LEA and the operator (NO/AN/SP).

Using an indirect identification, pointing to a target identity makes it easier to keep the knowledge about a specific target limited within the authorized operator (NO/AN/SP) and the handling agents at the LEA.

The LIID is a component of the CC delivery procedure and of the IRI records. It shall be used within any information exchanged at the handover interfaces HI2 and HI3 for identification and correlation purposes.

The LIID format shall consist of alphanumeric characters. It might for example, among other information, contain a lawful authorization reference number, and the date, when the lawful authorization was issued.

The authorized operator (NO/AN/SP) shall either enter, based on an agreement with each LEA, a unique LIID for each target identity of the target or a single LIID for multiple target identities all pertaining to the same target.

If more than one LEA intercepts the same target identity, there shall be unique LIIDs assigned relating to each LEA.

#### 15.X.2.3	Network Identifier

The network identifier (NID) is a mandatory parameter; it should be internationally unique. It consists of the following two identifiers.

1)	Operator- (NO/AN/SP) identifier (mandatory):
Unique identification of network operator, access network provider or service provider.

2)	Network element identifier NEID (optional):
The purpose of the network element identifier is to uniquely identify the relevant network element carrying out the LI operations, such as LI activation, IRI record sending, etc.

A network element identifier may be an IP address or other identifier. National regulations may mandate the sending of the NEID.

#### 15.X.2.3	Correlation Number

For a given target the Correlation Number is for a given MMS is unique across all MMS involving the target and used for the following purpose:

-	correlate CC with IRI.

NOTE:	The Correlation Number is at a minimum unique for each concurrent communication of a target within a lawful authorization.

### 15.X.3	Timing and quality

#### 15.X.3.1	Timing

As a general principle, within a telecommunication system, IRI, if buffered, should be buffered for as short a time as possible.

NOTE:	If the transmission of IRI fails, it may be buffered or lost.

Subject to national requirements, the following timing requirements shall be supported:

-	Each IRI data record shall be sent by the delivery function to the LEMF over the HI2 within seconds of the detection of the triggering event by the IAP at least 95% of the time.

-	Each IRI data record shall contain a time-stamp, based on the intercepting node's clock that is generated following the detection of the IRI triggering event.

#### 15.X.3.2	Quality

The quality of service associated with the result of interception should be (at least) equal to the highest quality of service of the original content of communication for all participants. This may be derived from the QoS class used for the original intercepted session, TS 23.107 [20]. However, when TCP is used as an OSI layer 4 protocol across the HI3, real time delivery of the result of the interception cannot be guaranteed. The QoS used from the operator (NO/AN/SP) to the LEMF is determined by what operators (NO/AN/SP) and law enforcement agree upon.

### 15.X.4	Security Aspects

#### 15.X.4.1	General

Security is defined by national requirements.

### 15.X.5	Quantitative Aspects

#### 15.X.5.1	General

The number of target interceptions supported is a national requirement.

The area of Quantitative Aspects addresses the ability to perform multiple, simultaneous interceptions within a provider's network and at each of the relevant intercept access points within the network. Specifics related to this topic include:

-	The ability to access and monitor all simultaneous communications originated, received, or redirected by the target;

-	The ability for multiple LEAs (up to five) to monitor, simultaneously, the same target while maintaining unobtrusiveness, including between agencies;

-	The ability of the network to simultaneously support a number of separate (i.e. multiple targets) legally authorized interceptions within its service area(s), including different levels of authorization for each interception (i.e. IRI only, or IRI and communication content), including between agencies.

### 15.X.6	IRI for MMS

#### 15.X.6.1	General

The IRI will in principle be available in the following phases of a MMS service transmission:

1)	When the target sends an MMS;

2)	When the target is notified of an incoming MMS;

3)	When the target retrieves an MMS;

4)	When the target forwards an MMS;

5)	When the target receives notifications regarding a sent MMS;

6)	When the target’s Proxy-Relay server sends notifications to other parties regarding an incoming MMS for the target.

The IRI may be subdivided into the following categories:

1.	Control information for HI2 (e.g. correlation information);

2.	Basic data communication information, for standard data transmission between two parties.

# The events defined in TS 33.107 [19] are used to generate records for the delivery via HI2.

There are multiple different event types received at DF2 level. According to each event, a Record is sent to the LEMF if this is required. The following table gives the mapping between event type received at DF2 level and record type sent to the LEMF.

Table 15.X.6.1.1: Mapping between MMS Events and HI2 records type

| Event | IRI Record Type |
| --- | --- |
| MMS Send | REPORT |
| MMS Notification & Confirmation | REPORT |
| MMS Retrieval Confirmation | REPORT |
| MMS Retrieval Acknowledgement | REPORT |
| MMS Forwarding | REPORT |
| MMS Store Request | REPORT |
| MMS Viewing Request | REPORT |
| MMS Viewing Response | REPORT |
| MMS Deletion Request | REPORT |
| MMS Cancel Request | REPORT |
| MMS Read Report | REPORT |

The Serving System and Serving Evolved Packet System events are reported as per Clause 6.5.1.1 and 10.5.1.1, respectively, of the present document.

A set of information is used to generate the records. The records used transmit the information from mediation function to LEMF. This set of information can be extended in the ICE or DF2 MF, if this is necessary in a specific country. The following table gives the mapping between information received per event and information sent in records.

Table 15.X.6.1.2: Mapping between Events information and IRI information

| Parameter | description | HI2 ASN.1 parameter |
| --- | --- | --- |
| Observed MSISDN | MSISDN of the target | partyInformation |
| Observed IMSI | IMSI of the target | partyInformation |
| Observed IMEI | IMEI of the target | partyInformation |
| Observed MMD Address | An address in a format as specified in [BB]. This is where a SIP URI would be included. | partyInformation |
| Observed IPv4/IPv6 Address | An IPv4 or IPv6 address of the target. | partyInformation |
| Event Type | Description which type of event is delivered: Send, Notification, Notification Response, Retrieval, Retrieval Acknowledgement, Forwarding, Store, Upload, Delete, Delivery, Read Reply From Target, Read Reply To Target, Cancel, View Request, View Confirm. | mMSEvent |
| Event Date | Date of the event generation in the MMS Proxy/Server. | timestamp |
| Event Time | Time of the event generation in the MMS Proxy/Server. Timestamp shall be based on the MMS Proxy/Server internal clock. |  |
| Correlation Number | The correlation number is used to correlate CC and IRI. | mMSCorrelationNumber |
| Adaptation Allowed | Identifies the members of a GCSE communications group who could potentially participate in an active GCSE communications group | adaptationAllowed |
| Applic-ID | Identification of the originating application of the original MM. | applicID |
| Aux Applic Info | Auxiliary application addressing information as indicated in the original MM. | auxApplicInfo |
| BCC Recipients | Address of a recipient; the "BCC" field may include addresses of multiple recipients. When address translation occurs, both the pre and post translated addresses (with appropriate correlation) are included. | mMSParties/bCCAddresses |
| Cancel id | This field includes the Message ID identifying the message to be cancelled. | cancelID |
| Cancel Status | Provides the status of the cancel request. | cancelStatus |
| CC Recipients | Address of a recipient; the "CC" field may include addresses of multiple recipients. When address translation occurs, both the pre and post translated addresses (with appropriate correlation) are included. | mMSParties/CCAddresses |
| Content Class | Classifies the content of the MM to the smallest content class to which the message belongs. | contentClass |
| Content Location | This field defines the location of the content to be retrieved. | contentLocation |
| Content Type | The content type of the MM. | contentType |
| correlation number | Provides a mechanism to correlate IRI for SMS with CC for SMS based on a single SMS event. | mMSCorrelationNumber |
| Delivery report | Specifies whether the originator MMS UE requests a delivery report from each recipient. | deliveryReport |
| Desired Delivery Time | Date and Time of desired delivery. Indicates the earliest possible delivery of the MM to the recipient. | desiredDeliveryTime |
| Distribution indicator | Identifies whether the originator (e.g., a Value Added Service Provider) allows the MM to be further distributed. A "No" value indicates to the user that the originator requested the content of the MM is not supposed to be distributed further. | distributionIndicator |
| DRM Content | Indicates if the MM contains any DRM-protected element. | dRMContent |
| Element descriptor | Contains the Content-Reference associated with the corresponding top level message content of the MM waiting for retrieval and MAY additionally contain the type/format of the message content. | elementDescriptor |
| Expiry | Length of time the MM will be stored in MMS Proxy- Relay or time to delete the MM. The field has two formats, either absolute or relative. | expiry |
| From address | Address of the sender of the MM or read reply. The sender may be the originator or a forwarding user. When address translation occurs (in the case of a token sent by the client and replaced with a proper address by the MMS Proxy/Relay), both the pre and post translated addresses (with appropriate correlation) are included. | mMSParties/from |
| Lawful interception identifier | Unique number for each lawful authorization. | lawfulInterceptionIdentifer |
| Message Class | Class of the MM. For example, a value of "auto" is automatically generated by the UE. If the field is not present, the class should be interpreted as "personal". | messageClass |
| Message ID | An ID assigned by the MMS Proxy-Relay to uniquely identify an MMS message. | messageID |
| MM State | Identifies the value of the MM State associated with a to be stored or stored MM. | mMState |
| MM State Flags | Identifies a keyword to add or remove from the list of keywords associated with a stored MM. | mMStateFlags |
| MMS Attributes | A list of information elements that should appear in the view for each selected message. | mMSAttributes |
| MMS Date/Time | Date and Time when the MM was last handled (either originated or forwarded). For origination, included by the sending MMS client or the originating MMS Proxy-Relay. | mMSDateTime |
| MMS Limit | A number indicating the maximum number of selected MMs whose information are to be returned in the response. If this is absent, information elements from all remaining MMs are to be returned. If this is zero then no MM-related information are to be returned. | mMSLimit |
| MMS Message Count | Identifies the number of messages in the content part of the PDU. | mMSMessageCount |
| MMS Quotas | Indicates a request for or the actual quotas for the user's MMBox in messages or bytes. | mMSQuotas |
| MMS Start | A number, indicating the index of the first MM of those selected to have information returned in the response. | mMSStart |
| MMS Status | Provides a MMS status. A status of "retrieved" is only signalled by the retrieving UE after retrieval of the MM. | mMSStatus |
| MMS Status Text | Text that qualifies the MMS Status. | mMSStatusText |
| MMS Totals | Indicates a request for or the actual count of messages currently stored in the MMBox. |  |
| MMS Version | The version of MMS used by the target. |  |
| Network Identifier | Operator ID plus unique identifier for the GCS AS. | networkIdentifer |
| Previously sent by | Address of the MMS Client that forwarded or originally sent the message and a sequence number. A higher sequence number indicates a forwarding event at a later point in time. This header field MAY appear multiple times. | previouslySentBy |
| Previously sent by Date/Time | Date and time of a forwarding or original send transaction of the message and a sequence number. The sequence number indicates the correspondence to the MMS Client's address in the "X-Mms-Previously- Sent-By" header field with the same sequence number. This header field MAY appear multiple times. | reviouslySentByDateTime |
| Priority | Priority of the MM assigned by the originator MMS Client. | priority |
| Read report | Specifies whether the originator MMS UE requests a read report from each recipient. | readReport |
| Read Status | Text explanation corresponding to the Read Status. | readStatus |
| Replace ID | This field indicates the reference (i.e. Message-ID) of the previous MM that is replaced by the current MM. | replaceID |
| Reply Applic ID | Identification of an application to which replies, delivery reports, and read reports are addressed. | replyApplicID |
| Report Allowed | Indication whether or not the sending of delivery report is allowed by the recipient MMS Client. | reportAllowed |
| Response Status | MMS specific status. | responseStatus |
| Response Status Text | Text that qualifies the Response Status. | responseStatusText |
| Retrieval Mode | Indicates whether manual retrieval mode is recommended for the MM. | retrievalMode |
| Retrieval Mode Text | Explains why manual retrieval mode is recommended for the MM. | retrievalModeText |
| Retrieve Status | MMS specific status. | retrieveStatus |
| Retrieve Status Text | Text that qualifies the Retrieve Status. | retrieveStatusText |
| Sender visibility | An indication that the sender's address should not be delivered to the recipient. | senderVisibility |
| Store | Specifies whether the originator MMS UE wants the submitted MM to be saved in the user's MMBox, in addition to sending it. | store |
| Store Status | Indicates if the MM was successfully stored in the MMBox. | storeStatus |
| Store Status Text | Text that qualifies the Store Status. | storeStatusText |
| To Recipients | Address of a recipient; the "To" field may include addresses of multiple recipients. When address translation occurs, both the pre and post translated addresses (with appropriate correlation) are included. | mMSParties/toAddresses |
| Transaction ID | An ID used to correlate an MMS request and response between the target and the MMS Proxy-Relay. | transactionID |

NOTE 1:	LIID parameter must be present in each record sent to the LEMF.

#### 15.X.6.2	Events and Event Information

##### 15.X.6.2.1	Overview

This clause describes the information sent from the Delivery Function (DF) to the Law Enforcement Monitoring Facility (LEMF) to support Lawful Interception (LI). The information is described as records and information carried by a record. This focus is on describing the information being transferred to the LEMF.

The IRI events and data are encoded into records as defined in the Table 15.X.6.1.1 Mapping between MMS Events and HI2 records type and Annex B.14 Intercept related information (HI2). IRI is described in terms of a 'causing event' and information associated with that event. Within each IRI record there is a set of events and associated information elements to support the particular service.

The communication events described in Table 15.X.6.1.1: Mapping between MMS Events and HI2 record type and Table 15.X.6.1.2: Mapping between Events information and IRI information convey the basic information for reporting the disposition of a communication. This clause describes those events and supporting information.

Each record described in this clause consists of a set of parameters. Each parameter is either:

mandatory (M)	-	required for the record,

conditional (C)	-	required in situations where a condition is met (the condition is given in the Description), or

optional (O)	-	provided at the discretion of the implementation.

The information to be carried by each parameter is identified. Both optional and conditional parameters are considered to be OPTIONAL syntactically in ASN.1 Stage 3 descriptions. The Stage 2 inclusion takes precedence over Stage 3 syntax.

##### 15.X.6.2.2	REPORT record information

The REPORT record is used to convey the events of MMS service interception.

The REPORT record shall be triggered when:

-	the target sends a MMS;

-	the target receives a notification of an incoming MMS;

-	the target responds to the notification of an incoming MMS;

-	an MMS is retrieved by the target;

-	the target acknowledges to the retrieved MMS;

-	the target forwards an MMS;

-	the target stores an MMS in the Mmbox;

-	the target uploads an MMS to the Mmbox;

-	the target deletes a MMS stored in the Mmbox;

-	the target receives a MMS Delivery report;

-	the target sends a MMS Read Reply;

-	the target receives a MMS Read Reply;

-	the target receives a MMS cancel request;

-	the target sends a MMbox view request;

-	the target receives a confirmation of a sent MMbox View request.

Table 15.X.6.2.2.1: MMS Send REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS Send). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| correlation number | C | Provide to allow correlation of CC and IRI when delivery of CC is authorized. |
| To Recipients | M | Shall be provided and shall include untranslated and translated addresses. |
| CC Recipients | C | Provide if available to identify CC recipients. When included, shall provide untranslated and translated addresses. |
| BCC Recipients | C | Provide if available to identify BCC recipients. When included, shall provide untranslated and translated addresses. |
| From address | M | Shall be provided (includes both target provided address and if translation occurs, network substituted post-translation address). |
| MMS Version | M | Shall be provided. |
| Transaction ID | M | Shall be provided. |
| Message ID | M | Shall be provided. |
| MMS Date/Time | M | Shall be provided. |
| Message Class | C | Provide if available to identify the message class. |
| Expiry | M | Shall be provided (either the signalled expiry or the default, whichever applies). |
| Desired Delivery Time | C | Provide when sent by the target to indicate the desired MMS delivery time. |
| Priority | C | Provide when sent by the target to indicate the desired MMS priority. |
| Sender visibility | C | Provide when sent by the target to indicate the target’s visibility to the other party or if not signalled by the target and the default is to not make target visible to the other party. |
| Delivery report | C | Provide when sent by the target to indicate the desired delivery report. |
| Read report | C | Provide when sent by the target to indicate the desired read report. |
| Store | C | Provide when sent by the target to indicate the MMS is to be stored. |
| Applic ID | C | Provide when sent by the target to identify the destination application. |
| Reply Applic ID | C | Provide when sent by the target to identify the application to which replies, delivery reports, and read reports are addressed. |
| Content Class | C | Provide when sent by the target to identify the class of the content. |
| DRM Content | C | Provide when sent by the target to indicate if the MM contains any DRM-protected element. |
| Adaptation Allowed | C | Provide when sent by the target to identify whether the target wishes the MM to be adapted or not. If overridden, an indication shall be included in the parameter. |
| Content Type | M | Shall be provided. |
| Content Location | C | Provide if signalled to the target in response to the target sending an MMS. |
| Response Status | C | Provide if signalled to the target in response to the target sending an MMS. |
| Response Status Text | C | Provide if signalled to the target in response to the target sending an MMS. |
| Store Status | C | Provide if signalled to the target in response to the target sending an MMS. |
| Store Status Text | C | Provide if signalled to the target in response to the target sending an MMS. |

Table 15.X.6.2.2.2: MMS Notification REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS Notification). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| correlation number | C | Provide to allow correlation of CC and IRI when delivery of CC is authorized. |
| To Recipients | M | Shall be provided and shall include untranslated and translated addresses. |
| CC Recipients | C | Provide if available to identify CC recipients. When included, shall provide untranslated and translated addresses. |
| BCC Recipients | C | Provide if available to identify BCC recipients. When included, shall provide untranslated and translated addresses. |
| From address | M | Shall be provided regardless of anonymity (includes both target provided address and if translation occurs, network substituted post-translation address). |
| MMS Version | M | Shall be provided. |
| Transaction ID | M | Shall be provided. |
| Message ID | M | Shall be provided. |
| MMS Date/Time | M | Shall be provided. |
| Message Class | M | Shall be provided. |
| Expiry | M | Shall be provided |
| Distribution indicator | C | If sent to the target, shall be included. |
| Element descriptor | C | If sent to the target, shall be included. |
| Retrieval Mode | C | If sent to the target, shall be included. |
| Retrieval Mode Text | C | If sent to the target, shall be included. |
| Sender visibility | C | If the originator indicated a desire to withhold their address from the MM recipient, then this parameter shall be included. |
| Delivery report | C | If sent to the target, shall be included. |
| Read report | C | Provide when sent by the target to indicate the desired read report. |
| Applic ID | C | If sent to the target, shall be included. |
| Reply Applic ID | C | If sent to the target, shall be included. |
| Aux Applic Info | C | If sent to the target, shall be included. |
| Content Class | C | If sent to the target, shall be included. |
| DRM Content | C | If sent to the target, shall be included. |
| Replace ID | C | If sent to the target, shall be included. |
| Content Location | C | If sent to the target, shall be included. |

Table 15.X.6.2.2.3: MMS Notification Response REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS Notification Response). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| MMS Version | M | Shall be provided. |
| Transaction ID | M | Shall be provided. |
| Message ID | M | Shall be provided. |
| MMS Status | M | Shall be provided. |
| Report Allowed | C | If sent by the target, shall be included. |

Table 15.X.6.2.2.4: MMS Retrieval REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS Retrieval). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| correlation number | C | Provide to allow correlation of CC and IRI when delivery of CC is authorized. |
| To Recipients | M | Shall be provided and shall include untranslated and translated addresses. |
| CC Recipients | C | Provide if available to identify CC recipients. When included, shall provide untranslated and translated addresses. |
| BCC Recipients | C | Provide if available to identify BCC recipients. When included, shall provide untranslated and translated addresses. |
| From address | M | Shall be provided regardless of anonymity (includes both target provided address and if translation occurs, network substituted post-translation address). |
| MMS Version | M | Shall be provided. |
| Transaction ID | M | Shall be provided. |
| Message ID | M | Shall be provided. |
| MMS Date/Time | M | Shall be provided. |
| Previously sent by | C | If available, shall be provided. |
| Previously sent by Date/Time | C | If available, shall be provided. |
| MM State | C | Provide if sent to the target. |
| Message Class | C | Provide if available to identify the message class. |
| Priority | M | Shall be provided (either the signalled priority or the default, whichever applies). |
| Delivery report | C | Provide when sent by the target to indicate the desired delivery report. |
| Read report | C | Provide when sent by the target to indicate the desired read report. |
| Sender visibility | C | If the originator indicated a desire to withhold their address from the MM recipient, then this parameter shall be included. |
| Retrieve Status | C | If sent to the target, shall be included. |
| Retrieve Status Text | C | If sent to the target, shall be included. |
| Distribution indicator | C | If sent to the target, shall be included. |
| Applic ID | C | If sent to the target, shall be included. |
| Reply Applic ID | C | If sent to the target, shall be included. |
| Aux Applic Info | C | If sent to the target, shall be included. |
| Content Class | C | If sent to the target, shall be included. |
| DRM Content | C | If sent to the target, shall be included. |
| Replace ID | C | If sent to the target, shall be included. |
| Content Type | C | If sent to the target, shall be included. |

Table 15.X.6.2.2.5: MMS Retrieval Acknowledgement REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS Retrieval Acknowledgement). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| MMS Version | M | Shall be provided. |
| Transaction ID | M | Shall be provided. |
| Message ID | M | Shall be provided. |
| Report Allowed | C | If sent to the target, shall be included. |

Table 15.X.6.2.2.6: MMS Forwarding REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS Forwarding). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| correlation number | C | Provide to allow correlation of CC and IRI when delivery of CC is authorized. |
| To Recipients | M | Shall be provided and shall include untranslated and translated addresses. |
| CC Recipients | C | Provide if available to identify CC recipients. When included, shall provide untranslated and translated addresses. |
| BCC Recipients | C | Provide if available to identify BCC recipients. When included, shall provide untranslated and translated addresses. |
| From address | M | Shall be provided (includes both target provided address and if translation occurs, network substituted post-translation address). |
| MMS Version | M | Shall be provided. |
| Transaction ID | M | Shall be provided. |
| Message ID | M | Shall be provided. |
| MMS Forward Req Date/Time | M | Shall be provided. |
| Message Class | C | Provide if available to identify the message class. |
| Expiry | M | Shall be provided (either the signalled expiry or the default, whichever applies). |
| Desired Delivery Time | C | Provide when sent by the target to indicate the desired MMS delivery time. |
| Priority | C | Provide when sent by the target to indicate the desired MMS priority. |
| Sender visibility | C | Provide when sent by the target to indicate the target’s visibility to the other party or if not signalled by the target and the default is to not make target visible to the other party. |
| Delivery report allowed | M | Shall be provided to report the target requested reporting to the original sender or the default, whichever applies. |
| Delivery report | C | Provide when sent by the target to indicate the desired delivery report. |
| Read report | C | Provide when sent by the target to indicate the desired read report. |
| Store | C | Provide when sent by the target to have the forwarded MM stored. |
| MM State | C | Provide when sent by the target to set the state for the forwarded MM when it is stored. |
| Content Location | M | Shall be provided. |
| Response Status | M | Shall be provided. |
| Response Status Text | C | If provided to the target, shall be included. |
| Store Status | C | If sent to the target, shall be included. |
| Store Status Text | C | If sent to the target, shall be included. |

Table 15.X.6.2.2.7: MMS Store REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS Forwarding). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| MMS Version | M | Shall be provided. |
| Transaction ID | M | Shall be provided. |
| MM State | C | If provided by the target, shall be included. |
| MM Flags | C | If provided by the target, shall be included. |
| Content Location | M | Shall be provided. |
| Store Status | M | Shall be provided. |
| Store Status Text | C | If sent to the target, shall be provided. |

Table 15.X.6.2.2.8: MMS Upload REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS Upload). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| correlation number | C | Provide to allow correlation of CC and IRI when delivery of CC is authorized. |
| MMS Version | M | Shall be provided. |
| Transaction ID | M | Shall be provided. |
| MM State | C | If sent by the target, shall be included. |
| MM Flags | C | If sent by the target, shall be included. |
| Content Type | M | Shall be provided. |
| Content Location | C | If available, shall be included. |
| Store Status | M | Shall be provided. |
| Store Status Text | C | If sent to the target, shall be included. |
| MMBox Description pdu | M | Shall be provided. The MMBox description PDU (as described in Table 15.X.6.2.2.16) corresponds to the particular MM being uploaded. |

Table 15.X.6.2.2.9: MMS Delete REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS Delete). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| MMS Version | M | Shall be provided. |
| Transaction ID | M | Shall be provided. |
| Content Location | M | Shall be provided. |
| Response Status | M | Shall be provided. |
| Response Status Text | C | If available, shall be provided. |

Table 15.X.6.2.2.10: MMS Delivery REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS Delivery). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| To Recipients | M | Shall be provided. |
| MMS date/time | M | Shall be provided. |
| Message id | M | Shall be provided. |
| MMS Status | M | Shall be provided. |
| MMS Status Text | C | If available, shall be included. |
| Applic-ID | C | If available, shall be provided. |
| Reply-Applic-id | C | If available, shall be provided. |
| Aux-Applic-Info | C | If available, shall be provided. |

Table 15.X.6.2.2.11: MMS Read Reply From Target REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS Read Reply From Target). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| To Recipients | M | Shall be provided. Identifies the recipient of the read report. |
| From address | M | Shall be provided and include target’s MMS address. |
| Message id | M | Shall be provided. |
| Read Status | M | Shall be provided. |
| MMS date time | M | Shall be provided. |
| Applic-ID | C | If available, shall be provided. |
| Reply-Applic-id | C | If available, shall be provided. |
| Aux-Applic-Info | C | If available, shall be provided. |

Table 15.X.6.2.2.12: MMS Read Reply To Target REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS Read Reply To Target). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| To Recipients | M | Shall be provided and include the address of the read recipient (i.e., the target). |
| From address | M | Shall be provided and include the address of read reply source. |
| Message id | M | Shall be provided. |
| Read Status | M | Shall be provided. |
| MMS date time | C | If available, shall be provided. |
| Applic-ID | C | If available, shall be provided. |
| Reply-Applic-id | C | If available, shall be provided. |
| Aux-Applic-Info | C | If available, shall be provided. |

Table 15.X.6.2.2.13: MMS Cancel REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS Cancel). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| MMS Version | M | Shall be provided. |
| Cancel id | M | Shall be provided. |
| Cancel Status | C | If available, shall be provided. |

Table 15.X.6.2.2.14: MMS View Request REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS View Request). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| MMS Version | M | Shall be provided. |
| Transaction id | M | Shall be provided. |
| MM State | C | If provided by the target, shall be included. |
| MM Flags | C | If provided by the target, shall be included. |
| Content Location | C | If provided by the target, shall be included. |
| MMS Start | C | If provided by the target, shall be included. |
| MMS Limit | C | If provided by the target, shall be included. |
| MMS Attributes | C | If provided by the target, shall be included. |
| MMS Totals | C | If provided by the target, shall be included. |
| MMS Quotas | C | If provided by the target, shall be included. |

Table 15.X.6.2.2.15: MMS View Confirm REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS View Confirm). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| correlation number | C | Provide to allow correlation of CC and IRI when delivery of CC is authorized. |
| MMS Version | M | Shall be provided. |
| Transaction id | M | Shall be provided. |
| MM State | C | Provide if sent to the target. |
| MM Flags | C | Provide if sent to the target. |
| Content Location | C | Provide if sent to the target. |
| MMS Start | C | Provide if sent to the target. |
| MMS Limit | C | Provide if sent to the target. |
| MMS Attributes | C | Provide if sent to the target. |
| MMS Totals | C | Provide if sent to the target. |
| MMS Quotas | C | Provide if sent to the target. |
| Response Status | M | Shall be provided. |
| Response Status Text | C | Provide if sent to the target. |
| MMS Message Count | C | Provide if sent to the target. |
| Content type | M | Shall be provided. |
| MMBox Description pdu | M | Shall provide one or more MMBox description PDUs where each MMBox description PDU (as described in Table 15.X.6.2.2.16) corresponds to a particular MM. |

Table 15.X.6.2.2.16: MMS Description PDU

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| correlation number | C | Provide to allow correlation of CC and IRI when delivery of CC is authorized. |
| To Recipients | M | Shall be included. |
| CC Recipients | C | Provide if available. |
| BCC Recipients | C | Provide if available. |
| From address | M | Shall be provided regardless of anonymity |
| Message ID | C | Shall be provided unconditionally for the MMS View Confirm report record and shall be included for the MMS Upload report record if a Message ID was previously assigned to the MM. In this latter case, if a Message ID was not previously assigned, this parameter is excluded. |
| MMS Date Time | C | If available for the MM, shall be provided. |
| Previously Sent by | C | If available for the MM, shall be provided. |
| Previously Sent by Date Time | C | If available for the MM, shall be provided. |
| MM State | C | Shall be included for the MMS View Confirm report record and shall be included in the MMS Upload report record if provided by the target. |
| MM Flags | C | If at least one keyword is associated with the MM, then this parameter shall be included. This parameter may convey all the keywords associated with the MM. If no keywords are associated with the MM, then this parameter may be excluded. |
| Message Class | C | If available, shall be included. |
| Priority | C | If available, shall be provided. |
| Delivery Time | C | If available, shall be provided. |
| Expiry | C | If available, shall be provided. |
| Delivery report | C | If available, shall be provided. |
| Read report | C | If available, shall be provided. |
| Message size | C | If available, shall be provided. |
| Content Location | C | Shall only be used in conjunction with the MMS View Confirm report record |
| Content type | M | Shall be provided. |

### 15.X.7	CC for MMS

#### 15.X.7.1	General

The CC for an MMS will be provided by the ICE to DF3 which will then handle delivery over HI3 to the LEMF.

Sufficient information will need to be included in the CC to allow for correlation of the CC to the IRI to which it pertains.  Table 15.X.7.1 shows the contents of the MMS CC PDU to be sent to the LEMF when CC is reported.

Table 15.X.7.1: MMS CC PDU

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed MMS Address |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type which corresponds to the MMS IRI event where the CC was captured. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| correlation number | M | Provide to allow correlation of CC and IRI when delivery of CC is authorized. |
| MMS Version | M | Shall be provided. |
| Transaction id | M | Shall be provided. |
| MMS Content | M | Shall be provided. |

*** END OF SECOND MODIFICATION ***

*** THIRD MODIFICATION ***

# B.2	3GPP object tree

Figure B.1: 3GPP object tree

*** END OF THIRD MODIFICATION ***

*** FOURTH MODIFICATION ***

# B.Y	Intercept related information (HI2 MMS)

Declaration of ROSE operation mms-sending-of-IRI is ROSE delivery mechanism specific. When using FTP delivery mechanism, data MmsIRIsContent must be considered.

# ASN1 description of IRI (HI2 interface)

MmsHI2Operations {itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulintercept(2) threeGPP(4) hi2mms(16) r14(14) version-0 (0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t(2) remote-operations(4) informationObjects(5) version1(0)}

LawfulInterceptionIdentifier,

TimeStamp,

Network-Identifier,

National-Parameters,

National-HI2-ASN1parameters,

DataNodeAddress,

IPAddress,

IP-value,

X25Address

FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulIntercept(2) hi2(1) version18(18)} -- Imported from TS 101 671v3.12.1

CivicAddress

FROM UmtsHI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulintercept(2) threeGPP(4) hi2(1)  r13 (13) version-0 (0)};

-- Imported from 3GPP TS 33.108, UMTS PS HI2

-- Object Identifier Definitions

-- Security DomainId

lawfulInterceptDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0)

securityDomain(2) lawfulIntercept(2)}

-- Security Subdomains

threeGPPSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP(4)}

hi2epsDomainId OBJECT IDENTIFIER	::= {threeGPPSUBDomainId hi2mms(16) r14(14) version-0 (0)}

mms-sending-of-IRI  OPERATION ::=

{

ARGUMENT	MmsIRIsContent

ERRORS		{ OperationErrors }

CODE		global:{threeGPPSUBDomainId hi2mms(16) opcode(1)}

}

-- Class 2 operation . The timer shall be set to a value between 3 s and 240 s.

-- The timer.default value is 60s.

-- NOTE:	The same note as for HI management operation applies.

MmsIRIsContent		::= CHOICE

{

mmsiRIContent			MmsIRIContent,

mmsIRISequence			MmsIRISequence

}

MmsIRISequence		::= SEQUENCE OF MmsIRIContent

-- Aggregation of MmsIRIContent is an optional feature.

-- It may be applied in cases when at a given point in time

-- several IRI records are available for delivery to the same LEA destination.

-- As a general rule, records created at any event shall be sent

-- immediately and not withheld in the DF or MF in order to

-- apply aggragation.

-- When aggregation is not to be applied,

-- MmsIRIContent needs to be chosen.

-- MmsIRIContent includes events that correspond to MMS.

MmsIRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,	-- not applicable for the present document

iRI-End-record			[2] IRI-Parameters, -- not applicable for the present document

iRI-Continue-record		[3] IRI-Parameters,	-- not applicable for the present document

iRI-Report-record		[4] IRI-Parameters	-- include at least one optional parameter

}

-- the MmsIRIContent may provide events that correspond to UMTS/GPRS as well as EPS.

unknown-version 		ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter-value	ERROR ::= { CODE local:2}

unknown-parameter		ERROR ::= { CODE local:3}

OperationErrors ERROR ::=

{

unknown-version |

missing-parameter |

unknown-parameter-value |

unknown-parameter

}

-- These values may be sent by the LEMF, when an operation or a parameter is misunderstood.

-- Parameters having the same tag numbers must be identical in Rel-14 and onwards modules.

IRI-Parameters		::= SEQUENCE

{

hi2mmsDomainId				[0]	OBJECT IDENTIFIER,  -- 3GPP HI2 MMS domain

lawfulInterceptionIdentifier	[1] LawfulInterceptionIdentifier,

-- This identifier is associated to the target.

timeStamp				[3] TimeStamp,

-- date and time of the event triggering the report.

locationOfTheTarget		[4] Location OPTIONAL,

-- location of the target

partyInformation 		[5] SET SIZE (1..10) OF PartyInformation OPTIONAL,

-- This parameter provides the concerned party, the identiy(ies) of the party

--)and all the information provided by the party.

mMSevent 				[7] MMSEvent OPTIONAL,

serviceCenterAddress	[8] PartyInformation OPTIONAL,

-- this parameter provides the address of the relevant MMS server

mMSParties				[9] MMSParties OPTIONAL,

-- this parameter provides the MMS parties (To, CC, BCC, and From) in the communication.

mMSVersion				[10] INTEGER OPTIONAL,

transactionID			[11] UTF8String OPTIONAL,

messageID				[12] UTF8String OPTIONAL,

-- In accordance with [bb] it is encoded as in email address as per [RFC2822]. The characters

-- "<" and ">" are not included.

mMSDateTime				[13] DateTime OPTIONAL,

messageClass			[14] MessageClass OPTIONAL,

expiry					[15] DateTime OPTIONAL,

distributionIndicator	[16] YesNo OPTIONAL,

elementDescriptor		[17] ElementDescriptor OPTIONAL,

retrievalMode			[18] YesNo OPTIONAL,

-- if retrievalMode is included, it must be coded to Yes indicating Manual retreival mode

-- recommended.

retrievalModeText		[19] EncodedString OPTIONAL,

senderVisibility		[20] YesNo OPTIONAL,

-- Yes indicates Show and No indicates Do Not Show.

deliveryReport			[21] YesNo OPTIONAL,

readReport				[22] YesNo OPTIONAL,

applicID				[23] UTF8String OPTIONAL,

replyApplicID			[24] UTF8String OPTIONAL,

auxApplicInfo			[25] UTF8String OPTIONAL,

contentClass			[26] ContentClass OPTIONAL,

dRMContent				[27] YesNo OPTIONAL,

replaceID				[28] UTF8String OPTIONAL,

contentLocation			[29] ContentLocation OPTIONAL,

mMSStatus				[30] MMSStatus OPTIONAL,

reportAllowed			[31] YesNo OPTIONAL,

previouslySentBy		[32] PreviouslySentBy OPTIONAL,

previouslySentByDateTime [33] PreviouslySentByDateTime OPTIONAL,

mMState					 [34] MMSState OPTIONAL,

desiredDeliveryTime		[35] DateTime OPTIONAL,

deliveryReportAllowed	[36] YesNo OPTIONAL,

store					[37] YesNo OPTIONAL,

responseStatus			[38] ResponseStatus OPTIONAL,

responseStatusText		[39] ResponseStatusText OPTIONAL,

storeStatus				[40] StoreStatus OPTIONAL,

storeStatusText			[41] EncodedString OPTIONAL,

mMState					[42] MMSState OPTIONAL,

mMFlags					[43] MMFlags OPTIONAL,

mMBoxDescriptionPdus	[44] SEQUENCE OF MMBoxDescriptionPdus  OPTIONAL,

cancelID				[45] UTF8String OPTIONAL,

cancelStatus			[46] YesNo OPTIONAL,

-- Yes indicates cancel successfully received and No indicates cancel request corrupted.

mMSStart				[47] INTEGER OPTIONAL,

mMSLimit				[48] INTEGER OPTIONAL,

mMSAttributes			[49] MMSAttributes OPTIONAL,

mMSTotals				[50] YesNo OPTIONAL,

mMSQuotas				[51] YesNo OPTIONAL,

mMSMessageCount			[52] INTEGER OPTIONAL,

messageSize				[53] INTEGER OPTIONAL,

mMSForwardReqDateTime	[54] DateTime OPTIONAL,

adaptationAllowed		[55] YesNo OPTIONAL,

priority				[56] Priority OPTIONAL

mMSCorrelationNumber	[57] MMSCorrelationNumber OPTIONAL,

-- this parameter provides MMS Correlation number when the event will also provide CC.

contentType				[58] OCTET STRING OPTIONAL,

national-Parameters		[59] National-Parameters OPTIONAL,

}

-- Parameters having the same tag numbers must be identical in Rel-14 and onwards modules

-- PARAMETERS FORMATS

Address::= EncodedString

Addresses::= SEQUENCE OF Address

ClassIdentifier ::= ENUMERATED

{

personal 			(0),

advertisement	 	(1),

informational		(2),

auto				(3),

…

}

ContentClass ::= ENUMERATED

{

text 			(0),

image-basic 	(1),

image-rich		(2),

video-basic		(3),

video-rich		(4),

megapixel		(5),

content-basic	(6),

content-rich	(7),

…

}

ContentLocation ::= SEQUENCE

{

contentLocationURI		[1] OCTET STRING,

-- See Clause 7.3.10 of [BB] for the coding of the contentLocationURI.

statusCount				[2] INTEGER OPTIONAL,

-- the statusCount is included only for the MMS Delete event.

...

}

ElementDescriptor ::= SEQUENCE

{

contentReferenceValue [1] UTF8String,

parameterName		  [2] ParameterName,

parameterValue		  [3] ParameterValue,

…

}

EncodedString::= CHOICE

{

text	[1] UFT8String,

encodedTextString	[2] EncodedTextString,

…

}

EncodedTextString::= SEQUENCE

{

stringType	[1] OCTET STRING,

-- stringType shall be encoded with MIBEnum values as registered with IANA as defined in [bb].

actualString	[2] OCTET STRING,

…

}

From::= SEQUENCE OF FromAddresses

FromAddresses::= CHOICE

{

actualAddress	[1] EncodedString,

insertToken		[2] NULL,

…

}

MessageClass::= CHOICE

{

classIdentifier	[1] ClassIdentifier,

tokenText		[2] OCTET STRING,

…

}

MMBoxDescriptionPdus::= SEQUENCE OF

{

mMSCorrelation				[1] MMSCorrelation OPTIONAL,

toAddresses					[2] Addresses,

cCAddresses					[3] Addresses OPTIONAL,

bCCAddresses				[4] Addresses OPTIONAL,

fromAddress					[5] From,

messageID					[6] UTF8String,

mMSDateTime					[7] DateTime OPTIONAL,

previouslySentBy			[8] PreviouslySentBy OPTIONAL,

previouslySentByDateTime	[9]	PreviouslySentByDateTime OPTIONAL,

mMState						[10] MMState OPTIONAL,

mMFlags						[11] MMFlags OPTIONAL,

messageClass				[12] MessageClass OPTIONAL,

priority					[13] Priority OPTIONAL,

deliveryTime				[14] DateTime OPTIONAL,

expiry						[15] DateTime OPTIONAL,

deliveryReport				[16] YesNo OPTIONAL,

readReport					[17] YesNo OPTIONAL,

messageSize					[18] INTEGER OPTIONAL,

contentLocation				[19] ContentLocation OPTIONAL,

contentType					[20] OCTET STRING OPTIONAL,

…

}

MMFlags::= SEQUENCE OF

{

tokenAction		[1] TokenAction,

mmFlagkeywords	[2] EncodedString,

}

MMSAttributes::= CHOICE

{

attributeApplicID					[1] UTF8String,

attributeAuxApplicInfo				[2] UFT8String,

attributeBCC						[3] Address,

attributeCC							[4] Address,

attributeContent					[5] OCTET STRING,

attributeContentType				[6] OCTET STRING,

attributeDate						[7] DateTime,

attributeDeliveryReport				[8] YesNo,

attributeDeliveryTime				[9]	DateTime,

attributeExpiry						[10] DateTime,

attributeFrom						[11] From,

attributeMessageClass				[12] MessageClass,

attributeMessageID					[13] UFT8String,

attributeMessageSize				[14] INTEGER,

attributePriority					[15] Priority,

attributeReadReport					[16] YesNo,

attributeTo							[17] Address,

attributeReplyApplicID				[18] UTF8String,

attributePreviouslySentBy			[19] PreviouslySentBy,

attributePreviouslySentByDateTime	[20] PreviouslySentByDateTime,

attributeAdditionalHeaders			[21] OCTET STRING,

…

}

MMSCorrelationNumber::=OCTET STRING

MMSEvent::= ENUMERATED

{

send 					(0),

notification			(1),

notificationResponse	(2),

retrieval				(3),

retrievalAcknowledgement(4),

forwarding				(5),

store					(6),

upload					(7),

delete					(8),

delivery				(9),

readReplyFromTarget		(10),

readReplyToTarget		(11),

cancel					(12),

viewRequest				(13),

viewConfirm				(14),

…

}

MMSParties::= SEQUENCE OF

{

toAddresses		[1] Addresses OPTIONAL,

cCAddresses		[2]	Addresses OPTIONAL,

bCCAddresses	[3] Addresses OPTIONAL,

fromAddresses	[4] From OTPTIONAL,

…

}

MMSState::= ENUMERATED

{

draft 		(0),

sent		(1),

new			(2),

retreived	(3),

forwarded	(4),

…

}

MMSStatus::= ENUMERATED

{

expired 		(0),

retrieved		(1),

rejected		(2),

deferred		(3),

unrecognised	(4),

indeterminate	(5),

forwarded		(6),

unreachable		(7),

…

}

ParameterName::= CHOICE

{

integername [1] INTEGER,

textName	[2] UTF8String,

…

}

ParameterValue::= CHOICE

{

integername [1] OCTET STRING,

textName	[2] UTF8String,

…

}

PreviouslySentby::= SEQUENCE OF

{

forwardedCount		[1] INTEGER,

forwardedPartyID	[2] EncodedString,

…

}

PreviouslySentbyDateTime::= SEQUENCE OF

{

forwardedCount		[1] INTEGER,

forwardedDateTime	[2] DateTime,

…

}

Priority ::= ENUMERATED

{

low 	(0),

normal 	(1),

high	(2),

…

}

ResponseStatus::= SEQUENCE

{

statusCount	[1]	EncodedString OPTIONAL,

-- the statusCount shall only be included for the Delete event.

actualResponseStatus	[2] ActualResponseStatus,

…

}

ResponseStatusText::= SEQUENCE

{

statusCount	[1]	EncodedString OPTIONAL,

-- the statusCount shall only be included for the Delete event.

actualResponseStatusText	[2] EncodedStringValue,

…

}

ActualResponseStatus ::= ENUMERATED

{

ok 										(0),

errorUnspecified						(1),

errorServiceDenied						(2),

errorMessageFormatCorrupt				(3),

errorSendingAddressUnresolved			(4),

errorMessageNotFound					(5),

errorNetworkProblem						(6),

errorContentNotAccepted					(7),

errorUnsuportedMessage					(8),

errorTransientFailure					(9),

errorTransientSendingAddressUnresolved	(10),

errorTransientMessageNotFound			(11),

errorTransientNetworkProblem			(12),

errorTransientPartialSucess				(13),

errorPermanentFailure					(14),

errorPermanentServiceDenied				(15),

errorPermanentMessageFormatCorrupt		(16),

errorPermanentSendingAddressUnresolved	(17),

errorPermanentMessageNotFound			(18),

errorPermanentContentNotAccepted		(19),

errorPermanentReplyChargingLimitationsNotMet	(20),

errorPermanentReplyChargingRequestNotAccepted	(21),

errorPermanentReplyChargingForwardingDenied		(22),

errorPermanentReplyChargingNotSupported			(23),

errorPermanentAddressHidingNotSupported			(24),

errorPermanentLackOfPrepaid						(25),

…

}

StoreStatus ::= ENUMERATED

{

success 		(0),

errorTransient 	(1),

high			(2),

…

}

TokenAction::= ENUMERATED

{

addToken 		(0),

removeToken 	(1),

filterToken		(2),

…

}

YesNo::= BINARY

-- 1 indicates Yes and 0 indicates No.

END -- OF MmsHI2Operations

*** END OF FOURTH MODIFICATION ***

*** START OF FIFTH MODIFICATION ***

# B.Y	Intercept related information (HI2 MMS)

Declaration of ROSE operation mms-sending-of-IRI is ROSE delivery mechanism specific. When using FTP delivery mechanism, data MmsIRIsContent must be considered.

Eps-HI3-PS {itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulintercept(2) threeGPP(4) hi3mms(17)  r14(14) version-0(0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

MMSCorrelationNumber, MMSEvent

FROM MmsHI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulintercept(2) threeGPP(4) 	hi2mms(16) r14(14) version-0(0)}    -- Imported from TS 33.108 v.14.0.0

LawfulInterceptionIdentifier,TimeStamp

FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) hi2(1) 	version18(18)};  -- from ETSI HI2Operations TS 101 671 v3.12.1

-- Object Identifier Definitions

-- Security DomainId

lawfulInterceptDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0)

securityDomain(2) lawfulIntercept(2)}

-- Security Subdomains

threeGPPSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP(4)}

hi3DomainId OBJECT IDENTIFIER  ::= {threeGPPSUBDomainId hi3mms(17) r14(14) version-0(0)}

CC-PDU	::= SEQUENCE

{

uLIC-header		[1] ULIC-header,

payload			[2] OCTET STRING

}

ULIC-header ::= SEQUENCE

{

hi3DomainId					[0]	OBJECT IDENTIFIER,  -- 3GPP HI3 Domain

lIID						[1] LawfulInterceptionIdentifier OPTIONAL,

mMSCorrelationNNumber		[2]	MMSCorrelationNumber,

timeStamp					[3]	TimeStamp,

t-PDU-direction				[4] TPDU-direction,

mMSEvent					[5] MMSEvent,

mMSVersion					[6]	INTEGER,

transactionID				[7]	UFT8String,

partyInformation			[8] SET SIZE (1..10) OF PartyInformation OPTIONAL,

national-HI3-ASN1parameters	[9] National-HI3-ASN1parameters OPTIONAL,

--  encoded per national requirements

ice-type					[10] ICE-type OPTIONAL,

-- The ICE-type indicates the applicable Intercepting Control Element(see ref [19]) in which

-- the T-PDU is intercepted.

…

}

TPDU-direction ::= ENUMERATED

{

from-target 	(1),

to-target 		(2),

unknown 		(3)

}

National-HI3-ASN1parameters	::= SEQUENCE

{

countryCode		[1] PrintableString (SIZE (2)),

-- Country Code according to ISO 3166-1 [39],

-- the country to which the parameters inserted after the extension marker apply

...

-- In case a given country wants to use additional national parameters according to its law,

-- these national parameters should be defined using the ASN.1 syntax and added after the

-- extension marker (...).

-- It is recommended that "version parameter" and "vendor identification parameter" are

-- included in the national parameters definition. Vendor identifications can be

-- retrieved from IANA web site. It is recommended to avoid

-- using tags from 240 to 255 in a formal type definition.

}

ICE-type ::= ENUMERATED

{

sgsn			(1),

ggsn			(2),

s-GW            (3),

pDN-GW          (4),

colocated-SAE-GWs (5),

ePDG            (6),

mMSProxyRelay	(7),

…

}

END-- OF Mms-HI3

*** END OF FIFTH MODIFICATION ***

*** END OF MODIFICATIONS ***