3GPP TSG-SA3 Meeting #64	    s3i170048

# Sophia-Antipolis, France, 24th Jan 2017 - 27th Jan 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | 0242 | rev | 1 | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Separate Delivery of MMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI14 |  |  |  |  |  | Date: |  |  | 2017-01-26 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Solution for MMS interception |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provides event triggers for MMS reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MMS interception will not be possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18.3.2, 18.3.3.2, 18.3.3.3, 18.3.3.4, 18.3.3.6, 18.3.3.9, 18.3.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

### 18.3.2	MMS Architecture IRI/CC Events

The MMS architecture is provided in Clause 5 of MMS Architecture [72]. The two interfaces considered in developing the events and event information to be reported are the MMSM interface (between the MMS client and the MMS Proxy-Relay) and the MMSR interface (i.e., between two MMS Proxy-Relay nodes).

For separate delivery of MMS, the following events shall be reported by the ICE to the DF:

1)	MMS Send

2)	MMS Notification & Confirmation

3)	MMS Retrieval Confirmation & Acknowledgement

4)	MMS Forwarding

5)	MMS Store Requests

6)	MMS Viewing Requests & Responses

7)	MMS Deletion Requests (delete from MMBox and Proxy-Relay)

8)	MMS Cancel Requests

9)	MMS Delivery Reports

10)	MMS Read Report

11)	Serving System (clause 7.4.9)

12)	Serving Evolved Packet System (clause 12.2.3.8)

The above events shall be reported from the ICE to the DF independent of any other services that may or may not be intercepted.

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from the nodes to DF2. Available IEs from this set of elements as shown below can be extended in the nodes, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option. Where MMS CC is available during an event, in addition to the IRI event reported to DF2, an MMS CC event is reported to DF3.

Table 18.3.2-1 Table of MMS Information Elements

Observed MSISDN MSISDN of the target.

Observed IMSI IMSI of the target.

Observed IMEI IMEI of the target

Observed MMS Address An address in a format as specified in [73]. This is where a SIP URI would be included.

Observed IPv4/IPv6 Address An IPv4 or IPv6 address of the target.

Event type Indicates which type of event is delivered: MMS Send, MMS Notification, MMS Notification Response, MMS Retrieval, MMS Retrieval Acknowledgement, MMS Forwarding, MMS Store, MMS Upload, MMS Delete from MMBox, MMS Delete from Proxy-Relay, MMS MMBox View, MMS Delivery, MMS Read Reply, MMS Cancel, MMS Cancel Confirm, MMS MMBox View Request, MMS MMBox View Confirm, Serving System, Serving Evolved System.

Event time Time of the event generation in the ICE. Time stamp shall be generated relative to ICE internal clock.

Event date Date of the event generation in the ICE.

Network Element Identifier Unique identifier for the ICE (MMS Relay/Proxy) reporting the event.

To Recipient(s) address(es) Address of a recipient; the "To" field may include addresses of multiple recipients. When address translation occurs, both the pre and post translated addresses (with appropriate correlation) are included.

CC Recipient(s) address(es) Address of a recipient; the "CC" field may include addresses of multiple recipients. When address translation occurs, both the pre and post translated addresses (with appropriate correlation) are included.

BCC Recipient(s) address(es) Address of a recipient; the "BCC" field may include addresses of multiple recipients. When address translation occurs, both the pre and post translated addresses (with appropriate correlation) are included.

From address Address of the sender of the MM or read reply. The sender may be the originator or a forwarding user. When address translation occurs (in the case of a token sent by the client and replaced with a proper address by the MMS Proxy/Relay), both the pre and post translated addresses (with appropriate correlation) are included.

MMS Version The version of MMS used by the target.

Transaction ID An ID used to correlate an MMS request and response between the target and the MMS Proxy-Relay.

Message ID An ID assigned by the MMS Proxy-Relay to uniquely identify an MMS message.

Message Reference A reference, e.g., URI, for the MM which refers to the stored MM within the MMS Proxy-Relay or the MMBox.

Stored Message Reference A reference, e.g., URI, for the MM which refers to the newly stored MM within the MMS Proxy-Relay or the MMBox.

MMS Date/Time Date and Time when the MM was last handled (either originated or forwarded). For origination, included by the sending MMS client or the originating MMS Proxy-Relay.

Subject Subject of the MM.

Message Class Class of the MM. For example, a value of "auto" is automatically generated by the UE. If the field is not present, the class should be interpreted as "personal".

Expiry Length of time the MM will be stored in MMS Proxy- Relay or time to delete the MM. The field has two formats, either absolute or relative.

Desired Delivery Time Date and Time of desired delivery. Indicates the earliest possible delivery of the MM to the recipient.

Priority Priority of the MM assigned by the originator MMS UE.

Sender Visibility An indication that the sender's address should not be delivered to the recipient.

Delivery Report Specifies whether the originator MMS UE requests a delivery report from each recipient.

Read Report Specifies whether the originator MMS UE requests a read report from each recipient.

Store Specifies whether the originator MMS UE wants the submitted MM to be saved in the user's MMBox, in addition to sending it.

Applic-ID Identification of the originating application of the original MM.

Reply-Applic-ID Identification of the destination application of the original MM.

Aux-Applic-Info Auxiliary application addressing information as indicated in the original MM.

Content Class Classifies the content of the MM to the smallest content class to which the message belongs.

DRM Content Indicates if the MM contains any DRM-protected element.

Adaptation Allowed Indicates if the originator wishes the MM to be adapted or not. This wish can be overridden by DRM protection rules or MMS service provider / network operator configuration.

Content Type The content type of the MM.

Previously Sent By Address of the MMS Client that forwarded or originally sent the message and a sequence number. A higher sequence number indicates a forwarding event at a later point in time. This header field MAY appear multiple times.

Previously Sent by Date/Time Date and time of a forwarding or original send transaction of the message and a sequence number. The sequence number indicates the correspondence to the MMS Client's address in the "X-Mms-Previously- Sent-By" header field with the same sequence number. This header field MAY appear multiple times.

MM State Identifies the value of the MM State associated with a to be stored or stored MM.

MM Flags Identifies a keyword to add or remove from the list of keywords associated with a stored MM.

Content Location This field defines the location of the content to be retrieved.

Response Status MMS specific status.

Response Status Text Text that qualifies the Response Status.

Store Status Indicates if the MM was successfully stored in the MMBox.

Store Status Text Text that qualifies the Store Status.

Message Size Identifies the size of the MM.

Distribution Indicator Identifies whether the originator (e.g., a Value Added Service Provider) allows the MM to be further distributed. A "No" value indicates to the user that the originator requested the content of the MM is not supposed to be distributed further.

Element Descriptor Contains the Content-Reference associated with the corresponding top level message content of the MM waiting for retrieval and MAY additionally contain the type/format of the message content.

Retrieval Mode Indicates whether manual retrieval mode is recommended for the MM.

Retrieval Mode Text Explains why manual retrieval mode is recommended for the MM.

Retrieve Status MMS specific status.

Retrieve Status Text Text that qualifies the Retrieve Status.

Replace ID This field indicates the reference (i.e. Message-ID) of the previous MM that is replaced by the current MM.

MMS Status Provides a MMS status. A status of "retrieved" is only signalled by the retrieving UE after retrieval of the MM.

MMS Status Text Text that qualifies the MMS Status.

Report Allowed Indication whether or not the sending of delivery report is allowed by the recipient MMS Client.

MMS Forward Req Date/Time Date and Time that the MM was requested to be forwarded.

Read Status Status of the MM regarding whether it was read or not, e.g. Read, Deleted without being read.

Read Status text Text explanation corresponding to the Read Status.

Cancel ID This field includes the Message ID identifying the message to be cancelled.

Cancel Status Provides the status of the cancel request.

MMS Start A number, indicating the index of the first MM of those selected to have information returned in the response.

MMS Limit A number indicating the maximum number of selected MMs whose information are to be returned in the response. If this is absent, information elements from all remaining MMs are to be returned. If this is zero then no MM-related information are to be returned.

MMS Attributes A list of information elements that should appear in the view for each selected message.

MMS Totals Indicates a request for or the actual count of messages currently stored in the MMBox.

MMS Quotas Indicates a request for or the actual quotas for the user's MMBox in messages or bytes.

MMS Message Count Identifies the number of messages in the content part of the PDU.

Correlation ID Provides a mechanism to correlate IRI for MMS with CC for MMS based on a single MMS event.

MMS Content The actual contents of the MM that is carried in the payload field of MMS messages.

Most of the parameters contained in Table 18.3.2-1 can be mapped to specific parameters defined in [73].

Editor's Note:	Specific mapping for the parameters of Table 18.3.2-1 should be added in an update to the present document.

Clause 8 of [73] defines procedures for address translation for MMS and it is those procedures that are referenced whenever Clause 18.3 of the present document refers to address translation.

*** END OF FIRST MODIFICATION ***

*** SECOND MODIFICATION ***

#### 18.3.3.2	MMS Notification & Response

The MMS Notification report event is generated at the MMS Proxy-Relay, when the MMS Proxy-Relay sends a MMS Notification (M-Notification.ind as defined in [73]) to the target indicating the arrival of an MMS for the target.

The elements of Table 18.3.3.3 will be delivered to the DF2, if available.

Table 18.3.3.3: Information Elements for MMS Notification Event

Observed MSISDN

Observed IMSI

Observed IMEI

Observed MMS Address

Observed IPv4/IPv6 Address

Event Type

Event Time

Event Date

Network Element Identifier

To Recipient(s) addresses

CC Recipient(s) addresses

BCC Recipient(s) addresses

From address (included regardless of anonymity)

MMS Version

Transaction ID

Message ID

MMS Date/Time (Timestamp when this MMS was last handled [either originated or forwarded])

Previously Sent By

Previously Sent By Date/Time (May appear multiple times)

MM State

Message Class

Priority

Message Size

Expiry

Distribution Indicator

Element Descriptor

Retrieval Mode

Retrieval Mode Text

Sender Visibility

Delivery Report

Applic ID

Reply Applic ID

Aux Applic Info

DRM Content

Replace ID

Content Location

Correlation ID (only if MMS CC is to be delivered)

The MMS Notification Response report event is generated at the MMS Proxy-Relay, when the MMS Proxy-Relay receives a MMS Notification Response (M-NotifyResp.ind as defined in [73]) from the target acknowledging the MMS Notification.

The elements of Table 18.3.3.4 will be delivered to the DF2, if available.

Table 18.3.3.4: Information Elements for MMS Notification Response Event

Observed MSISDN

Observed IMSI

Observed IMEI

Observed MMS Address

Observed IPv4/IPv6 Address

Event Type

Event Time

Event Date

Network Element Identifier

MMS Version

Transaction ID

Message ID

MMS Status

Report Allowed

*** END OF SECOND MODIFICATION ***

*** THIRD MODIFICATION ***

#### 18.3.3.3	MMS Retrieval & Acknowledgement

The MMS Retrieval report event is generated at the MMS Proxy-Relay when the MMS Proxy-Relay sends a Retrieve.conf PDU (as defined in [73]) to the target. According to [73], the MMS Proxy-Relay sends a Retrieve.conf PDU to the target in response to a MMS retrieval request from the target.

The elements of Table 18.3.3.5 will be delivered to the DF2, if available.

Table 18.3.3.5: Information Elements for MMS Retrieval Event

Observed MSISDN

Observed IMSI

Observed IMEI

Observed MMS Address

Observed IPv4/IPv6 Address

Event Type

Event Time

Event Date

Network Element Identifier

To Recipient(s) addresses

CC Recipient(s) addresses

BCC Recipient(s) addresses

From address (included regardless of anonymity)

MMS Version

Transaction ID

Message ID

MMS Date/Time (Timestamp when this MMS was last handled [either originated or forwarded])

Previously Sent By

Previously Sent By Date/Time (May appear multiple times)

MM State

Message Class

Priority

Sender Visibility

Delivery Report

Read Report

Retrieve Status

Retrieve Status Text

Distribution Indicator

Applic ID

Reply Applic ID

Aux Applic Info

Content Class

DRM Content

Replace ID

Correlation ID (only if MMS CC is also intercepted)

When the MMS Retrieval event is generated and CC interception is required, the MMS Retrieval CC event shall also be generated. In this case, the elements of Table 18.3.3.6 will be delivered to the DF3, if available.

Table 18.3.3.6: Information Elements for MMS Retrieval CC Event

Observed MSISDN

Observed IMSI

Observed IMEI

Observed MMS Address

Observed IPv4/IPv6 Address

Event Type

Event Time

Event Date

Network Element Identifier

MMS Version

Subject

Correlation ID

MMS Content

The MMS Retrieval Acknowledgement report event is generated at the MMS Proxy-Relay, when the MMS Proxy-Relay receives a MMS Retrieval Acknowledgement (M-Acknowledge.ind as defined in [73]) from the target acknowledging the transaction to the MMS Proxy-Relay.

The elements of Table 18.3.3.7 will be delivered to the DF2, if available.

Table 18.3.3.7: Information Elements for MMS Retrieval Acknowledgement Event

Observed MSISDN

Observed IMSI

Observed IMEI

Observed MMS Address

Observed IPv4/IPv6 Address

Event Type

Event Time

Event Date

Network Element Identifier

MMS Version

Transaction ID

Message ID

Report Allowed

*** END OF THIRD MODIFICATION ***

*** FOURTH MODIFICATION ***

#### 18.3.3.4	MMS Forwarding

The MMS Forwarding report event is generated at the MMS Proxy-Relay, when the MMS Proxy-Relay sends a M-Forward.conf (as defined in [73]) to the target. According to [73], the MMS Proxy-Relay sends a M-Forward.conf (as defined in [73]) to the target is response to a received a MMS Forwarding Request (M-Forward.ind as defined in [73]) from the target.

The elements of Table 18.3.3.8 will be delivered to the DF2, if available.

Table 18.3.3.8: Information Elements for MMS Forwarding Event

Observed MSISDN

Observed IMSI

Observed IMEI

Observed MMS Address

Observed IPv4/IPv6 Address

Event Type

Event Time

Event Date

Network Element Identifier

To Recipient(s) address(es) (untranslated and translated)

CC Recipient(s) address(es) (untranslated and translated)

BCC Recipient(s) address(es) (untranslated and translated)

From address (includes both target provided address and if translation occurs, network substituted post-translation target address.

MMS Version

Transaction ID

Message ID

MMS Forward Req Date/Time

Message Class

Expiry

Desired Delivery Time

Priority

Sender Visibility

Delivery Report Allowed

Delivery Report

Read Report

Store

MM State

Content Location

Response Status (from M-Forward.conf)

Response Status Text (from M-Forward.conf)

Store Status (from M-Forward.conf)

Store Status Text (from M-Forward.conf)

*** END OF FOURTH MODIFICATION ***

*** FIFTH MODIFICATION ***

#### 18.3.3.6	MMS Upload

The MMS Upload report event is generated at the MMS Proxy-Relay, when the MMS Proxy-Relay sends a M-Mmbox-Upload.conf (defined in [73]) to the target. According to [73], the MSM Proxy-Relay sends a M-Mmbox-Upload.conf to the target in response to a received MMS Upload Request (M-Mmbox-Upload.req as (defined in [73]) from the target.

The elements of Table 18.3.3.10 will be delivered to the DF2, if available.

Table 18.3.3.10: Information Elements for MMS Upload Event

Observed MSISDN

Observed IMSI

Observed IMEI

Observed MMS Address

Observed IPv4/IPv6 Address

Event Type

Event Time

Event Date

Network Element Identifier

Transaction ID

MMS Version

MM State

MM Flags

Content Type

MM Description PDU (see Table 18.3.3.19)

Content Location (from M-Mmbox-Upload.conf)

Store Status (from M-Mmbox-Upload.conf)

Store Status Text (from M-Mmbox-Upload.conf)

Correlation ID (only if MMS CC is also intercepted)

When the MMS Upload event is generated and CC interception is also required, an MMS Upload CC event is also generated at the MMS Proxy-Relay. In this case, the elements of Table 18.3.3.11 will be delivered to the DF3, if available.

Table 18.3.3.11: Information Elements for MMS Upload CC Event

Observed MSISDN

Observed IMSI

Observed IMEI

Observed MMS Address

Observed IPv4/IPv6 Address

Event Type

Event Time

Event Date

Network Element Identifier

MMS Version

Correlation ID

MMS Content

*** END OF FIFTH MODIFICATION ***

*** SIXTH MODIFICATION ***

#### 18.3.3.9	MMS Read Reply

The MMS Read Reply-From-Target report event is generated at the MMS Proxy-Relay, when the MMS Proxy-Relay receives a MMS Read Reply Notification (M-Read-Rec.ind as defined in [73]) from the target. The elements of Table 18.3.3.14 will be delivered to the DF2, if available.

The MMS Read Reply-To-Target report event is generated at the MMS Proxy-Relay, when the MMS Proxy-Relay sends a MMS Read Reply Notification (M-Read-Orig.ind as defined in [73]) to the target. The elements of Table 18.3.3.15 will be delivered to the DF2, if available.

Table 18.3.3.14: Information Elements for MMS Read Reply from Target Event

Observed MSISDN

Observed IMSI

Observed IMEI

Observed MMS Address

Observed IPv4/IPv6 Address

Event Type

Event Time

Event Date

Network Element Identifier

To (MMS sender's address)

From address (includes both target provided address and if translation occurs, network substituted post-translation target address).

Message ID

Read Status

Applic-ID

Reply-Applic-ID

Aux-Applic-Info

Table 18.3.3.15: Information Elements for MMS Read Reply to Target Event

Observed MSISDN

Observed IMSI

Observed IMEI

Observed MMS Address

Observed IPv4/IPv6 Address

Event Type

Event Time

Event Date

Network Element Identifier

To (target's address)

From address (address of read reply source).

Message ID

Read Status

Read Status text

Applic-ID

Reply-Applic-ID

Aux-Applic-Info

*** END OF SIXTH MODIFICATION ***

*** SEVENTH MODIFICATION ***

#### 18.3.3.12	MMS MMBox Viewing

The MMS MMBox View Request event is generated at the MMS Proxy-Relay when the MMS Proxy-Relay receives a MMS MMBox Viewing Request (M-Mbox-View.req as defined in [73]) from the target. In this case, the elements of Table 18.3.3.17 will be delivered to the DF2, if available.

The MMS MMBox View Confirm event is generated at the MMS Proxy-Relay when the MMS Proxy-Relay sends a M-Mbox-View.conf (defined in [73]). In this case, the elements of Table 18.3.3.18 will be delivered to the DF2, if available.

Table 18.3.3.17: Information Elements for MMS MMbox View Request Event

Observed MSISDN

Observed IMSI

Observed IMEI

Observed MMS Address

Observed IPv4/IPv6 Address

Event Type

Event Time

Event Date

Network Element Identifier

MMS Version

Transaction ID

MM State

MM Flags

Content Location

MMS Start

MMS Limit

MMS Attributes

MMS Totals

MMS Quotas

Table 18.3.3.18: Information Elements for MMS MMbox View Confirm Event

Observed MSISDN

Observed IMSI

Observed IMEI

Observed MMS Address

Observed IPv4/IPv6 Address

Event Type

Event Time

Event Date

Network Element Identifier

MMS Version

Transaction ID

MM State

MM Flags

Content Location

MMS Start

MMS Limit

MMS Attributes

MMS Totals

MMS Quotas

Response Status

Response Status Text

MMS Message Count

Content Type

Correlation ID (only if MMS CC is also intercepted)

MMBox Description PDU (see Table 18.3.3.19)

Table 18.3.3.19: MM Description PDU

Correlation ID

To Recipient(s) addresses

CC Recipient(s) addresses

BCC Recipient(s) addresses

From address (included regardless of anonymity)

Message ID

MMS Date/Time

Previously Sent By (May appear multiple times)

Previously Sent By Date/Time (May appear multiple times)

MM State

MM Flags

Message Class

Priority

Delivery Time

Expiry

Sender Visibility

Delivery Report

Read Report

Message Size

Content Location

Content Type

When the MMS MMBox View Confirm event is generated at the MMS Proxy-Relay and CC interception is also required, the MMS Proxy-Relay also generates a MMS MMBox View Confirm CC event. In this case, the elements of Table 18.3.3.20 will be delivered to the DF3, if available.

Table 18.3.3.20: Information Elements for MMS MMbox View Confirm CC Event

Observed MSISDN

Observed IMSI

Observed IMEI

Observed MMS Address

Observed IPv4/IPv6 Address

Event Type

Event Time

Event Date

Network Element Identifier

MMS Version

Correlation ID

MMS Content

*** END OF SEVENTH MODIFICATION ***

*** END OF MODIFICATIONS ***