ETSI DTS 103 221-1 V0.1.12 (2017-06)

Lawful Interception (LI);

Internal interface X1

This is being put forward as a draft for final checking. All comments are welcome.

<

TECHNICAL SPECIFICATION

Reference

DTS/LI-00104

Keywords

Interception, X1

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-préfecture de Grasse (06) N° 7803/88

Important notice

The present document can be downloaded from:
http://www.etsi.org

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the only prevailing document is the print of the Portable Document Format (PDF) version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at http://portal.etsi.org/tb/status/status.asp

If you find errors in the present document, please send your comment to one of the following services:
http://portal.etsi.org/chaircor/ETSI_support.asp

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.

The content of the PDF version shall not be modified without the written authorization of ETSI.

The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute yyyy.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM and the ETSI logo are Trade Marks of ETSI registered for the benefit of its Members.
3GPPTM and LTE™ are Trade Marks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
GSM® and the GSM logo are Trade Marks registered and owned by the GSM Association.

Logos on the front page

If a logo is to be included, it should appear below the title on the right hand side of the cover page

Copyrights on page 2

Reproduction is only permitted for the purpose of standardization work undertaken within ETSI.
The copyright and the foregoing restriction extend to reproduction in all media.

Contents (style TT)

Logos on the front page	3

Copyrights on page 2	3

If an additional copyright is necessary, it shall appear on page 2 after the ETSI copyright.	3

Intellectual Property Rights	5

Foreword	5

Multi-part documents	5

Modal verbs terminology	5

Executive summary	6

Introduction	6

1	Scope	6

2	References	6

2.1	Normative references	6

2.2	Informative references	7

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	7

3.3	Abbreviations	8

4	User defined clause(s) from here onwards	8

4.1	User defined subdivisions of clause(s) from here onwards	8

Proforma copyright release text block	9

Annexes		9

Annex <A>:	Title of annex 	9

Annex <B>:	Title of annex 	9

<B.1>	First clause of the annex	10

<B.1.1>	First subdivided clause of the annex	10

Annex <C>:	ATS in TTCN-2 	10

<C.1> The TTCN-2 Machine Processable form (TTCN.MP)	10

Annex <D>:	ATS in TTCN-3 	10

<D.1>TTCN-3 files and other related modules	10

<D.2>HTML documentation of TTCN-3 files	11

Annex <E>:	Bibliography 	11

Annex <F>:	Change History	11

History	12

A few examples:	12

Intellectual Property Rights

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (http://ipr.etsi.org).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

Foreword

This Technical Specification (TS) has been produced by ETSI Technical Committee Lawful Interception (LI).

Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "may not", "need", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# Scope

The present document defines an electronic interface for the exchange of information relating to the establishment and management of Lawful Interception. Typically this interface would be used between a central LI administration function and the network internal interception points.

Typical reference models for LI define an interface between law enforcement agencies (LEAs) and communication service providers (CSPs), called handover interface, and an internal network interface within the CSP domain between administration and mediation functions for lawful interception and network internal functions, which facilitate the interception of communication. This internal network interface typically consists of three interfaces for administration (called X1), transmission of intercept related information (X2) and transmission of content of communication (X3). The present document specifies the administration interface X1.

# References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at http://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication ETSI cannot guarantee their long term validity.

## Normative references

The following referenced documents are necessary for the application of the present document.

[1]	IETF RFC 4122: “A Universally Unique Identifier (UUID) URN Namespace”

[2]	W3C Recommendation 28 October 2004: “XML Schema Part 2: Datatypes Second Edition”

[3]	ETSI TS 103 280: “Dictionary for common parameters”

[4]	3GPP TS 23.003: “Numbering, addressing and identification”

[5]	IETF RFC 4282: “The Network Access Identifier”

[6]	IETF RFC 2865: “Remote Authentication Dial In User Service (RADIUS)”

[7]	IETF RFC 1738: “Uniform Resource Locators (URL)”

[8]	IETF RFC 2818: “HTTP over TLS”

[9]	IETF RFC 7230: “Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing”

[10]	IETF RFC 5246: “The Transport Layer Security (TLS) Protocol Version 1.2”

[11]	IETF RFC 6176: “Prohibiting Secure Sockets Layer (SSL) Version 2.0”

[12]	IETF RFC 7525: “Recommendations for Secure Use of Transport Layer Security (TLS) and Datagram Transport Layer Security (DTLS)”

[13]	IETF RFC 6125: “Representation and Verification of Domain-Based Application Service Identity within Internet Public Key Infrastructure Using X.509 (PKIX) Certificates in the Context of Transport Layer Security (TLS)”

[14]	IETF RFC 4519: “Lightweight Directory Access Protocol (LDAP): Schema for User Applications”

## Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	OWAS TLS Cheat Sheet: https://www.owasp.org/index.php/Transport_Layer_Protection_Cheat_Sheet

[i.2]	ETSI TS 103 308: “CYBER; Security baseline regarding LI and RD for NFV and related platforms”

[i.3]	ETSI GS NFV-SEC 009: “Network Functions Virtualisation (NFV); NFV Security; Report on use cases and technical approaches for multi-layer host administration”

[i.4]	ETSI GS NFV-SEC 012: “Network Functions Virtualisation (NFV); Release 3; Security; System architecture specification for execution of sensitive NFV components”

# Definitions and abbreviations

## Definitions

Activate Command: The request issued by an ADMF to start an interception.

Deactivate Command: The request issued by an ADMF to stop an interception.

Interception: a continuous instance of interception at a single NE carried out against a set of target identifiers, identified by an X1 Identifier, starting from an activate command and ending with a deactivate command or terminating fault.

Protocol error: An error at the X1 protocol level (rather than any fault with ADMF or NE).

NOTE: In the present document, the term “error” in general refers to a protocol error, whereas issues with systems not behaving correctly are called “faults”.

Terminating fault: A fault signalled from NE to ADMF which terminates the specific interception.

X1: LI interfaces internal to the CSP for management tasking

X2: LI interfaces internal to the CSP for IRI delivery

X3: LI interfaces internal to the CSP for CC delivery

X1 Identifier (XID): an identifier to uniquely identify a particular interception internally to the X1 interface

X1 Transaction ID: an identifier used to identify a specific request/response pair.

## Abbreviations

For the purposes of the present document, the following abbreviations apply:

ADMF	Administration Function

CC	Content of Communication

CSP	Communication Service Provider

HI	Handover Interface

IRI	Intercept Related Information

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

NE	Network Element – the element performing the interception.

XID	X1 Identifier

# Overview

## Reference model

The X1 interface is based on communication between two entities, the CSP Administration Function (ADMF) and the Network Element (NE) performing interception. The X1 reference model is shown in Figure 1.

NE

X1

Tasking interface e.g. HI-1 (out of scope)

NE

X1

Law Enforcement

ADMF

X1

NE

X1

NE

Figure 1: X1 reference model

Only one ADMF is able to make changes to tasking on a given NE. This is called the ADMF which is “responsible” for that NE.

Onward delivery of information from the NE is called X2 (for IRI) and X3 (for CC). The choice of protocols for X2 and X3 are out of scope of the present document.

NOTE: The present document may be used in direct delivery scenarios, in which the NE delivers directly to the LEMF. Any consequences of using direct delivery are out of scope of the present document.

## Reference model for X1: requesting and responding

Messages are sent as a request followed by a response.

Messages may be sent in either direction i.e. with the ADMF or NE initiating the request. The side initiating the request is called the “Requester”; this term is used when it is not specified whether it is the ADMF or NE making the request. The other side is called the “Responder”.

Requester (could be ADMF or NE)

Responder (could be ADMF or NE)

Generic terminology

Request

Response

Figure 2: Showing generic terminology

It is likely that in most routine situations, the ADMF will initiate the message i.e. to distribute information or request status. However it is possible that the NE will initiate the request in order to deliver fault reports etc.

CSP Administration Function (ADMF)

Network Element (NE)

CSP Administration Function (ADMF)

Network Element (NE)

Routine situation – ADMF is requester

Reporting situation – NE is requester

Figure 3: Showing two situations with either ADMF or NE as the requester

## Overview of security

Security is based on creating public/private keys for the ADMF and each NE for which it is responsible. All tasking over X1 is performed using the security procedures in clause 8, which provide assurance that communication only takes place between an NE and ADMF which have been populated with the relevant key material.

An NE may be controlled other than by X1 e.g. by logging on to the NE manually. Any manual action performed on the NE shall be captured on the NE logging. Also the new tasking shall be able to be seen and controlled by the ADMF that is responsible for the NE. i.e. the ADMF shall be able to stop the tasking over the X1 interface (e.g. it is required to have a unique XID). There may be broader consequences that are not covered by the present document if an NE is tasked independently of the X1 interface (e.g. security concerns).

## Relationship to other standards

The present document forms part of a family of internal interface documents covering all of X1, X2 and X3 which are handled as separate standards.

## Release management

This clause describes the release management requirements. The requirements are:

The version of the present document is defined as <major>.<minor>.<patch>

The major version should be incremented when making a backwards incompatible change.

The minor version should be incremented when adding backwards compatible functionality.

The patch version should be incremented when fixing a backwards compatible bug.

Once a major version has been incremented, the previous major version will be supported for 2 years after publication of the new version. Change requests issued to a version that is no longer supported will need to be issued for the latest supported major version.

# Basic concepts

## The lifecycle of an interception (identified by XID)

An interception relates to a single target identifier, and goes from the point an activate command is sent by the ADMF, though to the time a deactivate command is sent by the ADMF, or a “terminating fault” (see clause 5.4) occurs.

Each interception is uniquely identified by an X1 Identifier (XID) and it is handled independently of all others. The ADMF shall choose the XID as a UUID (version 4) as per RFC4122 [7]. The ADMF is responsible for correlating the XID to the identifiers used to communicate with Law Enforcement.

Once the interception has ended, the details of the interception may be discarded by the NE.

Either of the following situations shall cause an error:

Attempting to activate an existing XID

Attempting to deactivate a non-existent XID.

Intercepted traffic is delivered by the NE to a destination. Each destination is uniquely identified by a Destination Identifier (DID), and is handled independently from details of the interception. Each interception is associated with one or more destinations.

## The lifecycle of an X1 request/response (identified by X1TransactionID)

Each request and response shall be identified by an X1TransactionID.  The requester (may be ADMF or NE) shall choose an X1TransactionID as a UUID (version 4) as per RFC4122 [1].

The responder handles an X1 request by checking the message both syntactically and semantically and then sending a response.  The response shall be sent without undue delay and shall be sent within TIME1 of receiving the request.  TIME1 shall be configurable and by default TIME1 shall be five seconds.

For requests that affect an interception lifecycle (ACTIVATE, MODIFY, DEACTIVATE or DEACTIVATE ALL), if the Requester does not receive a response within TIME2, then it should consider requesting the status of the XID in question to see whether the prior request has been actioned.  TIME2 shall be configurable, it shall be at least twice TIME1 and by default shall be fifteen seconds.

For requests that do not change interception lifecycle, if the Requester does not receive a response within TIME2, then it should consider re-sending the original request (as a new request, with a new X1TransactionID).

For all requests, an error response shall be sent if the request message is not compliant syntactically (it does not match the schema) or semantically (it is not compliant or consistent with the existing state of the NE (e.g., activating an existing XID).

For all requests, an “OK – Acknowledged and Completed” response shall be sent if the request is fully understood, compliant and the request has been successfully completed within TIME1.  The NE and ADMF shall be designed so that information requested (status and tasking information) is in a data store which is readily available without undue delay and within TIME1.  If the request was a request for information (GET DETAILS, GET ALL DETAILS or LIST ALL INTERCEPTIONS) then all the information shall be delivered together as part of the “OK – Acknowledged and Completed” response message.

Because interception lifecycle affecting requests take time, if the request message can be parsed and it is compliant against the existing state of the NE, but if the action requested cannot be completed within TIME1, an “OK – Acknowledged” response shall be sent.  A status report shall be sent by the NE as soon as the action is completed or if it unsuccessful (see clause 6.4.2.2). This status report shall be sent as a new request/response pair, using the same XID but the status report shall have its own X1TransactionID.

Checks about downstream delivery of information are outside the scope of X1 e.g. the present document does not cover reporting that communications content has been seen and delivered, or that intercept-related-information has been delivered to the LEMF.

## Warnings and Faults

The present document uses the term “error” to mean a protocol error within the X1 protocol as defined in clause 6.6.

All other problems are categorised as “issues”, which may be warnings or faults.

Warnings are one-off messages i.e. sent by the NE and then not referred to again over X1. Warnings shall not be used for issues which are affecting traffic (i.e. losing content or intercept-related information). For example, warnings may include resources being nearly exhausted but not yet traffic-affecting. Warnings should include that keys/certificates are about to expire.

Faults are issues which the NE will continue to be aware of and which the NE is trying to manage and/or rectify. Any issue which loses traffic is categorised as a fault.

Warnings are reported using issue-reporting messages (clause 6.4) but then are not included in any future Status-Getting messages (clause 6.3). NE shall log any warnings for audit reasons.

The NE shall remember which of the XIDs are in fault and whether the NE itself is in a fault situation. An issue report (see clause 6.4) is required at the start of the fault. The NE shall also report faults when responding to the Status-Getting message defined in clause 6.3.

## Terminating and non-terminating faults

Faults for an XID are either non-terminating or terminating. A terminating fault will end the interception (see clause 5.1). The present document does not give a complete definition of when the NE should consider the fault terminating or non- terminating. The following shall always be considered terminating:

Heartbeat lost.

Expiry of certificates or keys.

Deleted by non-X1 input at NE (i.e. manual intervention).

Identifier or Traffic Type is not comprehensible or will never be seen on this NE.

NE lost target details (i.e. terminated because target definition has been lost).

Security concern at NE means target details were deleted.

# Message Structure and Data Definitions

## X1 Message details

X1 messages contain information as defined in Table 6.1 (the information is Mandatory, Optional or Conditional as shown in the last column).

Table 6.1 Message details

| Field | Description | Format | Mandatory (M), Optional (O) or Conditional (C) |
| --- | --- | --- | --- |
| ADMF Identifier | Uniquely identifies the ADMF to the NE. Required to match the details provided by the ADMF’s X.509 certificate (see clause 8) | token as per [2] section 3.4.2. Definition and assignment of identifiers is a deployment issue | M |
| NE Identifier | Uniquely identifies the NE to the ADMF. Required to match the details provided by the ADMF’s X.509 certificate (see clause 8) | token as per [2] section 3.4.2. Definition and assignment of identifiers is a deployment issue | M |
| MessageTimestamp | Timestamp indicating the time the message was sent by the requester. | See TS 103 280 [3] Qualified Microsecond Date Time. | M |
| Version | Version of this specification used for encoding the message. | Set to “v1.1.1”. See notes on release managed in clause 4.5. | M |
| X1TransactionID | Used to correlate Request and Response. Shall be omitted for “TopLevelError” situations as defined below this table but otherwise is mandatory. | An ID as defined in clause 5.2. | C |

In addition to the information in the table above, the X1 Request shall contain the following fields:

Type of Request (one of the list below).

Request Parameters for that type of request (as detailed in this clause).

If the Request was incomprehensible (no recognizable header or formatting) then the Response message shall be constructed with an ADMF and NE Identifier, MessageTimestamp and Version, and a “TopLevelError” flag but no other information.

If the Request was comprehensible then the X1 Response shall contain the following fields (in addition to the information in the table above):

Type of Request (one of the list below): the same as from the request.

Response Parameters for that type of request (as detailed in this clause).

A “RequestContainer” is used to contain one or more requests. All requests in a container are delivered at the same time, from the same Requester and to the same Responder. There is no implication about which order they are processed; for this reason, the ADMF should avoid sending ActivateTask and ModifyTask messages for the same XID in the same RequestContainer A “ResponseContainer” is used to contain all of the responses to the requests in the container. The ordering of these responses does not have a meaning. All responses are sent at the same time, from the same sender and to the same receiver. The RequestContainer and ResponseContainer shall be used even if there is one request and one response.

## Message definitions: starting, modifying and stopping tasks

### ActivateTask

#### Summary

DIRECTION: An ActivateTask request message goes from ADMF to NE.

USAGE: It is sent whenever the ADMF wants to add a new interception.

Table 6.2: ActivateTaskRequest message definition

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| Task details | Identifying the required target. | See clause 6.2.1.2 | M |

Table 6.3: ActivateTaskResponse message definition

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| OK or Error | The general errors in clause 6.6 apply. Also it is an error if the XID is already present at the NE. | See clause 6.6 | M |

There may be more than one different interception relating to the same target identifier (two distinct XIDs). The X1 interface supports delivery of this situation (i.e. it is not considered an error on the X1 interface).

NOTE: The recommended approach is that the ADMF tasks the NE only once for each particular target identifier (i.e. then the ADMF has the knowledge to instruct the MF to copy the traffic to two different places etc).

#### Task details

The task details shall include the following:

Table 6.4: Task details

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| XID | Uniquely identifies the task | UUIDv4 (see clause 5.1) | M |
| TargetIdentifiers | List of Target Identifiers that identify the relevant traffic. Where multiple identifiers are present, they shall be ANDED together i.e. all conditions are required to apply. | Each TargetIdentifier given follows one of the formats given in Table 6.Z | M |
| IRIorCC | Statement of whether to deliver IRI and/or CC | Enumerated value – one of “IRIOnly”, “CCOnly” and “IRIandCC” | M |
| DestinationDetails | Details of where to send the intercepted traffic | List of Destination Identifiers referencing the desired delivery destination records (see clause 6.2.1.3) | M |
| TaskServiceType | A statement of any further clarification or list of types of service to be intercepted or not intercepted (e.g. Voice, Data, Messaging) | If present it shall be specified as a free-text field, and shall only use values which have been agreed in advance (i.e. out of scope of the present document) by ADMF and NE | O |

The list of permissible TargetIdentifier formats is given below:

Table 6.5: Target Identifier Formats

| Format Name | Description | Format |
| --- | --- | --- |
| e164Number | E.164 Number in fully international format, written as decimal digits. | Given in TS 103 280 [3] InternationalE164 format. |
| IMSI | International Mobile Subscriber Identity, following the ITU E.212 numbering scheme, written as decimal digits. | Given in TS 103 280 [3] IMSI format |
| IMEI | International Mobile station Equipment Identity, following the numbering plan defined in 3GPP TS 23.003, written as decimal digits without the Luhn check digit | Given in TS 103 280 [3] IMEI format |
| MACAddress | A MAC address. | Given in TS 103 280 [3] MACAddress format |
| IPv4Address | IPv4 address in dotted decimal notation | Given in TS 103 280 [3] IPv4Address format |
| IPv6Address | IPv6 address written as eight groups of four hexadecimal digits separated by a colon | Given in TS 103 280 [3] IPv6Address format |
| IPv4CIDR | IPv4CIDR, written in dotted decimal notation followed by CIDR notation | Given in TS 103 280 [3] IPv4CIDR format |
| IPv6CIDR | IPv6CIDR written as eight groups of four hexadecimal digits separated by a colon, followed by CIDR notation | Given in TS 103 280 [3] IPv6CIDR format |
| TCPPort | TCP Port number, written in decimal notation | Given in TS 103 280 [3] TCPPort format |
| TCPPortRange | Range of TCP Ports, written as decimal numbers separated by a colon | Given in TS 103 280 [3] TCPPortRange format |
| UDPPort | UDP Port number, written in decimal notation | Given in TS 103 280 [3] UDPPort format |
| UDPPortRange | Range of UDP Ports, written as decimal numbers separated by a colon | Given in TS 103 280 [3] UDPPortRange format |
| EmailAddress | Email address following W3C HTML 5 Recommendation | Given in TS 103 280 [3] EmailAddress format |
| SIP-URI | SIP-URI according to the SIP URI scheme (see RFC 3261 / TS 24.229) | Given in TS 103 280 [3] SIPURI format |
| TEL-URI | TEL-URI according to the TEL URI scheme (see RFC 3966). Implementers should consider whether the value could be sent as an E.164 number (or one of the related types) instead. | Given in TS 103 280 [3] TELURI format |
| H323-URI | H323 URI according to the H323 URI scheme (see RFC 3508) | Given in H323Uri format (see XSD schema) |
| IMPU | IP Multimedia Public Identity, as per 3GPP TS 23.003 [4] | Given in IMPU format (see XSD schema) |
| IMPI | IP Multimedia Private Identity, as per 3GPP TS 23.003 [4] | Given in IMPI format (see XSD schema) |
| NAI | Network Access Identifier following RFC 4282 [5] format | Given in NAI format (see XSD schema) |
| RADIUS | Any Radius attribute that uniquely identifies the subscriber within the specific CSP (see NOTE 1). | Given as binary octets containing RADIUS AVP following RFC 2865 [5] clause 5. |
| GTPUTunnelId | GTP-U Tunnel Identifier | Given as a 32-bit integer |
| GTPCTunnelId | GTP-C Tunnel Identifier | Given as a 32-bit integer |
| CallPartyRole | Identifies the role of a party in a call. Intended for use in conjunction with e.g. E164Number | One of the values “Originating”, “Terminating”, “Forwarded-To”. |
| NonLocalIdentifier | Identifiers whether the identifier is local or non-local. Intended for use in conjunction with e.g. E164Number | One of the values “Local” or “NonLocal” |
| Pre-arranged format | Only used as an exception for types not already defined elsewhere. |  |

NOTE 1: Some Radius attributes are not usable for targeting, for example: attributes #4 (NAS-IP-Address), #32 (NAS-Identifier), #40 (Acct-Status-Type), #44 (Acct-Session-Id) and also Vendor Specific Attributes nested under Radius attribute #26.

NOTE 2: Future versions of the present document may need to consider temporary identifiers including pseudonyms or short-term identifiers which have been derived from the permanent identifiers.

The pre-arranged format shall only be used as an exception, for types that are not already defined elsewhere in table 6.5, strictly subject to agreement between the NE and ADMF

Table 6.6: Pre-arranged Format structure

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| IdentifierType | Identifies the format of the IdentifierValue field. | The mechanism for agreeing such formats is out of scope of the present document. | M |
| IdentifierValue | UTF-8 string containing the target identifier. Follows the format identified by the IdentifierType field. | UTF-8 string, matching the format given | M |

### ModifyTask

DIRECTION: A ModifyTask request message goes from ADMF to NE.

USAGE: Whenever an ADMF wants to modify and existing interception. All details for the interception shall be given (i.e. the modified details and the information that is unchanged) to totally replace the previous set of tasking details.

Table 6.7: ModifyTaskRequest message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| Task details | Identifying the required target (same as for Activate). | See clause 6.2.1.2 | M |

Table 6.8: ModifyTaskResponse message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| OK or Error | The general errors in clause 6.3.6 apply. Also it is an error if the XID is not already present. | See clause 6.3.6 | M |

### DeactivateTask

DIRECTION: A DeactivateTask request message goes from ADMF to NE.

USAGE: When the ADMF wants to deactivate (permanently stop and remove) an interception.

There is no concept of suspension or temporary deactivation. To stop an interception “temporarily”, ADMFs shall deactivate an interception and then activate a new interception.

Table 6.9: DeactivateTaskRequest message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| XID | See clause 5.1. | See clause 5.1 | M |

Table 6.10: DeactivateTaskResponse message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| OK or Error | The general errors in clause 6.3.6 apply. Also it is an error if the XID is not already present at the NE. | See clause 6.3.6 | M |

### DeactivateAllTasks command

DIRECTION: The DeactivateAllTasks command goes from ADMF to NE.

USAGE: To completely and permanently stop all interceptions on the NE.

Table 6.11: DeactivateAllTasksRequest message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| There shall be no request message parameters |  |  |  |

Table 6.12: DeactivateAllTasksResponse message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| OK or Error | The general errors in clause 6.3.6 apply. See below regarding whether “DeactivateAllTasks” is enabled; if Disabled then DeactivateAllTasks always triggers an error response. | See clause 6.3.6 | M |

The DeactiveAllTasks command shall be supported by all implementations of the present document.

It shall be agreed in advance as to whether the DeactivateAllTasks command is enabled or disabled. By default (if there has been no agreement in advance) then DeactivateAllTasks is disabled.

If DeactivateAllTasks is disabled, then a DeactivateAllTasks request shall always trigger an Error response of type “DeactivateAllTasks message is not enabled”.

If DeactivateAllTasks is enabled, then a DeactivateAllTasks request shall perform a DeactivateTask command on all XIDs on that NE, or it shall trigger an error for the general error conditions listed in clause 6.3.6.

## Message definitions: creating, modifying and removing Destinations

### CreateDestination

#### Summary

DIRECTION: A CreateDestination request message goes from ADMF to NE.

USAGE: It is sent whenever the ADMF wants to add a new destination.

Table 6.13: CreateDestinationRequest message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| Destination details | Details of the new destination | See clause 6.3.1.2 | M |

Table 6.14: CreateDestinationResponse message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| OK or Error | The general errors in clause 6.6 apply. Also it is an error if the DID is already present at the NE. | See clause 6.6 | M |

#### DestinationDetails

This clause covers delivery information as the information leaves the NE. Any further onward forwarding of data is handled by other Administrative, Mediation or Delivery Functions (out of scope of the present document).

The DestinationDetails structure is defined as follows:

Table 6.15: DestinationDetails

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| DID | Destination Identifier which uniquely identifies the destination | UUIDv4 (see clause 5.1) | M |
| FriendlyName | A human-readable name associated with the delivery destination | Free-text string | O |
| DeliveryType | Statement of whether to deliver IRI and/or CC to this destination | Enumerated value – one of “IRIOnly”, “”CCOnly” and “IRIandCC” | M |
| DeliveryAddress | One of the values from table 6.AA below. | As defined in table 6.11. | M |
| HandoverFormat | A format for delivery of the CC and IRI. | A free text field shall be populated with the name of the delivery format used. This shall match one of a pre-arranged set of values. The mechanism for agreeing these values is out of scope. NOTE: Once X2 and X3 are published, these will be the recommended standard for delivery from NE | M |

The DeliveryAddress structure is defined as follows:

Table 6.16: DeliveryAddress

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| IPAddressAndPort | This covers both IPv4 and IPv6 and contains a single IP Address and Port. | IPAddressAndPort from ETSI TS 103 280 [3]. | O |
| E164 Number | E.164 destination. | InternationalE164 (see ETSI TS 103 280 [3]). | O |
| FTPAddress | FTP address | URL as per anyURI (see [2] clause 3.2.17) but conformant to the FTP scheme defined in IETF RFC 1738 [7], which allows specification of hostname, port, path and username. | O |
| URL | URL destination | anyURI (see [2] clause 3.2.17) | O |
| FQDN | FQDN of the destination | LongString (see ETSI TS 103 280 [3]) | O |
| Email address | Email address of the destination | EmailAddress (see ETSI TS 103 280 [3]) | O |

NOTE: Once X2 and X3 are complete, Table 6.16 will be updated and some items (e.g. FTP and email address) may be removed from the list of delivery address details.

### ModifyDestination

DIRECTION: A ModifyDestination request message goes from ADMF to NE.

USAGE: Whenever an ADMF wants to modify and existing destination. All details for the destination shall be given (i.e. the modified details and the information that is unchanged) to totally replace the previous set of destination details.

Table 6.17: ModifyDestinationRequest message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| DestinationDetails | Updated details for the destination | See clause 6.3.1.2 | M |

Table 6.17: ModifyDestinationResponse message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| OK or Error | The general errors in clause 6.3.6 apply. Also it is an error if the DID is not already present. | See clause 6.3.6 | M |

### RemoveDestination

DIRECTION: A RemoveDestination request message goes from ADMF to NE.

USAGE: When the ADMF wants to remove a destination.

A destination may only be removed if it is not referenced by any active tasks. An NE shall respond with an appropriate error if the ADMF attempts to remove a destination that is referenced by an active task.

Table 6.18: RemoveDestinationRequest message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| DID | See clause 5.1. | See clause 5.1 | M |

Table 6.19: RemoveDestinationResponse message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| OK or Error | The general errors in clause 6.3.6 apply. Also it is an error if the XID is not already present at the NE. | See clause 6.3.6 | M |

### RemoveAllDestinations command

DIRECTION: The RemoveAllDestinations command goes from ADMF to NE.

USAGE: To completely and permanently stop all interceptions on the NE.

Table 6.20: RemoveAllDestinationsRequest message

| RemoveAllDestinations Message parameters |  |  |  |
| --- | --- | --- | --- |
| Field | Description | Format | M/C/O |
| There shall be no request message parameters |  |  |  |

Table 6.21: RemoveAllDestinationsResponse message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| OK or Error | The general errors in clause 6.3.6 apply. See below regarding whether “RemoveAllDestinations” is enabled; if Disabled then RemoveAllDestinations always triggers an error response. | See clause 6.3.6 | M |

The RemoveAllDestinations command shall be supported by all implementations of the present document.

It shall be agreed in advance as to whether the RemoveAllDestinations command is enabled or disabled. By default (if there has been no agreement in advance) then RemoveAllDestinations is disabled.

If RemoveAllDestinations is disabled, then a RemoveAllDestinations request shall always trigger an Error response of type “RemoveAllDestinations message is not enabled”

If RemoveAllDestinations is enabled, then a RemoveAllDestinations request shall perform a RemoveDestination command on all XIDs on that NE, or it shall trigger an error for the general error conditions listed in clause 6.3.6. Since a RemoveDestination command can only be issued against destinations that are not in use, an NE shall respond with an error if the ADMF sends a RemoveAllDestinations command while any of the destinations are referenced by active tasks.

## Message details: Getting status information from the NE

### Introduction

This clause defines messages for the ADMF to request status infromation from the NE. This is distinct from "Reporting Issues" where the NE pushes information to the ADMF (see clause 6.5).

The following messages shall be supported:

GetTaskDetails: to request details of a single interception

GetDestinationDetails: to request details of a single destination

GetAllDetails: requests details of all interceptions, destinations and the status of the NE itself

ListAllInterceptions: just gives the XIDs of all interceptions (i.e. not all the details)

### GetTaskDetails (of a particular interception)

#### Summary

DIRECTION: The GetTaskDetails command goes from ADMF to NE.

USAGE: For the ADMF to retrieve the details of a particular interception.

Table 6.22: GetTaskDetailsRequest message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| XID | See clause 5.1. | See clause 5.1 | M |

Table 6.23: GetTaskDetailsResponse message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| TaskResponseDetails | The task details are as per table 6.X, additionally containing a TaskStatus structure as per 6.3.2, unless there is an error, in which case see clause 6.6. If the XID is not present, this is an error (the appropriate error code shall be used, see clause 6.6). | See clause 6.2.1.2. | M |

#### Task Status

The Task Status relates only to the collection of the information internally to the NE.

Table 6.24: TaskStatus

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| ProvisioningStatus | Indicates whether the task has been provisioned (“complete”), has failed to provision (“failed”) or whether it is awaiting provisioning (“awaitingProvisioning”). | One of the values “awaitingProvisioning”, “failed” or “complete”. | M |
| ListOfFaults | List of all active faults on that task – a list of free text fields. |  |  |
| TimeOfLastIntercept | Time of last traffic intercepted if any (omit only if none seen so far). | See TS 103 280 [3] Qualified Microsecond Date Time. | C |
| AmountOfData | Data seen so far in Mb | Integer | C |
| TimeOfLastMod | Time of the last modification to the task (omit only if unmodified) | See TS 103 280 [3] Qualified Microsecond Date Time. | C |
| NumberOfMods | Number of modifications since start | Integer | C |

TimeOfLastIntercept, AmountOfData, TimeOfLastMod and NumberOfMods may be omitted if the NE has been configured to do so.

### GetDestinationDetails (of a particular destination)

#### Summary

DIRECTION: The GetDestinationDetails command goes from ADMF to NE.

USAGE: For the ADMF to retrieve the details of a particular destination.

Table 6.25: GetDestinationRequest message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| DID | See clause 5.1. | See clause 5.1 | M |

Table 6.26: GetDestinationResponse message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| DestinationResponseDetails | The destination details are as per table 6.10, additionally containing a DestinationStatus structure as per 6.4.3.2, unless there is an error, in which case see clause 6.6. If the XID is not present, this is an error (the appropriate error code shall be used, see clause 6.6). | See clause 6.2.1.2. | M |

#### DestinationStatus

The DestinationStatus relates only to the status of the delivery destination as seen by the NE.

Table 6.27: DestinationStatus

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| DestinationStatus | Status of destination (or two destinations, if CC and IRI are sent separately). Indicating whether the destination is active and working, or whether there is a delivery fault and traffic being lost. It is possible in the DeliveryFault state that some traffic is still being delivered – the determining factor is that issues with delivery to this destination is causing some traffic to be lost. | One of “ActiveAndWorking” or “DeliveryFaults” | M |
| ListOfFaults | List of all active faults on that destination – a list of free text fields. |  | M |

### GetAllDetails

#### Summary

DIRECTION: The GetDetails command goes from ADMF to NE.

USAGE: For the ADMF to determine the details of all interceptions and the status of the NE itself.

| Table 6.28: GetAllDetailsRequest messageField | Description | Format | M/C/O |
| --- | --- | --- | --- |
| THERE SHALL BE NO REQUEST MESSAGE PARAMETERS |  |  |  |

| Table 6.29: GetAllDetailsResponse messageGetAllDetails Message parameters |  |  |  |
| --- | --- | --- | --- |
| Field | Description | Format | M/C/O |
| NEStatusDetails | The NEStatusDetails for the NE. The NE Status shall be one of “OK” i.e. no NE faults, or “Faults” i.e. NE losing traffic (these are separate from delivery faults which are reported per XID). Additionally, a list of currently unresolved faults (list of free text items) shall be included (previous warnings are not included here). |  | M |
| ListOfTaskResponseDetails | The response shall include TaskResponseDetails structures for all interceptions present on the NE. If there are no interceptions, an empty list shall be returned – this is not an error. | See clause 6.2.1.2 and 6.3.2.2 | M |
| ListOfDestinationResponseDetails | The response shall include DestinationResponseDetails structures for all destinations present on the NE. If there are no destinations, an empty list shall be returned – this is not an error. | See clause 6.3.1.2 and 6.4,3,2 | M |

### ListAllInterceptions

#### Summary

DIRECTION: The ListAllInterceptions command goes from ADMF to NE.

USAGE: For the ADMF to retrieve the list of all XIDs and DIDs (i.e. a list of identifiers) but no details.

| Table 6.30: ListAllInterceptionsRequest messageField | Description | Format | M/C/O |
| --- | --- | --- | --- |
| There shall be no request parameters |  |  |  |

| Table 6.31: ListAllInterceptionsResponse messageField | Description | Format | M/C/O |
| --- | --- | --- | --- |
| List of XIDs | A list of all XIDs on the NE. If there are none, then an empty list is returned; this is not an error. | List of XIDs | M |
| List of DIDs | A list of all DIDs on the NE. If there is none, then an empty list is returned; this is not an error | List of DIDs | M |

## Message details: Reporting issues from the NE

### Introduction

This clause defines messages for the NE to report issues to the ADMF. It is distinct from “Getting Status”,  in which the ADMF retrieves information from the NE (see clause 6.4).

Issues may be:

Relating to a particular XID (including delivery issues with that XID).

Relating to the whole NE.

### ReportTaskIssue on given XID

#### Summary

DIRECTION: The ReportTaskIssue command goes from NE to ADMF.

USAGE: The NE shall send a ReportTaskIssue message when it becomes aware of an issue (warning or fault) relating specifically to a particular XID . It shall also be used to follow up on an “ACK” message, to signal that a request has been completed (clause 5.2) successfully or unsuccessfully.

Faults and warnings are defined in clause 5.3; see also clauses 5.1 and 5.4 about terminating and non-terminating faults.

If a non-terminating fault becomes terminating, the NE shall send another ReportTaskIssue.

If a non-terminating fault is cleared, the NE shall send another ReportTaskIssue indicating the fault is cleared.

Table 6.32: ReportInterceptionIssueRequest message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| XID | See clause 5.1. | See clause 5.1 | M |
| TaskReportType | Type of Issue | See clause 6.5.2.2 | M |
| TaskIssueErrorCode | Error code associated with the issue, if appropriate | See clause 6.6 | O |
| TaskIssueDetails | Further description of issue if appropriate. | Free text | O |

Table 6.33: ReportInterceptionIssueResponse message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| OK or Error | The general errors in clause 6.6 apply. | See clause 6.6 | M |

It is possible that the ADMF is not aware of the XID which is referenced in the NE message. The ADMF is required not to send an error back to the NE in this situation: it is for the ADMF to decide how to handle this (e.g. GetAllDetails or Deactivate the XID in question are possible approaches).

#### Task report types

The TaskReportType shall be one of the following:

All clear: non-terminating fault resolved.

Warning: not traffic-affecting.

Non-terminating fault (currently unable to collect traffic but not terminating).

Terminating fault. The message is used once from NE to ADMF and then the task is deactivated.

Actioned: Request has been fully actioned and was successful (to follow up on “ACK” message from clause 5.2).

Failed: Request has been fully actioned but was unsuccessful (to follow up on “ACK” message from clause 5.2). This is a terminating fault.

### ReportDestinationIssue on given DID

#### Summary

DIRECTION: The ReportDestinationIssue command goes from NE to ADMF.

USAGE: The NE shall send a ReportDestinationIssue message when it becomes aware of an issue (warning or fault) relating specifically to a particular DID.

Faults and warnings are defined in clause 5.3; see also clauses 5.1 and 5.4 about terminating and non-terminating faults.

If a non-terminating fault becomes terminating, the NE shall send another ReportDestinationIssue.

If a non-terminating fault is cleared, the NE shall send another ReportDestinationIssue indicating the fault is cleared.

Table 6.34: ReportDestinationIssueRequest message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| DID | See clause 5.1. | See clause 5.1 | M |
| DestinationReportType | Type of Issue | Same as TaskReportType, see clause 6.5.2.2 | M |
| DestinationIssueErrorCode | Error code for the issue, if appropriate | See clause 6.6 | O |
| DestinationIssueDetails | Further description of issue if appropriate. | Free text | O |

Table 6.35: ReportDestinationIssueResponse message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| OK or Error | The general errors in clause 6.6 apply. | See clause 6.6 | M |

### ReportNEIssue

DIRECTION: The ReportNEIssue command goes from NE to ADMF.

USAGE: The NE shall send a ReportNEIssue message when it becomes aware of an issue (warning or fault) relating to the whole NE.

NE issues can relate to:

Any hardware issue on NE (storage nearly full, power issue)

Current security issue on NE

Any issues with logging or audit material

Any report from manual changes to NE configuration

Table 6.36: ReportNEIssueRequest message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| NEIssueDetails | Description of issue being reported, including type of message (Warning, Fault Cleared, Fault Report) and description. | Description is a free text field. | M |

Table 6.37: ReportNEIssueResponse message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| OK or Error | The general errors in clause 6.6 apply. | See clause 6.6 | M |

## Message details: Pings and heartbeats

### Ping

DIRECTION: The Ping command goes in either direction.

USAGE: At any time from the ADMF or NE, to get a response over the X1 interface (does not test X2 or X3 or onward delivery).

Table 6.38: PingRequest message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| There shall be no request parameters |  |  |  |

Table 6.39: PingResponse message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| OK or Error | The OK message has no other content. The general errors in clause 6.6 apply. | Clause 6.6. | M |

### Heartbeat

DIRECTION: The Heartbeat command goes from ADMF to NE.

USAGE: See below.

Table 6.40: HeartbeatReqeust message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| There shal be no request parameters |  |  |  |

Table 6.41: HeartbeatResponse message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| OK or Error | The OK message has no other content. The general errors in clause 6.6 apply. | Clause 6.6. | M |

The Heartbeat functionality shall be supported by NE and ADMF. It is for prior agreement to determine whether heartbeats are enabled or disabled. By default (with no prior agreement) they are disabled.

If Heartbeats are enabled, the ADMF shall send out a Heartbeat message at least every TIME_P1 (by default TIME_P1 is 10 minutes) and no more than every TIME_P2 (by default TIME_P2 is 1 minute). Unless there is any specific reason (traffic shaping) it is recommended that the ADMF sends a Heartbeat consistently every TIME_P1, though it is acceptable to choose random times (e.g. in a window of 8 to 10 minutes) to prevent analysis of the traffic.

The NE shall respond with an OK for each Heartbeat. If the NE has not seen a Heartbeat message for TIME_P3 (by default TIME_P3 is 2 hours) then the NE shall perform a DeactivateAllTasks command i.e. deactivate all XIDs on the NE.

## Protocol error details

If the Responder is unable to perform an action requested as part of a Request Message, then it shall respond to that Request Message with an Error Response.

An error message is a response which has the information from clause 6.1, but the response body has an error code from the list below and a free text field for further information. It has the following structure:

Table 6.42: Error message

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| RequestMessageType | Indicates the type of Request Message that the Error Response message is a response to | One of the following: “ActivateTask”, “ModifyTask”, “DeactivateTask”, “DeactivateAllTasks”, “GetTaskDetails”, “CreateDestination”, “ModifyDestination”, “RemoveDestination”, “RemoveAllDestinations”, “GetDestination”, “GetAllDetails”, “ListAllInterceptions”, “ReportInterceptionIssue”, “ReportNEIssue”, “Ping”, “Heartbeat” | M |
| ErrorCode | Integer code indicating the type of error (see table X.1 below) | Integer | M |
| ErrorDescription | Free text field giving further details of the error | UTF-8 string | C |

This is used only as a response to a request which could not be actioned or understood. It is different from reporting on the status of the interception which are called “faults” and “warnings” but not “protocol errors”.

There are the following types of errors:

General errors (applying to all types of messages):

Security issue encountered.

Bad naming of sender or receiver (this may also show up as part of security checks).

Does not pass schema checks (NB if core parameters of message are incomprehensible then this is a separate case – see clause 5.1). Specifically look at identifying specific message element checks: required element missing, value change not allowed, improper value specified.

Message type not supported (e.g. if Deactivate All is requested but it has been agreed it is not supported).

Issue relating to different versions of the present document being used

Errors relating to the XID field:

XID not present (this is an error for those request types where it shall be present e.g. activate, get).

XID present (an error for those where it shall not be present e.g. list all).

XID not already activated (an error for those messages which relate to an existing XID e.g. get, modify, deactivate).

XID already activated (an error for activate message).

Specific errors for Activate/Modify:

End point not useable or not reachable (believed to be permanent). If this is a temporary error, then the interception could be tasked and a message issued to state that it is temporarily not working.

Target Identifier not useable. This could be because the format of it is not recognised or not able to be used in this context. Or it could be because it is clear that this target ID could not ever be seen on this NE.

Table 6.43: Error codes

| Error Code | Error Description | Message Element |
| --- | --- | --- |
| 3000 | General Error (i.e. not matching any other here) | Free text: Summary if possible |
| Errors for all messages |  |  |
| 3010 | Security issue | Free text: Description e.g. of stage that security checks failed. |
| 3011 | Bad naming of sender | Free text: Field that does not match |
| 3012 | Bad naming of receiver | Free text: Field that does not match |
| 3013 | XML schema check fail | Free text: Report from XML parser |
| 3014 | Message type not supported | None. |
| 3015 | Different version issue | None |
| 3016 | Response not received in time. | Free text field: Reference the timer that was exceeded (e.g. by referencing the clause number of the present document in which it is specified). |
| Errors relating to XID |  |  |
| 3020 | XID not present where it is required | None |
| 3021 | XID present where it is required not to be present | None |
| 3022 | Specified XID is already active | None |
| 3023 | Specified XID is not already active | None |
| Errors for Activate/Modify |  |  |
| 3030 | Specified endpoint cannot be reached | None |
| 3031 | Target identifier not useable | None |
| Other |  |  |
| 3100-3999 | Reserved for future Errors. | None |

# Transport and Encoding

## Encoding

XML encoding shall be used as defined in Annex C. An XSD schema is provided in [schema filename]. In the event of a discrepancy between the XSD schema and the present document, the present document shall be considered authoritative.

## Transport layer

### HTTPS and HTTP

HTTPS shall be used as per RFC 2818 [8]. The details relating to HTTP are given in this clause and the details relating to TLS are specified in clause 8.1.

NOTE: In this clause, the term HTTP is used (it is implicit that it is in fact HTTPS i.e. that the HTTP is used over TLS)

### How HTTP is used

The ADMF and NE shall both run HTTP clients and servers:

For messages where the ADMF is the requester (called the “routine” situation in clause 4), the ADMF shall use its HTTP client and the NE shall use its HTTP server.

For messages where the NE is the requester (called the “reporting” situation in clause 4), the NE shall use its HTTP client and the ADMF shall use its HTTP server.

Details in the request:

Each “RequestContainer” shall be sent as a HTTP request. It shall be a “POST” message (regardless of which type of X1 message it is) and the message body shall contain the X1Message RequestContainer as described in clause 6.

Details in the response:

Each “ResponseContainer” message shall be sent as a HTTP response.

The response shall indicate HTTP level errors within the range of HTTP error codes. If the HTTP level transaction is successful, then the response shall be a 200 OK message, with the X1Message ResponseContainer contained within the message body.

HTTP error codes shall only be used to indicate HTTP-level errors, and shall not be used to indicate errors with the X1 messages themselves. X1-level errors shall be indicated by correct use of the appropriate Error response message, encoded and returned as a HTTP 200 OK response.

### 7.2.3 Configuration and versions

HTTP version 1.1 shall be used as per RFC 7230 [9] and related specifications.

NOTE: HTTP/1.1 defaults to the use of "persistent connections" (see RFC 7230 [9] section 6.3). Implementers are encouraged to support the use of persistent connections

HTTP Pipelining shall not be used.

A Requester may issue multiple HTTP requests in parallel over multiple HTTP connections. However, such implementations should be aware that there is no guarantee of the order in which these requests are processed by the Responder. If such ordering is important to the Requester, it is responsible for ensuring the requests are sent out in the correct order, and for waiting for the response to each request before issuing the next one.  Transfer Coding shall not be applied to the HTTP Request or Response (see RFC 7230 [9] section 4) .

By default port 443 shall be used. If this is already in use, then the NE and ADMF shall be able to be configured with a port number, which shall be agreed prior to use of the standard.

By default the HTTP path for X1 request messages shall be “/X1/ADMF” for the ADMF and “/X1/NE” for the NE. An exception to the default shall only be made with strict agreement between NE and ADMF; however, implementers shall ensure that an X1 implementation can be configured with a different path if required.

# Security

## Transport Security

### Summary

TLS shall be used which provides authentication and authorization, integrity and confidentiality as well as replay protection between the TLS endpoints. Encryption shall be mandatory except for testing purposes.

### Profile

TLS shall be followed as defined in RFC5246 [10], and SSL2.0 shall not be used as described in RFC6176 [11].

RFC7525 [12] shall be followed.

### Key generation, deployment and storage

Apart from requirements given in clause 8.1.4, aspects concerning the generation, distribution, storage and revocation of key material and certificates are out of scope of the present document. Implementations are encouraged to support best practice e.g. the guidance given in OWASP TLS Cheat Sheet section 2.6. [i.1].

NOTE: It is assumed that the NE and ADMF are in a physically secure environment. For future uses e.g. NFV then this assumption would no longer be valid. Further details would then need to be added about the security of storage of key or certificate material e.g. TPM, Secure enclaves. See ETSI TC CYBER 103 308 [i.2] and ETSI NFV SEC009 [i.3] and SEC012 [i.4].

### Authentication

Implementations shall perform mutual authentication using X.509 certificates following RFC 6125 [13].

X1 implementations shall check that the UID part of the Subject field in the certificate (see RFC4519 [14]) provided matches the Sender or Receiver ID (whichever is provided by the other party in the communication). If a Responder receives an X1 message where these values do not match, it shall respond with an X1 error message indicating that the Requester is not authorised. If the Requester receives a response where these values do not match, then it shall disregard the response and log an appropriate error message.

## Additional security measures (beyond transport layer)

It will be important to follow general security best practice (e.g. use of firewalls and/or access lists to prevent denial-of-service attacks). This is out of scope of the present document.

The present document does not recommend that message-layer encryption or message-level message authentication codes are used in addition to the provisions in this clause. Of course there may be threat models in which additional encryption may be thought to be useful. The present document does not forbid adding message-layer encryption e.g. by encrypting the whole of the payloads of the request and response messages. The details of the changes needed to do this are outside the scope of the present document.

History

| Document history |  |  |
| --- | --- | --- |