3GPP TSG-SA3 Meeting #66	s3i170241

# Ljubljana, Slovenia, 25th Jul 2017 - 28th Jul 2017			revision of s3i140235

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | 0257 | rev | 1 | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Push to Talk over Cellular Service addition of PTC feature, part B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI14 |  |  |  |  |  | Date: |  |  | 2017-07-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of feature for Push to Talk over Cellular type services |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of informational elements and events to add further clarity for PTC type services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Will not be able to meet regulatory requirements since LI requirments for PTC is not currently defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause addition Z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Other CR’s related to this contribution are S3i170240 and S3i170249. |  |  |  |  |  |  |  |  |

*** Start of FIRST MODIFICATION ***

Z.3.3 	Structure of the Events

The LI event messages sent to the DF2/DF3 is triggered by different PTC session related and non-call related events/reports. Details for each message are described in the following clauses.

The following events are applicable to both the PoC and MCPTT service ICE:

- PTC Session Initiation

- PTC Session Abandon

- PTC Session Start

- PTC Session End

- PTC Start of Interception

- PTC Pre-Established Session

- PTC Instant Personal Alert

- PTC Party Join

- PTC Party Drop

- PTC Party Hold Retrieve

- PTC Media Modification

- PTC Group Advertisement

- PTC Floor Control

- PTC Target Presence

- PTC Associate Presence

- PTC List Management

- PTC Access Policy

- Group call request (Client – Server)

- Group call request (Server – Server)

- Group call request (Server – Client)

- Group call response (Server – Client)

- Group call response (Server – Server)

- Group call response (Client – Server)

- Group call notify (Server – Client)

- Group call release request (Server – Client)

- Group call release request (Server – Server)

- Group call release response (Client – Server)

- Group call release response (Server – Server)

- Group Rejoin Request (Client – Server)

- Group Rejoin Response (Server – Client)

- Group Join Request (Client – Server)

- Group Join Response (Server – Client)

- Group Call Leave Request (Server – Client)

- Group Call Leave Response (Client – Server)

- Group Interrogate Request (Server – Server)

- Group Interrogate Response (Server – Server)

The following events are applicable to the HLR:

-	PTC Serving System;

-	HLR subscriber record change;

-	Cancel location;

-	Register location;

-	Location information request.

The following events are applicable to the SIP Core:

-	PTC Service Registration

The following events are applicable to the MCPTT service only:

- MCPTT emergency group call request

- MCPTT emergency group call response

- MCPTT emergency group call cancel request

- MCPTT emergency group call cancel response

- MCPTT emergency alert request

- MCPTT emergency alert response

- MCPTT emergency state cancel request

- MCPTT emergency state cancel response

- MCPTT imminent peril group call request

- MCPTT imminent peril group call response

- MCPTT imminent peril group call cancel request

- MCPTT imminent peril group call cancel response

A set of elements as shown below can be associated with the events above. The events trigger the transmission of the information from PTC server, LI LCS Client or HLR to DF2, perhaps via a MF in the case of Packet Data Header Information. Available IEs from this set of elements as shown below can be extended in the 3G GSN or HLR, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option e.g. a unique number for each surveillance warrant.

## Z.3.4 	Structure of the Events

The LI event messages sent to the DF2/DF3 is triggered by different PTC session related and non-call related events/reports. Details for each message are described in the following clauses.

Within Table X: Information Elements for PTC Event Records, a provisioned target identity can be a SIP URI , TEL URI , MCPTT ID or an IMEI.

A PTC Client MAY support multiple PTC Addresses and be involved in one or more PTC Sessions at the same time using the same or different PTC Addresses.

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from PTC ICE to DF2. Available IEs from this set of elements as shown below can be extended in the PTC ICE, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option e.g. a unique number for each surveillance warrant.

Table X: Information Elements for PTC Event Records

AbandonCause  - Identifies the reason for the abandoned PTC Session.

Access_Policy_Type – Identifies the type of access policy list being managed or queried by the PTC Intercept target.

Access_PolicyFailure - Reports the error code or reason for failure when Access Policy Request is unsuccessful.

Ad-hoc PoC Group Session - A PoC Group Session established by a PoC User to PoC Users listed on the invitation. The list includes PoC Users or PoC Groups or both.

Alert indicator - Indicates an emergency alert sent, received or cancelled.

AssociatePresenceStatus - Provides the Associate Presence Status, which is a list of:

PresenceID: Identity of PTC Client(s) or PTC group, when known. 				PresenceType: Identifies type of ID [PTC Client(s) or PTC group]. 				PresenceStatus: Presence state of each ID.

Broadcast indicator - Indicates that the group call request is for a broadcast group call.

Contact_Identity - Identity of the contact in the list, one contact per Contact List or Group List.

Charging Correlation ID – PTC supports both subscription based charging and traffic based charging. Provide any charging events, e.g., service activation, correlation ID between the PoC service charging data and the packet data services.

Implicit floor request - When originating client requests the floor.

Observed SIP URI  - The provisioned target identity can be a SIP URI

Observed TEL URI  - The provisioned target identity can be a TEL URI

Observed IMEI - The provisioned International Mobile Equipment Identity target identity.

MCPTT ID –  Mission Critical Push-To-Talk IDentity.

Event Type - Description of which type of event is delivered: PTC Session Initiation/Abandon, PTC session end, PTC Party Join/Drop/Hold/Retrieve, MCPPT emergency/imminent group/peril etc.

Event Time - Time of the event generation in the PTC Server or Client

Event Date - Date of the event generation in the PTC Server or Client.

Emergency indicator - Indicates the request is an MCPTT emergency call.

InitationCause - The network receives an invitation from the PTC Intercept Target to initiate a PTC session.

Failure_Code – The reason or code for the failure or closing of the session.

FloorSpeakerIdentity - Identification of the PTC Client that has the Talk Burst.

IPAPartyIdentity - Instant Personal Alert a request for one PTC subscriber to initiate a PTC Session.

Invited_PTC_Client - A PTC Client that is invited to a PTC Session

Inviting_PTC_user – The PTC User who has been invited to a PTC Session.

Join_PTC_user – The PTC User who has joined the session, i.e., associate identity or targets.

Group_Ad_Sender – The group administrator who was the originator of the group call.

Group_Ad_Reveiver – The group administrator who was the receiver of the group call.

GroupAuthorizationRules - Identifies the action requested by the target to the PTC Group authorization rules.

Group_Characteristics –

Group_Identity - Identifies the PTC Group Identity, Nick Name, and characteristics.

Hold_retrieve_Indication - The PTC Session is put on hold (deactivate Media Bursts) or a new Primary PTC Session is activated or another PTC Session is locked for talking/listening.

Hold_Retrieve_user – The PTC Session is removed from hold by the PTC User and this indicates the PTC user identity.

Imminent peril indicator - Indicates that the PTC call is an imminent peril call.

Implicit floor request - When originating client requests the floor.

ListManagementType – Different PTC Group lists: ContactListManagementAttempt, GroupListManagementAttempt, and GroupListManagementResult.

ListManagmentAction –Identifies the action requested by the PTC Intercept target to the Contact Lists or Group Lists.

List_ManagementFailure - Reports the error code or reason for failure.

Network Element Identifier - Unique identifier for the network element reporting the event.

Max_TB_Time - Include the maximum duration value for the talkburst before the permission is revoked, provide when known.

Party_Drop - Member of a PTC Group Session and leaves the PTC Session, provide when known.

PTCCorrelationId - Uniquely identifies the SIP Session, correlates CII messages, and correlates CII and CC messages.

PTCSessionInfo - Provides PTC Session information such as PTC Session URI, PTC Session type, and Nickname.

TargetID - The PTC identifier for the PTC Intercept Target.

PTC group ID - The PTC group ID of the group on which the call is initiated.

PTCHost - Indentifies the PTC participant who has authority to initiate and administrate an active PTC Group Session. Provide when known.

PTC_ID_List - The list of PTC IDs of the PTC group members.

PTCOriginatingId - Identifies the originating party.

PTCParticipants - Identifies the invited PTC participants.

PTCUserAccessPolicy – Identifies the action requested by the PTC Intercept Target to the PTC user access policy.

PreEstablishedSessionID – Identifies the PTC Pre-established Session.

PreEstablishedStatus – Indicates if the Pre-Established Session is established (setup completed), modified, or released.

Queued_FloorControl - Indicates if queuing is supported by the PTC Server and the intercept target’s PTC Client.

Queued_Position_Status - If queued floor control is supported, indicates the queue position.

RTP_Setting – The IP address and the port number at the PTC Server for the RTP Session.

Location - Report when a PTC Session is initiated by the intercept target.This parameter is not reported when the PTC Intercept Target  receives an invitation to join a PTC Session;  rather this information is reported by the PTC Session Start message (see PTC Session Start message for usage). Include when reporting of the PTC Intercept Target’s location information is authorized and known.

MediaStream_Availability – Indicates if the PTC intercept target’s PTC Client is not able/willing to receive media streams immediately.  Provide when Pre-established session is established.

MCPTT ID - The Mission Critical Push To Talk identity.

MCPTT group ID - The Mission Critical Push To Talk group Identity.

MCPTT indicator – Indicates direction of the received request as either from the client or from the group to the client.

MCPTT Organization name – Name of the organization that the Mission Critical device belongs to.

TalkburstControl_Setting – The offered Talk Burst Control Protocol, e.g., Talk Burst parameter(s) and the port numbers.  Provide when Pre-established session is established.

TargetIdentity - The PTC identifier for the PTC Intercept Target.

TargetPresenceStatus - PTC-related presence information of the PTC intercept target.

Talk_burst_priority - If more than one level of priority is supported, indicates the talk burst priority level of the PTC Client.

Talk_burst_reason_code - The reason code for the denial or revoke of a talk burst.

TBCP_Request - Used by the PTC Client to request permission from the PTC Server to send a Talk Burst.

TBCP_Granted - Used by the PTC Server to notify the PTC Client that it has been granted permission to send a Talk Burst.

TBCP_Deny - Used by the PTC Server to notify a PTC Client that it has been denied permission to send a Talk Burst.

TBCP_Idle - Used by the PTC Server to notify all PTC Clients that no one has the permission to send a Talk Burst at the moment and that it may accept the TBCP Talk Burst Request message.

TBCP_Taken - Used by the PTC Server to notify all PTC Clients, except the PTC Client that has been given permission to send a Talk Burst, that another PTC Client has been given permission to send a Talk Burst.

TBCP_Revoke - Used by the PTC Server to revoke the media resource from a PTC Client and can be used for preemption functionality, but is also used by the system to prevent overly long use of the media resource.

TBCP_Queued - Indicates the request to talk is queued, if queued floor control is supported.  Include identification of the PTC Client that has the queued Talk Burst, if known.

TBCP_Release - Indicates the request to talk has completed.

RegistrationRequest - Identifies the type of registration request (e.g., register, re-register, de-register).

RegistrationOutcome - Identifies success or failure of registration and the failure reason.

SDP – Answer, offer and SDP parameter negotiations. Report when known.

PTCHost - Indentifies the PTC participant who has authority to initiate and administrate an active PTC Group Session.

PTCOriginatingId - Identifies the originating party. Provide when known.

PTCParticipants - Identifies the invited PTC participants, when known,  if other than the PTC Intercept Target.

AssociatePresence - Provides the Associate Presence Status, List of: 					PresenceID: Identity of PTC Client(s) or PTC group, when known. 				PresenceType: Identifies type of ID [PTC Client(s) or PTC group]. 				PresenceStatus: Presence state of each ID.

Location - Report when a PTC Session is initiated by the intercept target. This parameter is not reported when the PTC Intercept Target receives an invitation to join a PTC Session;  rather this information is reported by the PTC Session Start message (see PTC Session Start message for usage). Include when reporting of the PTC Intercept Target’s location information is authorized and known,

InitiationCause - Identifies the reason for the generation of the PTC Session Initiated message (e.g., SIP INVITE).

BearerCapability - Identifies the bearer characteristics (e.g., vocoder type).

Z.4	 PTC Surveillance Events

Z.4.1        PTC Service Registration event

The PTC Service Registration event occurs when the target registers, re-registers, or deregisters for the PTC service, regardless of whether it is successful or unsuccessful.

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event Type

Event Time

Event Date

Network Element Identifier

Any other IMPU or IMPI of the target (if available)

RegistrationRequest

RegistrationOutcome

Z.4.2 	PTC Serving System event

A PTC Serving System event is generated when there is a change to the SP serving the PTC target access network (i.e., for mobility). These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

Serving System Address (AVP name such as Visited-PLMN-Id)

Any other IMPU or IMPI of the target (if available)

Z.4.3 	PTC Session Initiation

When a PTC Session Initiation event occurs is when the PTC target initiates any type of PTC Session or the target receives an invitation to join a PTC Session regardless of the success of the initiation or the final disposition of the invitation. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCSessionInfo

TargetID

PTCHost

PTCOriginatingID

PTCParticipants

AssociatePresenceStatus

Location

InitationCause

BearerCapability

Z.4.4 	PTC Session Abandon

The PTC Session Abandon event is triggered when the PTC Session is not established and the request is abandoned before the PTC Session established end to end connectivity. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCorrelationID

PTCSessionInfo

TargetID

PTCOriginatingId

PTCParticipants

AssociatePresenceStatus:

Location

AbandonCause

Z.4.5 	PTC Session Start event

This event occurs when a PTC Session (e.g., One-to-One, One-to-Many, or One-to-Many-to-One) is answered, and voice communication begins.  These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCSessionInfo

TargetID

PTCOriginatingId

PTCParticipants

AssociatePresenceStatus:

Location

BearerCapability

Z.4.6 	PTC Session End event

The PTC Session End event occurs when the PTC Session is released for any reason (i.e., normal or abnormal release) and voice communications ends. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCSessionInfo

TargetID

PTCParticipants

Location

Cause

Z.4.7 	PTC Start of Interception

This event occurs when interception is started and the PTC Intercept target’s ongoing PTC Session(s) that is already in progress (e.g., the PTC target is a member of a Pre-Arranged PTC Group or Chat PTC Group), regardless of whether the PTC target is actively transmitting or receiving talk bursts. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCSessionInfo

TargetID

PTCOriginatingID

PTCHost

PTCParticipants

BearerCapability

Correlation information

Z.4.8 	PTC Pre-Established Session

This event occurs when a SIP Session is established, modified, or released between the PTC targets, PTC Client and the PTC Server that performs the Participating PTC Function.  These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

PTC_Server_URI

RTP_Setting

PTC_Media_Capability

PreEstablishSessionId

PreEstablishedStatus

TalkburstControl_Setting

MediaStream_Availability

Location

Failure_Code

Z.4.9 	PTC Instant Personal Alert

This event occurs when an Instant Personal Alert (i.e., a request for one PTC subscriber to initiate a PTC Session at a later time) is initiated or sent to the PTC target. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

IPAPartyIdentity

Z.4.10.	PTC Party Join event

This event occurs when a PTC target joins (or re-joins) a PTC Group Session (e.g., Pre-arranged PTC Group, Ad-hoc PTC Chat Group) already in progress. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationId

PTCSessionInfo

TargetIdentity

Inviting_PTC_user

Join_PTC_user

AssociatePresenceStatus

BearerCapability

Z.4.11 	PTC Party Drop

This occurs when the target is a participating member of a PTC Group Session and leaves the PTC Session in which the PTC target is also participating. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCSessionInfo

TargetIdentity

Party_Drop

AssociatePresenceStatus:

Z.4.12 	PTC Party Hold or Retrieve

If the PTC Client supports ‘on-hold’ procedures for PTC Sessions, a PTC Party Hold_Retrieve event occurs when the target places an ongoing PTC Session on hold or retrieves an ongoing PTC Session. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCSessionInfo

TargetIdentity

Hold_ Retrieve_Indication

Hold_Retrieve_user

Z.4.13 	PTC Media Modification

During the PTC Session, a PTC Client may modify the voice frame packetization or voice codec mode by Out-of-band signalling using SDP payload within SIP messages. The Media_Modification event is generated when a re-negotiation of the media parameters occurs during a PTC Session involving the target UE. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCSessionInfo

TargetIdentity

BearerCapability

Z.4.14 	PTC Group Advertisement

This event is generated when a PTC Intercept Target’s MS sends Group Advertisement information to a single PTC user, a list of PTC users or to all members of the Group using the Group Identity. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

Group_Ad_Sender

Group_Ad_Receiver

PTCHost

Group_Characteristics

Z.4.15 	PTC Floor Control

Floor Control arbitrates requests from the PTC Clients for the right to send media (i.e., the right to speak).  Note, the term “Floor Control” is used to mean the same as term “Talk Burst Control”. When the PTC target is participating in a PTC Session, a Floor Control event is generated when the target requests to speak (e.g., presses the PTT mechanism) or the target is given permission to speak in response to a request (e.g., the network responds positively to the PTC Subscriber’s request) or is refused the request to speak and when the target is finished speaking (e.g., the PTC Intercept target releases the PTT mechanism). These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

PTCCorrelationID

PTCSessionInfo

FloorActivity Choice of: - TBCP_Request - TBCP_Granted - TBCP_Deny - TBCP_Idle - TBCP_Taken - TBCP_Revoke - TBCP_Queued - TBCP_Release

FloorSpeakerIdentity

Queued_FloorControl

Queued_Position_Status

Max_TB_Time

Talk_burst_priority

Talk_burst_reason_code

Z.4.16 	PTC Target Presence

If the Presence functionality is supported by the PTC Server and the PTC Server assumes the role of a Presence Source, this event is generated when the PTC Server publishes network presence information to the Presence server on behalf of PTC target. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

TargetPresenceStatus

Z.4.17 	PTC Associate Presence

This event is generated when the PTC Server receives presence status notifications from the Presence Servers after having subscribed to the PTC presence status of other PTC Clients (i.e., Associates of the PTC Intercept targett). The default value of the Associate Presence event is ‘activate’. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

AssociatePresenceStatus:

Z.4.18	PTC List Management Events

This event is generated when the target PTC Client attempts to change their own contact list or their own PTC Group list(s).  This event is also generated when the network notifies the Intercept target’s PTC Client of changes made to their PTC-specific documents stored in the network (i.e., contact lists or PTC Group lists). These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

SIP message header offer

SIP message header answer

SDP offer

SDP answer

TargetIdentity

ListManagementType Choice of: - ContactListManagementAttempt - GroupListManagementAttempt - ContactListManagementResult - GroupListManagementResult - Request unsuccessful

ListManagmentAction Choice of: - Create - Modify - Retrieve - Delete - Notify

Contact_Identity

Group_Identity

PTCHost

List_ManagementFailure

Z.4.19 	PTC Access Policy event

This event is generated when the PTC Intercept target attempts to change the access control lists (e.g. PTC user access policy and PTC Group authorization rules).  In addition this event is generated when the network responds to a modification or query by the PTC Intercept target to the access control lists (e.g., PTC user access policy and PTC Group authorization rules). These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

Access_Policy_Type Choice of: - PTCUserAccessPolicyAttempt - GroupAuthorizationRulesAttempt - PTCUserAccessPolicyQuery - GroupAuthorizationRulesQuery - PTCUserAccessPolicyResult - GroupAuthorizationRulesResult - Request unsuccessful

PTCUserAccessPolicy Choice of: - Allow_Incoming_PTC_Session_request - Block_Incoming_PTC_Session_request - Allow_Auto_Answer_Mode - Allow_Override_Manual_Answer_Mode

GroupAuthorizationRules Choice of: - Allow_Initiating_Conference - Block_Initiating_Conference - Allow_Joining_Conference - Block_Joining_Conference - Allow_Add_Participants - Block_Add_Participants - Allow_Subscription_Conference_State - Block_Subscription_Conference_State - Allow_Anonymity - Forbid_Anonymity

Contact_Identity

Group_Identity

PTCHost

Access_PolicyFailure

Z.5	PTC Group Calls

These events occur when a PTC Session has a One-to-Many, or One-to-Many-to-One is requested, answered, joined, and voice communication begins, ends, or forced disconnected.

Z.5.1 	Group Call Request (Client to Server)

This event is generated when a PTC Group Call to join, rejoin, request or the release of the group call is generated from the targets PTC client to the PTC server is generated. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP offer

SDP Answer

Implicit floor request

Broadcast indicator

Z.5.2 	Group Call Request Event (Server to Server)

A Group Call Request event received at the PTC Server serving the target from a separate PTC server (outside the SP architecture) and the PTC Server serving the target would generate a Group Call Request server to server event. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP offer

Broadcast indicator

Z.5.3 	Group Call Request (Server to Client)

This event is generated when a PTC Group Call request from the PTC server serving the target to the targets PTC Client is received. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP offer

Broadcast indicator

Z.5.4 	Group Call Response (Server to Client)

A Group Call Response event received at the PTC Server serving the target from a PTC server (outside the SP architecture) and the PTC Server serving the target would generate a Group Call Response event. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP answer

Z.5.5 	Group call response (Server to Server)

A Group Call Response event generated at the PTC Server serving the target to a separate PTC server (outside the SP architecture) would generate a Group Call Response server to server event. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP answer

Z.5.6 	Group call response (Client to Server)

This event is generated when a PTC Group Call response is generated at the targets PTC server to the targets PTC Client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP answer

Z.5.7 	Group Call Notify request (Server to Client)

This event is generated when a PTC Group Call Notify request is generated from the PTC server to the PTC client is generated. These elements will be delivered to the DF2 if available:

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

PTC ID list

Z.5.8 	Group call release request (Server – Client)

This event is generated when a PTC Group Call release request is generated at the PTC Server to the Client is generated. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

PTC ID list

Z.5.9 	Group call release request (Server – Server)

This event is generated when a PTC Group Call release request is received at the PTC Server serving the target from a separate PTC server (outside the SP architecture) would generate a Group Call Release Request server event. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

Z.5.10 		Group call release response (Client – Server)

This event is generated when a PTC Group Call release response is generated at the targets PTC client to the targets PTC server. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

Z.5.11 		Group call release response (Server – Server)

This event is generated when a PTC Group Call release response is generated at the targets PTC server to the any other PTC server. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

Z.5.12 		Group Call Rejoin Request (Client – Server)

This event is generated when a PTC Group Call rejoin request is generated at the targets PTC client to the targets PTC server. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP offer

Z.5.13 	Group Call Rejoin Response (Server – Client)

This event is generated when a PTC Group Call rejoin response is generated at the targets PTC Server to the targets PTC client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP answer

Z.5.14 	Group Join Request (Client – Server)

This event is generated when a PTC Group Call rejoin response is generated at the targets PTC Server to the targets PTC client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

SDP offer

Z.5.15 	Group Join Response (Server – Client)

This event is generated when a PTC Group join response is generated from the PTC server to the PTC target client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP answer

Z.5.15 	Group Call Leave Request (Server – Client)

This event is generated when the group call leave request from the PTC server to the PTC client is generated. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

Z.5.16 	Group Call Leave Response (Client – Server)

This event is generated when the group call leave response is received at the PTC server from the PTC client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

Z.5.17 Group Interrogate Request (Server – Server)

This event is generated when the group interrogate request is received at the PTC Server serving the target. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

Z.5.18 Group Interrogate Response (Server – Server)

This event is generated when a PTC Group Interrogate response is generated at the targets PTC server to the any other PTC server. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

PTC ID list

Z.6 	MCPTT Priority Calls and Alerts Messages

Z.6.1 	Background

MCPTT emergency group calls are defined by TS 23.379 [3] and TS 22.179 [2]. The user profile/group configuration determines which MCPTT Group ID is used, if any.  Group calls are enabled in both on-network and off-network. This subclause contains events for group call across a single and multiple MCPTT servers, and associated functions such as emergency call, broadcast call and others.

There are a number of states that are key in managing aspects of MCPTT emergency group calls, which include:

- MCPTT emergency state: indicates that the MCPTT user is in a life-threatening situation.

- In-progress emergency group state: indicates whether or not there is an MCPTT emergency group call ongoing on the specified group.

- MCPTT emergency group (MEG) state: this is an internal state managed by the MCPTT client which tracks the in-progress emergency state of the group and managed by the controlling MCPTT function.

- MCPTT emergency group call (MEGC) state: this is an internal state managed by the MCPTT client which in conjunction with the MCPTT emergency alert state aids in managing the MCPTT emergency state and related actions.

- MCPTT emergency alert (MEA) state: this is also an internal state of the MCPTT client which in conjunction with the MCPTT emergency group call state aids in managing the MCPTT emergency state and related actions.

Z.6.2 	MCPTT Emergency Group Call Request

This event occurs when a MCPTT Emergency Group Call Request is received at the MCPTT server from the MCPTT client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Emergency indicator

Alert indicator

MCPTT group ID

MCPTT indicator

Z.6.3	MCPTT Emergency Group Call Response

This event is generated when received by a MCPTT Emergency Group Call Response from the MCPTT server to the MCPTT client is generated. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Emergency indicator

Alert indicator

MCPTT group ID

MCPTT indicator

Z.6.4 	MCPTT Emergency Group Call Cancel Request

This event is generated when a MCPTT Emergency group Call Cancel event is generated when the cancel request from the MCPTT client to the MCPTT server is recieved. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Emergency indicator

Alert indicator

MCPTT indicator

Z.6.5 	MCPTT Emergency Group Call Cancel Response

This event is generated when a MCPTT Emergency group Call Cancel response event is generated when the cancel response from the MCPTT server to the MCPTT client is generated. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Emergency indicator

Alert indicator

MCPTT indicator

Z.6.6 	MCPTT Emergency Alert Request

This event is generated when a MCPTT emergency alert request is generated from the MCPTT client to the MCPTT server. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Alert indicator

Organization name

Location

Z.6.7 	MCPTT Emergency Alert Response

This event is generated when a MCPTT emergency alert response is generated from the MCPTT server to the MCPTT client is generated. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Alert indicator

Organization name

Location

Z.6.8 	MCPTT Emergency State Cancel Request

This event is generated when the MCPTT Emergency State Cancel Request event is generated from the MCPTT client to the MCPTT server is generated. These elements will be delivered to the DF2 if available:

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Client emergency state cancel inform (NOTE)

Group emergency condition cancel request (NOTE)

NOTE: At least one of these information elements shall be present

Z.6.9 	MCPTT Emergency State Cancel Response

This event is generated when the MCPTT Emergency State Cancel Response event is generated from the MCPTT server to the MCPTT client. These elements will be delivered to the DF2 if available:

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Client emergency state cancel inform (NOTE)

Group emergency condition cancel request (NOTE)

NOTE: At least one of these information elements shall be present

Z.6.10 	MCPTT Emergency State Cancel Response

This event is generated when the MCPTT Emergency State Cancel Response event is generated from the MCPTT Serve to the MCPTT Client. These elements will be delivered to the DF2 if available:

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Client emergency state cancel response (NOTE)

Group emergency state cancel response (NOTE)

NOTE: At least one of these information elements shall be present

Z.6.11 	MCPTT Imminent Peril Group Call Request

This event is generated when a MCPTT Imminent Peril Group Call Request is generated from the MCPTT client to the MCPTT server. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Imminent peril indicator

MCPTT indicator

Z.6.12 	MCPTT Imminent Peril Group Call Response

This event is generated when a MCPTT Imminent Peril Group Call response event is generated from the MCPTT server to the MCPTT client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Imminent peril indicator

MCPTT indicator

Z.6.13 	MCPTT Imminent Peril Group Call Cancel Request

This event is generated when a MCPTT Imminent Peril Group request is generated from the MCPTT client to the MCPTT server. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Imminent peril indicator

MCPTT indicator

Z.6.14 	MCPTT Imminent Peril Group Call Cancel Response

This event is generated when a MCPTT Imminent Peril Group response is generated from the MCPTT server to the MCPTT client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Imminent peril indicator

MCPTT indicator

Z.7	Communication Content (CC) Delivery

Z.7.1 	Communication Content (CC)

When communication content (CC) delivery is authorized, it will be delivered through the DF3 interface to provide packetized CC for the PTC Service under surveillance.  The PTC_CC-Application Protocol Data Unit (APDU) event is used to encapsulate communications content packets for transfer over the CC delivery interface, in accordance with this standard.  These elements will be delivered to the DF3 if available and authorized:

Observed SIP URI

Observed TEL URI

Observed IMEI

Event type

Event Time

Event Date

Network Element Identifier

PTCSessionInfo

PTCCorrelationID

PTC CC

*** End of FIRST MODIFICATION ***

*** End of MODIFICATIONS ***