3GPP TS 33.126 V0.0.5 (2017-11)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Security;

Lawful Interception requirements

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

UMTS, LTE, IMS, LI, 5G, Security, Architecture

Delta between v004 and v005 are light bleu high lighted

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	9

3	Definitions and abbreviations	11

3.1 Definitions	11

3.2 Abbreviations	12

4	Jurisdiction specific Lawful Interception requirements	14

5	General interception model	15

6	Fundamental Requirements	16

6.1 Overview	16

6.2 Identification	16

6.3	Collection	17

6.4	Delivery	18

6.5	Lawfulness	20

6.6	Security	20

6.2.3 Specificity	22

6.2.11 Recordkeeping, Accounting and Auditing	22

7 Specific service, capability, and feature requirements	23

7.1 Customized Alerting Tone (CAT)	23

7.2 Customized Ringing Signal (CRS)	24

7.3 Home Node B and Home enhanced Node B (H(e)NB)	24

7.4 Location information	25

7.4.1 General	25

7.4.2 LI and Location Services	26

7.4.3 Lawful Access Usage of Location Services (LCS)	26

7.5 IMS VoIP Service	28

7.6 Messaging	28

7.7 Management of IMS supplementary services settings	28

7.7.1 IMS Video Service	29

7.7.2 IMS based Push-to-Talk	29

7.8 Interception within the Home and Visited Networks for roaming scenarios	29

Annex A (informative): Capability Issues	31

Annex A (informative): Bibliography	33

Annex B (informative): Change history	35

Editorial Parking Lot	36

# Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document has been produced by the 3GPP TSG SA to enable standardisation of Lawful Interception (LI) of telecommunications. This document provides requirements for lawful interception. The present document evolved from TS 33.106, in an attempt to update it for the architectural and functional changes introduced by 5G. No new requirements have been introduced. This is a re-arrangement of long-standing, well-established requirements, clarified, with non-obvious implications and couplings made explicit, in light of paradigm changes imposed by the evolution to 5G.

Editors´s note:	proir to publication historic reference in yellow to be removed

Laws of individual nations and regional institutions, and sometimes licensing and operating conditions, define a need to intercept targeted telecommunications traffic and related information in communication systems. Lawful interception shall be applied in accordance with applicable national or regional laws and technical regulations.

# 1	Scope

The present document specifies Stage 1 lawful interception requirements for 3GPP networks.

Regional interception requirements can be satisfied by meeting the correct subset of requirements from the present document. Which CSP services are subject to lawful interception is defined in national regulations.

Editor’s note: Wider fixed mobile confersion might need to taken into account. How should this be written in?

The presence of a requirement in this specification does not in itself infer or mandate that a 3GPP operator has an obligation to implement any network service capability, which is not otherwise required to meet LI obligation compliance in relation to specific regulated services, offered by that 3GPP operator. Only those specific requirements and sub-sections of the present document which are applicable to specific network and/or service capabilities implemented in a 3GPP operator’s network shall be considered in scope for that operator. In all cases, laws and regulations define which requirements are applicable to 3GPP operators in each country relative to the services offered by each 3GPP operator.

As such not all requirements in the current document will apply in all national jurisdictions or to all 3GPP operator deployments (e.g. if an operator does not offer voice services, then voice LI requirement in the current document do not apply ).

The interception system defined in the present document provides LI based on single target identifiers.

Editor’s note: review of the last sentence needed, to be done off line

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editors note: use of refs to be checked

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501 "Technical Specification Group Services and System Aspects; System Architecture for the 5G System; Stage 2".

[3]	ETSI GS NFV-SEC 012 System specification for execution of sensitive NFV components. VERSION MISSING

[4]	3GPP TS 33.127: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful interception architecture and functions".

Editor’s note: DOES NOT EXIST YET

[5]	3GPP TS 33.128: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Handover interface for Lawful Interception".

Editor’s note: DOES NOT EXIST YET

[6]	3GPP TS 33.107: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful interception architecture and functions".

[7]	3GPP TS 33.108: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Handover interface for Lawful Interception".

[8]	3GPP TS 22.220: "Service Requirements for Home NodeBs and Home eNodeBs"

[9]	3GPP TS 22.182: "Customized Alerting Tones (CAT) Requirements; Stage 1"

[10]	3GPP TR 23.872: "Study on Architecture of IP Mulimedia subsystem (IMS) based Customized Alerting Tone (CAT)"

[11]	3GPP TS 24.182: "IP Multimedia Subsystem (IMS) Customized Alerting Tones (CAT); Protocol Specificiation"

[12]	3GPP TR 29.882: "Customized Alerting Tone (CAT) in 3G CS Domain"

[13]	3GPP TS 22.183: "Customized Ringing Signal (CRS) Requirements; Stage 1"

[14]	3GPP TS 24.183: "IP Multimedia Subsystem (IMS) customized Ringing Signal (CRS); Protocol Specification"

[x]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[x]	OMA 	OMA-AD-MMS-V1_3-20110913-A.:"Multimedia Messaging Service Architecture".

[x]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[x]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[x]	3GPP TS 22 173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[x]	3GPP TS 22.179: "Mission Critical Push To Talk (MCPTT) over LTE; Stage 1".

[x]	OMA - OMA-TS-PoC_System_Description-V2_1-20110802-A: "OMA PoC System Description"

# 3	Definitions and abbreviations

## 3.1 Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [5] and the following terms apply.

Editor’s note:	Whole section to be checked with 905

Communication Service Provider: the entity that owns or operates the network that provides a service to a subscriber.

Content of Communication: information exchanged between two or more users of a communications service, excluding intercept related information. This includes information which may, as part of some communications service, be stored by one user for subsequent retrieval by another.

Context of Communication: Information needed to recreate the state known in the CSP’s network of the Target Communication. For example the direction of initiation on communication (to or from), direction of data flow (to or from), direction association with the identifiers to and from addresses), action taken by  the CSP on behalve of the target or identity translations.

Customized Alerting Tone: An indication that is customized by the called party or the calling party that is played to the calling party during call establishment or during an established call session indicating that the called party is being alerted, the progress of a communication request, or any alerting event during a call session. A Customized Alerting Tone may be a piece of recorded or composed music, greeting words, voice, advertisement or video.

Customized Ringing Signal: An indication to the called party as an incoming communication indication during the establishment of a communication that is customized by the calling party or the called party. A Customized Ringing Signal (CRS) may be a picture, a piece of recorded or composed music, greeting words, voice, advertisement, video or other elements.

Device: An entity that connects to a 3GPP network (e.g., Mobile Equipment[ME], User Equipment[UE]).

Intercept Related Information: information or data associated with communication services involving the target identity, specifically communication associated information or data (e.g. unsuccessful communication attempts), service associated information or data, and location information.

Lawful Interception Identifiers: Target identifying details as defined in ETSI TS 103 280

Lawful Interception (LI): Actions taken by the CSP that include: provisioning the target identity in the network to enable isolation of target communications (separating it from other users’ communications), duplicating the communications for the purpose of sending the copy to the LEA, and delivering the interception product to the LEA that presented the CSP with the warrant. An interception is associated with exactly one warrant.Interception Area: is a subset of the Public Lands Mobile Network (PLMN) service area comprised of a set of cells which define a geographical zone.

(Lawful) Interception Product: the resulting CC or IRI generated as a result of isolating the target’s communications for the purpose of delivery to the requesting LEA.

Lawful Access Location Services: action performed by a CSP of obtaining a user’s location information by means of Location Services (LCS), and providing that information to an LEA.

LCS (Location Services): LCS is a service concept in system (e.g. EUTRAN, New Radio[NR]) standardization. LCS specifies all the necessary network elements and entities, their functionalities, interfaces, as well as communication messages, necessary to implement the positioning functionality in a cellular network. Note that LCS does not specify any location based (value added) services except locating of emergency calls and Lawful Interception Access Services.

LI Delivery Latency: The time between isolation in the Point of Interception and delivery of the Product of Interception at the LEA at the agreed point of handover.

Location Dependent Interception: is interception within a PLMN service area that is restricted to one or several Interception Areas (IA).

Location Information: information relating to the geographic, physical or logical location of a target.

Product of Interception: The Content of Communication and/or the Intercept Related Information generated as a result of isolating the target’s communications for the purpose of delivery to the requesting LEA

Push-to-Talk: provides direct one-to-one and one-to-many voice communication service with speaker floor control based on half-duplex VoIP technology in the cellular network.

Editor’s note: do we need “target” definition

Target Based Interception: Interception that is invoked using a specific Target Identity

Target Communication: all communications, communication attempts (successful or not), and network interactions that originate from, are directed to, are controlled by, or are associated with, the target’s identifiers, equipment, facilities or services, including actions taken by the network on behalf of the target, that are available in the CSP’s network.

Target Identity: A string (sequence of characters) that uniquely identifies a target of interception. One target may have multiple identities.

Warrant: The formal mechanism to require lawful interception from a CSP on a single target identifier. Depending on jurisdiction also known as: intercept request, intercept order, lawful order, court order, lawful order or judidial order.

## 3.2 Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [19] and the following apply:

ADMF	Administration Function

CAT	Customized Alerting Tone

CC	Content of Communication

CRS	Customized Ringing Signal

CSG	Closed Subscriber Group

CSP	Communications Service Provider

DNN	Data Network Name

GUTI	Globally Unique Temporary Identifier

HeNB	Home eNodeB

H(e)NB	HNB and HeNB

HNB	Home NodeB

IA	Interception Area

IP	Internet Protocol

IRI	Intercept Related Information

LALS	Lawful Access Location Services

LDI	Location Dependent Interception

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

MMS	Multimedia Messaging Services

NF 	Network Function

PEI	Permanent Equipment Identifier

POI	Point Of Intercept

PTT	Push to Talk

SUPI	Subscription Permanent Identifier

UTC	Coordinated Universal Time

# 4	Jurisdiction specific Lawful Interception requirements

Lawful Interception requirements are subject to jurisdiction specific regulations and should be interpreted accordingly.

Requirements called out in jurisdiction specific lawful interception regulatory requirements are supported by the system defined in the present document.

Editor’s NOTE:	Individual requirements in clause 6 may conflict.  Need to check and remove conflict before publication

# 5	General interception lifecycle and model

## 5.1 	Model

Figure 1 depicts the general interception model. Lawful Interception (LI) is implemented in a 3GPP Communication Service Provider (CSP) network by the logical elements shown in the figure. Detailed LI architecture and functions are found in TS 33.127 [4], while delivery details are found in TS 33.128 [5].

ADMF

Target

IRI

CC

LEMF

mediation & delivery

POI

warrant

intercept management

LEA

3GPP CSP

Figure 5.1: Generic lawful interception model

## 5.2	Lifecycle

Figure 2 depicts the general Lawful Interception lifecycle.

Figure 5.2: Generic lawful interception lifecycle

The Lawful Interception lifecycle has five main stages. The first and last stages are administrative, and consist of provisioning and de-provisioning the Lawful Interception system.  In the middle three stages (labelled “production” in Figure 1) the Lawful Interception systems identify, capture and deliver intercepted material (“product”) to the LEA. These three production stages occur each time a targeted communication is identified, and therefore may happen many times during the lifecycle.

- Provisioning: The Law Enforcement Agency (LEA) presents the 3GPP CSP with a warrant. The warrant identifies the target, and the specific communications services to be intercepted and delivered. The CSP provisions the target identifiers through the Administration Function (ADMF) into the Points Of Intercet (POI)

- Identification The CSP detects that communications associated with the target identifier is occurring.

- , Capture: T he CSP Points Of Intercet (POI)all the necessary events and information associated with the targeted communications (as defined in TS 33.107[6] and TS 33127[4]).  The POIs pass all the content and associated information necessary for the LEA to understand the context of communication to the Mediation Function (MF)/Delivery Function (DF).

- Delivery:The MF/DF performs any necessary processing, correlation and mediation before sending IRI and CC to the Law Enforcement Monitoring Facility (LEMF) (as defined in TS 33.108 [7] and TS 33.128[5]).

- De-provisioning: The CSP de-provisions the target identifiers through the ADMF to the POI. This may happen in response to a further lawful order from the LEA, or automatically once the period specified in the original warrant has expired.

Editors note: 	Note on what applies to a CSP (e.g. no LI obligation for uncoorrrelated Wifi, obligationon for own services). Maybe in other chapter. Greg might make proposal

Editor’s note: Mark will take a look at the long sentence apart with Richerd

# 6	Fundamental Requirements

## 6.1 Overview

The fundamental  requirements in this chapter are derived from the fundamental lawful interception requirements presented in this clause.

Some requirements are cumulative in nature, and rely on implicit compliance with other requirements.

The CSP shall be able to provide a Lawful Interception capability which meets the relevant regulatory and operational obligations. In general, this gives rise to the following high-level requirements:

- Identification: The CSP shall be able to identify the traffic and metadata required to be intercepted, on the basis of target idenficiation information provided by the LEA and visible in the network

- Interception/Isolation/Collection: The CSP shall be able to intercept/isolate/capture  all  content and metadata (requiredto procuse IRI) associated with targeted communications as provided by the CSP, in order for the LEA to fully understand the context of the communication.

- Handover: The CSP shall be able to deliver intercepted product in agreed format to the LEA, such that the LEA can fully understand the context of the communication as provided by the CSP.

- Lawful: The CSP’s Lawful Interception capability shall comply with the relevant obligations, restrictions and reporting regimes in the lawful order, including (but not limited to) period, duration, locality.

- Security: Lawful Interception by the CSP shall be undetectable by any party not explicitly authorised to have knowledge of it by the relevant lawful order, and cannot be modified, altered or degraded by such a party.

## 6.2 Identification

Editor’s note:	numbering to be reviewd if the doicument becomes stable

R6.2.2 - 10	User identification–The CSP shall maintain an association among subscription identifiers or devices registered on the network, using private or public permanent or temporary available identifiers (e.g., SUPI, GUTI, MSISDN, IMEI, SIP-URI, IMSI, TEL-URL), such that LI can be performed at any time the target interacts with, or acts within, the CSP network, or the CSP network acts on behalf of the user. This requirement shall not be interpreted to conflict with regulations pertaining to unauthenticated emergency calls.

Editor’s note: requirement needs to be reviewed to some of the other requirements

R6.2.2 – 10BIS  Private and public identifiers–The CSP shall be able to support LI based on both private and public communications identifiers, if such identifiers are used or visible inside CSP’s network or platform or slice.

R6.2.2 - 20	Communication party identification–The CSP shall be capable of intercepting based on the identification of the target, regardless of the target role in the communication.

R6.2.2 - 30	Target communication identification: The network shall be able to distinguish specific usages of the network by the target (e.g., access or service) from all other usages in the network, based on the target identifier.

R6.2.2 - 40	Target identifier authentication - The CSP shall not be required to use target authentication procedures beyond those used in the normal course of business for user identities. Editor’s note:  to be removed somewere more appropriate

R6.2.2 - 50	Long Term Identifier - The CSP shall be able to intercept based on long term identifiers.

R6.2.2 - 60	Short Term Identifiers - The CSP shall be able to intercept based on valid short-term identifiers.

R6.2.2 - 62	Private Identifier - The CSP shall be able to intercept based on private identifiers.

R6.2.2 - 64	Public Identifiers - The CSP shall be able to intercept based on valid public identifiers.

R6.2.2 - 70	Short to Long Term Identifier Mapping - When a valid short-term identifier is provided in the warrant, the network shall be able to perform interception based on corresponding long-term identifiers.

R6.2.2 - 80	Long to Short Term Identifier Mapping - When a long-term identifier is provided in the warrant, the network shall be able to perform interception based on corresponding short-term identifiers.

R6.2.2 - 90	Non-local target identification - The CSP shall be able to isolate communications passing through its network based on a visible target identity, when not assigned, or managed, by the CSP.

R6.2.2 - 100	Target profile changes–The CSP shall be able to notify the LEA of changes to permanent identifiers in the target’s subscriber profile (e.g., IMEI change).

R6.2.2 - 110	Target service profile change; The CSP shall be able to notify the LEA of target’s service assoiation change events such as change of identifiers (e.g., association in a group call).
Editor’s note: how do we need to deal with group associations.

R6.2.2 - 120	Target Mapping–The CSP shall notify the LEA of modifications in parameters used for interception in relation to the warrant (e.g., target ID derivation).

R6.2.4 - 10	Isolation–The CSP shall isolate and intercept all Target Communications, as specified in the warrant.

R6.2.4 - 20	Completeness–The CSP shall intercept all Targeted Communications specified in the warrant

## 6.3	Collection

R6.2.1 - 30	Access Level Interception–The CSP shall be able to perform network access level interception.

R6.2.1 - 40	Service Level Interception–The CSP shall be able to perform service  level interception (e.g., IMS based VoIP).

R6.2.1 - 50	Third party assisted services–If a CSP uses third parties as part of its service chain, the CSP shall be responsible for ensuring that the overall service complies with applicable LI regulations.

R6.2.7 - 10	Multiple services per target–The CSP shall be capable of simultaneously performing LI for multiple services for a given target.

R6.2.7 - 20	Multiple targets–The CSP shall be capable of simultaneously performing intercepts on multiple independent targets.

R6.2.7 - 30	Multiple LEAs–The CSP shall be capable of simultaneously performing independent intercepts for any given target serving different warrants.

R6.2.7 - 40	Initial Resource Capacity–The CSP shall ensure necessary and sufficient resources are applied to start delivery of Interception Product to the LEA as soon as a warrant is received.

R6.2.7 - 50	Ongoing Resource Capacity–The CSP shall be able to allocate necessary and sufficient resources to allow the Interception continues uninterrupted and uncorrupted.

R6.2.9 - 10	Location–The CSP shall be able to deliver the  location information pertaining the target  to the LEA.

R6.2.9 - 20	Multiple location forms–If there are multiple forms of location information available in the CSP network, the CSP shall beable to reported those to the LEA.

R6.2.9 - 30	Location provenance–All location information shall be able to include provenance information (e.g., network-derived, UE-derived, access-derived).

R6.2.9 - 31	Enhanced location: if the altitude or any enhanced location information, of the target are available, such information has to be provided

Editor’s note:  are there other aspects of location that have to be reported.

R6.2.9 - 40	Location senescence–All location information shall be able to include a timestamp that indicates when the location was determined  (i.e.“age of location”).

R6.2.9 - 50	Predicted Location: The CSP shallbe able to indicate to the LEA whether the location information of the target is actual or predicted.

R6.2.9 - 60	Location reporting–Location reporting may either be part of interception of another service (e.g., VoLTE), or performed independently.

R6.2.9 - 70	Roaming location: In the case of roaming the visited CSP (the serving CSP) shall support location reporting using a local capability in the serving network.

R6.2.9 - 80	Location changes in the visited network–The home CSP shall notify the LEA of target-related location related events in the visited network if these are available in the home network as part of normal network operation.

R6.2.9 - 90	Location accuratecy–The CSP shall report the the most accurate target location available in the network.

R6.2.9 - 100	Location requests–The home CSP shall be able to notify the LEA of target-related location information requests from outside the home network when those requests are visible to the home subscriber registration database in the home network (e.g. HSS, UDM).

Editor’s note: does this fits better somewhere else?

Editor’s note: need to cover relay note requirements?

R6.2.10 - 10	Roaming targets–The visited CSP shall be able to perform interception of inbound roaming targets.

Editors’s note: All roaming targets can be 6.2.2 defined targets.

R6.2.10 - 20	Roaming - outbound–The CSP shall be able to notify the LEA whenever the  CSP becomes aware that the target has left, or entered, a visited CSP network.

R6.2.10 - 30	Roaming - inbound–The CSP shall be able to notify the LEA whenever the CSP becomes aware that the inbound roaming target has entered, or has left, the CSP network.

R6.2.10 - 40	Identity Verification: If the home CSP implements identity privacy, then the home CSP shall provide the corresponding real identy for all devices roaming to the vistited CSP.

R6.2.10 - 50 Additional Identity Verification: If any of the the real identity associated with roaming-in Devices as described in R6.2.10.40 are not able to be validated, the 3GPP system shall provide the visited CSP the ability to validate the real identity of roaming-in devices direct from the devices.

R6.2.10 - 60 Access network identity–If the target accesses the 3GPP access network via a non-CSP access network, the CSP shall provide the LEA the identity of the Access Network as known by the CSP.

R6.2.10 - 70	Handover–When the target’s communications or signalling information is no longer available to the CSP due to redirection or handover to another network operator, the CSP shall be able to provide the LEA with the identity of the network operator that has access to the target’s communications or signalling information.

R6.2.10 - 80	Serving network change–The home CSP shall notify the LEA of register location / registration / access control to a new serving network event, even if such information is coming from a 3GPP or non 3GPP access network; or from the previous serving network, happening after the provisioning of the target by the home CSP with the received warrant.

## 6.4	Delivery

R6.2.5 - 20	IRI to IRI correlation–The CSP shall deliver information to allow that all the  IRI can be correctly correlated with related IRI of the same Target Communication.

R6.2.5 - 30	CC to CC correlation–The CSP shall deliver information to allow all the  CC can be correctly correlated with related CC of the same Target Communication.

R6.2.5 - 40	IRI and CC correlation: The CSP shall deliver information to allow the  related IRI and CC of the same Target Communication can be correctly correlated.

R6.2.5 - 50	Context correlation: The CSP shall deliver information to allow the  LEA to correctly correlate all CC and IRI to the Context of Communications.

R6.2.5 - 60	POI Identification: The CSP shall be able to indicate to the LEA the POI source(s) of the Intercept Product.

R6.2.7 - 60	Delivery Reliability–The CSP shall be able to employ mechanisms (e.g. buffering) to limit the effect of delivery network failures or limitations to prevent loss of Interception Product.

R6.2.7 - 70	LI Delivery Latency–The CSP shall ensure that the Interception Product is delivered to the LEA without undue delay.

R6.2.7 - 80 	Timestamping–The CSP shall timestamp all Interception Product (both IRI and CC) with a timestamp of precision, resolution, and accuracy commensurate with the performance of the intercepted service.

R6.2.7 - 90 	UTC Offset–The CSP shall provide all timestamps with the UTC offset.

R6.2.7 - 100	Clock synchronization– The CSP shall maintain clock synchronization across its network such that the LEA LI context reflects the actual state of the service (e.g. maintain clock synchronization against an offical time reference).

R6.2.7 - 110	Ordering: The CSP shall provide a means to enable the LEA to correctly order the events of an intercepted service.

R6.2.7 - 120	Duplication: The CSP shall provide a means to avoid the duplicat delivery of Intercept Product for a single warrant. Any redundant information, CC or IRI, shall be avoided.

R6.2.8 - 10	Encryption– The CSP shall remove any encryption it provides or manages before delivery of the Interception Product to the LEA, or provide the LEA the information necessary to decrypt the intercepted communications (e.g. keys, algorithms, parameters) included with the Interception Product.

R6.2.8 - 20	CSP provided encryption keys–If the CSP provides encryption keys to the target, but does not provide the encryption itself, the CSP shall provide the keys to the LEA.

R6.2.8 - 30	Retroactive decryption–The CSP ensure that the crypto keys, algoritm and parameters delivered to  the LEA enable the LEA to decrypt encrypted Target Communications retroactively.

R6.2.8 - 40	Encryption key material lifecycle - creation– The CSP shall retain sufficient key material for the duration of any communications that use CSP provided or managed encryption..

R6.2.8 - 50	Encryption key material lifecycle - destruction–Once key material specifically retained for LI purposes is no longer required,  the CSP shall securely delete key material.

R6.2.8 - 60	Encoding–The CSP shall remove any CSP-controlled encoding before delivery to the LEA, or provide the LEA the information necessary to decode the intercepted communications concurrently with delivery of LI product.

R6.2.8 - 70	Compression–The CSP shall remove any CSP-controlled compression before delivery to the LEA, or provide the LEA the information necessary to decompress the intercepted communications concurrently with delivery of LI product.

R6.2.8 - 80	Context Comprehensibility: The CSP shall deliver information that allows the LEA to establish the Context of Communications.

R6.2.8 - 90	Context mapping: The CSP shall be able to map service specific parameters and records to generic parameters and records (Begin, Continue, End and Report).

R6.2.8 - 100	Service indication: The CSP shall deliver the indicator of the communication service as known by the CSP network.

R6.2.8 - 120	Independency of IRI and CC: The CSP shall deliver information that allows  to extract the available information within IRI without CC and vice versa (e.g. CC can be decoded without IRI).

R6.2.12 - 10	Quality of Service–The CSP shall enter into a bilateral agreement with the LEA that specifies Quality of service (QoS), capacity, and integrity of the delivered IRI and CC.

R6.2.11 - 80	Charging–The 3GPP system shall be able to generate LI charging event records .

## 6.5	Lawfulness

R6.2.1 - 10	Interception period–The CSP shall ensure that lawful interception is performed only for the period as specified in the warrant.

R6.2.1 - 20	Interception suspend/resume toggle–Inside the Interception Period the CSP shall be able to autonomously suspend and resume the delivery of the interception product for a target identifier of each interception based on specific criteria of which can be detected within the network (e.g., if interception is not permitted when the target is outside the jurisdiction, suspend interception when the target leaves the jurisdiction, and resume when the target re-enters the jurisdiction). In suspend state the interception product is discarded.

R6.2.1 - 60	LI Activation–The CSP shall be able to notify the LEA of interception activation.

R6.2.1 - 70	LI Changes–The CSP shall be able to notify the LEA of  O&M changes related to interception(?).

R6.2.1 - 80	LI Deactivation–The CSP shall be able to notify the LEA of interception deactivation.

R6.2.5 - 10	Warrant correlation–The CSP shall ensure all the Target Communications are correlated with the warrant.

R6.2.11 - 10	Recordkeeping–The CSP shall create and retain necessary and sufficient LI lifecycle records of the CSP’s mananement of the Interception (e.g., log files), to demonstrate, after the fact, that the interception was implemented correctly per the warrant.

## 6.6	Security

R6.2.6 - 10	Undetectability by the target–The CSP shall perform interception in such a manner that the target at any moment in time is unable to detect interception is taking place, before, during, or after the interception.

R6.2.6 - 20	Undetectability by other users–The CSP shall perform interception in such a manner that no other user can detect that interception is taking place, before, during, or after the interception.

R6.2.6 - 30	Undetectability by non-authorized service provision parties–The CSP shall perform Interception in such a manner that no unauthorized party can detect that Interception is taking place, before, during, or after Interception.

R6.2.6 - 40	Undetectability across LEAs–The CSP shall perform Interception in such a manner that no other LEA can detect that Interception is taking place, before, during, or after Interception.

R6.2.6 - 50	Undetectability across CSPs–The CSP shall be able to perform Interception such that no CSPs not obligated by the warrant, cannot detect that Interception is taking place unless there is a service/business relationship and not forbidden by law.

R6.2.6 - 60	Undetectability across Third Parties –The CSP shall be able to perform Interception such that any Third Parties not obligated by the warrant, cannot detect that Interception is taking place unless there is a service/business relationship and not forbidden by law.

R6.2.6 - 70	Undetectability across jusidictions: The CSP shall ensure the performance of interception cannot be detected in jurisdictioins other than the jurisdiction of the warrant.

R6.2.6 - 80	Interception Capability Undetectability–The CSP shall ensure that only authorized parties can have knowledge of operational use of Interception capabilities, Interception-related hardware and software.

R6.2.6 - 90	LI failure impact on target services–A failure of LI shall not impact on the target’s, or other users’ services.

R6.2.11 - 20	Recordkeeping access–LI lifecycle records of the CSP’s mananement interception (e.g., log files) shall only be visible to, and accessible by, authorized personnel.

R6.2.11 - 30	Tamper Prevention and Detection–The CSP shall employ a mechanism (e.g., cryptographic hashing) to provide assurance that LI lifecycle records of the CSP’s mananement interception (e.g., log files) cannot be unnoticeably altered / are tamper resistent.

Editor’s note: Requirement name must be improved

R6.2.11 - 40	Recordkeeping Retrospective Spoofing prevention–The CSP shall employ a mechanism (e.g., blockchain) that ensures LI implementation records cannot be spoofed.

R6.2.11 - 50	The Record Keeping: The record keeping period shall be configurable based on CSP record retention policy

R6.2.11 - 60	Recordkeeping lifecycle –Once the recordkeeping period expires, the CSP shall securely delete such records.

R6.2.11- 70	Auditing–The CSP shall implement jurisdiction-appropriate auditing procedures.

R6.2.12 - 20	Delivery Security–The CSP shall enter into a bilateral agreement with the LEA that specifies the security of the negotiated delivery mechanism from the CSP to the LEA.

R6.2.12 - 30	Authenticity: The delivery shall employ a mechanism to ensure the authenticity of the delivered Interception Product from the CSP to the LEA.

R6.2.12 - 40	Confidentiality: The delivery shall employ a mechanism to ensure the confidentiality of the Interception Product from the CSP to the LEA.

R6.2.12 - 50	Integrity: The delivery shall employ a mechanism to ensure the product of interception can not be altered from the CSP to the LEA without being detected at the LEA side.

R6.2.12 - 60	Spoofing prevention; The delivery shall employ a mechanism to ensure the product of interception cannot be spoofed, contemporaneously, or at a later time, and integrated into the legitimate interception product from the CSP to the LEA.

R6.2.12 - 70	DDOS prevention, editor’s note: Text needed

R6.2.13 - 10	Virtualization Security– When CSP networks are virtualized based on the ETSI suite of standards, the CSP LI implementation shall minimally comply with the NFV security requirements specified in ETSI GS NFV-SEC 012 System specification for execution of sensitive NFV components[3].

Editor’s note make RAN and core implementation requirements

Editor’s Note: Security section: many requirements missing!

R6.2.13.11	Security with legacy and virtual networks – As the surface of attack is higher with such configuration, CSP have to study any attack on the complete LI system using or from the legacy to the virtual part of its network, or from the virtual to the legacy part of its network.

R6.2.13.11 BIS   Security with slices, different virtual and infrastructure ownerships- If only part of the virtual or infrastructure is owned and/or managed by the CSP, the security of LI function or LI target information have be studied by the CSP as the surface of attacks and LI detectability by third parties is higher. The CSP have to ensure the highest level of security of LI system whatever the slices, such CSP is managing,  whatever the degree of control and of ownership provided by its partners or third party.

R6.2.13.13	Main assets to be protected – The most important assets to be secured by the CSP are at least of the target list(s), the LEA’s system addresses, the LI management system, the delivery or any LI internal interfaces or function to any known attacks.

R6.2.13.15	Role based LI management. If possible, any changes of LI function or management have to be made by different peoples inside the CSP. It is recommended that the people who are in charge to activate or deactivate a specific target are not those who are in charge of LI audit, or basic LI administrative support system (like update of OS, of some software…)

R6.2.13.16	Security solution to protect LI system.  The chosen solution by the CSP and/or with the authority/LEAS may have to be updated and based on the "start of the art" technology and at any ISO layer level from hardware up to the application level.

R6.2.13.17	Security at the edge of the CSP network, such as RAN- LI at the RAN shall be based on dynamic target provisioning to prevent risks to full static target list where full physical, and logical protection of the LI function cannot be guaranteed.

XXXXXXXXXXXXXXXXXXXXX          Left after reordering         XXXXXXXXXXXXXXXXXXXXX

### 6.2.3 Specificity

R6.2.3 - 10	Target specificity–The CSP shall ensure no communications are intercepted other than those of, or associated with, the target’s equipment, facilities or service.

R6.2.3 - 20	Interception specificity–The CSP shall ensure that only IRI, or IRI and CC, are intercepted and delivered to the LEA, as specified by the warrant.

Editor’s note: is CC only needed?

R6.2.3 - 30	Service specificity–The CSP shall ensure that only the communication services are intercepted required by the warrant.

### 6.2.11 Recordkeeping, Accounting and Auditing

Editor’s Note: Align all requirements with upcoming LI lifecycle diagram terms (provisioning/production, etc.)

# 7 Specific service, capability, and feature requirements

R7.1.1 1 location and relay- if a radio relay that could be based on another UE or on another cell, the location provided by each relay have to be provided with the information of the rank and of the nature of the relay (example: UE based or cell based).

R7.1.1.2 Presence- If presence service is provided by the CSP, the CSP shall be able to supress presence information provided to an LEA relating to target’s friends or contacts, on a per warrant basis.

Editor’s note: need to delete the general requirements from the specific section

Editor’s note: section 7 language is to adopt section 6 language

Editor’s note: potential new section on

off load on non 3GPP access networks (WiFi SIPTO)

MEC

M2M/IoT/BEST

D2D

V2X

Mission Critical Communication

Presence

Slicing

Relay

Enhanced location (5G and 5G features applied to 4G)

Local call/Local Switch

VCC

DDE

Additional messaging services (RCS)

Editors note: LI applies to all 3GPP services whether or not mentioned in 6 or 7

## 7.1 Customized Alerting Tone (CAT)

CAT is a service defined in TS 22.182 [9], TR 23.872 [10], TS 24.182 [11], and TR 29.882 [12]. The target may participate in the service as either the calling or the called party. The CSP providing the CAT service, and performing the interception, shall report the following:

-	When the target activates, modifies (e.g., changes to content, content descriptors, and timing descriptors), and deactivates CAT settings;

-	When the target invokes the function of copying of another subscriber’s CAT;

-	When the target invokes the up loading or down loading CAT and is not part of CAT delivery to the calling party, the CAT shall be delivered to the LEMF;

-	The identity whose CAT is played to the target.

Additionally when the target is a User, the CSP providing the CAT service, and doing the interception, shall report the following:

-	The CAT sent to the calling party;

-	When another subscriber copies the target’s CAT;

-	When available, the access method (e.g., via Device or web) the target used to activate, modify, and deactivate their CAT settings.

Intercepted CAT may, depending on national regulations, be:

-	"played" as part of the CC, during a call set up or;

-	Delivered as a file in the IRI Record.

Depending on national regulations, intercepted CAT media may be considered content or signalling.

## 7.2 Customized Ringing Signal (CRS)

CRS is a service defined in TS 22.183 [13] and TS 24.183 [14]. The target may participate in the service as either the calling or the called party. The CSP providing the CRS service, and doing the interception, should report the following:

-	The CRS, when invoked by the target, is sent to the called party;

-	When the target activates, modifies (e.g., changes to content, content descriptors, and timing descriptors), and deactivates their CRS settings;

-	When the target invokes the function of copying another subscriber’s CRS;

-	When the target invokes the up loading or down loading CRS, and is not part of CRS delivery to the called party the CRS shall be delivered;

The identity whose CRS is played to the target.

Additionally for when the target is a User, the CSP providing the CRS service, and doing the interception, shall report the following:

-	When another subscriber copies the target’s CRS;

-	When available, the access method (e.g., via Device or web) the target used to activate, modify, and deactivate their CRS settings.

Intercepted CRS may, depending on national regulations, be:

-	"played" as part of the CC, during a call set up or;

-	Delivered as a file in the IRI Record.

Depending on national regulations, intercepted CRS media may be considered content or signalling.

## 7.3 Home Node B and Home enhanced Node B (H(e)NB)

HNB and HeNB are jointly referred to as H(e)NB, as defined in TS 22.220 [8]. The location of the H(e)NB is the location information used by the operator to verify the location for H(e)NB activation.

For the purpose of LI, a target may be a user or user equipment attached to a H(e)NB, a Closed Subscriber Group (CSG), or it is a national option to allow targeting a H(e)NB itself.

The LI requirements for H(e)NB local routing, selected IP traffic offload (SIPTO) or local IP access (LIPA) are FFS.

Interception shall be done in such a manner to avoid detectability by the target or others.

When a target receives service from the PLMN via a H(e)NB, the following applies:

-	H(e)NB information (e.g., location and identification) shall also be provided to the LEMF;

-	If available, the location reported for the target attached to a H(e)NB is the H(e)NB’s location;

-	Target attachment to the H(e)NB and handovers to/from the H(e)NB shall be reported to the LEMF;

When the target is the CSG, the CSP shall report the following:

-	modifications (e.g., additions, deletions, changes in time limits for temporary CSG Members) of the CSG list for the H(e)NB;

-	When available, the access method (e.g. via Device or web) the H(e)NB Hosting Party used to modify the CSG list, if multiple access methods are allowed;

-	CSG member’s handovers to/from the H(e)NB;

-	CSG members attachments to the H(e)NB;

-	CSG members communications via the H(e)NB;

-	It is a national option whether interception on CSG members’ communications continues after handover occurs from the H(e)NB.

CSG requirements are For Further Study.

When the target is the H(e)NB, then the CSP shall report the following:

-	activation and deactivation of the targeted H(e)NB;

-	IP address information regarding the secure tunnel endpoints between the H(e)NB and the Femto Security Gateway in the home network;

-	modifications (e.g., additions, deletions, changes in time limits for temporary CSG Members) of the CSG list for the H(e)NB;

-	When available, the access method (e.g. via Device or web) the target used for the modification of the CSG list, if multiple access methods are allowed;

-	handovers to/from the H(e)NB;

-	Device registrations on the H(e)NB;

-	communications via the H(e)NB;

-	It is a national option whether interception on H(e)NB communications continues after handover occurs from the H(e)NB.

CSG requirements are For Further Study.

## 7.4 Location information

### 7.4.1 General

Depending on national requirements, the CSP may be required to report the location of the target at the beginning and end of sessions on a per intercept basis. It may also be a national requirement for the CSP to report the location of the target :

-	during on-going communications;

-	for any mobility management event detected in the CSP network which includes a target’s location change or update.

The location information associated with target communication reported to the LEMF shall be at least location information trusted by the 3GPP network (i.e. the location information is either 3GPP network derived or verified).

National regulation may require that the location information source be provided if known by the CSP.

The 3GPP access network derived or verified location information shall be the location(s) of the access point(s) to which the target is connected in the access network(s). The location shall be the access network identifier like the radio cell identity.

For non 3GPP access networks, the location information shall be at least the identity of entry point into the 3GPP core network (e.g. fire wall or security gateway). The location information of the non-3GPP access network shall be provided if this information is available to the CSP.

National regulation may require that third party provided location information associated with target communication, that may be available in the 3GPP network, is reported to the LEA.

If required by national regulation the geographic location and/or civic address information shall be reported to the LEMF. This can include additional radio coverage information.

### 7.4.2 LI and Location Services

In addition to the 3GPP network derived or verified location information as described in 5.11.1. target location information from Location Services (LCS, as described in 3GPP TS 22.071 [23] and 3GPP TS 23.271 [24]) may be used additionally to provide location information to the LEMF if available.

### 7.4.3 Lawful Access Usage of Location Services (LCS)

Additional Location information can be reported for various instances; the use of services; the use of network and or radio resources, the loss of a service; requested commercial Location Services (LCS) queries by the user, associates, or other application; or from network and radio functions providing varying degrees of granularity and accuracy.

Location Service may provide a higher degree of location granularity and accuracy than the existing network based Lawful Interception location capabilities described in clause 5.11.1.

National regulation may impose the following requirements defined in TS 22.071 [23] apply to LCS when invoked as part of LALS (Lawful Access Location Services):

-	Any Device (including inbound roamers) served by the PLMN and supported by the serving PLMN LCS (including coverage) shall be targetable by LALS;

-	 If the location positioning for the target Device has failed, the LCS server shall report the reason for failure, if known;

-	For LALS target Devices may be positioned under all circumstances required by national regulatory requirements;

-	The target Device user shall not be notified of any LALS location attempt;

-	LALS location information provided should be the most recent if requested and available;

-	The invocation, use and collection of LALS location should not cause target user service interruptions except for LCS priority described in clauses 4.5 and 7.3 of TS 22.071 [23].  In the case of LCS priority, national regulations may override the definitions in TS 22.071 [23];

-`	Support for LALS Location shall apply for both active and idle UEs;

-	LCS shall support LALS requests for the current (updated), or the last known position of an UE;

-	The network should be sufficiently flexible to accommodate evolving LCS enabling mechanisms and LALS service requirements;

-	Only authorized LALS network functions, resources and LI-LCS Clients shall provide LALS information;

For the purposes of this specification, the portion of LALS dealing with positioning of a target can be considered equivalent to lawful intercept use of LCS described in TS 22.071 [23]. Other aspects such as delivering a target's position to a LEA are beyond the scope of TS 22.071 [23].

National regulations may impose the following requirements in addition to those defined in TS 22.071 [23] to apply to LCS when invoked as part of LALS:

-	No LCS subscription is required for any DEVICE within the PLMN for providing LALS location;

-	The LALS positioning information shall be provided to the collection functions in a secure and reliable manner, such that the location information is neither lost, corrupted, nor made available to any unauthorized third party;

-	The LI-LCS Client shall support periodic target DEVICE location reporting to the LEA;

-	LALS shall support different location update periods per target for multiple LEAs;

-	LALS shall support identifying a target DEVICE using any one of the following:

-	MSISDN

-	Tel URI/SIP URI

-	IMPI / IMPU

-	IMSI

-	IMEI

-	Providing LALS location of an DEVICE attached to a 3GPP network, using an IMEI identifier is required by some national regulations. In such case, IMEI may be mapped dynamically by a suitable identity such as IMSI or MSISDN (but is beyond the scope of the present document). Target identities which may be provided by the LI-LCS client to the LCS server are specified by TS 22.071 [23] and TS 23.271 [24];

-	LALS shall re-use LCS supported positioning technologies defined in 3GPP, including positioning technology specific limitations, policies, procedures and operations.  The specific positioning technology selected and deployed to support LCS and LALS will vary from operator to operator and even PLMN to PLMN;

-	National Regulation may require LALS provide higher accuracy and security than commercial LCS Services;

-	LALS shall support multiple requests and obtain the location of the same target DEVICE at the same time from different LEAs.

The Location Services load which may include LALS for specific target UE(s) (e.g. multiple requests, multiple periodic updates, short period updates) may impact network and DEVICE performance (e.g. DEVICE battery performance) and may require establishing operator imposed limits to LALS Location Services requests.

## 7.5 IMS VoIP Service

The 3GPP network shall be able to support the delivery of IMS VoIP, and the IMS VoIP supplementary services (e.g., call forwarding), to the LEMF via one of the following two methods:

-	Intercepted IMS VoIP communications (e.g. IRI or IRI/CC) are delivered separately from other IMS services;

-	Intercepted IMS VoIP communications are delivered as part of all other services.

It is a national option as to which of the two options is applicable.

If a 3GPP network operator voice service replaces a legacy CS voice service, or is equivalent to a CS voice service, then it shall be considered to be a CS voice service for LI purposes. This also applies to new 3GPP networks without legacy CS voice service.

Subsequently, (at least) the following voice service LI capabilities shall be ensured in IMS VoIP; as they are in CS:

-	Location information shall be able to be reported to the LEMF;

-	If for a given implementation, the IRI related to the CC is not delivered in near real time (within the time delay allowance defined by national regulation), then the media related information (e.g. SDP) shall be delivered along with the CC to allow the CC content to be decoded or interpreted, without needing to wait for the IRI;

-	If available, activation or modifications of IMS supplementary services shall be reported to LEMF.

## 7.6 Messaging

The 3GPP network shall be able to support the separate delivery of intercept information (IRI or IRI/CC) for messaging services, to the LEMF from other targeted services.  This requirement is applicable for the following messaging services:

-	SMS (3GPP TS 23.040 [21]); and

-	MMS ([22]).

Message service delivery is independent from network access technology.

Editor’s note: not all SMS scenarios supported (SMS/IP, RCS)

## 7.7 Management of IMS supplementary services settings

The IMS network and related service platforms shall be able to support the reporting of IRI for the modification and management of the target’s IMS supplementary services settings.

### 7.7.1 IMS Video Service

In case of IMS video service, all clause 5 requirements shall apply.

### 7.7.2 IMS based Push-to-Talk

The 3GPP network shall be able to support interception of IMS based Push-to-Talk (PTT).

-	Services as defined in TS 22.179 [26] or OMA PoC System Description [27] and shall support Location reporting of the target;

-	Intercepted IMS PTT communications (e.g. IRI or IRI/CC) shall be able to be delivered separately from other IMS services;

-	Intercepted IMS PTT communications media types such as PTT Speech, PTT Audio (e.g. music), Video and Discrete Media (e.g. still image, formatted and non-formatted text, file) shall be able to be delivered as separated services.

The requirements for Mission Critical Push To Talk (MCPTT) service as defined in TS 22.179 [26] and the OMA PoC System Description [27], may also form the basis for a non-mission critical Push To Talk (PTT) service; those capabilities in either standard can be used as a basis for the service definitions. If the CSP offers both MCPTT and non-mission critical PTT services, the LI service shall be able to separate the interception based on the type of service offering.

## 7.8 Interception within the Home and Visited Networks for roaming scenarios

Editors not: move all missed in section 6 requirements to section 6 and than delete 7.8

The requirements in this clause are additional to the requirements described elsewhere in the present document which apply to the home network in a roaming scenario.

National regulations cover the definition of services and the definition of service provider categories which are subject to LI obligations. This can include which services are considered to be covered. For the purpose of roaming, IMS VoIP Service or other 3GPP operator services (CS voice replacement) shall be considered equivalent to a CS voice service and therefore all requirements applicable to legacy CS voice (e.g. interception of voice in isolation from other services) shall be applicable to IMS VoIP Service or other equivalent services.

It shall be possible to intercept all 3GPP defined services provided to a target by a CSP. The visited network shall be able to support the interception of all services without home network assistance or visibility, using the identifiers provided by an LEA as described in section 5.3.2. However, the visited network is not required to be able to intercept supplementary services (e.g. voicemail, home network based call forwarding) or 3rd party services not directly provided by the visited network.  However, national regulation may specify minimum LI capabilities, if such services are in the visited network then they shall be considered in scope for interception (subject to an applicable lawful authorisation). National regulations may require interception to take place in the home network for outbound roamers, where the user session is routed via the home network. There is no requirement to force traffic to the home network for this purpose.

National regulations may limit delivery of communications (CC and communications–associated IRI) of an outbound international roaming target by the HPLMN based on roaming specific interception constraints (e.g., roaming outside national border).  To support these scenarios, it is a national option for the HPLMN to be able to support a mechanism to apply on a per-warrant/per-intercept to limit the delivery of the communications (CC and communications–associated IRI) to LE of an outbound international roaming target based on roaming specific interception constraints (e.g., roaming outside a national border). The default setting is determined based on a national option. Communications originated by the roaming target are subject to this mechanism.  Communications incoming to the target are subject to this mechanism. Communications incoming to the target that do not reach the target (e.g., due to call forwarding) are not subject to this mechanism.  Non-communications-associated IRI (e.g., serving system event) are not affected by this requirement.

.

Annex A (informative): Capability Issues

This Annex contains capability issues not being formal LI requirements

RA.10	Delivery Performance–The CSP shall enter into a bilateral agreement with the LEA that specifies the availability and reliability of the near-real-time transport mechanism of the LI data from the CSP to the LEA.

RA.20	Overload reduction–The CSP shall employ a mechanism to reduce intercept product from overcollectionload on the LI equipment (e.g., exclusion of particular flows such as movies).

RA.30	Recordkeeping–The CSP shall create and retain necessary and sufficient LI lifecycle records of the CSP’s management of the Interception (e.g., log files), to demonstrate, after the fact, that the interception was implemented correctly per the warrant

RA.40 	Location of LI processes and associated sensitive data (see R6.2.13.13       Main assets to be protected ) The CSP shall ensure that LI processes and associated sensitive data such targets lists are self-contained within the legal jurisdiction of the warrant.

RA.50	Risk assessment -The CSP shall make  risks assessment and management on its LI system and its environment, based on some methods such as ISO 27000  family or TVRA of ETSI, in order to reduce risks on such LI system to the minimum possible. Such assessment have to be made regularly and at any new major change in terms of services or networks function.

RA.60	Traceability and access control to the LI system. Such system has to be securely protected and any attempts to it have to be logged.  The accesses to any target list or LEA machines addresses, or to LI administrative system, have to be isolated from the other network management and supervision. Secured access based control or ID based access control to LI system is recommended

Annex A (informative):
Bibliography

The documents listed below are not explicitly cited in this specification but are provided for background and for historical information.

3GPP TS 42.033: "Lawful Interception - stage 1".

3GPP TS 43.033: "Lawful Interception; stage 2".

3GPP TS 33.106: “Lawful Interception Requirements”3GPP TS 33.107: “Lawful interception architecture and functions”

3GPP TS 33.108:” Handover interface for Lawful Interception”

European Union Council Resolution on the Lawful Interception of Telecommunications (17 January 1995).

ETSI TS 101 331: "Lawful Interception (LI); Requirements of Law Enforcement Agencies".

ETSI ES 201 158: "Lawful Interception; Requirements for network functions".

Annex B (informative):
Change history

| Change history |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| TSG SA# | Version | CR | Tdoc SA | New Version | Subject/Comment |
| SA3LI#XX | 1.0.0 |  |  | 1.0.0 | Initial version |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |

# Editorial Parking Lot

- Alex Retzel: definition for CSP 5G network: All Elements used by the CSP to establish Telecommunication and to fulfil LI-requerements, belonging either to the CSP or to a third party provider.

- Robert Taylor: 1	Scope

The present document provides Stage 1 interception requirements within a 3GPP network.

The specification describes service requirements from a Law Enforcement point of view. The aim of this document is to define an interception system for 3GPP networks that supports regional interception regulations, but these regulations are not enumerated here as they vary. Regional interception requirements can be derived from this specification. Which PLMN services are subject to lawful interception is defined in national regulations.

The presence of a requirement in this specification does not in itself infer or mandate that a 3GPP operator has an obligation to implement any network service capability, which is not otherwise required to meet LI obligation compliance in relation to specific regulated services, offered by that 3GPP operator. Only those specific requirements and sub-sections of this specification which are applicable to specific network and/or service capabilities implemented in a 3GPP operator’s network shall be considered in scope for that operator. In all cases, national regulations define which requirements are applicable to 3GPP operators in each country relative to the services offered by each 3GPP operator.

As such not all requirements in the current document will apply in all national jurisdictions or to all 3GPP operator deployments (e.g. if an operator does not offer voice services, then voice LI requirement in the current document do not apply unless otherwise mandated in national regulations).

- Robert Taylor: did TR 23.872 graduate to a TS?

- Robert Taylor: same on TR 29.882

- Robert Taylor: Keep an eye (re: femto, unlicensed spectrum, etc.) on: Interception Area: is a subset of the Public Lands Mobile Network (PLMN) service area comprised of a set of cells which define a geographical zone.

- Robert Taylor: A lot of “national options” are left in 7.6 Encryption, compression and encoding

- Robert Taylor: timing is still messed up Again this section is still messed up.  Is it about timing requirements for delivery (timeliness) vs. accuracy & precision of timestamps for IRI & CC reports…

- Alex Markman: disambiguate third-party services: to the user vs. to the CSP

The CSP 3GPP notes that the 5G system is enabled by Network Function Virtualization and Software Defined Networking in TS 23.501[2].

While compliance with this reference document is necessary to achieve compliance with the majority of requirements in the present document, it is by no means sufficient. Consequently, the CSP shall complement its LI implementation with sound operational security policies, processes, and procedures