# 5G Lawful Interception Architecture

Contribution

s3i180073

3GPP SA3-LI Meeting, New Delhi, India

January 30, 2018

Source

OTD

Contact:
Scott C. Williams, SSA; scott.c.williams@ic.fbi.gov

Michael Bilca; michael@trideaworks.com

Abstract

This contribution describes the 5G Lawful Interception architecture.

# Introduction

This contribution describes the 5G Lawful Interception (LI) architecture. It defines a new network element, the Lawful Interception Control Function (LICF), which is an addition to the 4G LI architecture. It also takes a cue from the service-based architectural approach introduced by 3GPP SA2, and defines a new service based, bus architecture for LI.

# Acronyms and Definitions

ADMF – LI Administration Function

LEMF – Law Enforcement Monitoring Function

LICF – Lawful Interception Control Function

MF/DF – Mediation Function/Delivery Function

NF – Network Function

POI – Point of Interception

# Architecture

This clause describes the architecture of the 5G LI system.

## High Level LI Architecture

Figure 1 shows the high-level LI 5G architecture.

Figure 1. High Level LI Architecture

## Architectural Elements

This clause defines the LI architectural elements.

2.2.1 Network Function

The Network Functions (NF) in this figure are those defined by 3GPP SA2.

2.2.1 ADMF – LI Administrative Function

The 5G LI Administrative Function (ADMF) performs functions similar to pre-5G ADMFs, with the difference that it does not control the LI elements in the network directly, but through a newly defined LICF (see below). The ADMF pushes the target list to the LICF. The ADMF has no knowledge of session-level identifiers; that job is delegated to the LICF.

2.2.2 LICF – Lawful Interception Control Function

The Lawful Interception Control Function (LICF) manages the mapping between the target list pushed down by the ADMF and the session-level parameters that enable the choice, provisioning, and management of appropriate ICEs that have access to the target communications. The LICF maintains a mapping between ADMF subscription-level identifiers and session level identifiers used by the POIs. Critically, the LICF is an SBA client of the NRF. The NRF needs information from the NRF about the topology of the service chain for every session the target is involved in, so that it can ensure that the proper POIs are armed for every session.

2.2.3 POI– Point of Interception

Each network function has associated with it an POI that is provisioned and controlled by the LICF. The POI may query the LICF for the state of LI for a given subject identifier, or have the LICF push down said state. The POI also generates LI product by isolating the communications associated with the target and forwarding the LI product to the correct MF/DFs.

2.2.4 MF/DFs – Mediation/Delivery Functions

The Mediation/Delivery Functions (MF/DFs) aggregate information from upstream POIs, and, depending on implementation, correlate multiple streams, strip off network header parameters of no interest to the LEMF, and package and distribute the LI product to the correct LEMF.

2.2.5 LEMF – Law Enforcement Monitoring Function

The Law Enforcement Monitoring Function (LEMF) receives the LI product. The LEMF is out of scope of the present document and is completely outside the perimeter of the carrier’s network.

## Service Based LI Architecture

The LI system takes a cue from the service-based function approach introduced by 3GPP SA2, and defines a new service based, bus architecture for LI. Figure 2 provides a more detailed picture of the system.

Figure 2. 5G Service Based LI Architecture

3.3.1 Service Based Bus Architecture

The service-based architecture includes;

- two interception product busses,

  - one for Interception Related Information (IRI), depicted in Figure 2 as a solid blue line, and

  - one for Communication Content (CC), depicted as a triple blue line, and

- one control bus, depicted as a dotted blue line.

One of the benefits of a bus architecture is that POIs and MF/DFs can be instantiated dynamically, under the control of the LICF.

The POI on the UPF is doing double duty, pushing LI product both to the CC bus, and the IRI bus (e.g., for packet data header reporting).

Editor’s Note: Provisionally, all NFs are shown with associated POIs, but as this document evolves, it is expected that POIs that are not supported by use cases will be eliminated (for example, it is likely a POI will not be needed on the UDSF, but it is shown for now out of an abundance of caution, because it is easier to eliminate it later, rather than the other way around).

3.3.1 Startup

ICEs and MF/DFs register with the LICF upon startup and receive the necessary parameters to become functional. Among these parameters are target identifiers, and the addresses of the MF/DFs associates with each identifier. When the MF/DFs start up, they are provisioned by the LICF with the address of the LEMF associated with each target identifier.

3.3.3 Runtime

The lifecycle of all LI elements is under the control of the LICF, which serves as bus manager for both the IRI and CC busses, and the lifecycle events of the LI elements, including migrations.

# Security

The most sensitive information in the LI system is the target list. This is the list of all the subjects in the network currently under surveillance, whether active, suspended or in any other state.  The security measures used by the carrier to ensure unauthorized access to this list is not subject to standardization, but the architectural choices made in the design of the LI system do impact the security of the target list directly.

Since completeness of interception product is a legal requirement in most jurisdictions, the LI system must ensure that no events that are lawfully authorized for interception are missed. To ensure that no events are missed there are two architectural alternatives.

## Architectural Alternatives

4.1.1 Full Target List at every POI Node

A carrier may choose to deploy the full target list at all ICEs, such that when a UE arrives in the network and commences registration, the POI is fully armed and in position to recognize if the target identifier is in the target list. The choice to push the full list to every node is the simplest, and arguably the riskiest, since the compromise of any node will leak the complete target list.

4.1.2 Full Target List only in LICF

A service provider may choose not to spray the full target list wholesale in all ICEs in the network, but keep the full list in the LICF, and only provision at each POI the specific target identifiers that are present at the particular NF.

This choice introduces a race condition. When the UE appears, the POI must query the LICF to find out if the user identifier is part of the target list. As the registration sequence progresses, the NF POI is waiting for a response from the LICF.  When the reply arrives, the POI can now take action if the reply was positive. If the reply is negative, the ICE’s involvement ends.

If the reply is positive, depending on how long the POI-LICF-POI round trip for the query/reply took, it is possible that some reportable events are missed.  To mitigate this there are two further alternatives:

- The carrier may choose to suspend registration for the time it takes the LICF to answer, thus inducing a registration delay in all registrations, whether the user is a target or not, or

- The carrier may choose to cache the reportable registration events while the POI-LICF-POI query is running, and either report them if the answer is positive, or delete them if the answer is negative.

These are implementation choices at the discretion of the service provider, but the trade-off cannot be avoided.

4.1.3 Provisioning

When a new target is provisioned in the LI system, and the target is already registered in the SP network, the SP will have a similar implementation choice as in the previous clauses 4.1.1 and 4.1.2. The LICF has a choice to either wholesale arm every POI with the new target (and POIs would be expected to immediately start interception on the new target, as in 4.1.1), or, the LICF can poll every POI for all UEs, and only if a target UE is discovered at a particular NF, arm the associated POI (and start interception, as in 4.1.2). The second approach would take longer and would be expected to miss more of the pre-existing target communications than the first approach.