ETSI TS 103 221-3 V0.0.4 (2017-08)

Lawful Interception;

Internal Network Interface X3

The TS (ETSI Technical Specification) is the preferred deliverable when the document contains normative provisions and short time to "market", validation and maintenance are essential.

A TS may later be converted to an ES or an EN, or be used to publish the contents of a draft ES being sent for vote or a draft EN being sent for Public Enquiry or vote.

The guidance text (green) shall be removed when no longer needed or 
the skeleton without guidance text also available via the editHelp! website should be used.

<

TECHNICAL SPECIFICATION

Reference

<Workitem>

Keywords

<keywords>

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-préfecture de Grasse (06) N° 7803/88

Important notice

The present document can be downloaded from:
http://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the only prevailing document is the print of the Portable Document Format (PDF) version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx.

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.

The content of the PDF version shall not be modified without the written authorization of ETSI.

The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute yyyy.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM and the ETSI logo are Trade Marks of ETSI registered for the benefit of its Members.
3GPPTM and LTE™ are Trade Marks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
GSM® and the GSM logo are Trade Marks registered and owned by the GSM Association.

# Logos on the front page

If a logo is to be included, it should appear below the title on the right hand side of the cover page

# Copyrights on page 2

This paragraph should be used for deliverables processed before WG/TB approval and used in meetings.

Reproduction is only permitted for the purpose of standardization work undertaken within ETSI.
The copyright and the foregoing restriction extend to reproduction in all media.

# If an additional copyright is necessary, it shall appear on page 2 after the ETSI copyright notification.

The additional EBU copyright applies for EBU and DVB documents.

© European Broadcasting Union yyyy.

The additional CENELEC copyright applies for ETSI/CENELEC documents.

© Comité Européen de Normalisation Electrotechnique yyyy.

The additional CEN copyright applies for CEN documents.

© Comité Européen de Normalisation yyyy.

The additional WIMAX copyright applies for WIMAX documents.

© WIMAX Forum yyyy.

Contents (style TT)

To unlock the Table of Contents: select the Table of Contents, click simultaneously: Ctrl + Shift + F11.
To update the Table of Contents: F9.
To lock it: select the Table of Contents and then click simultaneously: Ctrl + F11.

Logos on the front page	3

Copyrights on page 2	3

If an additional copyright is necessary, it shall appear on page 2 after the ETSI copyright notification.	3

Intellectual Property Rights	5

Foreword	5

Multi-part documents	5

Modal verbs terminology	5

Executive summary	6

Introduction	6

1	Scope	6

2	References	6

2.1	Normative references	6

2.2	Informative references	7

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	8

3.3	Abbreviations	8

4	User defined clause(s) from here onwards	8

4.1	User defined subdivisions of clause(s) from here onwards	8

Proforma copyright release text block	9

Annex <A> (normative or informative): Title of annex	9

Annex <B> (normative or informative): Title of annex	10

<B.1> First clause of the annex	10

<B.1.1>	First subdivided clause of the annex	10

Annex <C> (normative or informative): ATS in TTCN-2	10

<C.1> Introduction	10

<C.2> The TTCN-2 Machine Processable form (TTCN.MP)	10

Annex <D> (normative or informative): ATS in TTCN-3	10

<D.1> Introduction	10

<D.2> TTCN-3 files and other related modules	11

<D.3> HTML documentation of TTCN-3 files	11

Annex <E> (informative): Bibliography	11

Annex <F> (informative): Change History	11

History	12

# Intellectual Property Rights.

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

# Foreword

This Technical Specification (TS) has been produced by ETSI Technical Committee for Lawful Interception (LI).

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# Introduction (style H1)

This unnumbered clause, if present, appears just before the "Scope". It is an optional informative element and shall not contain requirements.

Clause numbering starts hereafter.

Automatic numbering may be used in ETSI deliverables but it is highly recommended to use sequence numbering.

Check https://portal.etsi.org/Portals/0/TBpages/edithelp/Docs/EDR_Navigator.chm clauses 2.12.1.1 and 6.9.2 for help.

If the navigator does not work please run the following file AllowWebHTMLHelp.zip and please delete all the cookies of internet explorer. Please check with your IT administrator to make sure it is compliant with your company security policy.

# 1	Scope

The present document defines an electronic interface for the exchange of information relating to the establishment and management of Lawful Interception. Typically, this interface would be used between a central LI administration function and the network internal interception points.

Typical reference models for LI define an interface between law enforcement agencies (LEAs) and communication service providers (CSPs), called the handover interface. They also define an internal network interface within the CSP domain between administration and mediation functions for lawful interception and network internal functions, which facilitates the interception of communication. This internal network interface typically consists of three sub-interfaces; administration (called X1), transmission of intercept related information (X2) and transmission of content of communication (X3). The present document specifies the content of communication interface, X3.

# 2	References

## 2.1	Normative references (style H2)

Clause 2.1 shall only contain normative (essential) references, which are cited in the document itself. These references have to be publicly available and in English.

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are necessary for the application of the present document.

[1]		ETSI TS 103 221-1: “Lawful interception internal interface X1”

[2]		ETSI TS 103 280: “Dictionary for common parameters”

[3]		ETSI TS 102 232-1: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 1: Handover specification for IP delivery".

[4]		ETSI TS 103 221-2: “Lawful Interception; Internal Network Interface X2”

## 2.2	Informative references (style H2)

Clause 2.2 shall only contain informative references, which are cited in the document itself.

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

Use the EX style, add the letter "i" (for informative) before the number (which shall be in square brackets) and separate this from the title with a tab (you may use sequence fields for automatically numbering references, see clause A.4: "Sequence numbering") (see example).

EXAMPLE:

[i.1][tab]	<Standard Organization acronym>  <document number> <V#>: "<Title>".

[i.2][tab]	<Standard Organization acronym>  <document number>: "<Title>".

# 3	Definitions, symbols and abbreviations (style H1)

Delete from the above heading the word(s) which is/are not applicable, (see clause 2.11 of EDRs).

Definitions and abbreviations extracted from ETSI deliverables can be useful when drafting documents and can be consulted via the Terms and Definitions Interactive Database (TEDDI) (http://webapp.etsi.org/Teddi/).

## 3.1	Definitions (style H2)

For the purposes of the present document, the terms and definitions given in ETSI TS 103 221-1 [1] and the following apply:

EXAMPLE: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

## 3.2	Symbols

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in ETSI TS 103 221-1 [1] and the following apply:

# 4	Introduction and reference model

## 4.1	Reference model

The X3 interface is based on communication between:

The Network Function (NF), which performs interception

The Mediation Function (MF), which performs the necessary translation, correlation and mediation for onward handover over material to LEAs via the HI2 and HI3 interface.

The X3 reference model is shown in Figure 1.

Figure 1 – Reference Model

Implementation and deployment scenarios may be more complex. An illustrative list of deployment scenarios is considered in TS 103-221-2 [4] Annex C.

## 4.2	Assumptions

### 4.2.1	Architecture

The present document makes minimal assumptions about the LI architecture in which the X3 interface is deployed. The X3 interface is intended to be sufficiently flexible to be used as part of LI architectures defined elsewhere and assumes that the NF and MF are deployed following an LI architecture defined separately (e.g. by another SDO, industry body or local regulation).

As such, the present document makes no assumptions about the specific functional requirements on the NF with respect to e.g. buffering, de-duplication, filtering. It is expected that these requirements will be supplied by a combination of the relevant LI architecture and local regulation.

### 4.2.2	Implementation / realisation

The present document assumes that implementations of an LI architecture which utilise X1, X2 and X3 can be described by the following high-level model.

Figure 2 – Assumed Implementation Model

The model consists of the following entities:

An Implementation: This is a concrete realisation of one or more NFs as deployed by an implementer.

An NF: A function as defined by the relevant network and/or LI architecture (e.g. a P-GW in 3GPP LTE).

Control Function: The sub-function of the NF which accepts LI tasking messages. This may be supplied over a standardised interface (e.g, X1 as defined by TS 103 221 – 1). However, it is assumed that tasking may also be passed between NFs using other unspecified interfaces.

PoI (Point of Interception): The sub-function of the NF which performs interception and emits data. An NF may contain multiple PoIs; in this case it is assumed that the NF implementation will be responsible for multiplexing the output of these PoIs into a single X2 or X3 output stream.

The present document does not consider the means by which tasking information is communicated from a NF’s internal control function to the PoI sub-functions but provides the NF implementation a means by which to identify which NF and PoI originated each piece of data.

The present document assumes that the NF may be required to deliver high volumes of traffic (e.g. a broadband connection), and may be implemented on a platform with tight resource and/or performance constraints (e.g. a packet gateway), and as such X3 is required to minimise as far as is practical the amount of processing and additional bandwidth consumed (see clause A.1.4)

### 4.2.3	Deployment infrastructure

The present document assumes that the transport infrastructure between NF and MF is untrusted (see clause A.2.8), but assumes that the platform on which the NF and MF are realised are appropriately secured. It does not make any specific assumptions about whether the either the platform or transport infrastructure are virtualised.

The present document does not assume that clocks on different NFs are synchronised. It assumes that while X3 event timestamps may be required by local regulations and can be added to aid describing chronologies of events (e.g. in court), timestamps will not in general permit re-ordering or re-synchronisation of packets which have been intercepted at different NFs.

The present document assumes that X3 is required to provide sufficient information, together with X1, to detect loss of material over X3 (see clause A.1.8 and clause 4.2.4 below). However, it assumes that any NF behaviour regarding detection and recovery from link failures is out of scope, and that no additional application-layer mechanisms (e.g. X3 acknowledgement of data, or heartbeats) are required.

An illustrative list of deployment scenarios that have been considered as part of the design of the X2 interface is given in TS 103-221-2 [4] Annex C.

### 4.2.4	Regulatory assumptions

The present document assumes that material delivered over X3 may be wanted to be used as evidence in court. As such, it assumes that the X3 interface must make it clear when data has been lost over the X3 interface (see clause A.1.8), but recovery of this data (e.g. by buffering and retransmission) are out of scope (as described in clause 4.2.1).

The present document assumes that material over X3 is required to be delivered without undue latency (see clause A.1.5), but that any such latency requirements are not necessarily as stringent that those associated with the underlying communications session (e.g. there is no need for a latency which facilitates a two-way conversation, or remote haptic feedback, or vehicle avoidance measures).

## 4.3	Other standards in the X1, X2, X3 series

The present document forms part of an overall set of standards X1, X2 and X3.

# 5	Message contents and parameters

## 5.1	Overview

X3 PDUs are sent from the NF to the MF each time the NE intercepts data.

## 5.2	PDU Structure

The X3 PDU consists of two parts

A header providing information used by the NF and MF to correlate X3 and X2 messages which relate to a communication session, and what LI context they relate to (i.e what XID). See clause [5.3]

A content field, containing a copy of the intercepted material.

## 5.3	Header

### 5.3.1	Structure

The X3 PDU header consists of the following information elements:

Context information describing the source and and interception details associated with the X3 PDU (either as a full context structure as described in clause 5.3.1, or by giving the ContextIdentifier of a previously-seen Context as described in clause 5.3.2.7)

An optional sequence number (see clause 5.3.3)

An optional timestamp (see clause 5.3.4)

### 5.3.2	Context

#### 5.3.2.1	Description

An X3 Context describes a set of parameters associated with a set or flow of X3 PDUs, such as the target associated with the PDUs and the PoI that generated them.

The X3 PDU Context consists of the following information elements:

Table 1 – X3 PDU Context

| Field | Format | Clause | M/C/O |
| --- | --- | --- | --- |
| XID | XID (see TS 103 221-1 [1] clause 5.1.2) given as an unsigned 128-bit integer. | 5.3.2.2 | M |
| CorrelationID | 32-bit unsigned integer | 5.3.2.3 | O |
| NFID | String (?) | 5.3.2.4 | M |
| InterceptionPointID | String (?) | 5.3.2.5 | O |
| Direction | Enumerated | 5.3.2.6 | O |
| ContextIdentifier | 32-bit unsigned integer | 5.3.2.7 | O |

#### 5.3.2.2	XID

The NF shall populate the XID field with the XID associated with the intercepted product.

An XID is a UUID (see [1] clause 5.1.2). The XID shall be given as a 128-bit unsigned integer.

EDITOR’S NOTE – We may want to consider the binary representation of this more closely – RFC4122 shows that it is quite structured, and different platforms may behave differently (particularly endian-ness)

#### 5.3.2.2	CorrelationID

The NF shall ensure that X3 PDUs associated with the same communication session are given the same CorrelationID value. The value shall uniquely identify the communication within a given context. The scheme for choosing CorrelationIDs is an implementation matter. The NF may adopt a convention for choosing CorrelationIDs which enables correlation of the same communication session across multipled NFs (e.g. by using a charging identifier) but such decisions are out of scope of the present document

The CorrelationID value shall be given as a 32-bit unsigned integer.

If the X3 PDU is not associated with any other X2/X3 PDUs, or the NF is permitted to not correlate the X3 PDU with other X3 PDUs, the NFmay omit the field.

#### 5.3.2.4	NFID

The NF shall populate the NFID field with a value that identifies the NF to the MF. The format and content of the field is left to the relevant LI architecture to define. Depending on the architecture and deployment scenarios, it may be used as a way to identify the type of NF

The NF shall be given as a string.

EDITOR’S NOTE – may wish to consider this format further

#### 5.3.2.5	InterceptionPointID

The NF may optionally populate the InterceptionPointID field with a value that identifies the point of interception within the NF. The format and content of the field is left to the relevant LI architecture and implementation to define.

The InterceptionPointID shall be given as a string.

EDITOR’S NOTE – may wish to consider this format further

#### 5.3.2.6	Direction

The NF may optionally populate the Direction field with an indication of the direction of the set of X3 PDUs associated with the context. Permitted values are:

To Target

From Target

Combined

Unknown

The Direction shall be specified as an enumerand

EDITOR’S NOTE – we’ll worry about how this works when we decide on the encoding format, but it could just be an int.

#### 5.3.2.7	ContextIdentifier

The NF may optionally populate the ContextIdentifer field with a value that uniquely identifies the context.

After sending an X3 PDU with a context structure including a ContextTag field, the NF may use the ContextIdentifier value instead of a full context structure for subsequent X3 PDUs associated with the same context.

### 5.3.3	SequenceNumber

The NE may optionally populate the SequenceNumber field with the sequence number of the X3 PDU. If used, the SequenceNumber shall start at zero and increment by one for each X3 PDU in the same context.

EDITOR’S NOTE – maintaining sequence numbers requires additional resources at the NE, but are provided as a way of ensuring that loss of PDUs can be detected. If we feel that the delivery guarantees from the transport layer are good enough (for example, because you can’t accidentally lose a PDU from the middle of a TCP stream), then we can consider omitting the sequence number to minimise both message size and resource requirements at the NE

The SequenceNumber shall be given as a 32-bit unsigned integer. Once the maximum sequence number is reached, the NE shall restart the sequence number from zero.

### 5.3.4	Timestamp

The NE may optionallypopulate the Timestamp field with the time that the content for the PDU was intercepted.

The Timestamp field shall be given as an NTP timestamp format (see RFC 1305 [add ref later] section 3.1).

EDITOR’S NOTE – This is one possible encoding that gives a fixed width. Others can be chosen. We can decide if we care about the “Y2036” problem. If we choose other encoding methods, there may be a more natural choice (e.g. ASN.1 GeneralizedTime). Observation that RFC 5905 obsoletes 1305 and solves Y2036 problem.

## 5.4	Content

### 5.4.1	Structure

The Content field contains a set of binary octets representing the content intercepted by the NE. The Content field includes the following information elements:

| Field | Format | Clause | M/C/O |
| --- | --- | --- | --- |
| ContentLength | 32-bit unsigned integer | 5.4.2 | M |
| ContentFormat | 32-bit unsigned integer | 5.4.3 | M |
| ContentOctets | Binary data | 5.4.4 | M |

### 5.4.2 	LengthOfContent

The NF shall populate the LengthOfContent field with the length of the ContentOctets field in octets.

The LengthOfContent field shall be given as a 32-bit unsigned integer.

### 5.4.3	ContentFormat

#### 5.4.3.1	Description

The NF shall indicate the format and encoding of the ContentOctets field by setting the ContentFormat field to the appropriate ContentFormat identifier. A list of valid ContentFormat identifiers, and their definitions, is given in clauses 5.4.2 onward.

The ContentFormat shall be specific as a 16-bit unsigned integer

The following ContentFormats are defined:

Table 3 – ContentFormats

| ContentFormat | Description | ContentFormat identifier | Defined in |
| --- | --- | --- | --- |
| Unknown | Unknown format, unstructured OCTET STRING | 0 |  |
| IPv4 Packet | Content contains an IPv4 packet encoded as per RFC 791 [ref] | 1 | 5.4.3.2 |
| IPv6 Packet | Content contains an IPv4 packet encoded as per RFC 8200 [ref] | 1 | TBD |
| Ethernet Frame | Content contains an ethernet frame encoded as per IEEE 802.3 [ref] | 2 | 5.4.3.3 |
| RTP Packet | Content contains an RTP frame encoded as per RFC3550 [ref] | 3 | 5.4.3.4 |

EDITOR’S NOTE – This is just an example list. If we think this approach has merit, then most of the work in X3 will be collecting the correct list of content formats (such as IPv6, SRTP, packet-header-only variants of IPv4/6 etc). We also need to consider a mechanism that allows other SDOs (e.g. 3GPP) a means of defining them, and whether we allow vendor-specific definitions (which aids adoption but hurts interoperability).

EDITOR’S NOTE – There is a sense in which this indication simply says what the next layer of encapsulation is – it could be, for example, RTP inside UDP inside IPv4.

EDITOR’S NOTE – Check that X1 contains sufficient information to allow the NE to deliver the correct X3 content format.

#### 5.4.3.2 IPv4 Packet ContentFormat

If the IPv4 Packet ContentFormat is specified, the contents shall contain an IPv4 packet encoded as per RFC 791 [ref]

#### 5.4.3.3 Ethernet Frame Packet ContentFormat

If the Ethernet ContentFormat is specified, the contents shall contain an ethernet frame encoded as per IEEE 802.3 [ref]

#### 5.4.3.4 RTP Packet ContentFormat

If the RTP Packet ContentFormat is specified, the contents shall contain an RTP packet encoded as per RFC 3550 [ref], without any IP / UDP encapsulation.

Use of this ContentFormat is discouraged for new implementations – handing over RTP with IPv4/IPv6 encapsulation is much preferred.

#### 5.4.3.5 CCContents

If the CCContents ContentFormat is specified, the Contents field shall contain a BER-encoded CCContents structure, as described in TS 102 232-1, without any further encapsulation.

# 6	Transport and encoding

## 6.1	Encoding

X3 PDUs are encoded in a fixed-field binary format, as described in section 5.

EDITOR’S NOTE – This is just a suggestion, based on the requirements in Annex A. If it is agreeable, we’ll need to write more - it may make sense to merge the “encoding” section with the section above, since they can be described together simply. If we elect to adopt another encoding format using some kind of schema syntax (e.g, ASN.1 or Protocol Buffers) then we use this section to describe it and point to the relevant schema file.

EDITOR’S NOTE – Fixed-field binary may seem “elegant” or “fast” initially, but can become harder to maintain and requires more drafting effort. Depending on the complexity of the X3 structures, it may be that the “cost” of a more heavy-weight encoding/decoding mechanism is a price worth paying. Encoding/decoding performance is a very important consideration. May also need to consider two populations (high-speed packet-copiers and more complex higher-level implementations). See if 3GPP have a view.

## 6.2	Transport

To send X3 PDUs to the MF, the NE opens a TLS over TCP connection. TLS is used to perform mutual authentication and identification between the NE and MF (see clause 7).

EDITOR’S NOTE - Given the message format above, the transport is required to identification and authentication of the NE to the MF (and conceivably vice-versa) and also meet any delivery requirements involving reliability, confidentiality and integrity (such as A.1.7, A.2,3 and A.2.4). If we don’t want these requirements left the the transport, we need to introduce extra features to the previous section.

TLS is suggested based on the fact that it appears to meet the requirements and is what we used in X1. However, we should definitely consider alternatives. For example SCTP may allow us to get rid of some framing information, while QUIC is being considered by 3GPP even though it is not yet an RFC.

Initial assessment of QUIC from LI(17)R42: Concerns about timescale of standardisation and support in readily available network stacks in the timeframe required by X3. However, QUIC is being considered by 3GPP CT groups – if they choose it as part of 5G, support is likely to expand rapidly. We will know more by March. One of the concerns in 3GPP CT is the connection establishment time; this may not be relevant if we have persistent transport connections.

Initial assessment of SCTP: Also concerns about how widely available implementations are.

Initial assessment of TLS: It is in use in X1 and well-understood/deployed. It is in use in some vendors for X3.

Consider DTLS as an alternative to TCP-based approaches for performance considerations. It is in use in at least one vendor.

# 7	Security

EDITOR’S NOTE – as mentioned elsewhere, the idea here is to secure the transport, not each individual PDU (because doing otherwise is expensive and complicated to implement). TLS is one option for doing this, and is attractive because an NE will already need it for X1. However, others are possible (e.g. IPSec).

If your document contains figures, tables and/or mathematical formulae, this is the way they shall be prepared:

- Figures shall be prepared in accordance to clauses 7.5.2 and/or 7.1 of the EDRs. The figure number and title shall be below the figure. An explicit figure title is optional. See clause 5.1.5 if you need to include notes to figures.

Use TF style for the figure number and title.

Use FL style on the paragraph which contains the figure itself.

If applicable, the figure number is followed by a colon, a space and the table title.

Maximum width for figures is 17 cm and maximum height is 22 cm.

Should you wish to number figures automatically, "Sequence numbering and bookmarking" (see clause 6.9.2 of the EDRs) is highly recommended.

- Tables shall be prepared in accordance to clause 5.2 of the EDRs. If you have tables in your document, the table number and title shall be above the table itself. An explicit table title is optional.

Use TH style for the table number and title.

If applicable, the table number is followed by a colon, a space and the table title.

Should you wish to number tables automatically, "Sequence numbering and bookmarking" (see clause 6.9.2 of the EDRs) is highly recommended.

- Mathematical formulae shall be prepared in accordance to clause 5.3 of the EDRs.

Numbers given to the clauses, tables, figures and mathematical formulae of an annex shall be preceded by the letter designating that annex followed by a full-stop (e.g. figure B.1, table C.4). The numbering shall start afresh with each annex. A single annex shall be designated "Annex A".

NOTE:	For an easy application of the ETSI styles download "the ETSI styles toolbar" from editHelp! website.

The following text is to be used when appropriate:

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, ETSI grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

## Annexes

Each annex shall start on a new page (insert a page break between annexes A and B, annexes B and C, etc.).

Numbers given to the clauses, tables, figures and mathematical formulae of an annex shall be preceded by the letter designating that annex followed by a full-stop. The numbering shall start afresh with each annex. A single annex shall be designated "Annex A".

Clauses in annex A shall be designated "A.1", "A.2", "A.3", etc. (further details in clause 2.12.1 of the EDRs).

Use the Heading 8 style. Insert a line break ("shift" +  "enter") between the colon and the title.

For all annex clause headings use the appropriate Heading styles, starting from Heading 1, e.g. for clause A.1 use Heading 1, for clause A.1.1 use Heading 2. (See clause 6.1, table 8 of the EDRs).

Annex A (normative):
Requirements

## A.1 Protocol & Architecture requirements

EDITORS’ NOTE – comment from Heathrow rap meeting: We should consider whether there is a requirement to filter X3 at the NE to reduce traffic volumes.

EDITORS’ NOTE – comment from Heathrow rap meeting: We should consider whether there is a requirement to de-duplicate traffic in X3

EDITOR’S NOTE – comment from SA3-LI meeting: Ensure that the requirements are only about the protocol and not the wider interface.

### A.1.1	Basic Functionality

The interface shall be used for delivery of CC from the network element, which created the copy of the original content of communication, to the mediation function.

### A.1.2	Flexible

The X3 architecture and message exchange technique shall be flexible to allow implementation in both existing and future national and international operator network architectures.

### A.1.3	Extensible

The basic message exchange protocol shall allow limited extensibility to support parameter not currently supported by the base protocol.

### A.1.4	Lightweight

The protocol shall use a protocol containing minimal options or extensions which are not specifically required by X3.

### A.1.5	Delay

The X3 architecture and message exchange technique shall  not introduce unnecessary delay.

### A.1.6	Permanent and Dynamic Connections

The X3 architecture and message exchange technique shall support both permanent connection and dynamic link / connection scenarios.

### A.1.7	Reliability

The X3 architecture and message exchange technique shall provide reliable data transfer.

EDITOR’S NOTE – comment from Heathrow rap meeting: we should be more explicit about whether e.g. buffering is required at the NE, or whether this is left to the underlying transport protocol.

### A.1.8	Error detection

The X3 architecture and message protocol shall support error detection (ie in case of data loss) specifically only across the interface in question.

### A.1.9	Redundancy

The X3 architecture and message protocol shall support both 1 to 1 and 1 to 2 end point configurations (ie for redundancy).

### A.1.10	Correlation

The X3 protocol shall provide information necessary to allow correlation at the MF for information provided over HI2 and HI3.

### A.1.11	Mediation into HI2 / HI3

The X3 protocol shall provide information necessary to allow correlation at the MF to comply with the requirements of both the HI2 and HI3 interfaces, where applicable – at a minimum the packet-switched parts of TS 33.108 / 128 and TS 102 232 family

## A.2 Security requirements

### A.2.1	Authentication & Authorisation

The X3 architecture and message exchange technique shall provide authentication and authorisation of end points.

### A.2.2	Accounting / Audit

The X3 architecture and message exchange technique shall provide Accounting & Auditing.

### A.2.3	Integrity Protection

The X3 message exchange technique shall provide integrity protection for all messages exchanged between nodes in the X3 architecture. Use of Integrity protection shall be mandatory.

### A.2.4	Confidentiality Protection

The X3 message exchange technique shall provide confidentiality protection for all messages exchanged between nodes in the X3 architecture.

### A.2.5	Replay Protection

The X3 message exchange technique shall provide replay protection for all messages exchanged between nodes in the X3 architecture.

### A.2.6	Standalone interface

The X3 architecture and message exchange technique shall be designed as a standalone physically dedicated LI interface. The design and selection of the protocol shall where possible ensure vulnerabilities in non-LI interfaces on the same node shall not impact LI interfaces and security.

EDITOR’S NOTE: Need to revisit this for NFV or other virtualised situations. Noted that A.2.8 has the correct tone for NFV rather than A.2.6.

### A.2.7	Minimum Security Level

The X3 architecture and message exchange techniques shall provide a minimum level of security (including cypher suites and key length), which must be supported by all nodes. At least two algorithms shall be specified. The protocol and algorithms shall be resistant to bid down attack.

### A.2.8	Underlying Infrastructure Trust

The X3 architecture and message exchange techniques shall assume by default that the underlying network communication links and infrastructure are untrusted.

EDITOR’S NOTE – We need to look at undetectability requirements, both here and in X2/X1.

### A.2.9	Firewall and NAT Transversal

The X3 message exchange technique shall be compatible with existing operator firewall and NAT transversal architectures. The message exchange technique shall not require unrestricted opening of common ports (e.g. port 80 or 21). The message exchange technique shall not prohibit the development of future X3 aware firewall filtering to provide rejection of malicious X3 message at operator security gateways.

### A.2.10	Certificate and Key Management

The X3 architecture relies on (where applicable) Certificate and Key Management mechanisms (including Certificate and Key revocation) from X1.

EDITOR’S NOTE from SA3-LI Newport: This will need to be revisited for NFV, in line with NFV SEC 011. The “start-up” problem – how to get the initial keys/certificates – will need to be solved here and in NFV in the same way.

Annex B (informative):
Suggestions for solutions

Just a scratch area to make sure we don’t forget any ideas.

PROTOCOL

QUIC should be investigated for X3.

Some useful considerations sent in by Luke – make sure we think through all of these:

> 1) Explicit record boundary framing.
>
> We need explicit record boundary framing if we continue to use a TCP (stream) based handover.
> This would allow for more efficient receiving of PDUs via the stream from the initial
> handover as well as subsequent reprocessing.
>
> The existing BER requires decoding of received octets to see if the PDU is complete.
> Because BER permits indefinite length encoding, this may require a complete decode.
> An optimisation is to read the first few octets and manually decode the TLV
> and if the L is a definite length then that may be sufficient.
> Using DER instead of BER forces the use of definite length so that this optimisation would always work.
>
> Note that the framing issue is true for other encoding techniques.
> E.g. a lot of people who use Google protobuf reinvent the wheel and implement their own per-message framing for delivery over stream protocols, because the protobuf internally doesn't provide that.

I think this is still relevant for X3 if we end up using TCP (streams) for X3.
Less of an issue if record based (UDP protocol, SCTP) is used.


> 2) Optimise for decoding.
>
> Encoding only occurs once, and for packet-based CCs could arguably scale horizontally.
> Decoding occurs at least once and generally multiple times.
> I speculate that usually the receiver may want to do at least one pass of the decoding in a sequential manner.
> Therefore, in my opinion, optimising any handover should be for efficiency of network transport and receiver/decoding processing because that's where most of the CPU cycles will be spent.

Possibly less relevant for X3, but the point still stands:
- The sending NE only needs to encode once.
- Any middle boxes, receiving MFs, debug tools, etc, will result in decoding at least once if not more


> 3) Existing HI3 is too verbose.
>
> The existing HI3 payload is too verbose (in my opinion).
> Even with OER (which might reduce the TLVs to LV or V fields), there's still a lot of extra verbosity.

Not specifically an issue for X3 yet, since ASN.1 BER hasn't (yet) been chosen.


> Potential improvements include:
> - Fixed size 128bit UUID per active session to replace most of PSHeader ?
> - One timestamp format, ISO8601, possibly fixed-length binary encoded. (I know we can't change the standard now, but in my opinion microSecondTimeStamp is superfluous because timeStamp already supported fractional seconds...)
> - Fixed length header before each CC PDUs. Optional elements/extensions indicated with a header flag and encoded after the CC octets. I speculate that this will be more CPU cache friendly.

I want to give my "thought bubble" above more consideration, but I think it really has merit for X3.
Lightweight fixed-size header, then payload, then possibly extensions/options as trailers after the payload, to improve the efficiency of processing.


> 4) More interoperable extensions.
>
> The (national) extensions should be implemented similar to how 3GPP TS 29.002 (GSM MAP).
> This an optional container containing the OID of the extension and a binary bucket of the actual extension.
> This means that a generic decoder can decode all of the standard HI3 CC fields, and can gracefully skip the OID+octets of national extensions.
>
> This would be a useful change to make even in the existing -1 if at all possible.

Less of an issue for X3 (same as (2) above).
Worth investigating separately for HI3 / 102 232-1

Annex <F> (informative):
Change History (style H8)

The informative clause shall start on a new page and be the last annex before the "History" clause. It is an optional, informative element and shall not contain requirements.

If it is desired to keep a detailed record of the changes implemented in a new version it is recommended that this is done by inserting a "Change history/Change request" annex, see clause 2.15.

It shall be presented as a table. Apply the normal style format for tables (see clause 5.2.2 of the EDRs).

| Date | Version | Information about changes |
| --- | --- | --- |

# History (style H1)

This unnumbered clause shall start on a new page and be the last clause of an ETSI deliverable. It is a required informative element and shall not contain requirements.

The "History" identifies the major milestones in the life of an ETSI deliverable through the means of a table. The history box shall be provided by the ETSI Secretariat (all additional information will be removed at the publication stage).

| Document history |  |  |
| --- | --- | --- |
| <Version> | <Date> | <Milestone> |
| 0.0.1 | 2016-10-14 | Skeleton |