3GPP DTS 33.127 0.0.5a (2018-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Security;

Lawful Interception (LI) Architecture and Functions

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

5G, LTE, UMTS, Security, LI, Architecture

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Requirements Realisation	8

5	Functional Architecture	8

5.1	General	8

5.2	High Level Generic LI Architecture	8

5.3 	Functional Entities	10

5.3.1	Law Enforcement Agency (LEA)	10

5.3.2	Point of Interception (POI)	10

5.3.2.1	General	10

5.3.2.2	Separated Handling of Control Plane and User Plane	10

5.3.3	Mediation and Delivery Function (MDF)	11

5.3.4 	Administrative Function (ADMF)	11

5.3.5	System Information Retrieval Function (SIRF)	12

5.3.6	LEMF – Law Enforcement Monitoring Facility	12

5.4	LI Interfaces	12

5.4.1	General	12

5.4.2	Interface LI_SI	13

5.4.3	Interface LI_HI1	13

5.4.4	Interface LI_X1	13

5.4.4.1	General	13

5.4.4.2	LIPF and POI	13

5.4.4.3	LIPF and MDF2/MDF3	13

5.4.5	Interface LI_X2	14

5.4.6	Interface LI_X3	14

5.4.7	Interface LI_CPS	14

5.4.8	Interface LI_HI2	14

5.4.9	Interface LI_HI3	14

5.4.10	Interface LI_ADMF	14

5.4.11	Interface LI_MDF	15

6	Network Layer Based Interception	15

6.1	General	15

6.2	Network Layer based Points of Interception	15

6.3	5G	15

6.3.1	General	15

6.3.2	LI at AMF	17

6.3.2.1	Architecture	17

6.3.2.2	Target Identities	17

6.3.2.3	IRI Events	17

6.3.2.4	IRI Parameters	18

6.3.2.5	Parameters on each xIRI	18

6.3.2.6	Network Topologies	18

6.3.3	LI for SMF/UPF	18

6.3.3.1	Architecture	18

6.3.3.2	Target Identities	19

6.3.3.3	IRI Events	19

6.3.3.4	IRI Parameters	19

6.3.3.5	Parameters on each xIRI	20

6.3.3.6	Network Topologies	20

6.3.4	LI at UDM for 5G	20

6.3.5	LI at NRF	20

6.4	4G	21

6.5	3G	21

7	Service Layer Based Interception	21

7.1	General	21

7.2	Central Subscriber Database LI	21

7.2.1	General	21

7.2.2	LI at UDM	21

7.2.2.1	Architecture	21

7.2.2.2	Target Identities	22

7.2.2.3	IRI Events	22

7.2.2.4	IRI Parameters	22

7.2.2.5	Parameters on each xIRI	23

7.2.2.6	Network Topologies	23

7.2.3	LI at HSS	23

7.3	Location	23

7.3.1	General	23

7.3.2	Service Usage Location Reporting	23

7.3.3	Lawful Access Location Services (LALS)	23

7.3.3.1	General Architecture	23

7.3.3.2	Target Positioning	24

7.3.3.2.1	General	24

7.3.3.2.2	Immediate Location Provision	25

7.3.3.2.3	Periodic Location Provision	25

7.3.3.3	Enhanced Location for IRI	26

7.3.3.3.1	General	26

7.3.3.3.2	LALS Triggering Function	27

7.3.3.4	LI_X2-interface for Target Positioning and Enhanced Location	27

7.3.4	Cell Database Information Reporting	27

8	LI Security Considerations	27

8.1 Introduction	27

8.2	Architectural Alternatives	28

8.2.1	Full Target List at every POI Node	28

8.2.2	Full Target List only in LICF	28

8.2.3	Provisioning	28

8.2.3.1	Provisioning for Unregistered Users	28

8.2.3.2	Provisioning for Registered Users	28

Annex <A> (normative): <Normative annex title>	28

Annex <B> (informative): Implementation Considerations	29

B.1 General	29

B.2 Points of Interception	29

B.3	 5G LI Network Topology Views	30

B.3.1	Non-roaming scenario	30

B.3.1.1	General	30

B3.1.2	Service-based representation with point-to-point LI system	30

Annex Z (informative): Change history	32

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document has been produced by the 3GPP TSG SA to standardise Lawful Interception of telecommunications. This document describes the architecture and functions required to support Lawful Interception in 3GPP networks. Lawful Interception shall always be done in accordance with the applicable national or regional laws and technical regulations. Such national laws and regulations define the extent to which functional capacities in the present document are applicable in specific national laws.

# 1	Scope

The present document describes both the architectural and functional system requirements for Lawful Interception (LI) in 3GPP networks. The present document includes both LI architectures for network layer based and service layer based Interception.

The present document is inclusive of all LI architectural and functional capabilities required to support national Lawful Interception requirements globally, applicable to 3GPP networks. National regulatory requirements are responsible for defining the specific set of LI functional capabilities that are applicable to a specific 3GPP operator deployment. Editor’s Note: Initial focus on implementing 5G LI capabilities before importing existing 33.107 capabilities.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: " System Architecture for the 5G System".

[3]	3GPP TS 33.126: " Lawful Interception Requirements".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)”.

[6]	OMA MLP: "Mobile Location Protocol V3.3", [http://www.openmobilealliance.org].

[7]	ETSI TS 103 120: " Lawful Interception (LI); Interface for warrant information”.

[8]	ETSI TS 103 221-1: " Lawful Interception (LI); Part 1: Internal Network Interface X1 for Lawful Interception”.

Editor’s Note. References need to be check here and usage including 23.501 vs 23.502.

…

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

Editor’s Note: Import applicable definitions from 33.126 & 33.107.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

5GC	5G Core Network

5G-EIR	5G Equipment Identity Register

ADMF	LI Administration Function

AMF	Access Management Function

AUSF	Authentication Server Function

CC	Content of Communication

CPOI	Control plane Point Of Interception

CSP	Communication Service Provider

CUPS	Control and User Plane Separation

GPSI	Generic Public Subscription Identifier

IRI	Intercept Related Information

LADN	Local Area Data Network

LALS 	Lawful Access Location Services

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

LICF	Lawful Interception Control Function

LI_CPS	Lawful Interception CC interception rules Provisioning Signalling??

LI_HI1	LI_Handover Interface 1

LI_HI2	LI_Handover Interface 2

LI_HI3	LI_Handover Interface 3

LIPF	Lawful Interception Provisioning Function

LIR	Location Immediate Request

LI_SI	Lawful Interception System Information Interface

LI_X1	Lawful Interception Internal Interface 1

LI_X2	Lawful Interception Internal Interface 2

LI_X3	Lawful Interception Internal Interface 3

LMF	Location Management Function

LTF	Location Triggering Function

MDF	Mediation and Delivery Function

MDF2	Mediation and Delivery Function 2

MDF3	Mediation and Delivery Function 3

N3IWF	Non 3GPP Inter Working Function

NEF	Network Exposure Function

NPLI	Network Provided Location Information

NR	New Radio

NRF	Network Repository Function

NSSF	Network Slice Selection Function

PCF	Policy Control Function

PEI	Permanent Equipment Identifier

POI	Point Of Interception

UPF	User Plane Function

UPOI	User plane Point Of Interception

SIRF	System Information Retrieval Function

SMF	Session Management Function

SMSF	SMS-Function

SUPI	Subscriber Permanent Identifier

SX3LIF	Split X3 LI Interworking Function

TF	Topology Function

UDM	Unified Data Management

UDSF	Unstructured Data Storage Function

xCC	X3 Communications Content.

xIRI	X2 Intercept Related Information

Editor’s Note: Only add those abbreviations which are specifically needed and rely on 21.905 where possible.

# 4	Requirements Realisation

The LI architecture set out in the present document is designed to allow CSP deployments to meet the set of LI requirements described in TS 33.126 [3] that are determined to be applicable by the relevant national regulation for that deployment. For more details on the relationship between LI requirements and national legislation, see TS 33.126 [3] clause 4.

A CSP may deploy different network technologies or services considered in the present document. A CSP should consider each of these network technologies or services separately with respect to the present document, bearing in mind that a different subset of LI requirements may apply according to relevant national legislation, and that a warrant may require the CSP to intercept multiple network technologies or services.

# 5	Functional Architecture

Editor’s Note: This section shall include equivalent simplified text to both sections 4 and 5 of 33.107.

## 5.1	General

The following clauses describe the high-level architecture for LI for 3GPP-defined services and network technologies. It describes the architectural elements necessary for LI, their roles and responsibilities, and the interfaces and interactions between them.

Each 3GPP-defined service and network technology has its own specific configuration of the LI architecture. These are described in clauses 6 and 7. These clauses describe how the high-level LI architecture is configured and overlaid on to each service or network technology, including placement of LI architectural elements and which interfaces are used.  Not all LI architectural elements and interfaces are used in all network technologies and services.

## 5.2	High Level Generic LI Architecture

The overall conceptual view of LI architecture is shown in figure 5.2-1 below.

Figure 5.2-1: A high-level generic view of LI architecture

The functional entities of the architecture are described in more detail in clause 5.3 below. Details of the specific interfaces between these entities are described in clause 5.4.

## 5.3 	Functional Entities

### 5.3.1	Law Enforcement Agency (LEA)

The Law Enforcement Agency (LEA) is responsible for receiving the result of the interception sent by the MDF 2 or 3. In general the LEA is also responsible for submitting the warrant to the CSPs, although in some countries the warrant by be provided by a different legal entity (e.g. Judiciary).

### 5.3.2	Point of Interception (POI)

#### 5.3.2.1	General

The Point of Interception (POI) detects the target communication, derives the POI Output from the target communications and delivers the POI Output to the MDF2 or MDF3. A POI may be embedded within a Network Function (NF) or separate from a NF with which it is associated.

Multiple POIs may have to be involved in executing a warrant.

#### 5.3.2.2	Directly Provisioned and Triggered POIs

POIs can be divided into two categories according to how they receive intercept provisioning information

- Directly Provisioned POIs are provisioned by the LIPF directly.

- Triggered POIs are triggered by a Triggering Function (see clause 5.3.3) which is in turn provisioned by the LIPF.

#### 5.3.2.3	IRI-POIs and CC-POIs

POIs can be divided into two categories according to the type of intercepted data they send to the MDF (see clause 5.3.4)

- IRI-POI delivers xIRI to the MDF2

- CC-POI delivers xCC to the MDF3.

Both IRI-POIs and CC-POIs may be directly provisioned or triggered (see clause 5.3.2.2)

### 5.3.3	Triggering Function

The Triggering Function is responsible for triggering Triggered POIs in response to network and service events matching the criteria provisioned by the LIPF.

As a part of this triggering, the Trigger Function shall send all necessary interception rules (i.e. rules that allow the POIs to detect the target communications), forwarding rules (i.e. MDF2, MDF3 address), target identity, and the correlation information.

A Triggering Function may interact with other POIs to obtain correlation information. Details of this interface are not specified by the current document.

The Triggering Function that triggers CC-POI is referred to as a CC-TF and the Triggering Function that triggers an IRI-POI is referred to as IRI-TF.

### 5.3.4	Mediation and Delivery Function (MDF)

The Mediation and Delivery Function (MDF) delivers the Interception Product to the Law Enforcement Monitoring Facility (LEMF).

Two variations of MDF are defined – MDF2 and MDF3.

MDF2 generates the IRI messages from the xIRI and sends them to one or more LEMFs.  The MDF3 generates the CC from the xCC and delivers the same to one or more intercepting LEMFs. An overview of this is shown in figure 5.3-2 below.

Figure 5.3-2: MDF2 and MDF3

The MDF2 and MDF3 are provisioned by the LIPF with the intercept information necessary to deliver the IRI and CC to one or more LEMFs.

The LI_MDF interface between MDF2 and MDF3 (shown in figure 5.3-2) allows the MDF3 and MDF2 to exchange information between the two.

### 5.3.5 	Administrative Function (ADMF)

The Administration Function (ADMF), responsible for the overall management of LI system, includes the two logical functions:

- Lawful Intercept Control Function (LICF)

- Lawful Intercept Provisioning Function (LIPF).

The LICF controls the management of the end-to-end life cycle of a warrant. The LICF provides the intercept information derived from the warrant for provisioning at the POI, MDF2 and MDF3.

The LIPF provisions all the applicable POIs, MDF2 and MDF3.

### 5.3.6	System Information Retrieval Function (SIRF)

The System Information Retrieval Function (SIRF) is responsible for providing the LIPF with the system related information (e.g. topology) required to establish and maintain the interception.

### 5.3.7	LEMF – Law Enforcement Monitoring Facility

The Law Enforcement Monitoring Facility (LEMF) receives the Interception Product. The LEMF is out of scope of the present document.

## 5.4	LI Interfaces

### 5.4.1	General

An LI architecture diagram showing Point-to-Point LI interfaces is shown in figure 5.4-1 below.

Figure 5.4-1: Architecture diagram with Point-to-Point LI interfaces

### 5.4.2	Interface LI_SI

LI_SI is an interface between the SIRF and LIPF.  SIRF uses this interface to provide the system information to the LIPF.  The LIPF may request the SIRF for such information before sending the intercept provisioning information to the POIs. The SIRF may also notify the LIPF whenever the status of a system function changes (e.g. removed from service, migrating to another location, etc).

### 5.4.3	Interface LI_HI1

LI_HI1 is used to send warrant and other interception request information from the LEA to the CSP. This interface may be electronic or may be an offline manual process depending on national warrantry processes.

The following are some of the information included within the warrant sent over this interface:

- Target Identifier:  Used to identify the communications to be intercepted.

- Type of Intercept: Used to identify the scope of target communications to be delivered to the LEMF.

- LEMF Address: Used to deliver the Interception Product

- Lawful Interception Identifier: Used to associate the Interception Product with the issued warrant.

LI_HI1 interfaces shall support the use of ETSI TS 103 120 [7] for communication of warrant information between the LEA and CSP. However, default configurations, information element formats and other parameters as defined in the present document shall apply regardless of generic default options specified in ETSI TS 103 120 [7].

### 5.4.4	Interface LI_X1

#### 5.4.4.1	General

LI_X1 interfaces are used to configure LI target information on POIs and TFs in order to intercept target communications. LI_X1 interfaces are also used to configure MDFs with the necessary information to deliver those communications in the correct format to law enforcement.

LI_X1 interfaces shall support the use of ETSI TS 103 221-1 [8] for transport of X1 messages / information. However, default configurations, information element formats and other parameters as defined in the present document shall apply regardless of generic default options specified in TS 103 221-1 [8].

#### 5.4.4.2	LIPF and POI

The following are some of the information passed over LI_X1 to the POI as a part of intercept provisioning:

- Target Identifier

- Type of Intercept

- Address of MDF2 and MDF3.

The LI_X1 interface between LIPF (in the ADMF) and a Triggered POI shall be used only for audit and management purposes, and not for tasking purposes.

EDITOR’S NOTE: This section may require updating with respect to the inclusion of Triggering Functions

#### 5.4.4.3	LIPF and MDF2/MDF3

The following are some of the information passed over LI_X1 to the MDF2/MDF3 as a part of intercept provisioning:

- Target Identifier

- Lawful Interception Identifier

- Type of Intercept

- LEMF Address.

### 5.4.5	Interface LI_X2

The LI_X2 interfaces are used to pass xIRI from IRI-POIs to the MDF2.

The following are some of the information passed over this interface to the MDF2 as a part of xIRI:

- Target Identifier

- Time stamp

- Correlation Number

- IRI event resulting in xIRI details.

NOTE: 	Fully standardised definition of LI_X2 interface is not provided by the present document. Fully standardised interface will be supported in future versions once applicable ETSI TC LI_X2 specifications are completed.

### 5.4.6	Interface LI_X3

LI_X3 interfaces are used to pass teal-time content of communications (i.e. xCC) and associated metadata from CC-POIs to MDF3.

The following are some of the information passed over this interface to the MDF3 as a part of xCC:

- Target Identifier

- Time stamp

- Correlation Number

- User plane packets.

NOTE: 	Fully standardised definition of LI_X3 interface is not provided by the present document. Fully standardised interface will be supported in future versions once applicable ETSI TC LI_X3 specifications are completed.

### 5.4.7	Interface LI_T

#### 5.4.7.1	General

The LI_T interface is used to pass the triggering information from the Triggering Function to the POI. Depending on the POI type, LI_T can be classified as follows:

- LI_T2

- LI_T3.

LI_T2 is used when POI Output is sent over LI_X2 and LI_T3 is used when POI Output is sent over LI_X3.

#### 5.4.7.1	Interface LI_T2

The LI_T2 interface is from IRI-TF to IRI-POI.

The following are some of the information passed over this interface to the IRI-POI:

- Target Identifier

- IRI interception rules

- MDF2 address

- Correlation Information.

The IRI interception rules allow the IRI-POI to detect the target communication information to be intercepted.

#### 5.4.7.3	Interface LI_T3

LI_T3 interface is from CC-TF to CC-POI.

The following are some of the information passed over this interface to CC-POI:

- Target Identifier

- CC interception rules

- MDF3 address

- Correlation Information.

- The CC interception rules allow the CC-POI to detect the target communication information to be intercepted.

### 5.4.8	Interface LI_HI2

LI_HI2 is used to send IRI from the MDF2 to the LEMF.

### 5.4.9	Interface LI_HI3

LI_HI3 is used to send CC from the MDF3 to the LEMF.

### 5.4.10	Interface LI_ADMF

LI_ADMF is an interface between LICF and LIPF and is used by the LICF to send the intercept provisioning information to the LIPF. Further details about this interface is outside the scope of the present document.

### 5.4.11	Interface LI_MDF

LI_MDF is an interface between MDF2 and MDF3 and is used for MDF2 and MDF3 to interact with each other in the generation of IRI and CC.  Further details about this interface is outside the scope of the present document.

# 6	Network Layer Based Interception

Editor’s Note: This section will group together all PS and future edge, or other access based LI capabilities.

Editor’s Note: Support for CS LI using CS delivery will not be provided but CS support will be provided by reference to 33.107 and use of IP handover of CS to be define in this specification.

Editor’s Note: Access technology based capability should define access specific aspects (e.g. GPRS or LTE) in this section and then reference section 7 capabilities as appropriate (e.g. location, messaging).

Editor’s Note: Import and restructure appropriate 33.107 content.

## 6.1	General

## 6.2	Network Layer based Points of Interception

Editor’s Note: Master list of POI and applicable messages.

## 6.3	5G

Editor’s Note: To contain a mapping of POIs and messages in 6.2 that are specific to 5G.

### 6.3.1	General

Figure 6.3-1 depicts the 5G EPC anchored LI architecture. The network functions are depicted in grey, while the LI elements are depicted in blue.

Figure 6.3-1 5G EPC-Anchored LI Architecture

Figure 6.3-2 depicts the 5G Core anchored LI architecture. The network functions are depicted in grey, while the LI elements are depicted in blue.

Figure 6.3-2 5G Core-Anchored LI Architecture

### 6.3.2	LI at AMF

#### 6.3.2.1	Architecture

In the 5GC network, the AMF handles the access and mobility functions. The AMF shall have LI capabilities to generate the target UE’s network access, registration and connection management related xIRI.  Extending the generic LI architecture presented in clause 5, the figure 6.3-3 below gives a reference point representation of the LI architecture with AMF as a CP NF providing the IRI-POI functions.

Figure 6.3-3: LI Architecture for LI at AMF

The LICF present in the ADMF receives the warrant from an LEA, derives the intercept information from the warrant and provides the same to the LIPF.

The LIPF present in the ADMF provisions the IRI-POI (over LI_X1) present in the AMF and the MDF2.  The LIPF may interact with the SIRF (over LI_SI) present in the NRF to discover the AMFs in the network.

The IRI-POI present in the AMF detects the target UE’s access and mobility related functions (network access, registration and connection management), generates and delivers the xIRI to the MDF2 over LI_X2.  The MDF2 delivers the IRI messages as part of the Interception Product to the LEMF over LI_HI2.

#### 6.3.2.2	Target Identities

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the AMF:

- SUPI

- PEI

- MSISDN.

Editor’s Note: May need to change MSISDN to GPSI.

The interception performed on the above three identities are mutually independent, even though, an xIRI may contain the information about the other identities when available.

#### 6.3.2.3	IRI Events

The IRI-POI present in the AMF shall generate xIRI, when the AMF detects the following specific events or information.

Editor’s Note: Text will have to be drafted to this sub-clause and the new sub-clauses.

#### 6.3.2.4	IRI Parameters

A combined list of parameters that are included in the xIRI generated by the IRI-POI present in the AMF are shown below.

Editor’s Note: Text will have to be drafted to this sub-clause.

#### 6.3.2.5	Parameters on each xIRI

Editor’s Note: Text will have to be drafted to this sub-clause and new sub-clauses.

#### 6.3.2.6	Network Topologies

The AMF shall provide the IRI-POI functions in the following network topology cases:

- Non-roaming case

- Roaming case, in VPLMN

- Roaming case, in HPLMN for non-3GPP access.

### 6.3.3	LI for SMF/UPF

#### 6.3.3.1	Architecture

In the 5GC network, user plane functions are separated from the control plane functions. The SMF that handles control plane actions (e.g. establishing, modifying, deleting) for the PDU sessions shall have the LI capabilities to generate the related xIRI. The UPF that handles the user plane data shall have the capabilities to duplicate the user-plane packets from the PDU sessions based on the interception rules received from the SMF.  The figure 6.3-4 shows the LI architecture for SMF/UPF based interception..

Figure 6.3-4: LI Architecture showing LI at SMF/UPF

The LICF present in the ADMF receives the warrant from an LEA, derives the intercept information from the warrant and provides the same to the LIPF.

The LIPF present in the ADMF provisions IRI-POI (present in the SMF), MDF2 and MDF3 over the LI_X1 interfaces. To enable the interception of the target’s user plane packets (e.g. when the warrant requires the interception of communication contents), the CC-TF present in the SMF is also considered to be provisioned with the intercept data.

NOTE 1:	The IRI-POI and CC-TF represented in the figure 6.3-4 are logical functions, require a close coupling between the two and as such may be handled by the same process within the SMF.

The LIPF may interact with the SIRF (over LI_SI) present in the NRF to discover the SMFs and MDFs in the network.

The IRI-POI present in the SMF detects the PDU session establishment, modification, and deletion related events, generates and delivers the related xIRI to the MDF2 over LI_X2.  The MDF2 delivers the IRI messages to the LEMF over LI_H2.

When interception of communication contents is required, the CC-TF present in the SMF sends a trigger to the CC-POI  present in the UPF over the LI_T3 interface which can be based on N4 functionalities (between SMF and UPF) with LI specific security measures applied.

The trigger sent from the CC-TF to CC-POI includes the following information:

- User plane packet detection rules

- Target identity

- Correlation Number

- MDF3 address.

NOTE 2:	When LI_T3 is used, the LI-X1 between LIPF and CC-POI present in the UPF is used  to monitor the user plane data.

The CC-POI present in the UPF generates the xCC from the user plane packets, and delivers the xCC (that includes the correlation number and the target identity) to the MDF3. The MDF3 delivers the CC to the LEMF over LI_HI3.

A warrant that does not require the interception of communication contents, may require IRI messages that have to be derived from the user plane packets. To support the generation of related xIRI (i.e. that requires access to the user plane packets), the present document supports two implementation approaches:

- In approach 1, the IRI-POI responsible for the generation of such xIRI resides in the UPF. Such an IRI-POI requires a trigger to enable it to detect the user plane packets. The corresponding Triggering Function (IRI-TF) resides in the same SMF that has the IRI-POI for the other xIRI.

NOTE 3: The IRI-POI and IRI-TF present in the SMF may be handled by the same process in the SMF.

The trigger sent by the IRI-TF (present in the SMF) to the IRI-POI (present in the UPF) includes the following:

- User plane packet detection rules

- Target identity

- Correlation Number

- MDF2 address.

The IRI-POI present in the UPF generates the xIRI (that includes the correlation number and the target identity) from the user plane packets and send the same to the MDF2. The MDF2 generates the IRI messages and send the same to the LEMF.

- In approach 2, xCC is generated by the CC-POI present in the UPF as if the warrant involves the interception of communication contents. To enable this, the CC-TF presumed to be present in the SMF even when the warrant does not require the interception of communication contents. As explained before, the CC-POI generates the xCC and send the same to the MDF3. The MDF3 (based on the provisioned intercept information) does not generate and deliver the CC to the LEMF. Instead, the MDF3 forwards the xCC to the MDF2 over LI_MDF interface. The MDF2 then generates the IRI messages from xCC and delivers those IRI messages to the LEMF.

NOTE 4: 	When multiple warrants are active on a target with one requiring the interception of communication contents and the other not (in other words, this other one requiring xIRI from user plane packets), the first approach requires the UPF to have both CC-POI and IRI-POI and the SMF to have IRI-POI, IRI-TF and CC-TF.  Alternatively, the interception of communication contents is required anyway for one warrant, and hence, the second approach will become simpler and therefore, may be preferable.

NOTE 5: Directly provisioned CC-POI is not considered in the present document.

#### 6.3.3.2	Target Identities

The LICF provisions the intercept related information associated with the following target identities to the IRI-POI present in the SMF:

- SUPI

- PEI

- MSISDN.

The interception performed on the above three identities are mutually independent, even though, an xIRI may contain the information about the other identities when available.

#### 6.3.3.3	IRI Events

The IRI-POI present in the SMF shall generate xIRI, when the SMF detects the following specific events or information.

.

Editor’s Note: Text will have to be drafted to this sub-clause and the new sub-clauses.

#### 6.3.3.4	IRI Parameters

A combined list of parameters that are included in the xIRI generated by the IRI-POI present in the SMF are shown below.

Editor’s Note: Text will have to be drafted to this sub-clause.

#### 6.3.3.5	Parameters on each xIRI

Editor’s Note: Text will have to be drafted to this sub-clause and new sub-clauses.

#### 6.3.3.6	Network Topologies

The SMF shall provide the IRI-POI functions in the following network topology cases:

- Non-roaming case

- Roaming case, in VPLMN

- Roaming case, in HPLMN.

### 6.3.4	LI at UDM for 5G

In 5G packet core network, the UDM provides the unified data management for UE. The UDM shall have LI capabilities to generate the target UE’s service area registration related xIRI. See clause 7.2.2 for the details.

### 6.3.5	LI at NRF

In 5G, the network functions that may have virtualized deployment, register with the NRF upon instantiation. The NRF thus provides the network repository functions and is aware of all the NFs that have been instantiated. The present document refers to this as system information.

The SIRF present in the NRF provides the system information to LIPF present in the ADMF.

An architecture diagram depicting this LI at NRF is shown in figure 6.3.5-1 below.

Figure 6.3-5: LI Architecture depicting NRF as an SIRF

The figure 6.3-5 shows the architecture illustrating the SIRF functions within the NRF

The LIPF present in the ADMF interacts with the SIRF (over LI_SI) present in the NRF to obtain the system information.

### 6.3.6	External Data Storage

The UDSF or UDR as defined in 23.501 [2] are used to external store data relating to one or more NFs, separating the compute and storage elements of an NF. Where the NF contains a POI the following restrictions on the use of the UDSF/UDR shall apply;

- The UDSF/UDR shall be subject to the same location, geographic, security and other physical environment constraints as the NF POI for which it is storing data.

- No LI specific POI data (e.g. target list) shall be stored in the UDSF/UDR unless storage is directly under the control of the POI within the NF.

- LI data stored in a UDSF/UDR shall only be accessible by the specific individual POI for which the UDSF/UDR is storing data and that data shall not be shared between POIs unless specifically authorised by the ADMF.

- By default, LI data shall not be stored in a UDSF/UDR which is shared by multiple NFs unless specifically authorised by the ADMF.

- Any storage of LI data outside of the POI in the UDSF/UDR must be auditable by the ADMF.

- The interface between the POI/NF and the UDSF/UDR must be protected such that an attacker cannot identify targeted users based on observation of this interface. (i.e. access to the UDSF/UDR must be identical for both intercepted and non-intercepted user communications).

- The use and placement of a UDSF/UDR within an NF/POI design shall not introduce additional interception delay compared with non-separated compute and storage.

- Where the POI requires access to NF data that is stored in the UDSF/UDR, non-LI network functions and processes or non-LI authorised personnel shall not be able to detect POI access to that data in the UDSF/UDR.

- The POI and ADMF/MDF shall be responsible for managing encryption of LI data stored for the POI in addition to any default encryption applied by the NF.

## 6.4	4G

Editor’s Note: To contain a mapping of POIs and messages in 6.2 that are specific to 4G.

## 6.5	3G

Editor’s Note: To contain a mapping of POIs and messages in 6.2 that are specific to 3G.

# 7	Service Layer Based Interception

Editor’s Note: This section will contain service specific LI capabilities (e.g. IMS, VoLTE, LCLS, Location, HSS/HLR etc).

Editor’s Note: Service specific LI capability should define service specific aspects in this section and then reference section 6 capabilities as appropriate.

Editor’s Note: Import and restructure appropriate 33.107 content.

## 7.1	General

## 7.2	Central Subscriber Database LI

### 7.2.1	General

Editor’s Note: Need intro text linking to HSS / UDM reporting, serving system and other central cross service reporting.

### 7.2.2	LI at UDM

#### 7.2.2.1	Architecture

The UDM provides the unified data management for UE. The UDM shall have LI capabilities to generate the target UE’s registration area related xIRI.  Extending the generic LI architecture presented in clause 5, the figure 7.2-1 below gives a reference point representation the LI architecture with UDM as a CP NF providing the IRI-POI functions.

Figure 7.2-1: LI Architecture for LI at UDM

The LICF present in the ADMF receives the warrant from an LEA, derives the intercept information from the warrant and provides the same to the LIPF.

The LIPF present in the ADMF provisions IRI-POI (over LI_X1) present in the UDM and MDF2.  The LIPF may interact with the SIRF (over LI_SI) present in the NRF to discover the UDM in the network.

The IRI-POI present in the UDM detects the target UE’s service area registration and subscription related functions, generates and delivers the xIRI to the MDF2 over LI_X2.  The MDF2 delivers the IRI messages to the LEMF over LI_H2.

#### 7.2.2.2	Target Identities

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the UDM:

- SUPI

- PEI

- MSISDN

- SIP URI/TEL URI.

The interception performed on the above identities are mutually independent, even though, an xIRI may contain the information about the other identities when available.

#### 7.2.2.3	IRI Events

The IRI-POI present in the UDM shall generate xIRI, when the UDM detects the following specific events or information.

.

Editor’s Note: Text will have to be drafted to this sub-clause and the new sub-clauses.

#### 7.2.2.4	IRI Parameters

A combined list of parameters that are included in the xIRI generated by the IRI-POI present in the UDM are shown below.

Editor’s Note: Text will have to be drafted to this sub-clause.

#### 7.2.2.5	Parameters on each xIRI

Editor’s Note: Text will have to be drafted to this sub-clause and new sub-clauses.

#### 7.2.2.6	Network Topologies

The UDM shall provide the IRI-POI functions in the following network topology cases:

- Non-roaming case

- Roaming case, in HPLMN.

### 7.2.3	LI at HSS

Editor’s Note: Place holder for HSS LI capabilities.

## 7.3	Location

### 7.3.1	General

Editor’s Note: Intro text required to describe all location for all services / access technologies.

### 7.3.2	Service Usage Location Reporting

Editor’s Note: May need a better title.

Editor’s Note: Section to contain description of general communications related location reporting.

### 7.3.3	Lawful Access Location Services (LALS)

#### 7.3.3.1	General Architecture

LALS provides lawful access to the target's location using the Location Services (LCS) capabilities defined for 5G, and 4G access:

- in the clause 4.4.4  Location services of TS 23.501 [2],

- in the TS 23.271 [5] and OMA MLP TS [6] for  access such as UTRAN, for GERAN, for E-UTRAN and for non 3GPP access.

The present clause details architecture and functions that are needed to provide the LALS information to the MDF 2 for a target of interception for subsequent delivery to the LEMF.  LCS used by LALS shall meet Clauses 6.6 Security, and 6.3 Detect and Capture of TS 33.126 [3] at least and provide priority to Lawful Access Location Services (LALS) requests.

The subscriber location privacy settings shall be overridden see clause 9 of TS 23 271[5].

For roaming targets, both inbound and outbound, it is assumed that LALS accessing VPLMN GMLC avoids accessing HPLMN LCS elements (such as H-GMLC) to address potential detectability issues.

Depending on national requirements and LCS capabilities used by LALS of the network operator, the location information provided by LALS may vary in location information types (mobile network location format (including new 5G cell identities formats), location shape and geo-coordinates, civic address, or a combination of those), in the set of additional location parameters (map data, motion state, speed, etc.), as well as in the accuracy of provided location information.

As required by the R6.3 – 280, R6.3 – 380 and – 390 of TS 33.126[3], the output provided through the 5G Location Management Function (LMF) Services (see 5.2.15 LMF Services of TS 23501 [2]) to the LALS shall be Success/Failure indication. On top of when the location information was determined, It may be completed as option Geodetic Location, Civic Location, Position Methods Used, Failure Cause (See steps 6 and 8 of clause 4.13.5.3 of TS 23.502[4] for example of usage of this service operation), the LALS Correlation identifier, the location estimate, its age (R6.3 – 280 of TS 33.126[3]),  accuracy (R6.3 – 240 of TS 33.126 [3]) and may include information about the positioning method (R6.3 – 26 of TS 33.126[3] and from 4.13.5 Location Services procedures of TS 23.502[4]) . The LMF may access information from E UTRAN.

NOTE 1:	The accuracy of positioning for any particular technology is a trade-off for the location acquisition delay. It also depends on other technology specific factors.

The parameters controlling the LALS output are either delivered per warrant over LI_HI1/LI_X1 interface or pre-configured in the LI-LCS client.

There are two types of the location interception defined in the present specification: the Target Positioning and the Enhanced Location for IRI.

The Target Positioning is used to determine the target's location independently of the services used by the target.

The Enhanced Location for IRI is used to determine the LALS based location of the target when specific user service events related to the target occur.

The warrant for Target Positioning and for Enhanced Location for the same target may be independent of each other and may be overlapping in time or combined in a single intercept warrant by LEA.

There may be multiple active LALS warrants from different LEAs at any given time.

#### 7.3.3.2	Target Positioning

##### 7.3.3.2.1	General

As required by the R6.3 – 370 of TS 33.126, the provision variants supported in the current specification are the Immediate Location and the Periodic Location.

Figure 7.3-1 shows the architecture for the LALS where the LI LCS Client provides the target’s location and associated information towards the MDF 2 over the LI_X2 interface fulfilling the Target Positioning ADMF authorization delivered over LI_X1 interface.

Figure 7.3-1: LALS Model for Target Positioning

NOTE:	The Le interface is specified in 4.4.4.2 Reference point to support Location Services of TS 23.502[4] and in the 5.1 LCS access interfaces of TS 23 271[5]. It is completed by the reference points NLg and NLs, realized by service based interfaces (see TS 23.502 [4]).

##### 7.3.3.2.2	Immediate Location Provision

The warrant for Immediate Location provision is delivered to LI LCS Client over LI_X1 interface. Upon receiving the warrant the LI LCS Client initiates a request (see TS 23.502[4]) or Location Immediate Request (LIR, see TS 23.271 [5]) with the LCS Server/GMLC supporting LALS over Le interface and reports the acquired location to the MDF 2 over LI_X2.

During the period of active warrant for Immediate Location the LI LCS client may receive and process additional Immediate Location requests from ADMF over the LI_X1.

NOTE:	The LCS Server/GMLC supporting LALS may be optimized to provide the same single location estimation in response to multiple positioning requests arriving in temporal proximity of each other.

The resulting Immediate Location intercept product is delivered over LI_X2 to the MDF2 and propagated to the LEMF over LI_HI2.

##### 7.3.3.2.3	Periodic Location Provision

The warrant for Periodic Location provision is delivered to LI LCS Client over LI_X1 interface.

During the Periodic Location warrant the LI LCS Client shall produce the LALS Reports with the specified periodicity.

The periodicity shall be controlled by the LI LCS Client. The LI LCS Client shall issue a series of request (see TS 23.502[4]) or Location Immediate Requests (LIR, see TS 23.271 [5]) at required time intervals.

The LI LCS Client provides the acquired location reports to the MDF2 over LI_X2.

The Request for Periodic Location from ADMF to LI LCS Client may be accompanied by a set of parameters defining the time interval for reporting, report periodicity, etc. The description of the service response parameters is provided in clause 7.3.3.4. The Periodic Location intercept product is delivered over LI_X2 to the MDF2 and propagated to LEMF over LI_HI2.

#### 7.3.3.3	Enhanced Location for IRI

##### 7.3.3.3.1	General

The Enhanced Location for IRI refers to a capability providing LALS based location information when specific user service events related to the target of interception occur. An example of such service events are the events of IMS session initiation and termination.

Figure 7.3-2 depicts the architecture of Enhanced Location acquisition and delivery for the case when the Location Triggering Function (LTF) is associated with an CPOI.

Figure 7.3-2: LALS Model for Enhanced Location for IRI (CPOI/LTF option)

Figure 7.3-3 depicts the architecture of Enhanced Location acquisition and delivery for the case when the LTF is associated with a MDF2.

Figure 7.3-3: LALS Model for Enhanced Location for IRI (MDF/LTF option)

##### 7.3.3.3.2	LALS Triggering Function

The LALS Enhanced Location architecture in Figures 7.3-2 and 7.3-3 depicts the LALS Triggering Function (LTF). LTF is associated with an CPOI or with a MDF2 and is responsible for triggering the LI LCS Client when a specific event related to the target is observed at the CPOI, or received at the MDF2.

The request for Enhanced Location reporting for IRI is delivered from ADMF to either a CPOI over LI_X1 or to a MDF2 over LI_X1 interface along with other parameters of IRI intercept authorization/activation. The CPOI (s) or the MDF2 then arm the LTF(s).

The POI nodes that may have an associated LTF include AMF, AUSF, P/S-CSCF, PCF, IMS AS, HSS, MME, NSSF, SMF, SMSF, S/GGSN, P/S-GW, UPF, UDM.

The LTF triggers the LI LCS Client over the LALS_T interface.

The LALS intercept product is delivered to MDF2 from the LI LCS client over LI_X2 interface asynchronously with the associated xIRI reports generated by a CPOI. To enable correlation between the LALS Reports and the associated xIRI the LTF shall include the Correlation Identifier from the xIRI, if available, into the LALS_T trigger.

NOTE 1:	The xIRI may contain the location information obtained by other means, e.g. NPLI. The LALS reports are augmenting that information with extra details and accuracy.

The LALS_T interface for the LALS intercept trigger shall adhere to the security requirements outlined in Clause 8.

NOTE 2:	Detailed definition of the LALS_T interface is out of scope of the current specification.

#### 7.3.3.4	LI_X2-interface for Target Positioning and Enhanced Location

The following information needs to be transferred from the LI LCS Client to the MDF2 in order to allow a MDF2 to perform its functionality:

-	target identity;

-	other target identities, if available;

-	event date/time;

-	target location and extended location parameters, if available;

-	correlation information (in case of Extended Location for IRI reporting);

- 	error code, if the positioning fails.

### 7.3.4	Cell Database Information Reporting

Editor’s Note: Placeholder for Cell Site database reporting.

# 8	LI Security Considerations

## 8.1 Introduction

The most sensitive information in the LI system is the target list. This is the list of all the subjects in the network currently under surveillance, whether active, suspended or in any other state.  The security measures used by the carrier to ensure unauthorized access to this list is not subject to standardization, but the architectural choices made in the design of the LI system do impact the security of the target list directly.

Since completeness of interception product is a legal requirement in most jurisdictions, the LI system must ensure that no events that are lawfully authorized for interception are missed. To ensure that no events are missed there are two architectural alternatives.

## 8.2	Architectural Alternatives

### 8.2.1	Full Target List at every POI Node

A carrier may choose to deploy the full target list at all POIs, such that when a UE arrives in the network and commences registration, the POI is fully armed and in position to recognize if the target identifier is in the target list. The choice to push the full list to every node is the simplest, and arguably the riskiest, since the compromise of any node will leak the complete target list.

### 8.2.2	Full Target List only in LICF

A Communication Service Provider (CSP) may choose to selectively distribute specific target identifiers to specific POIs, rather than distributing the full target list to all POIs. This choice introduces a race condition. When the UE appears, the POI must query the ADMF/LICF to find out if the user identifier is part of the target list. As the registration sequence progresses, the NF POI is waiting for a response from the ADMF/LICF.  When the reply arrives, the POI can now take action if the reply was positive. If the reply is negative, the POI’s involvement ends.

If the reply is positive, depending on how long the POI-(ADMF/LICF)-POI round trip for the query/reply took, it is possible that some reportable events are missed.  To mitigate this there are two further alternatives:

- the carrier may choose to delay completion of the registration for all users for the time it takes the ADMF/LICF to answer, thus inducing a registration delay in all registrations, whether the user is a target or not, or

- the carrier may choose to cache the reportable registration events while the POI-(ADMF/LICF)-POI query is running, and either report them if the answer is positive, or delete them if the answer is negative.

These are choices at the discretion of the CSP, but the trade-off cannot be avoided.

Editor’s note: Choose better place (annex) to put detailed implementation choices.

### 8.2.3	Provisioning

Editor’s Note: Need to split out provisioning for both approaches.

#### 8.2.3.1	Provisioning for Unregistered Users

#### 8.2.3.2	Provisioning for Registered Users

When a new target is provisioned in the LI system, after the target is already registered in the CSP network, the CSP will be faced with the race condition consequences of the implementation choice made as described in the previous clauses. The ADMF/LICF has a choice to either wholesale pre-arm every POI with the new target (and expect every POI to immediately start interception on the new target, as in clause 8.1.1), or, the ADMF/LICF can poll every serving UDM POI for all target UEs, and arm the associated POI (and start interception, as in clause 8.1.2) only if a target UE is discovered to be served by that particular NF. The second approach would take comparatively longer and would be expected to miss more of the pre-existing target interactions with the network than the first approach.

Annex <A> (normative):
<Normative annex title>

Annexes are only to be used where appropriate:

Editor’s Note; Consider a new TS with informative annexes that contains the background guidance annexes in 33.107/108 rather than adding to 33.127/128.

Annex <B> (informative): Implementation Considerations

## B.1 General

Editor’s Note: Annex used to group implementation consideration text. May need to split and re-order sub-sections into other Annexes.

## B.2 Points of Interception

Communication Service Providers (CSPs) use a wide range of 3GPP Network Functions (NFs) to provide services to users. In order to intercept a service, Points Of Interception (POIs) are associated with specific NFs, as depicted in Figure B.2-1. The manner in which the POI obtains the required information from the NF depends on the service and can range from something as simple as a copy-and-forward mechanism, to sophisticated isolation and filtering. The present document describes implementation of the POI based on two basic approaches: POI embedded in the NF (depicted on the left) or external to the NF (depicted on the right), connected to its interfaces. The choice of one, the other, or both approaches is service specific. In the figures that follow the POI will be depicted straddling the edge of the NF to simultaneously indicate both approaches.

Figure B.2-1. Embedded vs. External POIs

Figure B.2-2 shows the basic job of a POI: to obtain the state of the intercepted service. As the NF changes state in the course of providing the service, the appropriate interceptable events or real-time content are transferred into the POI.

Figure B.2-2. POI State Capture

Although the POI has access to state data in the NF, the converse is prohibited, for obvious security reasons, as depicted in Figure B.2-3. If the POI is embedded, the implementation must prohibit LI state leakage back into the non-secure area of the NF. The same security principle applies if the POI is an external to the NF.

Figure B.2-3. POI State Capture Security

Generally, embedded POIs have full access to the state machine of the service they intercept, while external POIs have to infer the state of the intercepted service from the events detected on the interfaces or externally applied traffic filtering criteria.

Figure B.2-4 depicts a particular wrinkle that is introduced by the practice of storing state outside the NF itself, such as in a UDSF in a 5G network. Typically, this mechanism is used to facilitate migration or scale-out events.

Figure B.2-4. State Stored Externally to the NF

If interception is started during a session for which the relevant state is in the UDSF, but not in the NF, and the NF chooses to retrieve the state from the UDSF for no other reason than LI being started, particular attention has to be given to this event becoming detectable.

### B.3		5G LI Network Topology Views

Editors Note: Consider moving this to another annex and combine with similar content.

### B.3.1	Non-roaming scenario

#### B.3.1.1	General

In a non-roaming scenario, the POIs present in the following NFs provide the LI functions.

- AMF

- UDM

- SMF

- UPF

For the interception of PDU sessions, the EPC CUPS LI model is not extended to 5G where SMF and UPF are involved in delivering the xIRI and CC associated with the PDU sessions.

NOTE: 	The above list of NFs that provide the POI functions may have to be expanded once a deployment scenario for such a case is defined in the normative part of the present document.

#### B3.1.2	Service-based representation with point-to-point LI system

The overall network configuration for 5G in a non-roaming scenario with the LI aspects is shown in figure B.3.1-1 using the service-based representation (as shown in TS 23.501) with the use of point-to-point LI system.

Figure B.3-1: Network topology showing LI for 5G (service-based representation) with point-to-point LI system

The figure B.3-1 shows the network topology of 5G system in a service-based representation, however, all the LI-related interfaces remain to be point-to-point.

The IRI-POIs present in the AMF, UDM and SMF deliver the xIRI to the MDF2 and CC-POI present in the UPF delivers the xCC to the MDF3. The MDF3 address to CC-POI present in UPF is provided by the CC-TF present in the SMF over LI_N4 reference point.

The LIPF present in the ADMF provisions the IRI-POIs present in the NFs with the intercept related data. The LI_X1 interfaces between the LIPF and the UPF is to monitor the user plane data.

Annex Z (informative):
Change history

This is the last annex for TSs which details the change history using the following table.

This table can be used for recording progress during the WG drafting process till TSG approval of this TR.

For TRs under change control, use one line per approved Change Request

Date: use format YYYY-MM

TSG # : use format RAN#55

CR: four digits, leading zeros as necessary

Rev: blank, or number (max two digits)

Cat: use one of the letters A, B, C, D, F

Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover

New vers: use format n[n].n[n].n[n]

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-02 | SA3LI#68 | S3i180055 | - | - | - | First Skeleton Specification Draft with Scope | 0.0.0 |
| 2018-02 | SA3LI#68 | S3i180071 | - | - | - | Output of SA3LI#68 | 0.0.1 |
| 2018-05 | SA3LI#69bis | S3i180219 | - | - | - | Output of SA3LI#69bis | 0.0.2 |
| 2018-07 | SA3LI#70 | - | - | - | - | Agreed docs from SA3LI#70 plus Rapporteur structural changes required to implement agreed tdocs. | 0.0.3 |
| 2018-07 | SA3LI#70 | - | - | - | - | Additional editorial improvements throughout document. | 0.0.3a |
| 2018-08 | - | - | - | - | - | Further review comments from SA3-LI email list as input to conf call 15th Aug 2018. | 0.0.3b |
| 2018-08 |  |  |  |  |  | Further review comments from SA3-LI email list as input to conf call 15th Aug 2018. | 0.0.3c |
| 2018-08 |  |  |  |  |  | Output from SA3-LI 5G Conf Call 15th August 2018 | 0.0.3d |
| 2018-08 |  |  |  |  |  | Further review comments to version 003d | 0.0.3e |
| 2018-09 | 5G Conf Call 310818 |  |  |  |  | Implementing agreement from 5G conference call 31st August 2018, s3i180403,404,405,407 and some agreements from 416. Terms xIRI and xCC used. | 0.0.4 |
| 2018-09 |  |  |  |  |  | Editorial Review comments against v0.0.4 | 0.0.4a |
| 2018-09 | 5G Conf Call 180918 | - | - | - | - | Implementing agreement from 5G conference call 18th September 2018, s3i180411, 431, 425, 426, 427 and 428, with amendments as agreed on the call. | 0.0.5 |