ETSI TS 103 221-? V0.0.1 (2018-07)

Lawful Interception;

Internal Network Interface X2/X3

The TS (ETSI Technical Specification) is the preferred deliverable when the document contains normative provisions and short time to "market", validation and maintenance are essential.

A TS may later be converted to an ES or an EN, or be used to publish the contents of a draft ES being sent for vote or a draft EN being sent for Public Enquiry or vote.

The guidance text (green) shall be removed when no longer needed or 
the skeleton without guidance text also available via the editHelp! website should be used.

<

TECHNICAL SPECIFICATION

Reference

<DTS/LI-00104-2>

Keywords

INTERFACE, Lawful Interception

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-préfecture de Grasse (06) N° 7803/88

Important notice

The present document can be downloaded from:
http://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the only prevailing document is the print of the Portable Document Format (PDF) version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx.

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.

The content of the PDF version shall not be modified without the written authorization of ETSI.

The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute yyyy.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM and the ETSI logo are Trade Marks of ETSI registered for the benefit of its Members.
3GPPTM and LTE™ are Trade Marks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
GSM® and the GSM logo are Trade Marks registered and owned by the GSM Association.

Reproduction is only permitted for the purpose of standardization work undertaken within ETSI.
The copyright and the foregoing restriction extend to reproduction in all media.

Contents

Intellectual Property Rights.	6

Foreword	6

Modal verbs terminology	6

1	Scope	7

2	References	8

2.1	Normative references	8

2.2	Informative references	8

3	Definitions, symbols and abbreviations	9

3.1	Definitions	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Introduction and reference model	9

4.1	Reference model	9

4.2	Assumptions	10

4.2.1	Architecture	10

4.2.2	Implementation / realisation	10

4.2.3	Deployment infrastructure	11

4.2.4	Regulatory assumptions	11

4.3	Other standards in the X1, X2, X3 series	11

5	Message contents and parameters	11

5.1	Overview	11

5.2	PDU Header Fields	12

5.2.1	Version	12

5.2.2	PDU Type	12

5.2.3	Header Length	12

5.2.4	Payload Length	12

5.2.5	Payload Format	13

5.2.6	Direction	13

5.2.7	XID	13

5.2.8	Correlation ID	13

5.3	Conditional attribute fields	13

5.3.1	General structure	13

5.3.2	Domain ID (DID)	14

5.3.3	Network Function ID (NFID)	14

5.3.3	Interception Point ID (IPID)	14

5.3.4	Sequence Number	14

5.3.5	Timestamp	14

5.4	Payload	14

5.4.1	Overview	14

5.4.2	SDO Defined Payload	15

5.4.3	Proprietary Payload	15

5.4.4	IPv4 Packet	15

5.4.5	IPv6 Packet	15

5.4.6	Ethernet Frame Packet	15

5.4.7	RTP Packet	15

5.4.8 	SIP Message	15

5.4.9	DHCP Message	15

5.4.10	RADIUS Message	15

5.4.11	GTP-U Frame	16

6	Transport	16

6.1 Summary	16

6.2 Profile	16

6.3 Key generation, deployment and storage	16

6.4 Authentication	16

Annex A (normative): Requirements 	17

A.1 	X2 Protocol & Architecture requirements	17

A.1.1	Basic Functionality	17

A.1.2	Flexible	17

A.1.3	Extensible	17

A.1.4	Lightweight	17

A.1.5	Delay	17

A.1.6	Permanent and Dynamic Connections	17

A.1.7	Reliability	17

A.1.8	Error detection	17

A.1.9	Redundancy	17

A.1.10	Correlation	17

A.1.11	Mediation into HI2 / HI3	17

A.2 	X2 Security requirements	18

A.2.1	Authentication & Authorisation	18

A.2.2	Accounting / Audit	18

A.2.3	Integrity Protection	18

A.2.4	Confidentiality Protection	18

A.2.5	Replay Protection	18

A.2.6	Standalone interface	18

A.2.7	Minimum Security Level	18

A.2.8	Underlying Infrastructure Trust	18

A.2.9	Firewall and NAT Transversal	18

A.2.10	Certificate and Key Management	18

A.3 	X3 Protocol & Architecture requirements	19

A.3.1	Basic Functionality	19

A.3.2	Flexible	19

A.3.3	Extensible	19

A.3.4	Lightweight	19

A.3.5	Delay	19

A.3.6	Permanent and Dynamic Connections	19

A.3.7	Reliability	19

A.3.8	Error detection	19

A.3.9	Redundancy	19

A.3.10	Correlation	19

A.3.11	Mediation into HI2 / HI3	19

A.4 	X3 Security requirements	20

A.4.1	Authentication & Authorisation	20

A.4.2	Accounting / Audit	20

A.4.3	Integrity Protection	20

A.4.4	Confidentiality Protection	20

A.4.5	Replay Protection	20

A.4.6	Standalone interface	20

A.4.7	Minimum Security Level	20

A.4.8	Underlying Infrastructure Trust	20

A.4.9	Firewall and NAT Transversal	20

A.4.10	Certificate and Key Management	20

Annex B (informative): Illustrative deployment scenarios	21

B.1 Introduction	21

B.2 Simple deployment scenario	21

B.3 Individual X3 POIs with shared X2 POI	21

B.3 Separated interfaces	22

Annex Z (informative)	23

Change History	23

# Intellectual Property Rights.

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

# Foreword

This Technical Specification (TS) has been produced by ETSI Technical Committee for Lawful Interception (LI).

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# 1	Scope

The present document defines an electronic interface for the transmission of intercepted information as part of Lawful Interception. This interface is used from points of interception to LI mediation functions.

Typical reference models for LI define an interface between law enforcement agencies (LEAs) and communication service providers (CSPs), called the handover interface. They also define an internal network interface within the CSP domain between administration / mediation functions for lawful interception and network internal functions, which facilitates the interception of communication. This internal network interface typically consists of three sub-interfaces; administration (called X1), transmission of intercept related information (X2) and transmission of content of communication (X3). The present document specifies a protocol for delivering X2 and X3.

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are necessary for the application of the present document.

[1]		ETSI TS 103 221-1: "Lawful interception internal interface X1"

[2]		ETSI TS 102 232-1: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 1: Handover specification for IP delivery".

[3]	IEEE Std 1003.1-2008: "IEEE Standard for Information Technology - Portable Operating System Interface (POSIX)"

[4]	IETF RFC 791: "Internet Protocol"

[5]	IETF RFC 8200: "Internet Protocol, Version 6 (IPv6) Specification"

[6]	IEEE 802.3: "IEEE Standard for Ethernet"

[7]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications"

[8]		IETF RFC 3261: "SIP: Session Initiation Protocol"

[9]		IETF RFC 2131: "Dynamic Host Configuration Protocol"

[10]		IETF RFC 2865: "Remote Authentication Dial In User Service (RADIUS)"

[11]		3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)"

[12] 	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2"

[13] 	IETF RFC 7525: "Recommendations for Secure Use of Transport Layer Security (TLS) and Datagram Transport Layer Security (DTLS)".

[14] 	IETF RFC 6125: "Representation and Verification of Domain-Based Application Service Identity within Internet Public Key Infrastructure Using X.509 (PKIX) Certificates in the Context of Transport Layer Security (TLS)".

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1] 	OWASP TLS Cheat Sheet.

NOTE: Available at https://www.owasp.org/index.php/Transport_Layer_Protection_Cheat_Sheet.

[i.2] 	ETSI TR 103 308: "CYBER; Security baseline regarding LI and RD for NFV and related platforms".

[i.3] 	ETSI GS NFV-SEC 009: "Network Functions Virtualisation (NFV); NFV Security; Report on use cases and technical approaches for multi-layer host administration".

[i.4] 	ETSI GS NFV-SEC 012: "Network Functions Virtualisation (NFV) Release 3; Security; System architecture specification for execution of sensitive NFV components".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in ETSI TS 103 221-1 [1] and the following apply:

EXAMPLE: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

## 3.2	Symbols

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in ETSI TS 103 221-1 [1] and the following apply:

LI	Lawful Interception

MF	Mediation Function

NF	Network Function

POI	Point Of Interception

SDO	Standards Development Organisation

# 4	Introduction and reference model

## 4.1	Reference model

The X2/X3 interface is based on communication between:

The Point Of Interception (POI), which performs interception

The Mediation Function (MF), which performs the necessary translation, correlation and mediation for onward handover over material to LEAs via the HI2 and HI3 interface.

The X2/X3 reference model is shown in Figure 1.

Figure 1 – Reference Model

The POI produces internal interception product as part of its normal operation. This internal interception product may consist of copies of network traffic contain material related to Intercepted Related Information (IRI) or Content of Communication (CC), as defined in TS 102 232-1 [2]. Material related to IRI is transported via an X2 interface, while material related to CC is transported via an X3 interface.

Any given POI may have one or both interfaces, as specified by the relevant LI architecture. Implementation and deployment scenarios may be more complex. An illustrative list of deployment scenarios is considered in Annex B.

## 4.2	Assumptions

### 4.2.1	Architecture

The present document makes minimal assumptions about the LI architecture in which the X2/X3 interfaces are deployed. The X2/X3 interface is intended to be sufficiently flexible to be used as part of LI architectures defined elsewhere and assumes that the POI and MF are deployed following an LI architecture defined separately (e.g. by another SDO, industry body or local regulation).

As such, the present document makes no assumptions about the specific functional requirements on the POI with respect to e.g. buffering, de-duplication, filtering. It is expected that these requirements will be supplied by a combination of the relevant LI architecture and local regulation.

### 4.2.2	Implementation / realisation

The present document assumes that implementations of an LI architecture which utilise X1, X2 and X3 can be described by the following high-level model.

Figure 2 – Assumed Implementation Model

The model consists of the following entities:

An Implementation: This is a concrete realisation of one or more NFs as deployed by an implementer.

A NF: A function as defined by the relevant network and/or LI architecture (e.g. a P-GW in 3GPP LTE).

Control Function: The sub-function of the NF which accepts LI tasking messages. This may be supplied over a standardised interface (e.g. X1 as defined by TS 103 221 – 1). However, it is assumed that tasking may also be passed between NFs using other unspecified interfaces.

POI (Point of Interception): The sub-function of the NF which performs interception and emits data. An NF may contain multiple POIs; in this case it is assumed that the NF implementation will be responsible for multiplexing the output of these POIs into a single X2 or X3 output stream.

The present document does not consider the means by which tasking information is communicated from a NF’s internal control function to the POI sub-functions but provides the NF implementation a means by which to identify which NF and POI originated each piece of data.

The present document assumes that the NF may be required to deliver high volumes of traffic (e.g. a broadband connection), and may be implemented on a platform with tight resource and/or performance constraints (e.g. a packet gateway), and as such X2/X3 is required to minimise as far as is practical the amount of processing and additional bandwidth consumed (see clause A.1.4)

### 4.2.3	Deployment infrastructure

The present document assumes that the transport infrastructure between POI/NF and MF is untrusted (see clause A.2.8) but assumes that the platform on which the POI, NF and MF are realised are appropriately secured. It does not make any specific assumptions about whether the either the platform or transport infrastructure are virtualised.

The present document does not assume that clocks on different POIs are synchronised. It assumes that while X3 event timestamps may be required by local regulations and can be added to aid describing chronologies of events (e.g. in court), timestamps will not in general permit re-ordering or re-synchronisation of packets which have been intercepted at different NFs.

The present document assumes that X2/X3 is required to provide sufficient information, together with X1, to detect loss of material over X2/X3 (see clause A.1.8 and clause 4.2.4 below). However, it assumes that any POI behaviour regarding detection and recovery from link failures is out of scope, and that no additional application-layer mechanisms (e.g. X3 acknowledgement of data, or heartbeats) are required.

An illustrative list of deployment scenarios that have been considered as part of the design of the X2/X3 interface is given in Annex B.

### 4.2.4	Regulatory assumptions

The present document assumes that material delivered over X2/X3 may be used as evidence in court. As such, it assumes that the X2/X3 interface is capable of indicating when data has been lost over the X2/X3 interface (see clause A.1.8), but recovery of this data (e.g. by buffering and retransmission) are out of scope (as described in clause 4.2.1).

The present document assumes that material over X2/X3 is required to be delivered without undue latency (see clause A.1.5), but that any such latency requirements are not necessarily as stringent that those associated with the underlying communications session (e.g. there is no need for a latency which facilitates a two-way conversation, or remote haptic feedback, or vehicle avoidance measures).

## 4.3	Other standards in the X1, X2, X3 series

The present document forms part of an overall set of standards together with TS 103 221-1 (X1) [1].

# 5	Message contents and parameters

## 5.1	Overview

The POI sends data to the MF as a stream of X2/X3 Protocol Data Units (PDUs). Each PDU is formatted as described in the following clauses. Each PDU consists of three main sections:

A set of mandatory header fields containing identifiers, routing and correlation information – see clause 5.2

A set of additional optional attributes conveying additional metadata about the intercepted material – see clause 5.3

A copy of the intercepted material – see clause 5.4.

Table 2 – X2/X3 PDU Structure

| Field Name | Length (octets) | Defined in |
| --- | --- | --- |
| Version | 2 | 5.2.1 |
| PDU Type | 1 | 5.2.2 |
| Header Length | 2 | 5.2.3 |
| Payload Length | 4 | 5.2.4 |
| Payload Format | 1 | 5.2.5 |
| Direction | 1 | 5.2.6 |
| XID | 16 | 5.2.7 |
| Correlation ID | 4 | 5.2.8 |
| Conditional Attribute Fields NOTE: The list of conditional attribute fields is given in Clause 5.3 | Variable | 5.3 |
| Payload | Variable | 5.4 |

Each PDU is sent across an instance of either the X2 or X3 interface. The choice of which interface to use for any given PDU shall be given by the relevant LI architecture.

Definitions and encodings for the fields are given in the following sections. Unless otherwise specified by the present document or another referenced specification, header values shall be given in network byte order (i.e. big endian).

## 5.2	PDU Header Fields

### 5.2.1	Version

The POI shall populate the Version field with the version of the specification used to create the PDU, given as a 16-bit unsigned integer.

For PDUs created against the present document, this shall be set to the value 1.

### 5.2.2	PDU Type

The POI shall populate the PDU Type field to indicate the type of PDU, given as an 8-bit unsigned integer. It shall take one of the following values:

Table 3 – PDU Types

| Value | Meaning |
| --- | --- |
| 0 | X2 PDU |
| 1 | X3 PDU |

### 5.2.3	Header Length

The POI shall populate the Header Length field with the length of the header in octets, including any optional fields that have been populated, given as an unsigned 16-bit integer. Since this length includes the mandatory fields, the minimum value for this field is 30 octets.

### 5.2.4	Payload Length

The POI shall populate the Payload Length field with the length of the Payload field in octets, given as a 32-bit unsigned integer.

### 5.2.5	Payload Format

The POI shall indicate the format and encoding of the Payload field by setting the Intercepted Material Format field to the appropriate value, given as an 8-bit unsigned integer. A list of valid values, and their definitions, is given in clause 5.3.

### 5.2.6	Direction

The POI shall populate the Direction field with an indication of the direction of the intercepted data or event contained in the PDU.

The value of the Direction field shall be given as a 8-bit unsigned integer. Permitted values are:

Table 4 – General Optional Field Structure

| Direction Value | Meaning |
| --- | --- |
| 0 | The direction of the intercepted data or event is not known to the POI |
| 1 | The intercepted data or event was sent to (i.e. received by) the target |
| 2 | The intercepted data or event was sent from the target |
| 3 | The intercepted data or event is is a result intercepted data or events in more than one direction. This direction value shall not be used for X2 PDUs |
| 4 | The concept of direction is not applicable to this intercepted data or event |

### 5.2.7	XID

The POI shall populate the XID field with the XID associated with the intercepted product, as assigned by the relevant X1 interface (see TS 103 221-1 [1]).

An XID is a UUID (see [1] clause 5.1.2). The XID shall be given as a 128-bit unsigned integer.

### 5.2.8	Correlation ID

Where the POI correlates X2/X3 PDUS, the POI shall ensure that PDUs associated with the same communication session are given the same Correlation ID value. The value shall uniquely identify the communication within a given context. The scheme for choosing Correlation IDs is defined by the relevant LI architecture. The POI may adopt a convention for choosing Correlation IDs which enables correlation of the same communication session across multiple POIs  but such decisions are out of scope of the present document

The Correlation ID value shall be given as a 32-bit unsigned integer.

If the POI does not correlate the X2/X3 PDU with any other X2/X3 PDUs the POI shall set the field to zero.

## 5.3	Conditional attribute fields

### 5.3.1	General structure

The POI may provide a number of conditional attributes at the end of the header, as directed by the relevant LI architecture. Each of these attributes has the following Type-Length-Value (TLV) structure.

Table 5 – General Optional Attribute Structure

| Field Name | Description | Format |
| --- | --- | --- |
| Attribute Type | Indicates the type of field – see Table 6 | 8-bit unsigned integer |
| Attribute Length | Length of the attribute contents in octets | 8-bit unsigned integer |
| Attribute Contents | As defined by the relevant Field Type | Variable |

The present document specifies the following optional attribute types for use in the X2/X3 Content PDU:

Table 6 – Optional Attribute Types

| Attribute Type | Name | Defined in |
| --- | --- | --- |
| 0 | SDO Defined Attribute | 5.3.2 |
| 1 | Domain ID (DID) | 5.3.3 |
| 2 | Network Function ID (NFID) | 5.3.4 |
| 3 | Interception Point ID (IPID) | 5.3.5 |
| 4 | Sequence Number | 5.3.6 |
| 5 | Timestamp | 5.3.7 |

### 5.3.2	SDO Defined Attribute

If used, the POI shall contain an attribute defined by the relevant Standards Developing Organisation (SDO).

### 5.3.3	Domain ID (DID)

If used, the POI shall populate the DID field with a value that identifies the CSP, network or resource group in which the NF or POI exists. The format and content of the field is left to the relevant LI architecture to define.

### 5.3.4	Network Function ID (NFID)

If used, the POI shall populate the NFID field with a value that identifies the NF associated with the POI to the MF. The format and content of the field is left to the relevant LI architecture to define. Depending on the architecture and deployment scenarios, it may be used as a way to identify the type of NF.

### 5.3.5	Interception Point ID (IPID)

If used, the POI shall populate the IPID field with a value that identifies the POI within the NF. The format and content of the field is left to the relevant LI architecture and implementation to define.

### 5.3.6	Sequence Number

If used, the POI shall populate the Sequence Number field with the sequence number of the X2/X3 PDU. If used, the Sequence Number shall start at zero and increment by one for each X2/X3 PDU with the same XID, DID, NFID, IPID and correlation ID.

The Sequence Number shall be given as a 32-bit unsigned integer. Once the maximum sequence number is reached, the POI shall restart the sequence number from zero. Use of the field shall be determined by the relevant LI architecture.

### 5.3.7	Timestamp

If used, the POI shall optionally populate the Timestamp field with the time that the content for the PDU was intercepted.

The time shall be given in POSIX.1-2008 [3] timespec format (i.e. the number of elapsed seconds since the start of the UNIX epoch in UTC). The value shall be given as two successive 32-bit unsigned integers, with the first giving the integral part in seconds and the second giving the fractional part in nanoseconds.

## 5.4	Payload

### 5.4.1	Overview

The POI shall populate the Payload field with intercepted data, given in the format specified by the Payload Format field (see clause 5.2.5). Table 7 below defines the set of permissible Payload Formats and whether each is permitted for use in X2 or X3 PDUs:

Table 7 – Payload Formats

| Value | Intercepted Material Format | Permitted in X2 | Permitted in X3 | Defined in |
| --- | --- | --- | --- | --- |
| 0 | SDO Defined Payload | Yes | Yes | 5.4.2 |
| 3 | IPv4 Packet | Yes | Yes | 5.4.4 |
| 4 | IPv6 Packet | Yes | Yes | 5.4.5 |
| 5 | Ethernet Frame | No | Yes | 5.4.6 |
| 6 | RTP Packet | No | Yes | 5.4.7 |
| 7 | SIP Message | Yes | No | 5.4.8 |
| 8 | DHCP Message | Yes | No | 5.4.9 |
| 9 | RADIUS Message | Yes | No | 5.4.10 |
| 10 | GTP-U Frame | No | Yes | 5.4.11 |

### 5.4.2	SDO Defined Payload

If the SDO Defined Payload is specified, the Payload field shall contain data specified and encoded by the Standards Developing Organisation (SDO) responsible for the relevant LI architecture (e.g. 3GPP).

### 5.4.4	IPv4 Packet

If the IPv4 Packet Payload Format is specified, the Payload field shall contain an IPv4 packet encoded as per RFC 791 [4].

### 5.4.5	IPv6 Packet

If the IPv6 Payload Format is specified, the Payload field shall contain an IPv6 packet encoded as per RFC 8200 [5].

### 5.4.6	Ethernet Frame Packet

If the Ethernet Payload Format is specified, the Payload field shall contain an ethernet frame encoded as per IEEE 802.3 [5].

### 5.4.7	RTP Packet

If the RTP Packet Payload Format is specified, the Payload field shall contain an RTP packet encoded as per RFC 3550 [7], without any IP / UDP encapsulation.

Use of this Payload Format is discouraged for new implementations – handing over RTP with IPv4/IPv6 encapsulation is much preferred.

### 5.4.8 	SIP Message

If the SIP Message Payload Format is specified, the Payload field shall contain a SIP message encoded as per RFC 3261 [8].

### 5.4.9	DHCP Message

If the DHCP Message Payload Format is specified, the Payload field shall contain a DHCP message encoded as per RFC 1541 [9].

### 5.4.10	RADIUS Message

If the RADIUS Message Payload Format is specified, the Payload field shall contain a RADIUS message encoded as per RFC 2865 [10].

### 5.4.11	GTP-U Frame

If the GTP-U Frame Payload Format is specified, the Payload field shall contain a GTP Frame encoded as per TS 29.281 [911], without any IP / UDP encapsulation.

# 6	Transport

## 6.1 Summary

To send X2/X3 PDUs to the MF, the POI opens one or more TLS over TCP connections. TLS is used to perform mutual authentication and identification between the POI and MF, and provide confidentiality and integrity protection for X2/X3 PDUs.

NOTE: 	Schemes for distributing PDUs across multiple TLS connections are out of scope of the present document.

## 6.2 Profile

POIs and MFs shall support TLS as defined in IETF RFC 5246 [12] and support the recommendations given in IETF RFC 7525 [13].

## 6.3 Key generation, deployment and storage

Apart from requirements given in clauses 6.2, aspects concerning the generation, distribution, storage and revocation of key material and certificates are out of scope of the present document. Implementations are encouraged to support best practice e.g. the guidance given in OWASP TLS Cheat Sheet section 2.6 [i.1].

NOTE: It is assumed that the POI, NF and MF are in a physically secure environment. For future uses e.g. NFV then this assumption would no longer be valid. Further details would then need to be added about the security of storage of key or certificate material e.g. TPM, Secure enclaves. See ETSI TR 103 308 [i.2], ETSI GS NFV-SEC 009 [i.3] and ETSI GS NFV-SEC 012 [i.4].

## 6.4 Authentication

Implementations shall perform mutual authentication using client and server X.509 certificates following IETF RFC 6125 [14].

Annex A (normative):
Requirements

## A.1 	X2 Protocol & Architecture requirements

### A.1.1	Basic Functionality

The interface shall be used for delivery of IRI from the network element, which created the copy of the original content of communication, to the mediation function.

### A.1.2	Flexible

The X2 architecture and message exchange technique shall be flexible to allow implementation in both existing and future national and international operator network architectures.

### A.1.3	Extensible

The basic message exchange protocol shall allow limited extensibility to support parameter not currently supported by the base protocol.

### A.1.4	Lightweight

The protocol shall use a protocol containing minimal options or extensions which are not specifically required by X2.

### A.1.5	Delay

The X2 architecture and message exchange technique shall by design not introduce undue delay compared with existing proprietary X2 implementations.

### A.1.6	Permanent and Dynamic Connections

The X2 architecture and message exchange technique shall support both permanent connection and dynamic link / connection scenarios.

### A.1.7	Reliability

The X2 architecture and message exchange technique shall provide reliable data transfer.

### A.1.8	Error detection

The X2 architecture and message protocol shall support error detection (i.e. in case of data loss).

### A.1.9	Redundancy

The X2 architecture and message protocol shall support both 1 to 1 and 1 to 2 end point configurations (i.e. for redundancy).

### A.1.10	Correlation

The X2 protocol shall provide information necessary to allow correlation at the MF for information provided over HI2 and HI3

### A.1.11	Mediation into HI2 / HI3

The X2 protocol shall provide information necessary to allow correlation at the MF to comply with the requirements of both the HI2 and HI3 interfaces, where applicable.

## A.2 	X2 Security requirements

### A.2.1	Authentication & Authorisation

The X2 architecture and message exchange technique shall provide authentication and authorisation of end points.

### A.2.2	Accounting / Audit

The X2 architecture and message exchange technique shall provide Accounting & Auditing.

### A.2.3	Integrity Protection

The X2 message exchange technique shall provide integrity protection for all messages exchanged between nodes in the X2 architecture. Use of Integrity protection shall be mandatory.

### A.2.4	Confidentiality Protection

The X2 message exchange technique shall provide confidentiality protection for all messages exchanged between nodes in the X2 architecture.

### A.2.5	Replay Protection

The X2 message exchange technique shall provide replay protection for all messages exchanged between nodes in the X2 architecture.

### A.2.6	Standalone interface

The X2 architecture and message exchange technique shall be designed as a standalone physically dedicated LI interface. The design and selection of the protocol shall where possible ensure vulnerabilities in non-LI interfaces on the same node shall not impact LI interfaces and security.

### A.2.7	Minimum Security Level

The X2 architecture and message exchange techniques shall provide a minimum level of security (including cypher suites and key length), which shall be supported by all nodes. At least two algorithms shall be specified. The protocol and algorithms shall be resistant to bid down attack.

### A.2.8	Underlying Infrastructure Trust

The X2 architecture and message exchange techniques shall assume by default that the underlying network communication links and infrastructure are untrusted.

### A.2.9	Firewall and NAT Transversal

The X2 message exchange technique shall be compatible with existing operator firewall and NAT transversal architectures. The message exchange technique shall not require unrestricted opening of common ports (eg port 80 or 21). The message exchange technique shall not prohibit the development of future X2 aware firewall filtering to provide rejection of malicious X3 message at operator security gateways.

### A.2.10	Certificate and Key Management

The X2 architecture relies on (where applicable) Certificate and Key Management mechanisms (including Certificate and Key revocation) from X1.

## A.3 	X3 Protocol & Architecture requirements

### A.3.1	Basic Functionality

The interface shall be used for delivery of CC from the network element, which created the copy of the original content of communication, to the mediation function.

### A.3.2	Flexible

The X3 architecture and message exchange technique shall be flexible to allow implementation in both existing and future national and international operator network architectures.

### A.3.3	Extensible

The basic message exchange protocol shall allow limited extensibility to support parameter not currently supported by the base protocol.

### A.3.4	Lightweight

The protocol shall use a protocol containing minimal options or extensions which are not specifically required by X3.

### A.3.5	Delay

The X3 architecture and message exchange technique shall not introduce unnecessary delay.

### A.3.6	Permanent and Dynamic Connections

The X3 architecture and message exchange technique shall support both permanent connection and dynamic link / connection scenarios.

### A.3.7	Reliability

The X3 architecture and message exchange technique shall provide reliable data transfer.

### A.3.8	Error detection

The X3 architecture and message protocol shall support error detection (i.e. in case of data loss) specifically only across the interface in question.

### A.3.9	Redundancy

The X3 architecture and message protocol shall support both 1 to 1 and 1 to 2 end point configurations (i.e. for redundancy).

### A.3.10	Correlation

The X3 protocol shall provide information necessary to allow correlation at the MF for information provided over HI2 and HI3.

### A.3.11	Mediation into HI2 / HI3

The X3 protocol shall provide information necessary to allow correlation at the MF to comply with the requirements of both the HI2 and HI3 interfaces, where applicable – at a minimum the packet-switched parts of TS 33.108 / 128 and TS 102 232 family

## A.4 	X3 Security requirements

### A.4.1	Authentication & Authorisation

The X3 architecture and message exchange technique shall provide authentication and authorisation of end points.

### A.4.2	Accounting / Audit

The X3 architecture and message exchange technique shall provide Accounting & Auditing.

### A.4.3	Integrity Protection

The X3 message exchange technique shall provide integrity protection for all messages exchanged between nodes in the X3 architecture. Use of Integrity protection shall be mandatory.

### A.4.4	Confidentiality Protection

The X3 message exchange technique shall provide confidentiality protection for all messages exchanged between nodes in the X3 architecture.

### A.4.5	Replay Protection

The X3 message exchange technique shall provide replay protection for all messages exchanged between nodes in the X3 architecture.

### A.4.6	Standalone interface

The X3 architecture and message exchange technique shall be designed as a standalone physically dedicated LI interface. The design and selection of the protocol shall where possible ensure vulnerabilities in non-LI interfaces on the same node shall not impact LI interfaces and security.

### A.4.7	Minimum Security Level

The X3 architecture and message exchange techniques shall provide a minimum level of security (including cypher suites and key length), which shall be supported by all nodes. At least two algorithms shall be specified. The protocol and algorithms shall be resistant to bid down attack.

### A.4.8	Underlying Infrastructure Trust

The X3 architecture and message exchange techniques shall assume by default that the underlying network communication links and infrastructure are untrusted.

### A.4.9	Firewall and NAT Transversal

The X3 message exchange technique shall be compatible with existing operator firewall and NAT transversal architectures. The message exchange technique shall not require unrestricted opening of common ports (e.g. port 80 or 21). The message exchange technique shall not prohibit the development of future X3 aware firewall filtering to provide rejection of malicious X3 message at operator security gateways.

### A.4.10	Certificate and Key Management

The X3 architecture relies on (where applicable) Certificate and Key Management mechanisms (including Certificate and Key revocation) from X1.

Annex B (informative):
Illustrative deployment scenarios

## B.1 Introduction

This section provides a list of illustrative X2/X3 deployment scenarios for consideration during the drafting of this document.

## B.2 Simple deployment scenario

This scenario shows a simple deployment where separate POIs provide X2 and X3. A single MF connects to an X2 interface from one POI and an X3 interface from a second.

Figure B.1 – Simple deployment scenario

## B.3 Individual X3 POIs with shared X2 POI

This scenario shows a deployment scenario where a single physical POI provides X2 to multiple MFs, while each MF receives X3 from a different POI.

Figure B.2 – Individual X3 POIs with shared X2 POIs

## B.3 Separated interfaces

This scenario shows a deployment scenario where the X2 and X3 interfaces are delivered to different MFs

Figure B.3 – Separated interfaces

# Annex Z (informative)

# Change History

| Date | Version | Information about changes |
| --- | --- | --- |

History

| Document history |  |  |
| --- | --- | --- |
| <Version> | <Date> | <Milestone> |
| 0.0.1 | 2016-10-14 | Skeleton |