3GPP TSG-SA WG3 LI Meeting #70-bis	S3i180435

Sophia Antipolis; October 1-3, 2018

Source:	Ericsson

Title:	pCR to draft TS 33.127: LI events at the AMF

Document for:	Agreement

Agenda Item:	2.2

Work Item / Release:	FS_15LIS

Abstract of the contribution: This pCR proposes changes to the clause 6.3.2 to specify LI events to be intercepted at the AMF.

Discussion:

In SA3-LI#70 it was agreed to specify in DTS 33.127 LI events at each POI keeping high level description and including the main parameters, leaving more details to LI stage 3. This contribution proposes LI events and parameters for AMF.

**** First Change ****

### 6.3.2	LI at AMF

#### 6.3.2.1	Architecture

In the 5GC network, the AMF handles the access and mobility functions. The AMF shall have LI capabilities to generate the target UE’s network access, registration and connection management related xIRI.  Extending the generic LI architecture presented in clause 5, the figure 6.3-3 below gives a reference point representation of the LI architecture with AMF as a CP NF providing the IRI-POI functions.

Figure 6.3-3: LI Architecture for LI at AMF

The LICF present in the ADMF receives the warrant from an LEA, derives the intercept information from the warrant and provides the same to the LIPF.

The LIPF present in the ADMF provisions the IRI-POI (over LI_X1) present in the AMF and the MDF2.  The LIPF may interact with the SIRF (over LI_SI) present in the NRF to discover the AMFs in the network.

The IRI-POI present in the AMF detects the target UE’s access and mobility related functions (network access, registration and connection management), generates and delivers the xIRI to the MDF2 over LI_X2.  The MDF2 delivers the IRI messages as part of the Interception Product to the LEMF over LI_HI2.

#### 6.3.2.2	Target Identities

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the AMF:

- SUPI

- PEI

- MSISDN.

Editor’s Note: May need to change MSISDN to GPSI.

The interception performed on the above three identities are mutually independent, even though, an xIRI may contain the information about the other identities when available.

#### 6.3.2.3	IRI Events

The IRI-POI present in the AMF shall generate xIRI, when the AMF detects the following specific events or information:

- Registration

- Deregistration

- Location Update

- SMS

- Start of interception with registered UE

Location update shall trigger xIRI generation each time that the AMF detects an updated UE location by interaction with the UE/RAN (e.g. in case of Xn based inter NG-RAN handover). The xIRI generation for Location Update may be omitted if the updated UE location information is already included in other xIRIs provided by the AMF.

Start of interception with registered UE shall be provided in case interception is activated in the IRI-POI present in the AMF for a target identity which is already registered.

#### 6.3.2.4	IRI Parameters

A combined list of parameters that are included in the xIRI generated by the IRI-POI present in the AMF is shown below.

- Target id: can be one of SUPI, MSISDN/GPSI, PEI

- Additional observed identities: all the other identities which are related to the target id associated in the IRI-POI present in the AMF and associated to the target id. These includes SUPI, MSISDN/GPSI, PEI, SUCI, 5G-GUTI

- NF identifier: unique identifier for the NF

- Event type: Registration, Deregistration, Location Update, SMS, Start of interception with registered UE

- Registration type: Initial registration to the 5GS, Mobility Registration Update, Periodic Registration Update

- NSSAI: Network Slice Selection Assistance Information

- Location information: the target UE location, including the time at which the location was obtained by the IRI- POI present in the AMF

- Timestamp: the time at which the xIRI is generated by the IRI-POI present in the AMF

- Failure cause: provides the cause of failure of the network procedure as provided by the related protocols

- Initiator: information about the entity which initiated the procedure, i.e. UE or Network

- Access type: indicates whether the Deregistration procedure applies to the 3GPP access, to the non-3GPP access, or to both if the 3GPP access and non-3GPP access of the UE are served by the same AMF.

- SMS message: the body of SMS

#### 6.3.2.5	Parameters on each xIRI

Parameters to be included in each xIRI are included below.

##### 6.3.2.5.1	Registration

The following parameters shall be provided, if available, in the Registration xIRI from the AMF:

- Target id (SUPI, MSISDN/GPSI, PEI)

- Additional observed identities (e.g. SUCI, 5G-GUTI)

- NF identifier

- Event type

- Registration type

- NSSAI

- Location information

- Timestamp

- Failure cause (in case of failure only)

##### 6.3.2.5.2	Deregistration

The following parameters shall be provided, if available, in the Deregistration xIRI from the AMF:

- Target id (SUPI, MSISDN/GPSI, PEI)

- Additional observed identities (e.g. SUCI, 5G-GUTI)

- NF identifier

- Event type

- Initiator

- Access type

- Location information

- Timestamp

- Failure cause (only in case of failure)

##### 6.3.2.5.3	Location Update

The following parameters shall be provided, if available, in the Location Update xIRI from the AMF:

- Target id (SUPI, MSISDN/GPSI, PEI)

- Additional observed identities (e.g. SUCI, 5G-GUTI)

- NF identifier

- Event type

- Location information

- Timestamp

- Failure cause (only in case of failure)

##### 6.3.2.5.4	SMS

The following parameters shall be provided, if available, in the SMS xIRI from the AMF:

- Target id (SUPI, MSISDN/GPSI, PEI)

- Additional observed identities (e.g. SUCI, 5G-GUTI)

- NF identifier

- Event type

- SMS message

- Location information

- Timestamp

- Failure cause (only in case of failure)

##### 6.3.2.5.5	Start of interception with registered UE

##### The following parameters shall be provided, if available, in the Start of interception with registered UE xIRI from the AMF:

- Target id (SUPI, MSISDN/GPSI, PEI)

- Additional observed identities (e.g. SUCI, 5G-GUTI)

- NF identifier

- Event type

- NSSAI

- Location information

- Timestamp

**** End of Changes ****