3GPP TSG-SA WG3 LI Meeting #72	S3i190028

Sophia Antipolis (FRA), Jan 22nd - Jan 25th 2019

Source:	NTAC

Title:	pCR to resolve Editor’s Notes in 33.128 Clause 6.2.5 (LI at SMSF)

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	FS_15LIS

Abstract of the contribution: This pCR proposes text to resolve the Editor’s Notes in 33.128 in clause 6.2.3 (LI at SMF/UPF)

FIRST CHANGE (Clause 6.2.3)

#### 6.2.5.3	SMS

The IRI-POI in the SMSF shall generate an xIRI SMS for each SMS message.

The xIRI SMS message shall contain the following payload, encoded as per clause 5.3.2.

XIRISMSMessage ::= SEQUENCE

{

- See clause 6.2.5.3 for details of this structure

smsIdentity				[1] SMSParty		OPTIONAL,

nonLocalID				[2] E164Number		OPTIONAL,

otherPartyID			[3]	SMSParty,

direction				[4] SMSDirection,

transferStatus			[5] SMSTransferStatus,

otherMessage			[6] SMSOtherMessageIndication 	OPTIONAL,

location				[7]	LocationInformation 		OPTIONAL,

otherNFAddress			[9] SMSNFAddress				OPTIONAL,

otherNFType				[10] SMSNFType 					OPTIONAL,

payload					[11] SMSPayload					OPTIONAL

}

SMSParty ::= SEQUENCE

{

supi	[0] SUPI 	OPTIONAL,

pei		[1] PEI		OPTIONAL,

gpsi	[2] GPSI	OPTIONAL

}

SMSDirection ::= ENUMERATED

{

toTarget 	(0),

fromTarget 	(1)

}

SMSPayload ::= OCTET STRING (SIZE(1..270))

SMSNFAddress ::= CHOICE

{

ipAddress	[0] IPAddress,

e164Number	[1] E164Number
}

SMSNFType ::= ENUMERATED

{

sms-GMSC(0),

iwMSC(1),

sms-Router(2)

}

Table 6.2.5-1: Payload for SMS xIRI message

| Field name | Description | M/C/O |
| --- | --- | --- |
| smsIdentity | Identity of the target in communication with the SMSF. At least one of the elements (SUPI, PEI or GPSI) shall be populated, with the others populated. For all interceptions other than non-local-ID, the smsIdentity is mandatory. | C |
| nonLocalID | Non-local target identity associated with the SMS, if applicable | C |
| otherPartyID | The identifier of the other party in the SMS. | M |
| direction | Direction of the SMS with respect to the target | M |
| transferStatus | Indicates whether the transfer succeeded or not | M |
| otherMessage | In the event of a server-initiated transfer, indicates whether the server will send another SMS. May be omitted if the transfer is target-initiated. | M |
| otherNetworkFunctionAddress | Address of the other network function (SMS-GMSC/IWMSC/SMS-Router) involved in the communication of the SMS, if available | C |
| otherNetworkFunctionType | Type of the other network function (SMS-GMSC/IWMSC/SMS-Router) involved in the communication of the SMS, if available | C |
| location | Location information associated with the UE sending or receiving the SMS, if available | C |
| smsPayload | SMS TPDU, encoded as per TS 23.040 [18] clause 9. Shall be provided by the IRI-POI in the SMSF, but may be omitted or otherwise modified when forwarded by the MDF2 (see clause 6.2.5.4) depending on national regulation | M |

END OF CHANGES