3GPP TSG-SA WG3 LI Meeting #72	S3i190031

Sophia Antipolis (France), Jan 22nd – 25th, 2019

Source:	Ericsson

Title:	pCR to draft TS 33.128: Use of UDP/DTLS as transport protocol

Document for:	Agreement

Agenda Item:	2.2

Work Item / Release:	FS_15LIS

Abstract of the contribution: This pCR proposes to add UDP/DTLS as additional option to TCP as transport mechanism for LI_X3 and LI_HI3.

Discussion:

In the current version of TS 33.128, ETSI DTS 103 221-2 and TS 102 232-1 are adopted as standard interfaces respectively for LI_X3 and LI_HI3. For the transport protocol, DTS 103 221-2 specifies a default profile based on TCP and allows further profiles to be defined for LI_X3; TS 102 232-1 describes a transport layer based on TCP. In order to cope with 5G full rate from a given intercepted user and typical average rates from all intercepted users in a 5G scenario, it is proposed to add UDP/DTLS as alternative option, provided that an agreement exists between network operator and LEA on its use.

**** First Change ****

### 5.3.3	Usage for realising LI_X3

The POI sending xIRI messages over the LI_X2 interface shall set the PDU type field for these messages to “X3 PDU”. (see ETSI TS 103 221-2 [8] clause 5.1).

ETSI TS 103 221-2 [8] specifies in clause 6 a default profile, allowing further profiles to be defined. The default profile specified in ETSI TS 102 221-2 [8] shall be supported. However, based on agreement between network operator and LEA, in order to cope with 5G full rate from a given intercepted user and typical average rates from all intercepted users in a 5G scenario, UDP/DTLS may be used as alternative transport protocol for LI_X3.

Editor’s Note: Other areas where we may wish to define, or override defaults specified in TS 103 221-2 are:

- Whether we wish to define formats and hierarchies for the DID, NFID and IPID (clauses 5.3.3, 5.3.4 and 5.3.5)

- Whether sequence numbers are used (see clause 5.3.6). We might want to make this decision per-function. Also may wish to define procedures at the MF should sequence numbers not match up.

- Whether timestamps are used (see clause 5.3.7). We might want to make this decision per-function.

**** Next Change ****

### 5.5.3 	Usage for realising LI_HI3

The LI_HI3 messages are structured as a header and a payload. The header contains general information like LIID, Timestamp, correlation information (as for example defined in ETSI TS 102 232-1 [9]). The payload contains content of communication based on information that the MDF3 has received from sources in the network, such as the CC-POI as described in section 6 and 7 of the present document. Details of the LI_HI3 messages can be found in Annex B of the present document. Messages defined as passing over the LI_HI3 interface shall be passed as the payload of the threeGPPCC field (see TS ETSI 102 232 – 7 [10] clause 15).

Transport layer based on TCP as described in clause 6.4 of ETSI TS 102 232-1 [9] shall be supported. However, based on agreement between network operator and LEA, in order to cope with 5G full rate from a given intercepted user and typical average rates from all intercepted users in a 5G scenario, UDP/DTLS may be used as alternative transport protocol for LI_HI3.

Editor’s Note – Clause 15 does not yet exist, and will either be introduced by a CR or the text above will be revised.

Editor’s Note – this assumes we add a new dedicated CC contents field to TS 102 232 part 1 and refer to it in part 7

Editor’s Note – There are more options in TS 102 232 than can be further specified or overridden in this document, including (contribution to specify these details are welcome):

- Use of keepalives and length of wait timers (TIME1, TIME2, TIME3). See clause 6.3.4.

Support or use of various capabilities (encryption, payload aggregation, capability negotiation etc). See primarily clause 6.2.

**** End of Changes ****