3GPP TSG-SA3 Meeting #72-LI	s3i190035

# Sophia-Antipolis, France, 22nd Jan 2019 - 25th Jan 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0005 | rev | 0 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | N3IWF POI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI15 |  |  |  |  |  | Date: |  |  | 2019-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Without a POI on the N3IWF the IP Address used in the Access Network is not accesible by the LI system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution places a POI on the N3IWF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IP Address used by the target in the Access Network will not be accessible by the LI system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Modification of 6.2.1; addition of new clause between 6.2.2 and 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<< FIRST CHANGE  >>>>>>>>>>>>

## 6.2	5G

### 6.2.1	General

Figure 6.2-1 depicts the 5G EPC anchored LI architecture. The network functions are depicted in grey, while the LI elements are depicted in blue.

Figure 6.2-1: 5G EPC-anchored LI architecture

Figure 6.2-2 depicts the 5G core-anchored LI architecture. The network functions are depicted in grey, while the LI elements are depicted in blue.

Figure 6.2-2: 5G core-anchored LI architecture

<<<<<<<<<< END of FIRST CHANGE  >>>>>>>>>>>>

<<<<<<<<<< SECOND CHANGE  >>>>>>>>>>>>

### 6.2.2bis	LI at N3IWF

#### 6.2.2bis.1	Architecture

In the case of non-3GPP access, the only element in the network that has visibility of the IP address used in the access network is the N3IWF, as shown in Figure 8.3.2-1: User Plane via N3IWF from 33.501:

The yellow-highlighted (just above the “Lower layers“ in the N3IWF) IP layer in the N3IWF is the layer that contains the IP address associated with the interception subject used in the Untrusted non-3GPP access network, and necessary for Law Enforcement to identify the access network.  Since the N3IWF is the only place in the 5G network to obtain this address, the N3IWF is designated as a POI.

Figure 6.2-Y: LI architecture for LI at N3IWF

The LICF present in the ADMF receives the warrant from an LEA, derives the intercept information from the warrant and provides the same to the LIPF.

The LIPF present in the ADMF provisions the IRI-POI (over LI_X1) present in the N3IWF and the MDF2. The LIPF may interact with the SIRF (over LI_SI) present in the NRF to discover the N3IWFs in the network.

The IRI-POI present in the N3IWF detects the target UE's IP address used in the access network, generates and delivers the xIRI to the MDF2 over LI_X2. The MDF2 delivers the IRI messages as part of the Interception Product to the LEMF over LI_HI2.

#### 6.2.2bis.2	Target identities

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the N3IWF:

- SUPI

- PEI

- GPSI

- IMSI

- IMEI

- MSISDN

#### 6.2.2bis.3	IRI events

The IRI-POI present in the N3IWF shall generate xIRI, when it detects the following specific events or information:

- Registration.

- Deregistration.

- Location update.

- Start of interception with already registered UE.

- Unsuccessful communication attempt.

The registration xIRI is generated when the IRI-POI present in an N3IWF detects that a target UE has successfully registered to the 5GS via 3GPP NG-RAN or non-3GPP access.

The deregistration xIRI is generated when the IRI-POI present in an N3IWF detects that a target UE has deregistered from the 5GS. The deregistration xIRI shall indicate whether it was an UE-initiated or a network-initiated deregistration.

Location update xIRI is generated each time the IRI-POI present in an N3IWF detects that the target's UE location is updated due to target's UE mobility (e.g. in case of Xn based inter NG-RAN handover). The generation of such xIRI may be omitted if the updated UE location information is already included in other xIRIs (e.g. mobility registration) provided by the IRI-POI present in the same N3IWF.

The start of interception with already registered UE xIRI is generated when the IRI-POI present in an N3IWF detects that interception is activated on the target UE that has already been registered in the 5GS.

When additional warrants are activated on a target UE, MDF2 shall be able to generate and deliver the start of interception with already registered UE to the LEMF associated with the additional warrants without receiving a corresponding xIRI.

The unsuccessful communication attempt xIRI is generated when the IRI-POI present in an N3IWF detects that a target UE initiated communication procedure (e.g. session establishment, SMS) is rejected by the AMF before the proper NF handling the communication attempt itself is involved.

#### 6.2.2bis.4	Common IRI parameters

The detailed list of xIRI parameters are specified in TS 33.128. All xIRI shall include the following:

-	Target identity.

-	Time stamp.

#### 6.2.2bis.5	Specific IRI parameters

The detailed list of parameters in each xIRI are defined in TS 33.128. The following give a summary.

The registration xIRI shall include the following:

- Registration type information.

- Access type information.

- Access IP Address and Port Number.

- Requested slice information.

The deregistration xIRI shall include the following:

- UE initiated de-registration.

- Access type information.

- Access IP Address and Port Number.

- Network initiated de-registration.

The location update xIRI shall include the following:

- Location of the target UE (see clause 7.3).

The start of interception with already registered UE xIRI shall include the following:

- Access type information.

- Access IP Address and Port Number.

- Requested slice information.

The unsuccessful communication attempt xIRI shall include the following:

- Rejected type of communication attempt.

- Access type information.

- Access IP Address and Port Number.

- Failure reason.

#### 6.2.2bis.6	Network topologies

The N3IWF shall provide the IRI-POI functions in the following network topology cases:

- Non-roaming case.

- Roaming case, in VPLMN.

- Roaming case, in HPLMN for non-3GPP access.

In a roaming case, it is possible that the target UE may use non-3GPP access with the N3IWF present in the HPLMN.

<<<<<<<<<< END of SECOND CHANGE  >>>>>>>>>>>>