# Discussion

There are two fundamental ways to categorize LI identifiers when delivered over the HI interfaces. The two methods are table pivots of each other, and are fully equivalent in functionality, therefore either one is acceptable. There is no implication on the transport of IDs over the X interfaces, and that is left up to implementation.

The first leads with the provenance categories, but allows any particular ID to appear in more than one category. An alternative method is to lead the sequence with the ID itself, eliminating duplication, but making PROVENANCE a set.

### <<<<<<<<<<<<<<<<<<<< CHOICE 1 >>>>>>>>>>>>>>>>>>>>>

### 6.2.1bis Identity Provenance

The following example depicts a set of Target IDs and their associated provenance. Note that any particular ID can appear in more than one category. There is no implication on the transport of IDs over the X interfaces, and that is left up to implementation.

- LEA-PROVIDED IDs {

ID1: TYPE: IMPU

}

- MATCHED-ON IDs {

ID2; TYPE: SUPI

}

- Observed IDs {

ID1; TYPE: IMPU

ID3; TYPE: SUCI

}

- OTHER ASSOCIATED IDs {

ID4; TYPE: PEI

}

The ASN.1 definition associated with this provenance structure follows:

TargetIdentity ::= SEQUENCE {

leaProvidedIds	[0]	SEQUENCE OF Id,

matchedOnIds	[1]	SEQUENCE OF Id,

observedIds		[2]	SEQUENCE OF Id,

otherIds		[3]	SEQUENCE OF Id

}

Id	::=	SEQUENCE {

id		[0]	UTF8String,

idType	[1]	IdType

}

IdType	::=	CHOICE {

Supi	[0] 	NULL,

Suci	[1]	NULL,

Pei	[2]	NULL,

Gpsi	[3]	NULL,

Guti	[4]	NULL

}

Note: If an identifier appears in the matchedOnIds set, it implies it is also in the observedIds set, there is no need to make an entry in observedIds set in this case.

### <<<<<<<<<<<<<<<<<<<< END OF CHOICE 1 >>>>>>>>>>>>>>>>>>>>>

### <<<<<<<<<<<<<<<<<<<< CHOICE 2 >>>>>>>>>>>>>>>>>>>>>

### 6.2.1bis Identity Provenance

The following example depicts a set of Target IDs and their associated provenance. There is no implication on the transport of IDs over the X interfaces, and that is left up to implementation.

- ID1; TYPE: IMPU; PROVENANCE: {LEA-PROVIDED, OBSERVED}

- ID2: TYPE: SUPI; PROVENANCE: {MATCHED-ON}

- ID3: TYPE: SUCI; PROVENANCE: {OBSERVED}

- ID4: TYPE: PEI; PROVENANCE: {OTHER}

The ASN.1 definition associated with this provenance structure follows:

TargetIdentity ::= SEQUENCE OF Id

Id	::=	SEQUENCE	{

id		[0]	UTF8String,

idType	[1]	IdType,

provenance	[2]	SEQUENCE OF IdProvenance

}

IdType	::=	CHOICE {

Supi	[0] 	NULL,

Suci	[1]	NULL,

Pei	[2]	NULL,

Gpsi	[3]	NULL,

Guti	[4]	NULL

}

IdProvenance	::=	CHOICE {

LeaProvided	[0] 	NULL,

Observed	[1]	NULL,

MatchedOn	[2]	NULL,

Other		[3]	NULL

}

IdProvenance of MatchedOn implies it is also Observed, there is no need to make an entry for Observed in this case.

### <<<<<<<<<<<<<<<<<<<< END OF CHOICE 2 >>>>>>>>>>>>>>>>>>>>>