3GPP TSG-SA WG3 LI Meeting #72	S3i190059

Sophia Antipolis (FRA), Jan 22nd - Jan 25th 2019

Source:	NTAC

Title:	pCR to resolve Editor’s Notes in 33.128 Clause 5

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	FS_15LIS

Abstract of the contribution: This pCR proposes text to resolve the Editor’s Notes in 33.128 at the start of clause 5

FIRST CHANGE (Clause 5 title)

# 5	Transport and Communications Protocol

SECOND CHANGE (Clause 5.2)

## 5.2	Protocols for LI_X1 and LI_T interfaces

### 5.2.1 	General usage of ETSI TS 103 221-1

Functions having an LI_X1, LI_T2 or LI_T3 interface shall support the use of ETSI TS 103 221-1 [7] to realise the interface.

In the event of a conflict between ETSI TS 103 221-1 [7] and the present document, the terms of the present document shall apply.

The LIPF and MDF2/3 shall maintain a mapping between internal interception identifiers (XIDs) and external interception identifiers (LIIDs), as defined by TS 103 221-1 [7] clause 5.1.2. In the case of multiple interceptions for a single target identifier, it is an implementation decision for the LIPF/TF whether multiple XIDs are used (i.e. a one-to-one mapping between XID and LIID is maintained) or whether the single XID is used and mapped to multiple LIIDs at the MDF2/3.Clauses 6 and 7 give further details for specific networks or services (e.g. minimum supported Target Identifier Formats).

In the event that a request issued over the interface fails, or an error is reported, the LIPF should raise an alert in the appropriate LI O&M system. Further procedures (e.g. retrying a failed request) are left to CSP policy to define.

### 5.2.2 	Usage for realising LI_X1

For the purposes of realising LI_X1 between the LIPF and a POI or TF, the LIPF plays the role of the “ADMF” as defined in the TS 103 221-1 reference model (clause 4.2), and the POI or TF plays the role of the “NE”.

### 5.2.3	Usage for realising LI_X1 (Management)

For the purposes of realising LI_X1 between the LIPF and a Triggered POI, the LIPF plays the role of the “ADMF” as defined in the TS 103 221-1 reference model (clause 4.2), and the Triggered POI plays the role of the “NE”.

### 5.2.4	Usage for realising LI_T2

For the purposes of realising LI_T2 between a TF and a Triggered POI, the TF plays the role of the “ADMF” as defined in the TS 103 221-1 reference model (clause 4.2), and the Triggered POI plays the role of the “NE”.

### 5.2.5	Usage for realising LI_T3

For the purposes of realising LI_T3 between a TF and a Triggered POI, the TF plays the role of the “ADMF” as defined in the TS 103 221-1 reference model (clause 4.2), and the Triggered POI plays the role of the “NE”.

THIRD CHANGE (Clause 5.3)

## 5.3	Protocols for LI_X2 and LI_X3

### 5.3.1 	General usage of ETSI TS 103 221-2

Functions having an LI_X2 or LI_X3 interface shall support the use of ETSI TS 103 221-2 [8] to realise the interface.

In the event of a conflict between ETSI TS 103 221-2 [8] and the present document, the terms of the present document shall apply.

xIRI and xCC messages sent using ETSI TS 103 221-2 shall contain the appropriate XID as received in the relevant LI_X1 provisioning message (or LI_T2/3 triggering message, as appropriate).

### 5.3.2 	Usage for realising LI_X2

The POI sending xIRI over the LI_X2 interface shall set the PDU type field to “X2 PDU”. (see TS 103 221-2 [8] clause 5.1).

The TLS transport profile (see [8] clause 6) shall be supported and used by default.

Unless otherwise specified, xIRI messages shall include the Timestamp and Sequence Number conditional attribute fields, with the Timestamp value set to the time that the event occurred.

### 5.3.3	Usage for realising LI_X3

The POI sending xCC over the LI_X3 interface shall set the PDU type field to “X3 PDU”. (see TS 103 221-2 [8] clause 5.1).

FOURTH CHANGE (Clause 5.4)

## 5.4	Protocols for LI_HI1

### 5.4.1 	General

Functions having an LI_HI1 interface shall support the use of ETSI TS 103 120 [6] to realise the interface.

In the event of a conflict between ETSI TS 103 120 [6] and the present document, the terms of the present document shall apply.

FIFTH CHANGE (Clause 5.3.3)

### 5.5.3 	Usage for realising LI_HI3

The LI_HI3 PDUs are structured as a header and a payload. The header contains general information like LIID, Timestamp, correlation information (as for example defined in ETSI TS 102 232-1 [9]). The payload contains content of communication based on information that the MDF3 has received from sources in the network, such as the CC-POI as described in section 6 and 7 of the present document. Details of the LI_HI3 messages can be found in Annex B of the present document. Messages defined as passing over the LI_HI3 interface shall be passed as the payload of the threeGPPCC field (see ETSI TS 102 232–7 [10] clause 15).

Editor’s Note – ETSI TS 102 232-7 Clause 15 does not yet exist, and will either be introduced by a CR or the text above will be revised.

SIXTH CHANGE (Clause 5.6.2)

### 5.6.2 	Usage for realising LI_HI4

The LI_HI4 PDUs are structured as a header and a payload. The header contains general information like LIID, Timestamp (as for example defined in ETSI TS 102 232-1 [9]). The payload contains the administrative information such as a notification. Details of the LI_HI4 messages can be found in Annex B of the present document.

Where the LI_HI4 interface is present alongside an LI_HI2 or LI_HI3 interface, the LI_HI4 messages shall be transmitted along the same TCP or TLS connection as the LI_HI2 or LI_HI3 messages. Where ETSI TS 102 232-1 is used for LI_HI2 or LI_HI3, messages defined as passing over the LI_HI4 interface shall be passed as the contents of the threeGPP-HI1-Operation field.

The MDF2/3 shall support generation LI_HI4 notifications for at least the following events:

- Activation of an interception at the MDF2/3 via LI_X1.

- Modification of an interception at the MDF2/3 via LI_X1.

- Deletion of an interception at the MDF2/3 via LI_X1.

END OF CHANGES