3GPP TSG-SA WG3 LI Meeting #72	S3i190060

Sophia Antipolis (FRA), Jan 22nd - Jan 25th 2019

Source:	NTAC

Title:	pCR to resolve Editor’s Notes in 33.128 Clause 6.2.2 (LI at AMF)

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	FS_15LIS

Abstract of the contribution: This pCR proposes text to resolve the Editor’s Notes in 33.128 in clause 6.2 (LI at AMF)

FIRST CHANGE (Clause 6.2.2)

### 6.2.2 	LI at AMF

Editor’s Note – we should specify how correlation numbers should be set

#### 6.2.2.1	Provisioning over LI_X1

The IRI-POI present in the AMF is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The POI in the AMF shall support the following Target Identifier Formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 is not used):

- SUPI.

- PEI.

- GPSI.

#### 6.2.2.2	Generation of xIRI over LI_X2

##### 6.2.2.2.1	General

The IRI-POI present in the AMF shall emit X2 PDUs over LI_X2 for each of the events described in the following clauses.

xIRI messages pertaining to the same registration shall be given the same correlation number. Unless otherwise specified, the X2 “Matched Target Identifier” conditional attribute shall be set to indicate what target identity was matched to generate the xIRI message (see ETSI TS 103 221-2 [7] clause 5.3.18).

##### 6.2.2.2.2	Registration

The IRI-POI in the AMF shall generate an XIRIAMFRegistrationMessage when the IRI-POI present in the AMF detects that a UE matching one of the target identifiers provided via LI_X1 has successfully registered to the 5GS via 3GPP NG-RAN or non-3GPP access. Accordingly, the IRI-POI in the AMF generates the XIRIAMFRegistrationMessage when one of the following events are detected:

- AMF sends a REGISTRATION ACCEPT message to the target UE over the N1 reference point and the UE registration state within the AMF is changed to 5GMM-REGISTERED.

- When the REGISTRATION ACCEPT message is sent to the target UE includes the 5G-GUTI, AMF receives the REGISTRATION COMPLETE message from the target UE over the N1 reference point and the UE registration state within the AMF is 5GMM-REGISTERED.

The message shall contain the following payload, encoded as per clause 5.3.2.

XIRIAMFRegistrationMessage ::= SEQUENCE

{

-- See clause 6.2.2.2.2 for details of this structure

registrationType			[1] AMFRegistrationType,

registrationResult			[2]	AMFRegistrationResult,

sliceInformation			[3]	SliceInformation			OPTIONAL,

supi						[4]	SUPI,

suci						[5] SUCI 						OPTIONAL,

pei							[6] PEI,

gpsi						[7] GPSI 						OPTIONAL,

guti						[8]	GUTI,

location					[9]	LocationInformation 		OPTIONAL,

non3GPPAccessEndpoint 		[10] UEEndpointAddress 			OPTIONAL

}

AMFRegistrationType	::=	ENUMERATED

{

initial		(1),

mobility	(2),

periodic	(3),

emergency	(4)

}

AMFRegistrationResult	::= ENUMERATED

{

threeGPPAccess					(1),

nonThreeGPPAccess				(2),

threeGPPAndNonThreeGPPAccess	(3)

}

SliceInformation	::= SEQUENCE

{

allowedNSSAI					(1)	NSSAI			OPTIONAL,

configuredNSSAI					(2)	NSSAI			OPTIONAL,

rejectedNSSAI					(3)	RejectedNSSAI	OPTIONAL

}

Editor’s Note: TS 24.501 does not provide the ASN.1 definition for the referenced parameter types (e.g. NSSAI and rejected NSSAI). So, we will have to define it locally in the TS 33.128, or define as OCTET STRING and use CT spec value.

Editor’s Note – We may choose to refactor the ASN.1 into a single module later, but for now we will write each in the relevant section to make them easier to discuss. At that point we can also consider any other factorisation (e.g. grouping fields together) subject to keeping the number of OPTIONAL fields to a minimum, and remove any duplicate types.

Editor’s Note – Need to consider impacts of unauthenticated emergency calls and PARLOS. This editor’s note also applies to other events.

Table 6.2.2-1: Payload for XIRIAMFRegistrationMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| registrationType | Specifies the type of registration, see 3GPP TS 24.501 [13] clause 9.11.3.7. This is derived from the information received from the UE in the REGISTRATION REQUEST message. | M |
| registrationResult | Specifies the result of registration, see 3GPP TS 24.501 [13] clause 9.11.3.6. | M |
| sliceInformation | Provide, if available, one or more of the following: - allowed NSSAI (see 3GPP TS 24.501 [13] clause 9.11.3.37) - configured NSSAI (see 3GPP TS 24.501 [13] clause 9.11.3.37) - rejected NSSAI (see 3GPP TS 24.501 [13] clause 9.11.3.46) This is derived from the information sent to the UE in the REGISTRATION ACCEPT message. | C |
| supi | SUPI associated with the registration (see clause 6.2.2.4) | M |
| suci | SUCI used in the registration | C |
| pei | PEI provided by the UE during the registration. | M |
| gpsi | GPSI obtained in the registration, if available as part of the subscription profile. | C |
| guti | 5G-GUTI provided as outcome of initial registration or used in other cases, see 3GPP TS 24.501 [13] clause 5.5.1.2.2. | M |
| location | Location information determined by the network during the registration, if available | C |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, if available | C |

##### 6.2.2.2.3	Deregistration

The IRI-POI in the AMF shall generate an XIRIAMFDeregistrationMessage when the IRI-POI present in the AMF detects that a UE matching one of the target identifiers provided via LI_X1 has deregistered from the 5GS. Accordingly, the IRI-POI in AMF generates the XIRIAMFDeregistrationMessage when one of the following events are detected:

- For Network initiated de-registration, when the AMF receives the N1: DEREGISTRATION ACCEPT message from the target UE or when implicit deregistration timer expires; and in both cases the UE registration state within the AMF is changed to 5GMM-DEREGISTERED.

- For UE initiated de-registration, when the AMF sends the N1: DEREGISTRATION ACCEPT message to the target UE or when the AMF receives the N1: DEREGISTRATION REQUEST message from the target UE with deregistration type value of “switch off”; and in both cases the UE registration state within the AMF is changed to 5GMM-DEREGISTERED.

The message shall contain the following payload, encoded as per clause 5.3.2.

XIRIAMFDeregistrationMessage ::= SEQUENCE

{

-- See clause 6.2.2.2.3 for details of this structure

deregistrationDirection	[1]	AMFDirection,

accessType				[2]	AMFAccessType,

supi					[3]	SUPI					OPTIONAL,

suci					[4]	SUCI					OPTIONAL,

pei						[5]	PEI						OPTIONAL,

gpsi					[6] GPSI					OPTIONAL,

guti					[7]	GUTI					OPTIONAL,

cause					[8]	FiveGMMCause			OPTIONAL,

location				[9] LocationInformation		OPTIONAL,

non3GPPAccessEndpoint 		[10] UEEndpointAddress 			OPTIONAL

}

AMFDirection	::=	ENUMERATED

{

networkInitiated 	(1),

ueInitiated			(2)

}

AMFAccessType	::=	ENUMERATED

{

threeGPPAccess					(1),

nonThreeGPPAccess				(2),

threeGPPandNonThreeGPPAccess	(3)

}

FiveGMMCause	::= INTEGER (0..255)

Table 6.2.2-2: Payload for XIRIAMFDeregistrationMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| deregistrationDirection | Indicates whether the deregistration was initiated by the network or by the UE | M |
| accessType | Indicates the access for which the deregistration is handled, see 3GPP TS 24.501 [13] , clause 9.11.3.20. | M |
| supi | SUPI associated with the deregistration (see clause 6.2.2.4), if available | C |
| suci | SUCI used in the deregistration, if available (see NOTE 1) | C |
| pei | PEI used in the deregistration, if available (see NOTE 1) | C |
| gpsi | GPSI associated to the deregistration, if available as part of the subscription profile | C |
| guti | 5G-GUTI used in the deregistration, if available, see 3GPP TS 24.501 [13], clause 5.5.2.2.1 (see Note 1) | C |
| cause | Indicates the 5GMM cause value for network-initiated deregistration, see 3GPP TS 24.501 [13], clause 9.11.3.2 | C |
| location | Location information determined by the network during the deregistration, if available | C |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, if available | C |

NOTE 1: At least one among SUCI, PEI and GUTI shall be provided.

##### 6.2.2.2.4	Location Update

The IRI-POI in the AMF shall generate an AMFLocationUpdateMessage each time the IRI-POI present in an AMF detects that the target’s UE location is updated due to target’s UE mobility (e.g. in case of Xn based inter NG-RAN handover).  The generation of such xIRI may be omitted if the updated UE location information is already included in other xIRIs (e.g. mobility registration) provided by the IRI-POI present in the same AMF at the same time. The message shall contain the following payload, encoded as per clause 5.3.2.

AMFLocationUpdateMessage ::= SEQUENCE

{

-- See clause 6.2.2.2.4 for details of this structure

supi				[1]	SUPI,

suci				[2]	SUCI				OPTIONAL,

pei					[3]	PEI					OPTIONAL,

gpsi				[4] GPSI				OPTIONAL,

guti				[5]	GUTI				OPTIONAL

location			[6] LocationInformation,

non3GPPAccessEndpoint 		[7] UEEndpointAddress 			OPTIONAL

}

Table 6.2.2-3: Payload for AMFLocationUpdateMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| supi | SUPI associated with the location update (see clause 6.2.2.4) | M |
| suci | SUCI associated with the location update, if available, see 3GPP TS 24.501 [13]. | C |
| pei | PEI associated with the location update, if available. | C |
| gpsi | GPSI associated with the location update, if available as part of the subscription profile. | C |
| guti | 5G-GUTI associated with the location update, if available, see 3GPP TS 24.501 [13]. | C |
| location | Updated location information determined by the network. | M |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, if available | C |

##### 6.2.2.2.5	Start Of Interception With Registered UE

The IRI-POI in the AMF shall generate an XIRIAMFStartOfInterceptionWithRegisteredUEMessage when the IRI-POI present in the AMF detects that interception is activated on a target UE that has already been registered in the 5GS (see clause 6.2.2.4 on Identity Privacy). The message shall contain the following payload, encoded as per clause 5.3.2.

XIRIAMFStartOfInterceptionWithRegisteredUEMessage ::= SEQUENCE

{

-- See clause 6.2.2.2.5 for details of this structure

registrationResult			[1]		AMFRegistrationResult,

registrationType			[2]		AMFRegistrationType			OPTIONAL,

requestedSliceInformation	[3]		RequestedSliceInformation 	OPTIONAL,

supi						[4]		SUPI,

suci						[5] 	SUCI						OPTIONAL,

pei							[6] 	PEI,

gpsi						[7] 	GPSI						OPTIONAL,

guti						[8]		GUTI,

location					[9]		LocationInformation 	OPTIONAL,

non3GPPAccessEndpoint 		[10] 	UEEndpointAddress 		OPTIONAL,

timeOfRegistration			[11]	Timestamp 				OPTIONAL

}

AMFRegistrationResult	::= ENUMERATED

{

threeGPPAccess					(1),

nonThreeGPPaccess				(2),

threeGPPAndNonThreeGPPAccess	(3)

}

AMFRegistrationType	::=	ENUMERATED

{

initial		(1),

mobility	(2),

periodic	(3),

emergency	(4)

}

Table 6.2.2-4: Payload for XIRIAMFStartOfInterceptionWithRegisteredUEMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| registrationResult | Specifies the result of registration, see 3GPP TS 24.501 [13], clause 9.11.3.6 | M |
| registrationType | Specifies the type of registration, see 3GPP TS 24.501 [13] clause 9.11.3.7, if available | C |
| requestSliceInformation | Slice requested for the registration, if available Editor’s Note – need to define valid format / values for this. This may be different to normal registration case. | C |
| supi | SUPI associated with the registration (see clause 6.2.2.4) | M |
| suci | SUCI used in the registration | C |
| pei | PEI provided by the UE during the registration | M |
| gpsi | GPSI obtained in the registration, if available as part of the subscription profile. | C |
| guti | 5G-GUTI provided as outcome of initial registration or used in other cases, see 3GPP TS 24.501 [13], clause 5.5.1.2.2. | M |
| location | Location information, if available | C |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, if available | C |
| timeOfRegistration | Time at which the last registration occurred, if available | C |

##### 6.2.2.2.6	AMF Unsuccessful Procedure

The IRI-POI in the AMF shall generate an XIRIAMFUnsuccessfulProcedure when the IRI-POI present in the AMF detects an unsuccessful procedure for a UE matching one of the target identifiers provided via LI_X1.

Unsuccessful registration shall be reported only if the target UE has been successfully authenticated.

The message shall contain the following payload, encoded as per clause 5.3.2.

XIRIAMFUnsuccessfulProcedure ::= SEQUENCE

{

-- See clause 6.2.2.2.6 for details of this structure

failedprocedureType			[1] FailedAMFProcedureType,

failurecause				[2]	AMFFailureCause,

requestedSliceInformation	[3]	NSSAI	OPTIONAL,

supi						[4]	SUPI						OPTIONAL,

suci						[5] SUCI						OPTIONAL,

pei							[6] PEI							OPTIONAL,

gpsi						[7] GPSI						OPTIONAL,

guti						[8]	GUTI						OPTIONAL,

location					[9]	LocationInformation 		OPTIONAL,

non3GPPAccessEndpoint 		[10] UEEndpointAddress 			OPTIONAL

}

FailedAMFProcedureType ::= ENUMERATED

{

registration            (1),

deregistration          (2),

locationupdate          (3),

sms                     (4),

pDUSessionEstablishment (5)

}

AMFFailureCause	::= CHOICE

{

fiveGMMCause 	[1]		FiveGMMCause,

fiveGSMCause	[2]		FiveGSMCause

}

FiveGMMCause	::= INTEGER (0..255)

FiveGSMCause	::= INTEGER (0..255)

Table 6.2.2-5: Payload for XIRIAMFUnsuccessfulProcedure

| Field name | Description | M/C/O |  |
| --- | --- | --- | --- |
| failedprocedureType | Specifies the procedure which failed at the AMF | M |  |
| failurecause | Provides the value of the 5GSM or 5GMM cause, see 3GPP TS 24.501 [13], clauses 9.11.3.2 and 9.11.4.2. | M |  |
| requestedSliceInformation | Slice requested for the procedure, if available, given as an NSSAI (a list of S-NSSAI values as described in 3GPP TS 24.501 [13] clause 9.11.3.37). | C |  |
| supi | SUPI associated with the procedure, if available (see Note) | C |  |
| suci | SUCI used in the procedure, if applicable and if available (see Note) | C |  |
| pei | PEI used in the procedure, if available (see Note) | C |  |
| gpsi | GPSI used in the procedure, if available (see Note) | C |  |
| guti | 5G-GUTI used in the procedure, if available, see 3GPP TS 24.501 [13], clause 9.11.3.4 (see Note) | C |  |  |
| location | Location information determined during the procedure, if available | C |  |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, if available |  | C |  |

Note: At least one identity shall be provided, the others shall be provided if available.

#### 6.2.2.3	Generation of IRI over LI_HI2

When an AMF xIRI message is received over LI_X2, the MDF2 shall emit an IRI message over LI_HI2 without undue delay.

The timestamp field of the psHeader structure shall be set to the time that the AMF event was observed (i.e. the Timestamp field of the X2 PDU).

The payload of the xIRI messages shall be used as the payload of the IRI contents.

When additional warrants are activated on a target UE, MDF2 shall be able to generate and deliver the Start of Interception with Already Registered UE message to the LEMF associated with the additional warrants without receiving a corresponding xIRI.

#### 6.2.2.4	Identity Privacy

The AMF shall ensure for every registration (including re-registration) that SUPI has been provided by the UDM to the AMF and that the SUCI to SUPI mapping has been verified as defined in TS 33.501 [11]. This shall be performed regardless of whether the SUPI is a target of interception, and whether the null encryption algorithm is used for the SUCI. The AMF shall maintain the SUPI to SUCI mapping for at least the lifetime of the registration in order to allow interception on SUPI after the initial registration.

END OF CHANGES