3GPP TSG-SA WG3 LI Meeting #72	S3i190062

Sophia Antipolis (FRA), Jan 22nd - Jan 25th 2019

Source:	NTAC

Title:	pCR to resolve Editor’s Notes in 33.128 Clause 6.2.3 (LI at SMF/UPF)

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	FS_15LIS

Abstract of the contribution: This pCR proposes text to resolve the Editor’s Notes in 33.128 in clause 6.2.3 (LI at SMF/UPF)

FIRST CHANGE (Clause 6.2.3)

### 6.2.3	LI for SMF/UPF

Editor’s Note – we need to specify how correlation numbers are managed.

#### 6.2.3.1	Provisioning of SMF over LI_X1

The IRI-POI and CC-TF present in the SMF are provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2. If an SMF and UPF are implemented as a single function, then this interface may be sufficient to provision both (see section 6.2.3.3).

The POI in the SMF shall support the following Target Identifier Formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 is not used):

- SUPI.

- PEI.

- GPSI.

#### 6.2.3.2	Generation of xIRI at SMF over LI_X2

##### 6.2.3.2.1	General

The IRI-POI present in the SMF shall emit X2 PDUs over LI_X2 for each of the events described in the following clauses.

##### 6.2.3.2.2	PDU Session Establishment

The IRI-POI in the SMF shall generate an XIRISMFPDUSessionEstablishmentMessage when the IRI-POI present in the SMF detects that a PDU session has been established for the target UE. The message shall contain the following payload, encoded as per clause 5.3.2.

EDITOR’S NOTE: The ASN.1 structures are currently in the body of the text (this makes drafting and pCRs easier, as the ASN.1 is next to the tables which give the full definition of the information). For the final published version it is agreed that the ASN.1 structures will be put together i.e. all in one place. It is agreed that there will be only one place in which the published ASN.1 exists

EDITOR’S NOTE: The EDITOR’S NOTEs on the following parameters have been put into the Table below, not inline with the ASN.1.

XIRISMFPDUSessionEstablishmentMessage ::= SEQUENCE

{

-- See clause 6.2.3.2.2 for details of this structure

supi					[0]	SUPI 							OPTIONAL,

supiUnauthenticated 	[1] SUPIUnauthenticatedIndication	OPTIONAL,

pei						[2] PEI 							OPTIONAL,

gpsi					[3] GPSI 							OPTIONAL,

pduSessionID			[4] PDUSessionID,

gtpTunnelID				[5] FTEID,

pduSessionType				[6] 	PDUSessionType,

sNSSAI					[7] SNSSAI 							OPTIONAL,

ueEndpoint				[8] SEQUENCE OF UEEndpointAddress 	OPTIONAL,

non3GPPAccessEndpoint 	[9] UEEndpointAddress 				OPTIONAL,

locationInformation		[10] 	Location 						OPTIONAL,

dnn						[11] DNN,

amfID					[12] AMFID							OPTIONAL,

hSMFURI					[13] HSMFURI						OPTIONAL,

requestType				[14] FiveGSMRequestType,

accessType				[15] AMFAccessType 					OPTIONAL,

ratType					[16] RATType						OPTIONAL,

smPDUDNRequest			[17] SMPDUDNRequest 				OPTIONAL

}

PDUSessionType ::= ENUMERATED

{

ipv4 			(1),

ipv6 			(2),

ipv4v6 			(3),

unstructured		(4),

ethernet		(5)

}

PDUSessionID ::= INTEGER (0..255)

SNSSAI ::= SEQUENCE

{

sliceServiceType 	[0] INTEGER (0..255),

sliceDifferentiator	[1]	OCTET STRING (SIZE(3)) OPTIONAL

}

FTEID ::= SEQUENCE

{

teid		[0] INTEGER (0.. 4294967295),

ipv4Address	[1] IPv4Address OPTIONAL,

ipv6Address	[2] IPv6Address OPTIONAL

}

UEEndpointAddress ::= CHOICE

{

ipv4Address		[1] Ipv4Address,

ipv6Address		[2] Ipv6Address,

ethernetAddress	[3] MACAddress

}

SMPDUDNRequest ::= OCTET STRING

DNN ::= UTF8STRING

SUPIUnauthenticatedIndication ::= BOOLEAN

AMFID ::= SEQUENCE

{

amfRegionID		[1] INTEGER (0..255),

amfSetID		[2] INTEGER (0..1023),

amfPointer		[3] INTEGER (0..63)

}

HSMFURI ::= UTF8String

RATType ::= ENUMERATED

{

nr(0),

eutra(1),

wlan(2),

virtual(3)

}

FiveGSMRequestType ::= ENUMERATED

{

initialRequest(0),

existingPDUSession(1),

initialEmergencyRequest(2),

existingEmergencyPDUSession(3),

modificationRequest(4),

reserved(5)

}

Table 6.2.3-1: Payload for XIRISMFPDUSessionEstablishmentMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| supi | SUPI associated with the PDU session (e.g. as provided by the AMF in the associated Nsmf_PDU_Session_CreateSMContext service operation). Shall be present except for PEI-only unauthenticated emergency sessions. | C |
| supiUnauthenticated | Shall be present if a SUPI is present in the message, and set to “true” if the SUPI has not authenticated, or “false” if it has been authenticated. | C |
| pei | PEI associated with the PDU session if available | C |
| gpsi | GPSI associated with the PDU session if available | C |
| pduSessionID | PDU Session ID as assigned by the AMF, as defined in TS 24.007 [14] clause 11.2.3.1b Editor’s Note (2018-12-05)– This appears to be only one byte long (and actually only uses four bits of that byte); is this the correct reference? | M |
| gtpTunnelID | Contains the F-TEID identifying the GTP tunnel used to encapsulate the traffic, as defined in TS 29.244 [15] clause 8.2.3. Non-GTP encapsulation is for further study. | M |
| pduSessionType | Identifies selected PDU session type, see TS 24.501 clause 9.11.4.11. | M |
| sNSSAI | Slice identifier associated with the PDU session, if available. See TS 23.003 [16] clause 28.4.2 and TS 23.501 [11] clause 5.12.2.2. | C |
| ueEndpoint | UE endpoint address(es) if available | C |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, if available | C |
| locationInformation | Location information provided by the AMF, if available | C |
| dnn | Data Network Name associated with the target traffic, as defined in TS 23.003 [16] clause 9A and described in TS 23.501 [11] clause 4.3.2.2 | M |
| amfID | Identifier of the AMF associated with the target UE, as defined in TS 23.003 [z] clause 2.10.1, if available. | C |
| hSMFURI | URI of the Nsmf_PDUSession service of the selected H-SMF, if available. See TS 29.502 clause 6.1.6.2.2 | C |
| requestType | Type of request as described in 3GPP TS 24.501 [13] clause 9.11.3.47 if available. | C |
| accessType | Access type associated with the session (i.e. 3GPP or non-3GPP access) if provided by the AMF (see 3GPP TS 24.501 [13] clause 9.11.3.11). | C |
| ratType | RAT Type associated with the access if provided by the AMF as part of session establishment (see TS 23.502 [4] clause 4.3.2). Values given as per TS 29.571 [17] clause 5.4.3.2 | C |
| smPDUDNRequest | Contents of the SM PDU DN Request container, if available, as described in TS 24.501 [13] clause 9.11.4.15 | C |

At least one of the SUPI, PEI or GPSI fields shall be present.

##### 6.2.3.2.3	PDU Session Modification

The IRI-POI in the SMF shall generate an XIRISMFPDUSessionModificationMessage when the IRI-POI present in the SMF detects that a PDU session has been modified for the target UE. The message shall contain the following payload, encoded as per clause 5.3.2.

XIRISMFPDUSessionModificationMessage ::= SEQUENCE

{

-- See clause 6.2.3.2.3 for details of this structure

supi						[0]	SUPI 							OPTIONAL,

supiUnauthenticated		[1] SUPIUnauthenticatedIndication	OPTIONAL,

pei						[2] PEI 							OPTIONAL,

gpsi					[3] GPSI 							OPTIONAL,

sNSSAI					[4] SNSSAI 							OPTIONAL,

non3GPPAccessEndpoint 	[5] UEEndpointAddress 				OPTIONAL,

locationInformation		[6]	Location 						OPTIONAL,

requestType				[7] FiveGSMRequestType,

accessType				[8] AMFAccessType 					OPTIONAL,

ratType					[9] RATType 						OPTIONAL

}

Table 6.2.3-2: Payload for XIRISMFPDUSessionModificationMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| supi | SUPI associated with the PDU session (e.g. as provided by the AMF in the associated Nsmf_PDU_Session_CreateSMContext service operation). Shall be present except for PEI-only unauthenticated emergency sessions. | C |
| supiUnauthenticated | Shall be present if a SUPI is present in the message, and set to “true” if the SUPI was not authenticated, or “false” if it has been authenticated. | C |
| pei | PEI associated with the PDU session if available | C |
| gpsi | GPSI associated with the PDU session if available | C |
| sNSSAI | Slice identifier associated with the PDU session, if available. See TS 23.003 [16] clause 28.4.2 and TS 23.501 [11] clause 5.12.2.2. | C |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, if available | C |
| locationInformation | Location information provided by the AMF, if available | C |
| accessType | Access type associated with the session (i.e. 3GPP or non-3GPP access) if provided by the AMF (see 3GPP TS 24.501 [13] clause 9.11.3.11). | C |
| ratType | RAT Type associated with the access, if available. Values given as per TS 29.571 [17] clause 5.4.3.2 | C |

##### 6.2.3.2.4	PDU Session Release

The IRI_POI in the SMF shall generate an XIRISMFPDUSessionReleaseMessage when the IRI-POI present in the SMF detects that a PDU Session has been released for the target UE. The message shall contain the following payload, encoded as per clause 5.3.2.

XIRISMFPDUSessionReleaseMessage ::= SEQUENCE

{

-- See clause 6.2.3.3.4 for details of this structure

supi				[0]	SUPI,

pei					[1] PEI OPTIONAL,

gpsi				[2] GPSI OPTIONAL,

pduSessionID		[3] PDUSessionID,

timeOfFirstPacket	[4] GeneralizedTime OPTIONAL,

timeOfLastPacket	[5] GeneralizedTime OPTIONAL,

uplinkVolume		[6] INTEGER OPTIONAL,

downlinkVolume		[7] INTEGER OPTIONAL,

locationInformation	[8]	Location OPTIONAL

}

Table 6.2.3-3: Payload for XIRISMFPDUSessionReleaseMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| supi | SUPI associated with the PDU session | M |
| pei | PEI associated with the PDU session if available | C |
| gpsi | GPSI associated with the PDU session if available | C |
| pduSessionID | PDU Session ID as assigned by the AMF | M |
| timeOfFirstPacket | Time of first packet as reported in the Usage Report IE if available in the UPF deletion response (see TS 29.244 [15] clause 7.5.7.2) | C |
| timeOfLastPacket | Time of last packet as reported in the Usage Report IE if available (see TS 29.244 [15] clause 7.5.7.2) | C |
| uplinkVolume | Number of uplink octets, as reported in the Volume Report IE if available (see TS 29.244 [15] clause 7.5.7.2) | C |
| downlinkVolume | Number of downlink octets, as reporting the Volume Report IE if available (see TS 29.244 [15] clause 7.5.7.2) | C |
| locationInformation | Location information, if available. | O |

##### 6.2.3.2.5	Start of Interception with an Established PDU Session

The IRI_POI in the SMF shall generate an XIRISMFStartOfInterceptionWithEstablishedPDUSessionMessage when the IRI-POI present in the SMF detects that a PDU session has already been established for the target UE when interception starts. The message shall contain the following payload, encoded as per clause 5.3.2.

XIRISMFStartOfInterceptionWithEstablishedPDUSessionMessage ::= SEQUENCE

{

-- See clause 6.2.3.3.5 for details of this structure

supi					[0]	SUPI 							OPTIONAL,

supiUnauthenticated 	[1] SUPIUnauthenticatedIndication	OPTIONAL,

pei						[2] PEI 							OPTIONAL,

gpsi					[3] GPSI 							OPTIONAL,

pduSessionID			[4] PDUSessionID,

gtpTunnelID				[5] FTEID,

pduSessionType			[6]	PDUSessionType,

sNSSAI					[7] SNSSAI 							OPTIONAL,

ueEndpoint				[8] SEQUENCE OF UEEndpointAddress,

non3GPPAccessEndpoint 	[9] UEEndpointAddress 				OPTIONAL,

locationInformation		[10] Location 						OPTIONAL,

dnn						[11] DNN,

amfID					[12] AMFID							OPTIONAL,

hSMFURI					[13] HSMFURI						OPTIONAL,

requestType				[14] FiveGSMRequestType,

accessType				[15] AMFAccessType 					OPTIONAL,

ratType					[16] RATType 						OPTIONAL,

smPDUDNRequest			[17] SMPDUPDNRequest				OPTIONAL

}

Table 6.2.3-4: Payload for XIRISMFStartOfInterceptionWithEstablishedPDUSessionMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| supi | SUPI associated with the PDU session (e.g. as provided by the AMF in the associated Nsmf_PDU_Session_CreateSMContext service operation). Shall be present except for PEI-only unauthenticated emergency sessions. | C |
| supiUnauthenticated | Shall be present if a SUPI is present in the message, and set to “true” if the SUPI has not authenticated, or “false” if it has been authenticated. | C |
| pei | PEI associated with the PDU session if available | C |
| gpsi | GPSI associated with the PDU session if available | C |
| pduSessionID | PDU Session ID as assigned by the AMF, as defined in TS 24.007 [14] clause 11.2.3.1b | M |
| gtpTunnelID | Contains the F-TEID identifying the tunnel used to encapsulate the traffic, as defined in TS 29.244 [15] clause 8.2.3. Non-GTP encapsulation is for further study. | M |
| pduSessionType | Identifies selected PDU session type, see TS 24.501 clause 9.11.4.11. | M |
| sNSSAI | Slice identifier associated with the PDU session, if available. See TS 23.003 [16] clause 28.4.2 and TS 23.501 [11] clause 5.12.2.2. | C |
| ueEndpoint | UE endpoint address(es) if available | C |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, if available | C |
| locationInformation | Location information provided by the AMF at session establishment, if available | C |
| dnn | Data Network Name associated with the target traffic, as defined in TS 23.003 [16] clause 9A and described in TS 23.501 [11] clause 4.3.2.2 | M |
| amfID | Identifier of the AMF associated with the target UE, as defined in TS 23.003 [z] clause 2.10.1. | M |
| hSMFURI | URI of the Nsmf_PDUSession service of the selected H-SMF, if available. See TS 29.502 clause 6.1.6.2.2 | C |
| requestType | Type of request as described in 3GPP TS 24.501 [13] clause 9.11.3.47 if available. | C |
| accessType | Access type associated with the session (i.e. 3GPP or non-3GPP access) if provided by the AMF (see 3GPP TS 24.501 [13] clause 9.11.3.11). | C |
| ratType | RAT Type associated with the access if provided by the AMF as part of session establishment (see TS 23.502 [4] clause 4.3.2). Values given as per TS 29.571 [17] clause 5.4.3.2 | C |
| smPDUDNRequest | Contents of the SM PDU DN Request container, if available, as described in TS 24.501 clause 9.11.4.15 | C |

#### 6.2.3.3	Triggering of the UPF from SMF over LI_T3

When interception of communication contents is required, the CC-TF present in the SMF sends a trigger to the CC-POI  present in the UPF over the LI_T3 interface.

When the CC-TF in the SMF detects that a PDU session has been established for a target UE, it shall send an activation message to the CC-POI in the UPF over the LI_T3 interface. The activation message shall contain the correlation identifiers that the CC-POI in the UPF shall use when emitting xCC. This can be achieved by sending an ActivateTask message as defined in TS 103 221-1 [7] clause 6.2.1 with the following details:

Table 6.2.3-5: ActivateTask message for triggering the CC-POI in the UPF

| TS 103 221-1 Field name | Value | M/C/O |
| --- | --- | --- |
| XID | Set to the same XID associated with the interception in the SMF. NOTE: It is for further study how the CC-TF and IRI-POI in the SMF agree on the XID. | M |
| TargetIdentifiers | Packet detection criteria as determined by the CC-TF in the SMF, which enables the UPF to isolate target traffic. The UPF CC-POI shall support at least the following identifier types: GTP Tunnel ID IPv4 address IPv6 address NOTE: This value is the target identifier for the UPF CC-POI, and may be different from the target identifier specified in the warrant. | M |
| DeliveryType | Set to “X3Only” | M |
| ListOfDIDs | Delivery endpoints for LI_X3. These delivery endpoints shall be configured by the CC-TF in the SMF using the CreateDestination message as described in TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| CorrelationNumber | Correlation number to assign to X3 PDUs generated by the CC-POI in the UPF | M |

When the CC-TF in the SMF detects that a targeted PDU session has changed in a way which requires changes to the interception by the CC-POI in the UPF, the CC-TF  shall modify the interception at the CC-POI in the UPF over the LI_T3 interface. This is achieved by sending an ModifyTask message as defined in TS 103 221-1 [7] clause 6.2.2 with the following details:

Table 6.2.3-6: ModifyTask message for updating interception at the CC-POI in the UPF

| Field name | Value | M/C/O |
| --- | --- | --- |
| XID | Set to the XID associated with the interception | M |
| TargetIdentifiers | Updated packet detection criteria as determined by the CC-TF in the SMF. NOTE: See notes on TargetIdentifiers in Table 6.2.3-5 | M |

When the CC-TF in the SMF detects that the PDU session has been released for a target UE, it shall send a deactivation message to the CC-POI in the UPF over the LI_T3 interface. When using TS 103 221-1 [7] this is achieved by sending a DeactivateTask message with the XID field set to the XID associated with the interception, as described in TS 103 221-1 [7] clause 6.2.3.

#### 6.2.3.4	Generation of xIRI at UPF over LI_X2

Editor’s Note – Someone who knows about Packet Data Header Reporting probably ought to write this…

Editor’s Note – also need a section on use of LI_T2 to trigger Packet Data Header Reporting in the UPF.

#### 6.2.3.5	Generation of xCC at UPF over LI_X3

The CC-POI present in the UPF shall emit X3 PDUs over LI_X3 for each IP packet matching the criteria specified in the Triggering message received over LI_T3 from the CC-TF in the SMF.

NOTE: Implementers are reminded of the completeness and non-duplication requirements (see TS 33.127 [5]).

Editor’s Note: What do we do with packets that reached the UPF on N6 from the DN, but were not delivered to the UE?

Each X3 PDU shall contain the contents of the GTP-U packet given using the GTP-U payload format.

#### 6.2.3.6	Generation of IRI over LI_HI2

When an SMF xIRI PDU is received over LI_X2, the MDF2 shall send an IRI message over LI_HI2 without undue delay.

The timestamp field of the TS 102 232-1 [6] PSHeader structure shall be set to the time that the AMF event was observed (i.e. the Timestamp field of the LI_X2 PDU). The LIID and CID fields shall correctly reflect the target identity and communication session to which the IRI belongs.

The threeGPPIRIContents field shall be populated with the BER-encoded PDU.

#### 6.2.3.7	Generation of CC over LI_HI3

When a UPF xCC PDU is received over LI_X3, the MDF3 shall emit a CC PDU over LI_HI3 without undue delay.

The timestamp field of the TS 102 232-1 [6] PSHeader structure shall be set to the time that the UPF observed the data (i.e. the Timestamp field of the LI_X3 PDU). The LIID and CID fields shall correctly reflect the target identity and communication session to which the CC PDU belongs.

The threeGPPCCContents field shall be populated with the BER-encoded PDU as follows.

UPFPDURecord ::= OCTET STRING

END OF CHANGES