3GPP TSG-SA WG3 LI Meeting #72	S3i190063

Sophia Antipolis (FRA), Jan 22nd - Jan 25th 2019

Source:	NTAC

Title:	pCR to resolve Editor’s Notes in 33.128 Clause 6.2.5 (LI at SMSF)

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	FS_15LIS

Abstract of the contribution: This pCR proposes text to resolve the Editor’s Notes in 33.128 in clause 6.2.3 (LI at SMF/UPF)

FIRST CHANGE (Clause 6.2.3)

#### 6.2.5.3	SMS

The IRI-POI in the SMSF shall generate an xIRI SMS for each SMS message.

The XIRISMSMessage shall contain the following payload, encoded as per clause 5.3.2.

XIRISMSMessage ::= SEQUENCE

{

- See clause 6.2.5.3 for details of this structure

originatingSMSParty		[1] SMSParty,

terminatingSMSParty		[2] SMSParty,

direction				[3] SMSDirection,

transferStatus			[4] SMSTransferStatus,

otherMessage			[5] SMSOtherMessageIndication 		OPTIONAL,

location				[6]	LocationInformation 				OPTIONAL,

otherNFAddress			[7] SMSNFAddress 				OPTIONAL,

otherNFType				[8] SMSNFType 					OPTIONAL,

smsTPDUData							[9] SMSTPDUData

}

SMSParty ::= SEQUENCE

{

supi	[0] SUPI 	OPTIONAL,

pei		[1] PEI		OPTIONAL,

gpsi	[2] GPSI	OPTIONAL

}

SMSDirection ::= ENUMERATED

{

toTarget 	(0),

fromTarget 	(1)

}

SMSTransferStatus ::=  ENUMERATED

{

transferSucceeded 	(0),

transferFailed		(1),

undefined 			(2)

}

SMSOtherMessageIndication ::= BOOLEAN

SMSNFAddress ::= CHOICE

{

ipAddress	[0] IPAddress,

e164Number	[1] E164Number
}

SMSNFType ::= ENUMERATED

{

sms-GMSC	(0),

iwMSC		(1),

sms-Router	(2)

}

SMSTPDUData ::= CHOICE

{

smsTPDU [0] SMSTPDU

}

SMSTPDU ::= OCTET STRING (SIZE(1..270))

Table 6.2.5-1: Payload for XIRISMSMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| originatingSMSParty | Identity of the originating SMS party | M |
| terminatingSMSParty | Identity of the terminating SMS party | M |
| direction | Direction of the SMS with respect to the target | M |
| transferStatus | Indicates whether the transfer succeeded or not | M |
| otherMessage | In the event of a server-initiated transfer, indicates whether the server will send another SMS. May be omitted if the transfer is target-initiated. | M |
| otherNetworkFunctionAddress | Address of the other network function (SMS-GMSC/IWMSC/SMS-Router) involved in the communication of the SMS, if available | C |
| otherNetworkFunctionType | Type of the other network function (SMS-GMSC/IWMSC/SMS-Router) involved in the communication of the SMS, if available | C |
| location | Location information associated with the UE sending or receiving the SMS, if available | C |
| smsTPDUData | SMS TPDU, encoded as per TS 23.040 [18] clause 9. EDITOR’S NOTE: The IRI-POI in the SMSF is required to support suppression of TP-User-Data, but the mechanism for handing this over is FFS. | M |

END OF CHANGES