3GPP TSG-SA WG3 LI Meeting #72	S3i190066

Sophia Antipolis (France), Jan 22nd – 25th, 2019                                         (revision of S3i190030)

Source:	Ericsson

Title:	pCR to draft TS 33.128: PEI availability in registration xIRIs

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	FS_15LIS

Abstract of the contribution: This pCR proposes to consider PEI conditionally available in registration related xIRIs

Discussion:

In the current version of TS 33.128, PEI is considered mandatory in Registration and Start of Interception with registered UE xIRIs. However, there are scenario where PEI could be not available; this may happen, e.g. in case the UE does not support any 3GPP access or in case the use of PEI for LI is not required by national regulations. In order to cope with these cases and ensure that the xIRIs can be correctly reported even in case PEI is missing, PEI is proposed to be reported under the condition that it is available.

**** First Change ****

##### 6.2.2.2.1	Registration

The IRI_POI in the AMF shall generate an xIRI AMF Registration message when the IRI-POI present in the AMF detects that a UE matching one of the target identifiers provided via LI_X1 has successfully registered to the 5GS via 3GPP NG-RAN or non-3GPP access. Accordingly, the IRI_POI in the AMF generates the xIRI AMF Registration message when one of the following events are detected:

- AMF sends a REGISTRATION ACCEPT message to the target UE over the N1 reference point and the UE registration state within the AMF is changed to 5GMM-REGISTERED.

- When the REGISTRATION ACCEPT message is sent to the target UE includes the 5G-GUTI, AMF receives the REGISTRATION COMPLETE message from the target UE over the N1 reference point and the UE registration state within the AMF is 5GMM-REGISTERED.

The message shall contain the following payload, encoded as per clause 5.3.2.

XIRIAMFRegistrationMessage ::= SEQUENCE

{

- See clause 6.2.2.2.1 for details of this structure

registrationType			[1] AMFRegistrationType,

registrationResult			[2]	AMFRegistrationResult,

sliceInformation	[3]	SliceInformation	OPTIONAL,

supi						[4]	SUPI,

suci						[5] SUCI 						OPTIONAL,

pei							[6] PEI							OPTIONAL,

gpsi						[7] GPSI 						OPTIONAL,

guti						[8]	GUTI,

location					[9]	LocationInformation 		OPTIONAL

}

AMFRegistrationType	::=	ENUMERATED

{

initial		(1),

mobility	(2),

periodic	(3),

emergency	(4)

}

AMFRegistrationResult	::= ENUMERATED

{

threeGPPAccess					(1),

nonThreeGPPAccess				(2),

threeGPPAndNonThreeGPPAccess	(3)

}

SliceInformation	::= SEQUENCE

{

allowedNSSAI					(1)	NSSAI			OPTIONAL,

configuredNSSAI					(2)	NSSAI			OPTIONAL,

rejectedNSSAI					(3)	RejectedNSSAI	OPTIONAL

}

Editor’s Note: TS 24.501 does not provide the ASN.1 definition for the referenced parameter types (e.g. NSSAI and rejected NSSAI). So, we will have to define it locally in the TS 33.128, or define as OCTET STRING and use CT spec value.

Editor’s Note – We may choose to refactor the ASN.1 into a single module later, but for now we will write each in the relevant section to make them easier to discuss. At that point we can also consider any other factorisation (e.g. grouping fields together) subject to keeping the number of OPTIONAL fields to a minimum.

Editor’s Note – we may want to qualify whether each field is directly observed or obtained from some internal state within the NF associated with the POI.

Editor’s Note – we also need to indicate which identifier (e.g. SUPI) was used to isolate/identify the traffic at the POI?

Editor’s Note – Need to consider impacts of unauthenticated emergency calls and PARLOS. This editor’s note also applies to other events.

Table 6.2.2-1: Payload for AMF Registration xIRI message

| Field name | Description | M/C/O |
| --- | --- | --- |
| registrationType | Specifies the type of registration, see 3GPP TS 24.501 [13] clause 9.11.3.7. This is derived from the information received from the UE in the REGISTRATION REQUEST message. | M |
| registrationResult | Specifies the result of registration, see 3GPP TS 24.501 [13] clause 9.11.3.6. | M |
| sliceInformation | Provide, if available, one or more of the following: - allowed NSSAI (see 3GPP TS 24.501 [13] clause 9.11.3.37) - configured NSSAI (see 3GPP TS 24.501 [13] clause 9.11.3.37) - rejected NSSAI (see 3GPP TS 24.501 [13] clause 9.11.3.46) This is derived from the information sent to the UE in the REGISTRATION ACCEPT message. | C |
| supi | SUPI associated with the registration (see clause 6.2.2.4) | M |
| suci | SUCI used in the registration | C |
| pei | PEI provided by the UE during the registration, if available | C |
| gpsi | GPSI obtained in the registration, if available as part of the subscription profile. | C |
| guti | 5G-GUTI provided as outcome of initial registration or used in other cases, see 3GPP TS 24.501 [13] clause 5.5.1.2.2. | M |
| location | Location information determined by the network during the registration, if available | C |

**** Next Change ****

##### 6.2.2.2.4	Start Of Interception With Registered UE

The IRI_POI in the AMF shall generate an xIRI AMF Start of Interception with Registered UE message when the IRI-POI present in the AMF detects that interception is activated on a target UE that has already been registered in the 5GS (see clause 6.2.2.4 on Identity Privacy). The message shall contain the following payload, encoded as per clause 5.3.2.

XIRIAMFStartOfInterceptionWithRegisteredUEMessage ::= SEQUENCE

{

- See clause 6.2.2.2.1 for details of this structure

registrationResult			[1]		AMFRegistrationResult,

registrationType			[2]		AMFRegistrationType			OPTIONAL,

requestedSliceInformation	[3]		RequestedSliceInformation 	OPTIONAL,

supi						[4]		SUPI,

suci						[5] 	SUCI						OPTIONAL,

pei							[6] 	PEI							OPTIONAL,

gpsi						[7] 	GPSI						OPTIONAL,

guti						[8]		GUTI,

location					[9]		LocationInformation 		OPTIONAL,

timeOfRegistration			[10]	Timestamp 					OPTIONAL

}

AMFRegistrationResult	::= ENUMERATED

{

threeGPPAccess					(1),

nonThreeGPPaccess				(2),

threeGPPAndNonThreeGPPAccess	(3)

}

AMFRegistrationType	::=	ENUMERATED

{

initial		(1),

mobility	(2),

periodic	(3),

emergency	(4)

}

Editor’s note: Type definitions above have been duplicated for easier check, needs to be removed from the final version.

Table 6.2.2-4: Payload for AMF Start Of Interception With Registered UE xIRI message

| Field name | Description | M/C/O |
| --- | --- | --- |
| registrationResult | Specifies the result of registration, see 3GPP TS 24.501 [13], clause 9.11.3.6 | M |
| registrationType | Specifies the type of registration, see 3GPP TS 24.501 [13] clause 9.11.3.7, if available | C |
| requestSliceInformation | Slice requested for the registration, if available Editor’s Note – need to define valid format / values for this. This may be different to normal registration case. | C |
| supi | SUPI associated with the registration (see clause 6.2.2.4) | M |
| suci | SUCI used in the registration | C |
| pei | PEI provided by the UE during the registration, if available | C |
| gpsi | GPSI obtained in the registration, if available as part of the subscription profile. | C |
| guti | 5G-GUTI provided as outcome of initial registration or used in other cases, see 3GPP TS 24.501 [13], clause 5.5.1.2.2. | M |
| location | Location information, if available | C |
| timeOfRegistration | Time at which the last registration occurred, if available | C |

**** End of Changes ****