3GPP TSG-SA WG3 LI Meeting #72 Bis	S3i190108

Dusseldorf (Germany), Apr 9th – 12th, 2019

Source:	Ericsson

Title:	pCR to draft TS 33.128: Corrections and Stage 3 description for unsuccessful procedures at AMF

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	FS_15LIS

Abstract of the contribution: Derigistration and location update should not be considered unsuccessful at the AMF. Stage 3 text for triggering XIRIAMFUnsuccessfulProcedure added.

Discussion

In the current version of TS 33.128, the IRI-POI at the AMF is required to generate an XIRI when the AMF detects un unsuccessful procedure for the target UE.

However, some specific cases currently listed need further consideration.

- Deregistration: According to TS 24.501, no scenario where this procedure fails can be identified.

- Location Update: By definition, the related XIRI is defined to report cases where the AMF receives updated location information, not provided as part of other xIRI. So, the unsuccessful case seems also not applicable

Based on that, it is proposed to delete the cases above from the XIRI definition.

In addition, this p-CR adds stage 3 text for triggering the XIRI.

**** First Change ****

##### 6.2.2.2.6	AMF unsuccessful procedure

The IRI-POI in the AMF shall generate an XIRIAMFUnsuccessfulProcedure when the IRI-POI present in the AMF detects an unsuccessful procedure for a UE matching one of the target identifiers provided via LI_X1.

Unsuccessful registration shall be reported only if the target UE has been successfully authenticated.

Accordingly, the IRI-POI in the AMF generates the XIRIAMFUnsuccessfulProcedure when one of the following events are detected:

-	AMF sends a N1: REGISTRATION REJECT message to the target UE and the UE 5G Mobility Management (5GMM) state within the AMF is changed to 5GMM-DEREGISTERED.

-	AMF aborts a registration procedure before the UE 5G Mobility Management (5GMM) state within the AMF is changed to 5GMM-REGISTERED.

-	AMF sends a SERVICE REJECT message to the target UE including a PDU session establishment reject message type.

-	AMF aborts a UE-initiated NAS transport procedure with payload container type IE is set to "SMS".

Table 6.2.2-5: Payload for XIRIAMFUnsuccessfulProcedure

| Field name | Description | M/C/O |
| --- | --- | --- |
| failedprocedureType | Specifies the procedure which failed at the AMF. | M |
| failurecause | Provides the value of the 5GSM or 5GMM cause, see TS 24.501 [13], clauses 9.11.3.2 and 9.11.4.2. | M |
| requestedSliceInformation | Slice requested for the procedure, if available, given as a NSSAI (a list of S-NSSAI values as described in TS 24.501 [13] clause 9.11.3.37). | C |
| supi | SUPI associated with the procedure, if available (see Note). | C |
| suci | SUCI used in the procedure, if applicable and if available (see Note). | C |
| pei | PEI used in the procedure, if available (see Note). | C |
| gpsi | GPSI used in the procedure, if available (see Note). | C |
| guti | 5G-GUTI used in the procedure, if available, see TS 24.501 [13], clause 9.11.3.4 (see Note). | C |  |
| location | Location information determined during the procedure, if available. | C |

NOTE:		At least one identity shall be provided, the others shall be provided if available.

**** Next Change ****

Annex A (normative): Structure of the Internal Interface

….
….

FailedAMFProcedureType ::= ENUMERATED

{

registration            (1),

sms                     (2),

pDUSessionEstablishment (3)

}

**** End of Changes ****