3GPP TSG-SA WG3 LI Meeting #72 Bis	S3i190156

Dusseldorf (Germany), Feb 26th – 28th, 2019                                               (revision of S3i190127)

Source:	Ericsson

Title:	Unsuccessful procedures at SMF

Document for:	Agreement

Agenda Item:	2

Work Item / Release:	FS_15LIS

Abstract of the contribution: This pCR proposes to add a new xIRI event to report unsuccessful procedures at the SMF.

Discussion

In the current version of TS 33.128, there is no IRI message specified to report the occurrence of unsuccessful procedure at the SMF. This contribution proposes to add a new subclause and the related ASN.1 definition.

**** First Change ****

##### 6.2.2.2.6	SMF unsuccessful procedure

The IRI-POI in the SMF shall generate an XIRISMFUnsuccessfulProcedure when the IRI-POI present in the SMF detects an unsuccessful procedure or error condition for a UE matching one of the target identifiers provided via LI_X1.

Accordingly, the IRI-POI in the SMF generates the XIRISMFUnsuccessfulProcedure when one of the following events are detected:

-	SMF sends a PDU SESSION ESTABLISHMENT REJECT message to the target UE.

-	SMF sends a PDU SESSION MODIFICATION REJECT message to the target UE.

-	SMF sends a PDU SESSION RELEASE REJECT message to the target UE.

-	SMF receives a PDU SESSION MODIFICATION COMMAND REJECT message from the target UE.

-	An ongoing SM procedure is aborted at the SMF, due to e.g. a 5GSM STATUS message sent from or received by the SMF.

Table 6.2.2-5: Payload for XIRISMFUnsuccessfulProcedure

| Field name | Description | M/C/O |
| --- | --- | --- |
| failedProcedureType | Specifies the procedure which failed or is aborted at the SMF. | M |
| failureCause | Provides the value of the 5GSM cause, see TS 24.501 [13], clause 9.11.4.2. In case the procedure is aborted due to a 5GSM STATUS message, the 5GSM cause is the one included in the 5GSM status message. | M |
| requestedSliceInformation | Slice requested for the procedure, if available, given as a NSSAI (a list of S-NSSAI values as described in TS 24.501 [13] clause 9.11.3.37). | C |
| initiator | Specifies whether the network (SMF) or the UE is initiating the rejection or indicating the failure. | M |
| supi | SUPI associated with the procedure, if available (see Note). | C |
| supiUnauthenticated | Shall be present if a SUPI is present in the message and set to “true” if the SUPI has not authenticated, or “false” if it has been authenticated. | C |
| pei | PEI used in the procedure, if available (see Note). | C |
| gpsi | GPSI used in the procedure, if available (see Note). | C |
| pduSessionID | PDU Session ID See clause 9.4 of TS 24.501 [13], if available | C |  |
| ueEndpoint | UE endpoint address(es) if available. | C |  |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, if available. | C |  |
| locationInformation | Location information provided by the AMF, if available. | C |  |
| dnn | Data Network Name associated with the target traffic, as defined in TS 23.003 [19] clause 9A and described in TS 23.501 [2] clause 4.3.2.2, if available. | C |  |
| amfID | Identifier of the AMF associated with the target UE, as defined in TS 23.003 [19] clause 2.10.1 when available. | C |  |
| hSMFURI | URI of the Nsmf_PDUSession service of the selected H-SMF, if available. See TS 29.502 [16] clause 6.1.6.2.2. | C |  |
| requestType | Type of request as described in TS 24.501 [13] clause 9.11.3.47 if available. | C |  |
| accessType | Access type associated with the session (i.e. 3GPP or non-3GPP access) if provided by the AMF (see TS 24.501 [13] clause 9.11.3.11). | C |  |
| ratType | RAT Type associated with the access if provided by the AMF as part of session establishment (see TS 23.502 [4] clause 4.3.2). Values given as per TS 29.571 [17] clause 5.4.3.2. | C |  |
| smPDUDNRequest | Contents of the SM PDU DN Request container, if available, as described in TS 24.501 [13] clause 9.11.4.15. | C |  |

##### NOTE:		At least one identity shall be provided, the others shall be provided if available.

**** Next Change ****

Annex A (normative): Structure of the Internal Interface

….
….

XIRISMFUnsuccessfulProcedure ::= SEQUENCE

{

-- See clause X.X.X.X.X for details of this structure

failedProcedureType			[1] 	FailedSMFProcedureType,

failureCause				[2]		FiveGSMCause,

initiator					[3]		Initiator,

requestedSliceInformation	[4]		NSSAI							OPTIONAL,

supi						[5]		SUPI							OPTIONAL,

supiUnauthenticated 		[6] 	SUPIUnauthenticatedIndication	OPTIONAL,

pei							[7] 	PEI								OPTIONAL,

gpsi						[8] 	GPSI							OPTIONAL,

pduSessionID				[9] 	PDUSessionID,					OPTIONAL,

ueEndpoint					[10] 	SEQUENCE OF UEEndpointAddress	OPTIONAL,

non3GPPAccessEndpoint	 	[11] 	UEEndpointAddress 				OPTIONAL,

dnn							[12]	DNN								OPTIONAL,

amfID						[13] 	AMFID							OPTIONAL,

hSMFURI						[14] 	HSMFURI							OPTIONAL,

requestType					[15] 	FiveGSMRequestType,				OPTIONAL,

accessType					[16] 	AMFAccessType 					OPTIONAL,

ratType						[17] 	RATType 						OPTIONAL,

smPDUDNRequest				[18] 	SMPDUDNRequest 					OPTIONAL,

locationInformation			[19]	Location				 		OPTIONAL

}

FailedSMFProcedureType ::= ENUMERATED

{

pDUSessionEstablishment	            (1),

pDUSessionModification					(2),

pDUSessionRelease							(3)

}

Initiator ::=	ENUMERATED

{

uE				(1),

network			(2),

unknown			(3)

}

**** End of Changes ****