3GPP SA3LI#72 BIS	s3i190162 (was s3i190152)

Dusseldorf, DE (Germany), 26-28 February 2019

Source:	OTD

Title:	pCR on TS33.128 PDHR/PDSR Contribution

Document for:	Agreement

Agenda Item:	 10

Work Item / Release:	LI15

Abstract of the contribution: Interception of Packet Data Headers, modified due to comments from Wednesday, Feb 27th meeting.

#### <<<<<<<<<<<<<<<<<<<< FIRST CHANGE >>>>>>>>>>>>>>>>>>>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: " System Architecture for the 5G System".

[3]	3GPP TS 33.126: " Lawful Interception Requirements".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 33.127: "Lawful Interception (LI) Architecture and Functions".

[6]	ETSI TS 103 120: " Lawful Interception (LI); Interface for warrant information".

[7]	ETSI TS 103 221-1: " Lawful Interception (LI); Part 1: Internal Network Interface X1 for Lawful Interception".

[8]	ETSI TS 103 221-2: "Lawful Interception: Internal Network Interface X2/X3"

[9]	ETSI TS 102 232-1: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 1: Handover specification for IP delivery"

[10]	ETSI TS 102 232-7: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 7: Service-specific details for Mobile Services"

[11]	3GPP TS 33.501: "Security Architecture and Procedures for the 5G System"

[12]	3GPP TS 33.108: "3G security; Handover interface for Lawful Interception (LI)"

[13]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS)".

[14]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General Aspects".

[15]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

[16]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[17]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[18]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[19]	3GPP TS 23.003: " Numbering, addressing and identification ".

[20]	OMA-TS-MLP: "Open Mobile Alliance; Mobile Location Protocol, Version 3.4", OMA-TS-MLP-V3-4-20150512-A

[21]	IETF RFC 815, "IP DATAGRAM REASSEMBLY ALGORITHMS".

[22]	IETF RFC 2460, "Internet Protocol, Version 6 (IPv6) Specification".

[23]	IETF RFC 793, "TRANSMISSION CONTROL PROTOCOL".

[24]	IETF RFC 768, "User Datagram Protocol".

[25]	IETF RFC 4340, "Datagram Congestion Control Protocol (DCCP)".

[26]	IETF RFC 4960, "Stream Control Transmission Protocol".

[27]	IANA (www.iana.org) Assigned Internet Protocol Numbers, "Protocol Numbers".

[28]	IETF RFC 6437, "IPv6 Flow Label Specification".

[29] 	IETF RFC 791, "Internet Protocol".

#### <<<<<<<<<<<<<<<<<<<< END of FIRST CHANGE >>>>>>>>>>>>>>>>>>>>

#### <<<<<<<<<<<<<<<<<<<< SECOND CHANGE >>>>>>>>>>>>>>>>>>>>

##### 6.2.3.3	Triggering of the UPF from SMF over LI_T3

When interception of communication contents is required, the CC-TF present in the SMF sends a trigger to the CC-POI present in the UPF over the LI_T3 interface.

When the CC-TF in the SMF detects that a PDU session has been established for a target UE, it shall send an activation message to the CC-POI in the UPF over the LI_T3 interface. The activation message shall contain the correlation identifiers that the CC-POI in the UPF shall use when emitting xCC. This can be achieved by sending an ActivateTask message as defined in TS 103 221-1 [7] clause 6.2.1 with the following details:

Table 6.2.3-5: ActivateTask message for triggering the CC-POI in the UPF

| TS 103 221-1 Field name | Description | M/C/O |
| --- | --- | --- |
| XID | Set to the same XID associated with the interception in the SMF | M |
| TargetIdentifiers | Packet detection criteria as determined by the CC-TF in the SMF, which enable the UPF to isolate target traffic. The UPF CC-POI shall support at least the following identifier types: - GTP Tunnel ID - IPv4 address - IPv6 address NOTE: This value is the target identifier for the UPF CC-POI, and may be different from the target identifier specified in the warrant. | M |
| DeliveryType | Set to “X3Only” | M |
| ListOfDIDs | Delivery endpoints for LI_X3. These delivery endpoints shall be configured by the CC-TF in the SMF using the CreateDestination message as described in TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| CorrelationNumber | Correlation ID to assign to X3 PDUs generated by the CC-POI in the UPF | M |

When the CC-TF in the SMF detects that a targeted PDU session has changed in a way which requires changes to the interception by the CC-POI in the UPF, the CC-TF shall modify the interception at the CC-POI in the UPF over the LI_T3 interface. This is achieved by sending an ModifyTask message as defined in TS 103 221-1[ [7] clause 6.2.2 with the following details:

Table 6.2.3-6: ModifyTask message for updating interception at the CC-POI in the UPF

| Field name | Description | M/C/O |
| --- | --- | --- |
| XID | Set to the XID associated with the interception | M |
| TargetIdentifiers | Updated packet detection criteria as determined by the CC-TF in the SMF. NOTE: – See notes on TargetIdentifiers in Table 6.2.3-5 | M |

When the CC-TF in the SMF detects that the PDU session has been released for a target UE, it shall send a deactivation message to the CC-POI in the UPF over the LI_T3 interface. When using TS 103 221-1 [7] this is achieved by sending a DeactivateTask message with the XID field set to the XID associated with the interception, as described in TS 103 221-1 [7] clause 6.2.3.

#### 6.2.3.4 UPF IRI-POI triggering over LI_T2

When interception of Packet Data Headers is required, the SMF IRI-TF sends a trigger to the UPF IRI-POI over the LI_T2 interface.

When the SMF IRI-TF detects that a PDU session has been established for a target UE, it shall send an activation message to the UPF IRI-POI over the LI_T2 interface. The activation message shall contain the correlation identifiers that the UPF IRI-POI shall use when emitting xIRI. This shall be achieved by sending an ActivateTask message as defined in TS 103 221-1 [7] clause 6.2.1 with the following details:

Table 6.2.3-6: ActivateTask message for triggering the UPF IRI-POI

| TS 103 221-1 Field name | Description | M/C/O |
| --- | --- | --- |
| XID | Set to the same XID associated with the interception in the SMF | M |
| TargetIdentifiers | Packet detection criteria as determined by the SMF IRI-TF, which enable the UPF IRI-POI to isolate target traffic. The UPF IRI-POI shall support at least the following identifier types: - GTP Tunnel ID - IPv4 address - IPv6 address NOTE: This value is the target identifier for the UPF IRI-POI, and may be different from the target identifier specified in the warrant. | M |
| DeliveryType | Set to “X2Only” | M |
| TaskDetailsExtensions/ HeaderReporting | Header reporting-specific tag to be carried in the TaskDetailsExtensions field of TS 103 221-1 | M |
| ListOfDIDs | Delivery endpoints for LI_X2. These delivery endpoints shall be configured by the SMF IRI-TF using the CreateDestination message as described in TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| CorrelationNumber | Correlation ID to assign to LI_X2 PDUs generated by the IRI-POI in the UPF | M |

The following table adds new necessary parameters to trigger packet header reporting.

Table 6.2.3-7: ActivatePDHReporting Parameters

| Field name | Description | M/C/O |
| --- | --- | --- |
| pDHType | This field shall be set to either: - “PDHR,” for packet-by-packet reporting - “PDSR,” for summarized reporting | M |
| pDSRType | If pDHType is PDSR, this field shall be set to one of the following triggers: - timer expiry (along with a timer value and unit) - packet count (along with a value for the number of packets detected before a summary is to be triggered) - byte count (along with a value for the cumulative byte size reached across all packets belonging to the summary before said summary is to be triggered) Summary reports shall not be cumulative, i.e., each summary report shall describe only the packets contained in its respective range, and each new summary shall start its count (of whichever attribute from the numbered list above applies) from zero, i.e., the information in the (n+1)’th summary report starts immediately after the end of the n’th summary report. | C |

When the SMF IRI-TF detects that a targeted PDU session has changed in a way which requires changes to the interception by the UPF IRI-POI, the SMF IRI-TF shall modify the interception at the UPF IRI-POI over the LI_T2 interface. This is achieved by sending an ModifyTask message as defined in TS 103 221-1[ [7] clause 6.2.2 with the following details:

Table 6.2.3-8: ModifyTask message for updating interception at the IRI-POI in the UPF

| Field name | Description | M/C/O |
| --- | --- | --- |
| XID | Set to the XID associated with the interception | M |
| TargetIdentifiers | Updated packet detection criteria as determined by the SMF IRI-TF. NOTE: – See notes on TargetIdentifiers in Table 6.2.3-6 | M |

When the SMF IRI-POI detects that the PDU session has been released for a target UE, it shall send a deactivation message to the UPF IRI-POI over the LI_T2 interface. When using TS 103 221-1 [7] this is achieved by sending a DeactivateTask message with the XID field set to the XID associated with the interception, as described in TS 103 221-1 [7] clause 6.2.3.

#### <<<<<<<<<<<<<<<<<<<< END of SECOND CHANGE >>>>>>>>>>>>>>>>>>>>

#### <<<<<<<<<<<<<<<<<<<< THIRD CHANGE >>>>>>>>>>>>>>>>>>>>

#### 6.2.3.5	Generation of xIRI at UPF over LI_X2

#### 6.2.3.4.1 Packet data header reporting

The UPF IRI-POI generates packet data header information either in per-packet form, as Packet Data Header Reports (PDHRs), or in summary form, as Packet Data Header Summary Reports (PDSRs).

##### 6.2.3.4.2 Fragmentation

If the UPF IRI-POI is placed on a link which fragmented the original IP packet (see IETF RFC 791[29] for basic fragmentation rules, and IETF RFC 815 [21] for more complex re-assembly rules), a situation may occur in which only the first fragment can be sensibly reported in a PDHR, while the subsequent fragments may be missing essential fields that are mandatory, which may cause simplistic implementations to mis-report them, or omit them altogether.

In this case, the UPF CC-POI shall report the first fragment of a fragmented IP packet, including the port numbers when they are included within this first fragment, using the length of the fragment to determine if the port numbers are indeed encoded within this first fragment. The following fragments are reported without port information. This technique relieves the UPF from having to reassemble the original IP packet (at line speed) at the cost of accuracy of the reported fields.

##### 6.2.3.4.3 Packet Data Header Reporting (PDHR)

If the per-packet form of packet data header reporting, i.e. PDHR, is used, the UPF IRI-POI extracts the following information from each packet:

Table 6.2.3-9: PDHR information extracted by the UPF IRI-POI

| Field name | Description | M/C/O |
| --- | --- | --- |
| pDUSessionID | The PDU session ID received from the SMF IRI-TF. | M |
| sourceIPAddress | Shall contain the source address of the packet from the 32-bit “Source Address” field in IPv4, as defined in IETF RFC 791 [29], or from the 128-bit “Source Address” field in IPv6, as defined in IETF RFC 2460 [22]. | M |
| sourcePortNumber | Shall contain the “Source Port” number that indicates an application or service running on top of the transport, if the “Protocol” IP field (see the nextLayerProtocol field below in this table) is one of: - Transmission Control Protocol (TCP), IP “Protocol” field decimal “6”; see TRANSMISSION CONTROL PROTOCOL [23]. - User Datagram Protocol (UDP), IP “Protocol” field decimal “17”; see User Datagram Protocol [24]. - Datagram Congestion Control Protocol (DCCP), IP “Protocol” field decimal “33”; see Datagram Congestion Control Protocol [25]. - Stream Control Transmission Protocol (SCTP), IP “Protocol” field decimal “132”; Stream Control Transmission Protocol [26]. For further details on Layer four protocols, see the IANA Assigned Internet Protocol Numbers [27]. | C |
| destinationIpAddress | Shall contain the destination address of the packet from the 32-bit “Destination Address” field in IPv4, as defined in IETF RFC 791 [29], or from the 128-bit “Destination Address” field, as defined in IETF RFC 2460 [22]. | M |
| destinationPortNumber | Shall contain the “Destination Port” number that indicates an application or service running on top of the transport, if the “Protocol” IP field (see the nextLayerProtocol field below in this table) is one of: - Transmission Control Protocol (TCP), IP “Protocol” field decimal “6”; see TRANSMISSION CONTROL PROTOCOL [23]. - User Datagram Protocol (UDP), IP “Protocol” field decimal “17”; see User Datagram Protocol [24]. - Datagram Congestion Control Protocol (DCCP), IP “Protocol” field decimal “33”; see Datagram Congestion Control Protocol [25]. - Stream Control Transmission Protocol (SCTP), IP “Protocol” field decimal “132”; Stream Control Transmission Protocol [26]. For further details on Layer four protocols, see the IANA Assigned Internet Protocol Numbers [27]. | C |
| iPNextLayerProtocol | Shall contain the contents of the IP “Protocol” field as defined in IETF RFC 791 [29] (bits 72..79 in the IP header), and is one of the assigned Internet protocol numbers defined in IANA Assigned Internet Protocol Numbers [27]. | M |
| flowLabel | If the IP addresses in the report are IPv6, this field shall contain the 20-bit IPv6 “Flow Label” as defined in: - IPv6 IETF RFC 2460 [22], and - IPV6 Flow Label Specification IETF RFC 6437 [28]. | C |
| direction | Shall contain the direction of the intercepted packet, and it indicates either “from target” or “to target.” | M |
| packetSize | Shall contain the value of the “Total Length” IP header field if IPv4 is used, as defined in IETF RFC 791 [29], or the value of the “Payload Length” field if IPv6 is used, as defined in IETF RFC 2460 [22]. | M |

##### 6.2.3.4.4 Packet Data Summary Reporting (PDSR)

If the summary form of the packet data header reporting, i.e. PDSR, is used, the UPF IRI-POI extracts from each packet the following information, and aggregates it in summaries:

Table 6.2.3-10: PDSR information extracted by the UPF IRI-POI

| Field name | Description | M/C/O |
| --- | --- | --- |
| pDUSessionID | The PDU session ID received from the SMF IRI-TF. | M |
| sourceIPAddress | Shall contain the source address of the packet from the 32-bit “Source Address” field in IPv4, as defined in IETF RFC 791 [29], or from the 128-bit “Source Address” field in IPv6, as defined in IETF RFC 2460 [22]. | M |
| sourcePortNumber | Shall contain the “Source Port” number that indicates an application or service running on top of the transport, if the “Protocol” IP field (see the nextLayerProtocol field below in this table) is one of: - Transmission Control Protocol (TCP), IP “Protocol” field decimal “6”; see TRANSMISSION CONTROL PROTOCOL [23]. - User Datagram Protocol (UDP), IP “Protocol” field decimal “17”; see User Datagram Protocol [24]. - Datagram Congestion Control Protocol (DCCP), IP “Protocol” field decimal “33”; see Datagram Congestion Control Protocol [25]. - Stream Control Transmission Protocol (SCTP), IP “Protocol” field decimal “132”; Stream Control Transmission Protocol [26]. For further details on Layer four protocols, see the IANA Assigned Internet Protocol Numbers [27]. | C |
| destinationIpAddress | Shall contain the destination address of the packet from the 32-bit “Destination Address” field in IPv4, as defined in IETF RFC 791 [29], or from the 128-bit “Destination Address” field, as defined in IETF RFC 2460 [22]. | M |
| destinationPortNumber | Shall contain the “Destination Port” number that indicates an application or service running on top of the transport, if the “Protocol” IP field (see the nextLayerProtocol field below in this table) is one of: - Transmission Control Protocol (TCP), IP “Protocol” field decimal “6”; see TRANSMISSION CONTROL PROTOCOL [23]. - User Datagram Protocol (UDP), IP “Protocol” field decimal “17”; see User Datagram Protocol [24]. - Datagram Congestion Control Protocol (DCCP), IP “Protocol” field decimal “33”; see Datagram Congestion Control Protocol [25]. - Stream Control Transmission Protocol (SCTP), IP “Protocol” field decimal “132”; Stream Control Transmission Protocol [26]. For further details on Layer four protocols, see the IANA Assigned Internet Protocol Numbers [27]. | C |
| iPNextLayerProtocol | Shall contain the contents of the IP “Protocol” field as defined in IETF RFC 791 [29] (bits 72..79 in the IP header), and is one of the assigned Internet protocol numbers defined in IANA Assigned Internet Protocol Numbers [27]. | M |
| flowLabel | If the IP addresses in the report are IPv6, this field shall contain the 20-bit IPv6 “Flow Label” as defined in IPv6 [22] and the IPV6 Flow Label Specification IETF RFC 6437 [28]. | C |
| direction | Shall contain the direction of the intercepted packet, and it indicates either “from target” or “to target.” | M |
| summaryTrigger | Shall contain the trigger that caused the summary report to be emitted, which is one of the following: - timer expiry - packet count - byte count | M |
| firstPacketTimestamp | Shall contain the timestamp that represents the time that the UPF IRI-POI detected the first packet in the set represented by this summary. | M |
| lastPacketTimestamp | Shall contain the timestamp that represents the time that the UPF IRI-POI detected the last packet in the set represented by this summary. | M |
| packetCount | Shall contain the number of packets detected during the creation of this summary | M |
| byteCount | Shall contain the number of bytes summed across all packets that belong to this summary. For IPv4 it is the sum of the “Total Length” fields across all packets in the summary as defined in Internet Protocol IETF RFC 791[29], while for IPv6 it is the sum of the “Payload Length” fields across all packets in the summary as defined in Internet Protocol, Version 6 (IPv6) Specification, IETF RFC 2460 [22]. | M |

#### <<<<<<<<<<<<<<<<<< END of THIRD CHANGE >>>>>>>>>>>>>>>>>>>

#### <<<<<<<<<<<<<<<<<<<< FOURTH CHANGE >>>>>>>>>>>>>>>>>>>>

#### 6.2.3.1	Provisioning of SMF over LI_X1

The IRI-POI and CC-TF present in the SMF are provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2. If an SMF and UPF are implemented as a single function, then this interface may be sufficient to provision both (see section 6.2.3.3).

The POI in the SMF shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used):

-	SUPI.

-	PEI.

-	GPSI.

If packet header reporting is required, parameters specified in table 6.2.3-7: ActivatePDHReporting Parameters shall be provided as part of the LI_X1 provisioning message.

#### <<<<<<<<<<<<<<<<<<<< END of FOURTH CHANGE >>>>>>>>>>>>>>>>>>>>

#### <<<<<<<<<<<<<<<<<<<< FIFTH CHANGE >>>>>>>>>>>>>>>>>>>>

# Annex A (normative): Structure of the Internal Interface

PDHR ::= SEQUENCE {

pDUSessionID			[0] PDUSessionID,

sourceIPAddress			[1]	IPAddress,

sourcePort				[2]	INTEGER (SIZE(0..65535))	OPTIONAL, -- 16 bits

destinationIPAddress	[3]	IPAddress,

destinationPort			[4]	INTEGER (SIZE(0..65535))	OPTIONAL, -- 16 bits

nextLayerProtocol		[5] INTEGER (SIZE(0..255)),			      --  8 bits

iPv6flowLabel			[6] INTEGER (SIZE(0..1048575))	OPTIONAL, -- 20 bits

direction				[7] Direction,

packetSize				[8] INTEGER

}

PDSR ::= SEQUENCE {

pDUSessionID			[0] PDUSessionID,

sourceIPAddress			[1]	IPAddress,

sourcePort				[2]	INTEGER (SIZE(0..65535))	OPTIONAL, -- 16 bits

destinationIPAddress	[3]	IPAddress,

destinationPort			[4]	INTEGER (SIZE(0..65535))	OPTIONAL, -- 16 bits

nextLayerProtocol		[5] INTEGER (SIZE(0..255)),               --  8 bits

iPv6flowLabel			[6] INTEGER (SIZE(0..1048575))	OPTIONAL, -- 20 bits

direction				[7] Direction,

pDSRSummaryTrigger		[8] PDSRSummaryTrigger,

firstPacketTimestamp	[9] Timestamp,

lastPacketTimestamp		[10] Timestamp,

packetCount				[11] INTEGER,

byteCount				[12] INTEGER

}

PDSRSummaryTrigger ::= ENUMERATED {

timerExpiry	(0),

packetCount	(1),

byteCount	(2)

}

IPAddress	::=	CHOICE {

ipv4 	[0] OCTET STRING(SIZE(4)),

ipv6	[1]	OCTET STRING(SIZE(16))

}

Direction ::= ENUMERATED {

fromTarget	(0),

toTarget	(1)

}

#### <<<<<<<<<<<<<<<<<<<< END of FIFTH CHANGE >>>>>>>>>>>>>>>>>>>>