3GPP TSG-SA3 Meeting #73-LI	s3i190235

# La Jolla (California), United States, 9th Apr 2019 - 12th Apr 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.126 | CR | 0006 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | ● |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Artificial Intelligence in network automation: draft rules related to LI in TS 33126 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ministère Economie et Finances |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LI16 |  |  |  |  |  | Date: |  |  | 2019-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Artificial Intelligence Network Automation / ETSI ISG Zero Touch Service Management Such automation could be used to complement 5GC and may impact LI 5G system. Some related rules could be added to TS 33.126 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Two draft proposed rules are: - to create in LI system, indication on target related meta data or modified CC that such AI system could induce - to avoid any modification or alteration by such system on LI / LALS processes, specially in confidentiality or integrity, as automation could detect LI/LALS as attack on the target. Automation design should take into account LI/LALS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory issues for MNOs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3; 6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | ● | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | ● | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | ● | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change****

## 6.3	Detect and Capture

R6.3 - 10	Access Level Interception - The CSP shall be able to perform network access level interception in both the core and on the edge of the network (e.g. IP-CAN level interception).

R6.3 - 20	Service Level Interception - The CSP shall be able to perform service level interception in both the core and on the edge of the network (e.g. IMS based VoIP).

R6.3 - 30	Multi Party Service Interception - CSP shall be able to report the multi-party service Interception Product of targeted group communications and its users.

R6.3 - 40	Third Party Assisted Services - If a CSP uses Third Parties as part of its service provision, the CSP shall be responsible for ensuring that the overall service complies with applicable LI regulations and requirements.

R6.3 - 50	Third Party ME or UE Interception - To the extent that a CSP manages a Third Party ME or UE, the CSP shall be able to report communications of such Third Party ME or UE (e.g. status of devices with a relay or forward function).

R6.3 - 60	Third Party ME or UE Users Interception - To the extent that a CSP manages Third Party ME or UE, the CSP shall be able to report communications of the end users connected to the CSP network via a Third Party ME or UE that is managed by the CSP (e.g. status of users communicating via ME or UE with a relay or forward function).

R6.3 - 70	Modification of services – Any change to any target service settings, as known to the CSP, shall be able to be reported.

R6.3 - 80	Multiple Services Per target - The CSP shall be able to simultaneously perform LI for multiple services for a given target.

R6.3 - 90	Multiple Targets - The CSP shall be able to simultaneously perform intercepts on multiple independent targets.

R6.3 - 100	Multiple LEAs - The CSP shall be able to simultaneously perform independent intercepts for any given target under different warrants.

R6.3 - 110	Roaming Targets - The visited CSP shall be able to perform interception of inbound roaming targets.

R6.3 - 120	Roaming – Outbound - The CSP shall be able to notify the LEA whenever the CSP becomes aware that the target has left, or entered, a visited network.

R6.3 - 130	Roaming – Inbound - The CSP shall be able to notify the LEA whenever the CSP becomes aware that the inbound roaming target has entered, or has left, the network.

R6.3 - 140	Serving CSP change - When the target changes serving CSP, the CSP that is served the warrant shall be able to provide the LEA with the identity of the new CSP if known.

R6.3 - 150	Roaming Identifiers Visited CSP - The visited CSP shall be able to obtain and validate the long term 3GPP identifiers of all inbound roamers from the home CSP regardless of the use of privacy mechanisms (based on roaming agreements).

R6.3 - 160	Roaming Identifiers Home CSP - The home CSP shall provide the long term 3GPP identifiers to the visited CSP for outbound roamers (based on roaming agreements).

R6.3 - 170	Outbound Roaming Home Network - CSPs shall be able to intercept its outbound roamers, if the communication pass through the home CSP's network.

R6.3 – 180	Access Network Identity - The CSP shall provide the LEA the identity of the 3GPP or non 3GPP Access Network as known by the CSP.

R6.3 - 190	Location - The CSP shall be able to obtain and report the location of the target.

R6.3 - 200	Location Triggers - The CSP shall be able to obtain and report the target location at certain network events associated with the target.

R6.3 - 210	Communication Location Reporting - The CSP shall be able to obtain and report the target location at start and end of communication, as well as during the communication including periodically and per event.

R6.3 - 220	Location Reporting - The CSP shall be able to obtain and report the target location for both active and idle MEs or UEs triggered either by UE-Action (e.g. UE cell site change) or on a periodic basis or on demand by the LEA.

R6.3 - 230	Location Reporting Independency - Location information may be reported as part of interception of a service (e.g. VoLTE, RCS), or independently.

R6.3 - 240	Location Accuracy - The CSP shall report the most accurate target location available to the CSP.

R6.3 - 250	Multiple Location Sources - The CSP shall be able to report the source of each location information report provided to the LEMF (e.g. cell site identifier, GPS).

R6.3 - 260	Location Positioning Methods - The CSP shall be able to report the positioning method used to obtain location information (e.g. network-based, UE-based, access-based).

R6.3 - 270	Additional Location Information - If the CSP has additional location information of the target beyond cell site identifier (e.g. altitude, civic address, geo-coordinates), the CSP shall be able to provide this.

R6.3 - 280	Location Senescence - The CSP shall provide information that indicates when the location was determined (e.g. age of location, timestamp).

R6.3 - 290	Trusted/Untrusted Location - The location information reported to the LEMF shall be location information trusted by the 3GPP network (i.e. the location information is either 3GPP network derived or verified), if available. The CSP shall also be able to report target location information from untrusted sources (e.g. user provided) in addition to or in absence of the trusted location information.

R6.3 - 300	Location Trust Indication - The CSP shall be able to indicate to the LEA whether the location information is trusted or untrusted.

R6.3 - 310	Projected Location - The CSP shall be able to indicate to the LEA whether the location information of the target is measured or possible.

R6.3 - 320	Non 3GPP access - For non 3GPP access the CSP shall be able to provide the identity and location of the non 3GPP access function serving the UE as known by the CSP.

R6.3 - 330	Roaming Location - In the case of inbound roaming, the visited CSP that was served a warrant shall be able to provide location information without assistance from the home CSP.

R6.3 - 340	Location Changes in the Visited Network - In the case of roaming, the home CSP that was served a warrant shall be able to provide location information as visible in the home network.

R6.3 - 350	Location Requests - The home CSP shall be able to provide notification of target-related location information requests received from outside the home network when these requests are visible to the home network as part of normal network operations.

R6.3 – 360	LCS Use - The CSP shall be able to use LCS, if available, in support of LALS for an LCS-targetable UE (with or without target LCS subscription).

R6.3 – 370	LALS Reporting – The CSP shall be able to provide on-demand and periodic LALS reports of the target's location independent of the target's communication state.

R6.3 - 380	Up-to-date LALS location - LALS shall report either the current (updated) location, or if the current location is unavailable the last known location of a target's UE.

R6.3 - 390	LALS failure notification - If the location is unavailable, LALS shall be able to report a failure reason, as to why the location is unavailable.

R6.3 - 400	Target specificity - The CSP shall ensure no communications are intercepted other than those of, or associated with, the target's equipment, facilities or services.

R6.3 - 410	Service specificity - The CSP shall ensure that only the communication services specified by the warrant are intercepted.

R6.3 – 420	Service Scope - All CSP based services shall be in scope of LI including mission critical services and non-mission critical services.

R6.3 - 430	Service Activation - The CSP shall report service activation.

R6.3 - 440	Service Invocation - The CSP shall report service invocation.

R6.3 - 450	Service Modification - The CSP shall report service modifications (e.g., changes to content, content descriptors, timing descriptors, group participation, copy of service content).

R6.3 - 460	Service Deactivation - The CSP shall report service deactivation.

R6.3 - 470	Service Up/Download - The CSP shall report service related uploading or downloading.

R6.3 - 480	Service Access Method - The CSP shall report the access method used by the target to interact with the service (e.g., via ME, UE or web).

R6.3 - 490	Early media - The CSP shall be able to intercept early media (e.g., CAT, CRS).

R6.3 - 500	Context Comprehensibility - The CSP shall include in Interception Product information that allows the LEA to establish the Context of Communications.

R6.3 - 510	Service Indication - The CSP shall include in Interception Product an indication of the communication service as known by the CSP network.

R6.3 - 520	Interdependency of IRI and CC - The CSP shall ensure IRI containing CC metadata is delivered in a timely and accurate manner such that it shall be possible to decode CC in real time.

R6.3 - 530	Reporting Post Session Established Digits - The CSP shall support extracting and reporting dialled digits after the session is established (e.g. user dialled, signalled) via the CSP services, on a per-warrant basis.

R6.3 - 540	Post Session Established Digit Reporting for IRI and CC Intercepts - The CSP shall be able to support extracting and reporting digits after the session is established for IRI-only intercepts, as well as for intercepts that report both IRI and CC.

R6.3 - 550	Toggle for Post Session Established Digit Extraction - The CSP shall support the Post Session Established Digit Extraction capability with a toggle feature that can activate/deactivate this capability, per warrant.

R6.3 - 560	Charging - The 3GPP system shall be able to generate LI charging event records.

R6.3 - 570	Automation Indication - The CSP shall be able to indicate to the LEA whether the origin of the LI/LALS meta data information or modified content is from automated processes controlled by Artificial Intelligence and not directly or indirectly by the target usages, to avoid evidence issue in the court without such dedicated indication.

**** Second Change****

## 6.6	Security

R6.6 - 10	Undetectability by the Target - The CSP shall perform interception in such a manner that the target is unable to detect interception is taking place, before, during, and after the interception.

R6.6 - 20	Undetectability by Other Users - The CSP shall perform interception in such a manner that no other users of CSP's services can detect that interception is taking place, before, during, and after the interception.

R6.6 - 30	Undetectability by Non-Authorized Parties - The CSP shall ensure that non unauthorized personnel or processes that are part of the service cannot detect that interception is taking place, before, during, and after interception.

R6.6 - 40	Undetectability across LEAs - The CSP shall perform interception in such a manner that no other LEA can detect that interception is taking place, before, during, and after interception.

R6.6 - 50	Undetectability across CSPs - The CSP shall be able to perform interception such that no CSP not obligated by the warrant can detect that interception is taking place.

R6.6 - 60	Undetectability Across Third Parties - The CSP shall be able to perform interception such that any Third Parties, not obligated by the warrant, cannot detect that interception is taking place.

R6.6 - 70	Undetectability Across Countries - The CSP shall ensure the performance of interception in one country cannot be detected in other countries.

R6.6 - 80	Interception Capability Undetectability - The CSP shall ensure that only authorized parties can have knowledge of operational use of interception capabilities, interception-related hardware and software.

R6.6 - 90	LI Failure Impact on Target Services - A failure of LI shall not impact the target's, or other users' services.

R6.6 - 100	Recordkeeping Access - The CSP's record retention policy shall ensure that LI records of the CSP's management of interception (e.g. log files) are only visible to, and accessible by, authorized personnel.

R6.6 - 110	Alteration Prevention and Detection - The CSP shall employ a mechanism (e.g. cryptographic hashing) to provide assurance that LI records of the CSP's management of interception (e.g. log files) cannot be unnoticeably altered.

R6.6 - 120	Authenticity - The delivery shall employ a mechanism to provide assurance of the authenticity of the delivered Interception Product from the CSP to the LEA.

R6.6 – 130	Confidentiality - The delivery shall employ a mechanism to provide assurance of the confidentiality of the Interception Product from the CSP to the LEA.

R6.6.- 140	Integrity - The delivery shall employ a mechanism to provide assurance that the Interception Product cannot be altered from the CSP to the LEA.

R6.6.- 150	Mutual authentication - The CSP and the LEA shall provide assurance that any communications between the CSP and LEA can mutually authenticate.

R6.6 - 160	Virtualization Security - When CSP networks are virtualized, the CSP LI implementation shall at a minimum comply with the NFV security requirements specified in ETSI GS NFV-SEC 012 System specification for execution of sensitive NFV components [2].

R6.6 - 170	Limited Security POI - Where interception cannot be implemented at a fully secure location, such that physical and logical security of the POI cannot be guaranteed, the CSP shall employ methods to reduce LI security risks.

R6.6 - 180	Network Automation Security – Network Automation, based on Artificial Intelligence shall be designed to let LI/LALS processes unaltered, unless agreed by the LEA, in order to keep its confidentiality towards unauthorized persons and to avoid to stop any processes of the LI lifecycle. The CSP shall employ methods to reduce LI security risks regarding to such tool(s).

**** Last Change ****