3GPP TSG-SA3 Meeting #73-LI	s3i190240

# La Jolla (California), United States, 9th Apr 2019 - 12th Apr 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0012 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service scoping missing stage two tables |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI15 |  |  |  |  |  | Date: |  |  | 2019-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As the clause is presently written, it is missing the necessary stage two tables, and is as such unimplementable in a standard fashion |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds the necessary stage two tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not approving the present CR will cause implementations to diverge, making them non-interoperable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.4 (very minor editorial), 5.2.4.6 (new clause that adds the necessary tables) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<< DISCUSSION >>>>>>>>>>>>>>>>>>>>>>>>>>>>

This contribution adds a table of new parameters to be included in each ActivateTask or ModifyTask mesasge used to trigger interception in any POI, TF, or MDF.

<<<<<<<<<<<<<<<<<<<<<<<<<< END OF DISCUSSION >>>>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<<<<<<<< START OF CHANGE >>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 5.2.4	Service scoping

#### 5.2.4.1	General

The CSP shall support the following specified options on a per-intercept (per LIID) basis, to ensure strict delivery of only the interception data that an LEA is authorised to receive. These options are not exclusive, i.e., none, one, or more than one item in any given category may be turned on for any given intercept (LIID).

#### 5.2.4.2	CSP service type

-	Voice.

-	Data.

-	Messaging (e.g. SMS/MMS).

-	Push-to-Talk (including MCPTT).

#### 5.2.4.3	Interception type

-	IRI.

-	CC.

#### 5.2.4.4	Location type

-	Report location at the beginning and end of a session.

-	Report location every time the network detects a change in target location (including location update with no physical change of location).

-	LALS.

#### 5.2.4.5	Roaming

-	Stop interception for non HPLMN RAN.

#### 5.2.4.6 Service Scoping Details

Tables 5.2.4.5-1, 5.2.4.5-2, and 5.2.4.5-3 introduce addional parameters to be appended to every ActivateTask or ModifyTask mesasge used to trigger interception in any POI, TF, or MDF:

Table 5.2.4.6-1: TaskDetails parameter of the ActivateTask or ModifyTask messages.

| ETSI TS 103 221-1 field name | Description | Format | M/C/O |
| --- | --- | --- | --- |
| XID | Uniquely identifies the task. | UUIDv4 | M |
| TargetIdentifiers | Packet detection criteria as determined by the IRI-TF in the SMF, which enable the UPF IRI-POI to isolate target traffic. The IRI-POI in the UPF shall support at least the following identifier types: - GTP Tunnel ID. - IPv4 address. - IPv6 address. NOTE: This value is the target identifier for the IRI-POI in the UPF, and may be different from the target identifier specified in the warrant. | see TS 103 221-1 | M |
| DeliveryType | Set to “X2Only” – IRI only, no CC | Enum | M |
| ListOfDIDs | Address of the corresponding MDF2. | String (?) (IPv4/6) | M |
| ListOfMediationDetails | See Table 6.2.2-2 | See Table 6.2.2-2 | M |
| CorrelationID | Correlation identifier to assign to intercepted material for this Task. Intended for use in triggering scenarios, and shall be ignored by non-mediation function NEs. | Unsigned Integer | M |
| ImplicitDeactivationAllowed | Disallowed (need explicit deactivation from ADMF) | Boolean | M |
| TaskDetailsExtensions | Not Used | N/A | O |

Table 5.2.4.6-2: Additional parameters used for triggering any POI, TF, or MDF using the MediationDetailsExtension extension mechanism for the ListOfMediationDetails parameter in the TaskDetails parameter of the ActivateTask or ModifyTask messages.

| Field name | Description | Format | M/C/O |
| --- | --- | --- | --- |
| LIID | Lawful Interception Identifier associated with the Task (multiple instances of this structure may be used to indicate that multiple LIIDs are associated with the Task) | LIID as defined in TS 103 280 [4] | M |
| DeliveryType | “HI2” – IRI only, no CC | Enum | M |
| StartTime | Start time associated with the activation of interception or mediation for the Task | Timestamp | O |
| EndTime | End time associated with the deactivation of interception or mediation for the Task | Timestamp | O |
| ListOfDIDs | Address of the LEMF | String (?) (IPv4/6) | M |
| MediationDetailsExtension | “serviceScoping” - Set to or more ServiceScopeDetail objects (see Table 6.2.2-3) | Enum | M |

Table 5.2.4.6-3: serviceScoping structure.

| Field name | Description | Format | M/C/O |
| --- | --- | --- | --- |
| serviceType | This field shall be set to NO MORE THAN ONE of: - “voice” - “data” - “messaging” - “pushToTalk” | Enum | M |
| interceptionType | This field shall be set to ONE OR MORE of: - “IRI” - “CC” | Enum | M |
| locationType | This field shall be set to ONE OR MORE of: - “beginningAndEnd” - “uponChange” - “LALS” | Enum | M |
| suspend | This field shall be set either to NULL or to: - “suspendOnOutboundInternationalRoaming” | Enum | M |

<<<<<<<<<<<<<<<<<<<<<<<<<< END OF CHANGE >>>>>>>>>>>>>>>>>>>>>>>>>>>>