3GPP TSG-SA3 Meeting #73-LI	s3i190243

# La Jolla (California), United States, 9th Apr 2019 - 12th Apr 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0015 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Branching UPF interception correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI15 |  |  |  |  |  | Date: |  |  | 2019-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The addition of the CC interception at PDU session anchor UPFs causes the clause in this CR to contain redundant and conflicting information, possibly causing incomplete or duplicate packet interception. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR corrects the affected clause to remove redundant and conflicting information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If this CR is not accepted, incomplete or duplicate packet interception may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.3.3 (assumes CR 14 was accepted, and changed the clause number from 6.2.3.3.2 -> 6.2.3.3.3. If CR14 was not accepted, the clause number this CR applies to is 6.2.3.3.2) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 33.128, CR 14 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Depending on the order the contributions are applied, the affected clause in the present CR15 may have been changed by CR14 from 6.2.3.3.2 to 6.2.3.3.3. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<< DISCUSSION >>>>>>>>>>>>>>>>>>>>>>>>>>>>

Requirement R6.2-180 in TS 33.126 [3] requires that the complete communications of a target be intercepted:

“R6.2 - 180          Completeness - The CSP shall be able to intercept all Target Communications as specified in the warrant."

Further, clause 6.2.3.6 Network topologies in TS 33.127 [5], also requires that the complete communications of a target be intercepted:

When a target UE's PDU session involves multiple Data Network (DN) connections, the generation and delivery of xCC shall be done in such a way that:

- All applicable user plane packets are captured and delivered.

As a consequence, if content interception is required for a given target, then all Data Network connections managed by the CSP for that target shall be intercepted. There is no case in which only some DNs are to be intercepted, while some are excluded. TS 33.128 should be modified to reflect this consequence of the completeness requirement.

<<<<<<<<<<<<<<<<<<<<<<<<<< END OF DISCUSSION >>>>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<<<<<<<< START OF CHANGE >>>>>>>>>>>>>>>>>>>>>>>>>>>>

##### 6.2.3.3.3	CC interception with multi-homed PDU session

When a target UE accesses multiple Data Networks (DNs) via a multi-homed PDU session (see TS 23.501 [2] clause 5.6.4.3), multiple UPFs are involved in providing the PDU Session Anchors, with one UPF providing the Branching Point functionality. The Branching Point UPF may, or may not, be a PDU Session Anchor UPF (see TS 33.127 [5] Annex A3.2). The CC-TF present in the SMF shall send the CC intercept trigger to the CC-POI present in an UPF if and only if that UPF is selected to provide the CC-POI functions.

When the target UE is involved in multi-homed PDU session, the CC-TF present in the SMF (i.e. in the SMF that establishes the PDU session) shall determine which UPF(s) is(are) more suitable to provide the CC-POI functions adhering to the following requirements specified in TS 33.127 [5]:

- All user plane packets are captured and delivered.

- Duplicate delivery of CC is suppressed to the extent possible.

This clause assumes that a PDU session contains only one Branching Point UPF (with N3 reference point toward the target UE) and one PDU Session Anchor UPF for each DN connection).

Since the CC interception of all DN connections is required, the SMF may choose either all the PDU Session Anchor UPFs or the Branching Point UPF to provide the CC-POI functions.

The Branching Point UPF may be chosen when all user plane packets pass through the Branching Point UPF, and the CC-TF present in the SMF may choose the Branching Point UPF to provide the CC -POI function and accordingly, send the CC interception trigger to the CC-POI present in the Branching Point UPF. The CC intercept trigger shall include the packet detection rules. An example of these rules is:

-	Generate the xCC from all the incoming and outgoing user plane packets to the target UE

In this case, the CC-TF present in the SMF shall not select any of the PDU Session Anchor UPFs to provide the CC-POI functions.

When a Branching Point UPF is chosen to provide the CC-POI functions, and if the Branching Point UPF is removed from the user plane path during a PDU session, then the CC POI functions will have to be moved to the PDU Session Anchor UPFs.

The xCC delivered to the MDF3 shall be correlated to the PDU session related xIRI messages. The use of Correlation Id shall be on a user-plane path basis, which means that the xCC generated at different UPFs that belong to different PDU sessions may need to have separate Correlation Ids, each correlating to their own PDU session related xIRI messages.

<<<<<<<<<<<<<<<<<<<<<<<<<< END OF CHANGE >>>>>>>>>>>>>>>>>>>>>>>>>>>>