3GPP TSG-SA3 Meeting #73-LI	s3i190275

# La Jolla (California), United States, 9th Apr 2019 - 12th Apr 2019          was s3i190238

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0010 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | In-bound roaming interception at anchor UPFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI15 |  |  |  |  |  | Date: |  |  | 2019-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The inbound roaming interception case differs from the home network case enough to possibly cause incomplete interception |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds text to precisely define the interception point for inbound roaming interception |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If this CR is not accepted, interception may not be complete, violating requirement R6.2-10 Completeness from TS 33.126 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<< START OF CHANGE >>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 6.2.3.3	Triggering of the CC-POI from CC-TF over LI_T3

##### 6.2.3.3.1	LI_T3 interface specifics

When interception of communication contents is required, the CC-TF present in the SMF sends a trigger to the CC-POI  present in the UPF over the LI_T3 interface.

When the CC-TF in the SMF detects that a PDU session has been established for a target UE (i.e. when the SMF sends the N4: Session Establishment Request and N4: Session Modification Request to the UPF), it shall send an activation message to the CC-POI in the UPF over the LI_T3 interface. The activation message shall contain the correlation identifiers that the CC-POI in the UPF shall use with the xCC. This can be achieved by sending an ActivateTask message as defined in ETSI TS 103 221-1 [7] clause 6.2.1 with the following details.

Table 6.2.3-6: ActivateTask message for triggering the CC-POI in the UPF

| ETSI TS 103 221-1 field name | Description | M/C/O |
| --- | --- | --- |
| XID | Set to the same XID associated with the interception in the SMF. | M |
| TargetIdentifiers | Packet detection criteria as determined by the CC-TF in the SMF, which enables the UPF to isolate target traffic. The CC-POI in the UPF shall support at least the following identifier types: - GTP Tunnel ID. - IPv4 address. - IPv6 address. NOTE: This value is the target identifier for the CC-POI in the UPF, and may be different from the target identifier specified in the warrant. | M |
| DeliveryType | Set to “X3Only”. | M |
| ListOfDIDs | Delivery endpoints for LI_X3. These delivery endpoints shall be configured by the CC-TF in the SMF using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| CorrelationNumber | Correlation ID to assign to X3 PDUs generated by the CC-POI in the UPF. | M |

When the CC-TF in the SMF detects that a targeted PDU session has changed (i.e. when the SMF sends the N4 Session Modification Request to the UPF) in a way which requires changes to the interception by the CC-POI in the UPF, the CC-TF shall modify the interception at the CC-POI in the UPF over the LI_T3 interface. This is achieved by sending a ModifyTask message as defined in ETSI TS 103 221-1 [7] clause 6.2.2 with the following details.

Table 6.2.3-7: ModifyTask message for updating interception at the CC-POI in the UPF

| ETSI TS 103 221-1 field name | Description | M/C/O |
| --- | --- | --- |
| XID | Set to the XID associated with the interception. | M |
| TargetIdentifiers | Updated packet detection criteria as determined by the CC-TF in the SMF. NOTE: See notes on TargetIdentifiers in Table 6.2.3-6. | M |

When the CC-TF in the SMF detects that the PDU session has been released (i.e. when the SMF sends the N4: Session Release Request to the UPF) for a target UE, it shall send a deactivation message to the CC-POI in the UPF over the LI_T3 interface. When using ETSI TS 103 221-1 [7] this is achieved by sending a DeactivateTask message with the XID field set to the XID associated with the interception, as described in ETSI TS 103 221-1 [7] clause 6.2.3.

##### 6.2.3.3.3	CC interception with multi-homed PDU session

Another option is the following: When a target UE accesses multiple Data Networks (DNs) via a multi-homed PDU session (see TS 23.501 [2] clause 5.6.4.3), multiple UPFs are involved in providing the PDU Session Anchors with one UPF providing the Branching Point functionality. The Branching Point UPF may, or may not, be a PDU Session Anchor UPF (see TS 33.127 [5] Annex A3.2). The CC-TF present in the SMF shall send the CC intercept trigger to the CC-POI present in an UPF if and only if that UPF is selected to provide the CC-POI functions.

When the target UE is involved in multi-homed PDU session, the CC-TF present in the SMF (i.e. in the SMF that establishes the PDU session) shall determine which UPF(s) is(are) more suitable to provide the CC-POI functions adhering to the following requirements specified in TS 33.127 [5]:

- All applicable user plane packets are captured and delivered.

- Duplicate delivery of CC is suppressed to the extent possible.

To meet the above requirements, the CC-TF present in the SMF shall follow the rules specified below:

NOTE 1: 	The below rules are based on the assumption that a PDU session has one Branching Point UPF (with N3 reference point toward the target UE) and one PDU Session Anchor UPF for each DN connection).

Scenario 1: CC Interception of all DN connections is not required.

- CC interception at PDU Session Anchor UPF.

If the CC interception of all DN connections is not required, then the CC-TF present in the SMF shall choose one or more of the PDU Session Anchor UPFs (that are on the user plane path of DN connections that require CC interception) to provide the CC-POI functions.

In this case, the CC intercept trigger shall be sent to the CC-POIs present in the selected PDU Session Anchor UPFs and shall include the packet detection rules. Examples of the packet detection rules are:

-	Generate the xCC from the user plane packets sent and received from the DN connection that requires the interception, or

-	Generate the xCC from all the incoming user plane packets.

In this case, the CC-TF present in the SMF shall select the Branching Point UPF to provide the CC-POI functions unless the Branching Point UPF is also a PDU Session Anchor UPF.

- Branching Point UPF is also a PDU Session Anchor UPF.

If the CC interception of all DN connections is not required and if the UPF playing the role of Branching Point is also a PDU Session Anchor UPF, and if the CC interception for the related DN connection is required, then SMF shall choose the Branching Point/PDU Session Anchor UPF to provide the CC-POI functions for the related DN connection. The CC interception trigger sent to CC-POI present in the Branching Point/PDU Session Anchor UPF shall include the following packet detection rule:

-	Generate the xCC from the user plane packets that are sent and received from the DN network to which the Branching Point UPF is providing the PDU Session Anchor PDU function.

In this case, the user plane packets sent/received on the common path (i.e. over the N3 interface) shall not be intercepted since such an interception, if performed, will result in a duplicate delivery of CC intercepted at the CC-POIs present in the PDU Session Anchor UPFs.

Scenario 2: CC interception of all DN connections is required.

If the CC interception of all DN connections is required, then the SMF may choose either all the PDU Session Anchor UPFs or the Branching Point UPF to provide the CC-POI functions.

- PDU Session Anchor UPFs are chosen.

With this method, to ensure that all user-plane packets are intercepted, the CC-TF present in the SMF will have to choose all PDU Session Anchor UPFs to provide the CC-POI functions and accordingly, send the CC interception trigger to CC-POIs present in all the PDU Session Anchor UPFs. The CC interception trigger shall include the packet detection rules as applicable to respective CC-POIs and the examples of those rules are the same as described under scenario 1, a.

In this case, the CC-TF present in the SMF shall select the Branching Point UPF to provide the CC-POI functions if, and only if, the Branching Point UPF is also a PDU Session Anchor UPF and the packet detection rules included within the CC interception trigger sent to the CC-POI present in the Branching Point/PDU Session Anchor UPF shall be according to scenario 1, b.

- Branching Point UPF is chosen.

When all user plane packets pass through the Branching Point UPF, the CC-TF present SMF may choose the Branching Point UPF to provide the CC -POI function and accordingly, send the CC interception trigger to the CC-POI present in the Branching Point UPF. The CC intercept trigger shall include the packet detection rules and examples of the same are:

-	Generate the xCC from all the incoming and outgoing user plane packets to the target UE, or

-	Generate the xCC from all the incoming user plane packets.

In this case, the CC-TF present in the SMF shall not select any of the PDU Session Anchor UPFs to provide the CC-POI functions.

NOTE 2: 	When Branching Point UPF is chosen to provide the CC-POI functions, and if the Branching Point UPF is removed from the user plane path during a PDU session, then the CC POI functions will have to be moved to the PDU Session Anchor UPFs.

The xCC delivered to the MDF3 shall be correlated to the PDU session related xIRI messages. The use of Correlation Id shall be on a user-plane path basis, which means that the xCC generated at different UPFs may need to have separate Correlation Ids, each correlating to their own PDU session related xIRI messages.

##### 6.2.3.3.3	CC interception at PDU session anchor UPFs

An option is to intercept a copy of the packets sent and received on the N6 interface [2] side of the PDU Anchor UPF (for each UL classifier in case of selective routing or Service and Session Continuity mode 3) for all DNs the subject is connected to. In the in-bound roaming case for home-routed roaming, the CSP shall deliver a copy of the packets sent and received on the N9 side of the PDU Anchor UPF towards the serving network.

<<<<<<<<<<<<<<<<<<<<<<<<<< END OF CHANGE >>>>>>>>>>>>>>>>>>>>>>>>>>>>