3GPP SA3LI#74	S3i190434

# Wroclaw, Poland, 16-19 July 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0031 | rev | 001 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CC-PAG Pairing and Location determination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Cisco Systems, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3-LI |  |  |  |  |  |  |  |  |  |
| Work item code: | LI16 |  |  |  |  |  | Date: |  |  | 2019-7-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Proposal to ensure a validated set of options for CC-PAG usage in deployment are clarified and documented in Clause 8.6.2. Additionally propose an option to change the pic in Figure 8.6-1 to show only 2 deployment options instead of 3 shown in the current version, ie. deprecating the Hybrid mode (dual-homed UPF) option in current version of the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying options for CC-PAG deployment in Clause 8.6.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of clarity in the deployment options when CC-PAG is present |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

***************** FIRST CHANGE **********************

### 8.6.2	CC-PAG

This clause introduces CC-PAG (CC-POI Aggregator) as an architectural extension that is located between the MDF3 and CC-POI. The CC-PAG performs the function of aggregating the xCC from different CC-POIs towards the MDF3 and is shown in Figure 8.6-1.

Figure 8.6-1: LI architecture showing CC-PAG.

The CC-PAG is an optional LI function and may be deployed in networks that need aggregation of xCC from potentially large number of CC-POIs towards the MDF3. The CC-POI may send xCC directly to the MDF3 or to CC-PAG as provisioned by the CC-TF (see Figure 8.6-1). The CC-PAG appears to the CC-POI as if it is an MDF3. The LI_X3A interface is the same as LI_X3 interface but may be used with other transport protocol options as described in ETSI TS 103 221-2 [16]. The CC-PAG is configured by the LIPF via the LI_X1 interface with the address of MDF3 and aggregates and forwards the xCC from the different CC-POIs. The xCC is not modified.

The CC-PAG may be deployed closer to the UPFs, to reduce the impact of latencies, packet drops, and buffering on UPFs for lawful interception of highspeed user plane traffic . The system resources such as hardware interfaces, CPUs and memory for the CC-PAG node may be tuned to balance the forwarding/reception capabilities of CC-POI and MDF3.

The CC-POI in the UPFs can have either (1) LI_X3A to a CC-PAG, or (2) LI_X3 to MDF3.

- For (1):

  - CC-PAG address is given by the LIPF to the CC-TF (in SMF) during the provisioning of intercept data as the destination address for LI_X3A for all warrants.

  - The MDF3 address is given by the LIPF to the CC-PAG as the destination address for LI_X3 on a per warrant basis.

- For (2):

  - MDF3 address is given by the LIPF to the CC-TF (in SMF) during the provisioning of intercept data as the destination address for LI_X3 on a per warrant basis.

Editor’s Note : The method used in LIPF to determine (1) or (2) is for further study.

In order to give CC-PAG address to the CC-TF (in SMF) for all warrants, then the following configuration is required in LIPF per SMF basis:

- LIPF Data

  - CC-PAG: YES/NO

  - If YES, use CC-PAG address as the destination address for the LI_X3A for all warrants

  - IF NO, use MDF3 address as the destination address for LI_X3 on a per warrant basis