3GPP TSG-SA3 Meeting #74-LI	s3i190450

# Wroclaw, Poland, 16th July 2019 - 19th July 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0044 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Task Details required for Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI16 |  |  |  |  |  | Date: |  |  | 2019-07-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the Triggering over LI_T2 interface, the ActivateTask message includes in the TaskDetailExtensions paramter PositioningParameters. This parameter is labeled as Conditional “C” in the MOC column. However, no condition is specified for its inclusion. This should be corrected. Also, the first paragraph mentions a correlationID, but none is shown in Table 7.3.1.3-1. A correlationID parameter should be included. Finally, an erroneous reference to 7.3.3.2.2 (which does not exist) is made in the first paragraph. This should be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to define the inclusion conditions for the TaskDetails in Table 7.3.1.3-1, adds a correlationID parameter in the table, and requests help in fixing the erroneuous reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent implementations as the conditions for including the PositioningParameters would not be specified. Also without a correlation parameter, the CSP may not be able to deliver the location ifnroamtion to the LEMF correlated to the triggering event. tThese can lead tto th CSP not fulfiling their LI obligations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<< START OF FIRST CHANGE >>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 7.3.1.3	Triggering over LI_T2

An LTF, provisioned as described in clause 7.3.1.2.2, triggers the triggered IRI-POI provided by the LI-LCS client using the LI_T2 protocol as described in clause 5.2.4. The “TaskDetailsExtensions” in the LI_T2 “ActivateTask” message carries the positioning parameters mapped from the LTF provisioning over the LI_X1. The LI_T2 “ActivateTask” message header may include a correlation ID from the triggering xIRI event, if available.

Prior to issuing one or more “ActivateTask” requests towards an LI-LCS Client, the LPF shall provision the LI-LCS client with the LI_X2 destinations by using the “CreateDestination” operation(s), as per clause 5.2.2. The LI-LCS client shall implicitly deactivate the task upon issuing the final xIRI message for the trigger. There is no DeactivateTask operation on the LI_T2 for the LI-LCS client.

The Table 7.3.1.3-1 shows the details of the LI_T2 ActivateTask message used by the LTF to trigger LI-CS client for the triggered location service.

Table 7.3.1.3-1: ActivateTask message from LTF to LI-LCS client for the triggered location service triggering

| ETSI TS 103 221-1 field name | Description | M/C/O |
| --- | --- | --- |
| XID | The same value as in the LTF provisioning (clause 7.3.3.2.2). | M |
| TargetIdentifiers | One of the following, per LTF provisioning: - SUPI. - PEI. - GPSI. | M |
| DeliveryType | Set to “X2Only”. | M |
| ListOfDIDs | Delivery endpoints identifiers for LI-LCS Client LI_X2 messages. These delivery endpoints are configured in LTF using the CreateDestination message as described in ETSI TS 103 221-1 [7], clause 6.3.1 prior to the task activation. | M |
| correlationID | Correlates the requested location to the triggering xIRI event, if available. | C |
| TaskDetailsExtensions/ PositioningParameters | Set of parameters (requested location type, requested response type (SYNC vs ASYNC) and timing, QoS (max location age, accuracy and QoS Class), mapData request, velocity request), as per OMA-TS-MLP-V3-4-20150512-A [20], as specified in the LTF provisioning message over LI_X1. This parameter is included to override any system defaults. It may be omitted if system defaults are to be used. | C |

<<<<<<<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>>>>>>>>>