3GPP SA3LI#74	S3i190475

# Wroclaw, Poland, 16-19 July 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0030 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fix picture for CC POI Aggregator for 5GC LI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Cisco Systems, Nokia, Nokia Shanghai Bell, Verizon) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI16 |  |  |  |  |  | Date: |  |  | 2019-7-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR fixes the picture for the CC-POI Aggegator(CC-PAG) to the 5GS LI architecture by adding the LI-X2 connections between the SMF and MDF2 for the options shown in the Sec 8.6.2, figure 8.6-1: LI architecture showing CC-PAG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fixes discrepancy in the deployment option picture in Sec 8.6.2 in current revision of 33.127 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect architectural options in the deployment section |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

***************** FIRST CHANGE **********************

### 8.6.2	CC-PAG

This clause introduces CC-PAG (CC-POI Aggregator) as an architectural extension that is located between the MDF3 and CC-POI. The CC-PAG performs the function of aggregating the xCC from different CC-POIs towards the MDF3 and is shown in Figure 8.6-1.

Figure 8.6-1: LI architecture showing CC-PAG.

The CC-PAG is an optional LI function and may be deployed in networks that need aggregation of xCC from potentially large number of CC-POIs towards the MDF3. The CC-POI may send xCC directly to the MDF3 or to CC-PAG as provisioned by the CC-TF (see Figure 8.6-1). The CC-PAG appears to the CC-POI as if it is an MDF3. The LI_X3A interface is the same as LI_X3 interface but may be used with other transport protocol options as described in ETSI TS 103 221-2 [16]. The CC-PAG is configured by the LIPF via the LI_X1 interface with the address of MDF3 and aggregates and forwards the xCC from the different CC-POIs. The xCC is not modified.

The CC-PAG may be deployed closer to the UPFs, to reduce the impact of latencies, packet drops, and buffering on UPFs for highspeed LI. The system resources such as hardware interfaces, cpus and memory for the CC-PAG node may be tuned to balance the forwarding/reception capabilities of CC-POI and MDF3.