3GPP TSG-SA3 Meeting #74-BIS LI	s3i190514

# Basking Ridge (NJ), United States, 4th Sep 2019 - 6th Sep 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0040 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LI Virtualisation Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (BT plc, NTAC) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI16 |  |  |  |  |  | Date: |  |  | 2019-08-27 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In R15 only limited decription of LI function instantiation for virtualised deployments was provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | LI instantiation procedures using LI NFV Controller and LI_X0 are provided in line with references already cited in the existing specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LI virtualised deployment instantiation procedures continue not to be fully detailed causing inter-operability and security issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of 1st Change ****

## 5.6	LI in a virtualised environment

### 5.6.1	General

Virtualisation is one of the 3GPP network deployment options for NFs containing LI functions as described in the present document. In virtualised deployments, many of the initial deployment and configuration actions performed manually in non-virtualised deployments need to be automated and occur in near real-time. This clause outlines the basic architecture enhancements to support virtualised LI in 3GPP networks. Security aspects relating to virtualisation are described in clause 8.

The architecture enhancements in this clause are intended to apply to any virtualised 2G, 3G, 4G, 5G scenario including IMS that needs to support LI. Where legacy network functions defined in TS 33.107 [11] are virtualised, the architecture in figure 5.6-1 shall be applied, with legacy TS 33.107 [11] reference points and functional elements substituted for their equivalent in the present document (e.g. POI is equivalent to ICE and LI_X2 is equivalent to X2 in TS 33.107 [11],).

### 5.6.2	Virtualised deployment architecture

Figure 5.6-1 shows the necessary extensions to the basic LI architecture described in clause 5.2 required to support real-time deployment of virtualised LI functions. Figure 5.6-1 is a simplified version of the virtual LI function deployment procedures.

Figure 5.6-1: Simplified virtualised LI system with provisioning infrastructure for a direct provisioned POI

NOTE 1:	Figure 5.6-1 shows the LI NFV controller and NFV Management and Orchestration functions (MANO). However, for the purposes of the present document, other than needing to acknowledge that these functions exist within a CSP virtual network implementation, they are only described at a high level in clause 5.6.3.

### 5.6.3	LI function instantiation and lifecycle management procedures

#### 5.6.3.1	Controller virtualisation layer and MANO procedures

##### 5.6.3.1.1	Responsibilities

The 5G NRF is not involved in the initial discovery of LI functions. NFs containing LI functions shall only be discoverable by the NRF / SIRF once all LI initialisation steps in this clause have been completed and the OSS/BSS/ MANO informed that LI operation is ready. The process by which the NRF / SIRF is notified by the OSS/BSS/MANO is out of scope of the present document.

##### 5.6.3.1.2	General procedures

When the 3GPP network OSS/BSS makes a request to MANO to instantiate, modify or terminate a 3GPP NF MANO shall notify the LI NFV controller of the request using procedures as described in ETSI GR NFV IFA 026 [X] or equivalent. The NFV LI controller shall be able to send all applicable NF changes to the LIPF, so that the LICF is able to maintain an understanding of network topology. In 5G the ADMF also maintains understanding of active use of NF via the NRF / SIRF.

In addition, MANO is required to send notifications of non OSS/BSS triggered (e.g. MANO automated VNF relocation, or software image update) as described in ETSI GR NFV IFA 026. The LI controller shall also be able to provide applicable notifications to the ADMF of such changes.

NOTE: The precise list of information required for the ADMF to maintain understanding of network topology is implementation specific and therefore outside the scope of the present document.

In deployments where the implementation supports data centre / location verification for NFs being instantiated or modified, subject to operator policy the LI NFV Controller shall not allow instantiation or modification of LI functions or associated NFs which do not comply with LI location constraints set by the ADMF.

##### 5.6.3.1.3	Instantiation

Where an NF being instantiated contains one or more LI functions (e.g. POI, TF, MDF) the LI NFV controller shall handle any necessary steps to allow the LI functions to be instantiated by MANO and the LI functions to be added to the LI environment, so that initial contact between the LI functions and the ADMF can be established. The LI NFV controller shall provide details of the new LI functions to the LICF (via the LIPF), including the identity of the new LI functions, so that the ADMF is aware of the existence of the LI functions.

In deployments where ADMF signing of LI function software images has been implemented, the LI NFV Controller shall provide the signatures to the ADMF for verification and shall only authorise MANO to continue instantiation of the LI function if the ADMF has successfully verified the signatures.

NOTE : Once this instantiation step is completed an LI function is considered to be ready for configuration by the ADMF but is not ready to become a live LI function.

##### 5.6.3.1.4	Modification

When the NF containing LI functions is being modified (e.g. scaled or relocated) the LI NFV controller shall manage the necessary interactions with MANO (using the procedures in ETSI NFV IFA 026 [X] or equivalent) to allow the LI functions to also be modified in alignment with changes to their parent NF. The LI NFV shall notify the LICF via the LIPF of the subsequent modifications. Where the modifications result in a new LI function being instantiated (e.g. where a scale up exceeds the existing capabilities of the existing LI functions), the LI NFV controller shall notify the LICF about the existence of the new LI function and indicate to which existing NF the new LI function is associated.

##### 5.6.3.1.5	Termination

When an NF containing LI functions is terminated, the LI NFV Controller shall manage the necessary interactions with MANO (using the procedures in ETSI NFV IFA 026 [X] or equivalent) and notify the LICF via the LIPF that the LI functions have been removed from the system. The LICF shall ensure that certificates associated with those LI functions are appropriately revoked.

#### 5.6.3.2	ADMF LI_X0 procedures

Only once an LI function has been instantiated and the ADMF informed of that function's existence can that function be managed by the ADMF. Such notification is achieved as described in clause 5.6.3.1 and occurs prior to any NRF (or equivalent) NF discovery processes.

The LI_X0 interface is used to manage LI functions such they are made ready for LI use and subsequent provisioning over LI_X1.

After a virtual function is instantiated (e.g. using the procedures in ETSI NFV-SEC 011 [10] or equivalent), it is necessary to automatically configure the LI functions (e.g. POI, TF, MDF) before use (i.e. to initialise it to a state where it can accept LI_X1 messages). To achieve this the LI Function  shall contact the LIPF over the LI_X0 interface and LIPF will notify the LICF that a new potential LI function has contacted the LIPF. The LIPF shall only accept incoming connections from new LI functions that have previously been notified to the LIPF/LICF by the LI NFV Controller.

The LICF, through the LIPF, shall verify the authenticity of the LI function over LI_X0 in order to verify that the new LI function has been instantiated from a valid software image. If the LI function software image has been partly encrypted as described in ETSI NFV SEC 011 [10], then once the LIPF has verified the integrity of the LI function it shall provide any necessary keys to decrypt the LI function to complete instantiation of that LI function.

Once a trust relationship has been established between the LICF and new LI function, the LICF shall issue the LI function with an LI identity (e.g. POI CSCF number 42 or LI System FQDN) and provide the other necessary certificates and configuration information to allow the new LI function to be configured for LI use on LI_X1.

In the case of triggered POIs which are not directly provisioned by the ADMF over LI_X1, the ADMF is still responsible for LI_X0 configuration of the POI including identity manage and all necessary identity / communication certificates.

Once the LI function has been made fully ready for provisioning over LI_X1 using LI_X0, then ADMF shall notify the LI NFV Controller that the LI function is ready for service and MANO may advise the OSS/BSS that the NF associated with the LI functions is ready for service and discovery by the NRF.

NOTE:	The full procedure for notify the OSS/BSS that LI is ready and the NF can be notified to the NRF (in the case of 5G SBA) is out of scope of the present document and is left to operator deployment choice.

During normal system operation LI_X0 shall be used by the ADMF to maintain the LI function throughout the LI function’s lifecycle. In-life certificate updates, identity changes, LI_X1/2/3 credential changes and other similar configuration changes shall be supported by both the ADMF and LI functions over LI_X0.

Figure 5.6-2 shows an example of what the procedures described in this clause look like when instantiating a new NF and associated LI functions.

Figure 5.6-2: Example simplified flow-diagram for instantiation procedures

### 5.6.3.3	Exception Procedures

If during normal LI system operation the ADMF detects or is informed of abnormal LI function behaviour, then subject to operator policy the ADMF shall be able via the LI NFV Controller to request immediate termination or quarantine of the LI function to MANO as defined in ETSI GR NFV IFA 026 [X].

If during normal operation the ADMF is notified of a NF modification or instantiation event which does not comply with operator LI policy (e.g. NF location is not within allowed locations or LI functionality is not authorised for a given deployment scenario) the ADMF shall be able to deny MANO and the OSS/BSS authorisation to complete the system change.

**** End of 1st Change ****

**** Start of 2nd Change ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.126: "Lawful interception requirements".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[6]	OMA MLP: "Mobile Location Protocol V3.3".

NOTE:	Available at http://www.openmobilealliance.org.

[7]	ETSI TS 103 120: "Lawful Interception (LI); Interface for warrant information".

[8]	ETSI TS 103 221-1: "Lawful Interception (LI); Part 1: Internal Network Interface X1 for Lawful Interception".

[9]	3GPP TS 33.501: "Security Architecture and Procedures for the 5G System".

[10]	ETSI GR NFV-SEC 011: "Network Functions Virtualisation (NFV); Security; Report on NFV LI Architecture".

[11]	3GPP TS 33.107: "3G Security; Lawful interception architecture and functions".

[12]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[13]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[14]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[15]	3GPP TS 33.128: "Protocol and Procedures for Lawful Interception; Stage 3".

[16]	ETSI TS 103 221-2: "Lawful Interception: Internal Network Interface X2/X3".

[X]	ETSI GS NFV-IFA 026: "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Architecture enhancement for Security Management Specification".

**** End of Changes ****