3GPP TSG-SA3 Meeting #74-BIS LI	s3i190519

# Basking Ridge (NJ), United States, 4th Sep 2019 - 6th Sep 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0043 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Securing Location Capabilities in 5G |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI16 |  |  |  |  |  | Date: |  |  | 2019-08-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Location capability in 5G is not secure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution ensures that obtaining location information in 5G does not break fundamental security detectability requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Obtaining the location of the target will be detectable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | changes 6.2.1, adds 6.2.5bis, changes 7.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <<<<<<<<<<<< BEGIN FIRST CHANGE >>>>>>>>>>>>>

### 6.2.1	General

Figure 6.2-1 depicts the 5G EPC anchored LI architecture. The network functions are depicted in grey, while the LI elements are depicted in blue.

Figure 6.2-1: 5G EPC-anchored LI architecture

Figure 6.2-2 depicts the 5G core-anchored LI architecture. The network functions are depicted in grey, while the LI elements are depicted in blue.

Figure 6.2-2: 5G core-anchored LI architecture

### <<<<<<<<<<<< END FIRST CHANGE >>>>>>>>>>>>>

### <<<<<<<<<< BEGIN SECOND CHANGE (NEW SECTION) >>>>>>>>>>>

### 6.2.5bis	LI at LMF

#### 6.2.5.1	Architecture

In the 5GC network, the LMF provides location services for its clients. Since request to the LMF regarding the target cannot be made in the open without leaking information about the target, the LMF is outfitted with a POI that can perform exactly the same location services that the open LMF provides, but it does so entirely in the LI security domain.

Extending the generic LI architecture presented in clause 5, figure 6.2-5bis below gives a reference point representation of the LI architecture with LMF POI providing the location functions.

Figure 6.2-5: LI architecture for LI at LMF

The ADMF LICF receives the warrant from an LEA, derives the intercept information from the warrant and provides it to the LIPF.

The ADMF LIPF provisions the LMF IRI-POI and the MDF2 over LI_X1 interfaces. The LIPF may interact with the NRF SIRF (over LI_SI) to discover the LMFs in the network.

The LMF IRI-POI obtains the target UE's location, generates and delivers the xIRI to the MDF2 over LI_X2. The xIRI will contain the location. The MDF2 shall support the capability to deliver the IRI messages including the location of the UE as part of the Interception Product to the LEMF over LI_HI2.

The MDF2 has final responsibility to ensure the intercept product delivered to LE complies with all applicable national regulations.

#### 6.2.5.2	Target identities

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the LMF:

- SUPI.

- PEI.

- GPSI.

The interception performed on the above three identities are mutually independent, even though an xIRI may contain other identities when available.

#### 6.2.5.3	IRI events

The LMF IRI-POI shall generate xIRI when it detects the following specific events or information:

- requests for UE location from LI NFs.

The LMF xIRI is generated when the LMF IRI-POI obtains the UE location.

#### 6.2.5.4	Common IRI parameters

The list of xIRI parameters are specified in TS 33.128 [15]. The xIRIs shall include at the minimum the following information:

- Target identity.

- Time stamp.

- Location information.

#### 6.2.5.5	Specific IRI parameters

The parameters in each xIRI are defined in TS 33.128 [15].

#### 6.2.5.6	Network topologies

The LMF POI shall provide the IRI-POI functions in the following network topology cases:

- Non-roaming case.

### Roaming case, in VPLMN.

### <<<<<<<<<<<< END SECOND CHANGE >>>>>>>>>>>>>

### <<<<<<<<<<<< BEGIN THIRD CHANGE >>>>>>>>>>>>>

### 7.3.3	Lawful Access Location Services (LALS)

#### 7.3.3.1	General

LALS provides lawful access to the target's location. LALS is based on the Location Services (LCS) capabilities defined in the TS 23.271 [5] and in the OMA MLP specification [6]. The 5G Core Network support of LCS is described in clause 4.4.4 of TS 23.501 [2] and clause 4.13.5 of TS 23.502 [4].

LALS shall adhere to the requirements in clauses 6.6 (Security) and 6.3 (Detect and Capture) of TS 33.126 [3]. The LCS IRI-POI supporting LALS shall be able to provide priority over open LALS requests. The subscriber location privacy settings (see clause 9 of TS 23.271 [5]) shall be overridden for LALS.

For inbound roaming targets, the VPLMN LCS functional entities fulfilling LALS requests should not communicate with the target's HPLMN, as it will cause detectability issues.

Depending on national requirements and LCS capabilities of the CSP, the location information provided by LALS may vary in location information types (mobile network cell ID, location shape and geo-coordinates, civic address, or a combination of those), in the set of additional location parameters (map data, motion state, speed, etc.), as well as in the accuracy of provided location information.

NOTE:	The accuracy of positioning is, usually, a trade-off for the location acquisition delay. It also depends on other positioning technology specific factors.

The parameters controlling the LALS output are either delivered per warrant over the LI_X1 interface from the ADMF to the LI-LCS Client, or to the Location Triggering Function (LTF, see Clause 7.3.3.3), or are pre-configured in the LI-LCS Client. The LI-LCS Client is an IRI-POI in the CSP network fulfilling the role of the LCS client for LALS purposes.

To comply with the security requirements in clause 6.6 (Security) of TS 33.126 [3] the requests for UE location cannot be made in the open to the LCS Server or the GMLC. Therefore, the LCS Server and GMLC must be outfitted with IRI-POIs that can perform all the functions that the servers provide in the open, but do so entirely in the LI security domain.

There are two types of the location interception defined in the present document: target positioning and triggered location.

Target positioning determines the target's location independently of the services used by the target.

Triggered location determines the LALS based location of the target when specific network or service events related to the target occur.

The warrant for target positioning and for triggered location of the same target may be independent of each other and may be overlapping in time or combined in a single intercept warrant by the LEA.

There may be multiple active LALS warrants from different LEAs at any given time.

#### 7.3.3.2	Target positioning

##### 7.3.3.2.1	General

As required by the R6.3 – 370 of TS 33.126 [3], the location provision variants supported in the current document are immediate location and periodic location.

Figure 7.3-1 shows the architecture for LALS where the LI-LCS client provides the target's location and associated information towards the MDF2 over the LI_X2 interface as per the ADMF request for target positioning delivered over LI_X1 interface.

Figure 7.3-1: LALS model for target positioning

NOTE:	The Le interface is specified in the OMA MLP specification [6]. However, it runs entirely in the LI security domain, as it is terminated on both sides by LI domain functions. The downstream interactions of the LCS Server/GMLC IRI-POI with the AMF IRI-POI and LMF IRI-POI remain entirely in the LI security domain.

##### 7.3.3.2.2	Immediate location provision

The request for immediate location provision is delivered to the LI-LCS client over the LI_X1 interface. Upon receiving the request, the LI-LCS client initiates a Location Immediate Request (LIR, see TS 23.271 [5]) to the LCS Server/GMLC IRI-POI supporting LALS over the secure LI security domain Le interface and reports the acquired location to the MDF2 over LI_X2.

While waiting for response to an LIR from the LCS Server/GMLC IRI-POI, the LI-LCS client may receive and process additional LIRs from the ADMF over the LI_X1.

NOTE:	The LCS Server/GMLC IRI-POI supporting LALS may be optimized to provide the same single location estimation in response to multiple positioning requests arriving in temporal proximity of each other.

The resulting immediate location information is delivered over LI_X2 to the MDF2 and propagated to the LEMF over LI_HI2.

##### 7.3.3.2.3	Periodic location provision

The request for periodic location provision is delivered to the LI-LCS client over the LI_X1 interface.

The request for periodic location from the ADMF to the LI-LCS client may include a set of parameters defining the duration of reporting, report periodicity, etc. The description of the service response parameters is provided in clause 7.3.3.4. The periodic location result is delivered over LI_X2 to the MDF2 and propagated to the LEMF over LI_HI2.

The periodicity of the LALS reports shall be controlled by the LI-LCS client. The LI-LCS client shall issue a series of Location Immediate Requests (LIR, see TS 23.271 [5]) at required time intervals.

The LI-LCS client provides the acquired location reports to the MDF2 over LI_X2.

#### 7.3.3.3	Triggered location

The Triggered location is the capability of providing LALS based location information when specific network or service events related to the target occur. While IRI generated by the event that also triggers the LALS may have the location information included (in the form of cell ID), the LALS may provide additional location parameters, such as the target geo-location, velocity, etc. (see R6.3 – 270 of TS 33.126 [3]).

The LALS triggered location architecture in Figures 7.3-2 and 7.3-3 depicts the LTF. The LTF is an IRI-TF embedded into an IRI-POI (e.g. AMF, etc.), or into an MDF2. The LTF is responsible for triggering the LI-LCS Client when a specific event related to the target is observed at the IRI-POI, or received at the MDF2.

Figure 7.3-2 depicts the architecture of Triggered Location for IRI acquisition and delivery for the case when the LTF is embedded into an IRI-POI reporting IRI events for the target.

Figure 7.3-2: LALS model for triggered location (POI/LTF option)

Figure 7.3-3 depicts the architecture of triggered location acquisition and delivery for the case when the LTF is embedded into an MDF2.

Figure 7.3-3: LALS Model for triggered location (MDF/LTF option)

The request for triggered location is delivered from the ADMF to either an IRI-POI or to a MDF2 over LI_X1 interface along with other parameters of IRI intercept authorization/activation. The IRI-POI (s) or the MDF2 then arm the LTF(s).

The LTF triggers the LI-LCS client over the LI_T2 interface.

The LALS result is delivered to MDF2 from the LI-LCS Client over the LI_X2 interface asynchronously with the associated IRI events delivered by the IRI-POI. To enable correlation between the LALS reports and the associated IRI events, the LTF shall include the correlation information of the IRI event, if provided by the IRI-POI, into the LI_T2 trigger.

NOTE:	The IRI events may contain the location information obtained by other means, e.g. NPLI. The LALS reports augment that information with extra details and accuracy.

#### 7.3.3.4	LI_X2 interface for target positioning and triggered location

The following information needs to be delivered from the LI-LCS Client to MDF2 in order to enable the MDF2 to format and deliver LALS intercept product to LEMF:

- 	Target identity.

-	Target reported location(s).

-	Date/time(s) location(s) established by reporting function.

-	Additional location parameters based on operator policy.

-	Correlation information.

### <<<<<<<<<<<< END THIRD CHANGE >>>>>>>>>>>>>